3GPP TSG RAN WG1 Meeting #100bis	R1-2003117

# e-Meeting, 20th – 30th April 2020

Agenda Item:	6.2.1.4

Source:	Moderator (Ericsson)

Title:	TP for 36.213 on SRS handling for LTE-MTC resource reservation

Document for:	Discussion, Decision

# Introduction

RAN1#100bis-e discussed SRS handling for Rel-16 LTE-MTC resource reservation in the email discussion “[100b-e-LTE-eMTC5-Coex-NR-01]”. For more background information, see “Issue #5” in the email discussion summary in R1-2002797. This contribution presents a 36.213 TP based on the outcome of the discussion.

| Reason for change: | SRS transmission that falls into a reserved UL resource should be dropped, which is not clear from the current specification. |
| --- | --- |
| Summary of change: | It is clarified that SRS transmission falling into a reserved UL resource is dropped. |
| Consequences if not approved: | Resource reservation may be incorrectly implemented for SRS, which may cause collision with other transmissions. |
| Clauses affected: | 8.2 |

# Text proposal for 36.213

#### 8.2	UE sounding procedure

<Unchanged parts are omitted>

For a BL/CE UE not configured with the higher layer parameter srs-UpPtsAdd, for a SRS transmission in subframe n and if the UE transmits PUSCH/PUCCH in subframe n and/or n+1, the UE shall not transmit the SRS in subframe n if the SRS transmission bandwidth in subframe n is not completely within the narrowband of PUSCH/PUCCH in subframe n and/or n+1

A BL/CE UE not configured with the higher layer parameter srs-UpPtsAdd shall not transmit SRS in UpPTS if SRS frequency location is different from DwPTS reception narrowband in the same special subframe.

For a BL/CE UE, the SRS transmission that falls into the reserved symbol of a BL/CE UL subframe is dropped.

<Unchanged parts are omitted>