3GPP TSG RAN WG1 Meeting #100b-e
e-Meeting, April 20th – 30th, 2020
Title:
RAN1 Chairman’s Notes

31
Opening of the meeting (Day 1: 9.00 AM)


31.1
Call for IPR


31.2
Competition Law Statement


31.3
Network Usage Conditions


41.4
Check-in for Registered Delegates


41.5
Aspects related to RAN1 Meeting Management


42
Approval of Agenda


43
Highlights from RAN plenary


44
Approval of Minutes from previous meetings


45
Incoming Liaison Statements (11+3=14)


75.1
RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (3)


85.2
RAN1 Aspects for RF Requirement Enhancements for FR2


86
E-UTRA


86.1
Maintenance of E-UTRA Releases 8 – 15


86.2
Maintenance of E-UTRA Release 16


96.2.1
Maintenance of Additional MTC Enhancements


96.2.1.1
UE-group wake-up signal


96.2.1.2
Support for transmission in preconfigured UL resources


96.2.1.3
Scheduling of multiple DL/UL transport blocks


96.2.1.4
Coexistence of LTE-MTC with NR


96.2.1.5
Support of quality report in Msg3 and Connected Mode


96.2.1.6
MPDCCH performance improvement


96.2.1.7
CE mode A and B improvements for non-BL UEs


106.2.1.8
Use of LTE Control Channel Region for DL Transmission


106.2.1.9
Use of RSS for measurement improvements


106.2.1.10
Others


106.2.2
Maintenance of Additional Enhancements for NB-IoT


106.2.2.1
UE-group wake-up signal


106.2.2.2
Support for transmission in preconfigured UL resources


106.2.2.3
Scheduling of multiple DL/UL transport blocks


106.2.2.4
Coexistence of NB-IoT with NR


106.2.2.5
Support of Quality report in Msg3 and Connected Mode


106.2.2.6
Presence of NRS on a non-anchor carrier for paging


106.2.2.7
Others


106.2.3
Maintenance of DL MIMO efficiency enhancements for LTE


106.2.3.1
Support of enhancing SRS capacity and coverage


116.2.3.2
Others


116.2.4
Maintenance of LTE-based 5G Terrestrial Broadcast


116.2.4.1
New numerology(ies) for PMCH for support of rooftop reception


116.2.4.2
Necessity and detailed enhancements to the physical channels and signals in the CAS


116.2.4.3
New numerology for support of mobility of up to 250km/h


116.2.4.4
Others


116.2.5
LTE Rel-16 UE Features (8)


116.2.5.1
UE features for additional MTC enhancements (2)


126.2.5.2
UE features for additional enhancements for NB-IoT (1)


126.2.5.3
UE features for DL MIMO efficiency enhancements for LTE (3)


126.2.5.4
UE features for LTE-based 5G terrestrial broadcast (1)


126.2.5.5
Others


126.2.6
Others


127
NR Maintenance


127.1
Maintenance of Release 15 NR


127.1.1
Maintenance for Initial access and mobility


137.1.2
Maintenance for MIMO


137.1.3
Maintenance for Scheduling/HARQ aspects


147.1.4
Maintenance for NR-LTE co-existence


147.1.5
Maintenance for UL power control


147.1.6
Others


147.2
Maintenance of Release 16 NR


157.2.1
Maintenance of Two step RACH for NR (4)


157.2.1.1
Channel Structure for Two-Step RACH


157.2.1.2
Procedure for Two-step RACH


167.2.1.3
Others


167.2.2
Maintenance of NR-based Access to Unlicensed Spectrum


167.2.2.1
Physical Layer Signals and Channels


177.2.2.2
Physical Layer Procedure


187.2.2.3
Others


187.2.3
Maintenance of Integrated Access and Backhaul for NR (4)


197.2.3.1
Extensions of SSBs for inter-IAB-node discovery and measurements


197.2.3.2
Extensions of RACH occasions and periodicities for backhaul RACH resources


197.2.3.3
Mechanisms for resource multiplexing among backhaul and access links


197.2.3.4
Mechanism to support the “case-1” OTA timing alignment


207.2.3.5
Others


207.2.4
Maintenance for 5G V2X with NR sidelink (1+4+11+4+1+4+2+1=27)


207.2.4.1
Physical layer structure for sidelink (4)


217.2.4.2
Resource allocation for NR sidelink (11)


237.2.4.3
Sidelink synchronization mechanism (4)


247.2.4.4
In-device coexistence between LTE and NR sidelinks (1)


257.2.4.5
Physical layer procedures for sidelink (4)


267.2.4.6
QoS management for sidelink (2)


267.2.4.7
Support of NR Uu controlling LTE sidelink (1)


267.2.4.8
Others


267.2.5
Maintenance of Physical Layer Enhancements for NR URLLC (3+3+4+2+3+2+3=20)


267.2.5.1
PDCCH enhancements (3)


277.2.5.2
UCI enhancements (3)


287.2.5.3
PUSCH enhancements (4)


297.2.5.4
Enhancements to scheduling/HARQ (2)


297.2.5.5
Enhanced inter UE Tx prioritization/multiplexing (3)


307.2.5.6
Enhanced UL configured grant transmission (2)


317.2.5.7
Others (3)


317.2.6
Maintenance of Enhancements on MIMO for NR


317.2.6.1
CSI Enhancement for MU-MIMO Support


327.2.6.2
Enhancements on Multi-TRP/Panel Transmission


327.2.6.3
Enhancements on Multi-beam Operation


327.2.6.4
Full TX Power UL transmission


337.2.6.5
Low PAPR RS


337.2.6.6
Others


337.2.7
Maintenance of UE Power Saving for NR (5)


337.2.7.1
PDCCH-based power saving signal/channel (2)


347.2.7.2
Procedure of cross-slot scheduling power saving techniques (2)


357.2.7.3
UE adaptation to maximum number of MIMO layers


357.2.7.4
Others


357.2.8
Maintenance of NR positioning support


357.2.8.1
DL Reference Signals for NR Positioning


357.2.8.2
UL Reference Signals for NR Positioning


357.2.8.3
UE and gNB measurements for NR Positioning


357.2.8.4
Physical-layer procedures to support UE/gNB measurements


367.2.8.5
Others


367.2.9
Maintenance of NR Mobility Enhancements


367.2.9.1
Physical Layer Aspects for Mobility Enhancements during HO and SCG Change


367.2.9.2
Others


367.2.10
Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (7)


367.2.10.1
Uplink Power Control for Supporting NR-NR Dual-Connectivity (1)


367.2.10.2
Potential Enhancements to Single Tx Switched Uplink Solution for EN-DC (1)


377.2.10.3
Support of efficient and low latency serving cell configuration/activation/setup (2)


377.2.10.4
Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH (1)


377.2.10.5
Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA (1)


387.2.10.6
Support of cross-carrier scheduling with different numerology (1)


387.2.10.7
Others


387.2.11
NR Rel-16 UE Features (1+2+6+5+4+10+6+5+5+4+5+1+2+2=58)


387.2.11.1
UE features for two-step RACH (2)


397.2.11.2
UE features for NR-U (6)


407.2.11.3
UE features for IAB (5)


417.2.11.4
UE features for 5G V2X (4)


417.2.11.5
UE features for URLLC/IIoT (10)


457.2.11.6
UE features for eMIMO (6)


467.2.11.7
UE features for UE power savings (5)


467.2.11.8
UE features for NR positioning (5)


487.2.11.9
UE features for NR mobility enhancements (4)


487.2.11.10
UE features for MR-DC/CA (5)


497.2.11.11
UE features for CLI/RIM (1)


507.2.11.12
UE features for TEIs (2)


517.2.11.13
Others (2)


527.2.12
Others (3)




1 Opening of the meeting (Day 1: 9.00 AM)
R1-1911417
Statement Regarding Engagement with Companies Added to the U.S. Export Administration Regulations (EAR) Entity List in 3GPP Activities
RAN1 Chair

1.1 Call for IPR

I draw your attention to your obligations under the 3GPP Partner Organizations' IPR policies.  Every Individual Member organization is obliged to declare to the Partner Organization or Organizations of which it is a member any IPR owned by the Individual Member or any other organization which is or is likely to become essential to the work of 3GPP.

	The attention of the delegates to this meeting was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates were asked to take note that they were thereby invited:

· to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP. 

· to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Statement and the Licensing declaration forms (http://www.etsi.org/WebSite/document/Legal/IPRForms.doc). 


1.2 Competition Law Statement

	The attention of the delegates to this meeting was drawn to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel. The present meeting will be conducted with strict impartiality and in the interests of 3GPP.  Furthermore, delegates were reminded that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.


1.3 Network Usage Conditions

	Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws.

Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network.

Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself.

1. DON’T place your WiFi device in ad-hoc mode 

2. DON’T set up a personal hotspot in the meeting room 

3. DO try 802.11a if your WiFi device supports it 

4. DON’T manually allocate an IP address 

5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 

6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)


1.4 Check-in for Registered Delegates

	The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary. 


1.5 Aspects related to RAN1 Meeting Management
	Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.


2 Approval of Agenda
R1-2001500
Draft Agenda of RAN1#100bis-e meeting
RAN1 Chair
Email approval till 4/21/2020, no dedicated email thread. Please use email thread “RAN1#100b-e: meeting management” if any concerns/comments. Automatically endorsed if no comments by 4/21/2020
R1-2002749
Additional Guidelines for RAN1#100b e-Meeting Management
RAN1 Chair

R1-2002748
RAN1#100bis-e Meeting_x00B_Timelines, Scope, Process
RAN1 Chair, MCC

NOTE: for all email threads, agreements/conclusion may be achieved earlier than the deadline via conference calls (particularly possible for UE feature related email threads)

3 Highlights from RAN plenary

R1-2001501
Highlights from RAN#87-e relevant to RAN1
RAN1 Chair

Noted

4 Approval of Minutes from previous meetings
R1-2001502
Report of RAN1#100-e meeting
ETSI MCC

[100b-e-MeetingMinutes100e] Email approval till 4/21/2020 (Patrick, MCC)
5 Incoming Liaison Statements (10+3=13)
R1-2002704
RAN1#100b-e preparation phase on incoming LS
RAN1 Chair
R1-2002736
RAN1#100b-e preparation phase on incoming LS
RAN1 Chair
[100b-e-LS-01] Email approval of the reply LS for R1-2001505 by 4/23 (Ericsson, Mattias)
[100b-e-LS-02] Email approval of the reply LS for R1-2001507 by 4/22 (Huawei, ???)
[100b-e-LS-03] Email approval of the reply LS for R1-2001508 by 4/23 (Ericsson, Zhipeng)

[100b-e-LS-04] Email approval of the reply LS for R1-2001509 by 4/22 (ZTE, Xingguang)
[100b-e-LS-05] Email approval of the reply LS for R1-2001511 by 4/23, included a potential TP (ZTE, Li).
[100b-e-LS-06] Email approval of the reply LS for R1-2001514 by 4/24 (OPPO, Zhisong)
[100b-e-LS-07] Email approval of the reply LS for R1-2001519 by 4/24 (DCM, Yuki)
[100b-e-LS-08] Email discussion/approval for a potential reply LS to R1-2000165 by 4/24 (Ericsson, Claes)
[100b-e-LS-09] Email approval of the reply LS for R1-2001236 by 4/23 (ZTE, Li)
[100b-e-LS-10] Email approval of the reply LS for R1-2001237 by 4/23 under 7.2.2.2.2 (Ericsson, Zhipeng)

[100b-e-LS-101] Email discussion on aspects (including feasibility) related to received interference power measurement, including a potential reply LS,  in response to LS in R1-1909950 by 4/23 (Huawei, ???)

// LTE

R1-2001517
LS on open PUR issues for NB-IoT/eMTC
RAN2, Ericsson

R1-2001849
Discussion on RAN2 LS on open PUR issues
ZTE

R1-2002501
On the LS on open PUR issues for NB-IoT/eMTC
Ericsson

R1-2002603
Draft reply LS on open PUR issues for NB-IoT/eMTC
Huawei, HiSilicon

R1-2001518
LS on NR coexistence
RAN2, Qualcomm

R1-2001848
Discussion on RAN2 LS on NR coexistence
ZTE

R1-2002502
On the LS on NR coexistence for NB-IoT/eMTC
Ericsson

R1-2002602
Draft reply LS on NR coexistence
Huawei, HiSilicon

// NR
R1-2001505
LS on eMIMO RRC parameters
RAN2, Ericsson

R1-2001591
Draft reply LS on eMIMO parameters
ZTE

R1-2001637
Draft reply LS on eMIMO RRC parameters
vivo

R1-2001744
Discussion on eMIMO RRC parameters
OPPO

R1-2001909
Draft reply LS on eMIMO RRC parameters
LG Electronics

R1-2002058
Draft reply LS on MIMO RRC parameters
CATT

R1-2002099
Draft reply to RAN2 LS on eMIMO RRC parameters
Samsung

R1-2002285
Draft LS reply on eMIMO RRC parameters
Ericsson

R1-2002289
Draft Reply LS to RAN2 on multi-TRP
Nokia, Nokia Shanghai Bell

R1-2002672
[Draft] Reply LS on eMIMO RRC parameters
Huawei, HiSilicon

R1-2001506
LS on random access procedure in NR-U
RAN2, InterDigital

R1-2001641
Discussion on random access procedure in NR-U
vivo

R1-2001718
Discussion on the LS for the random access procedure in NR-U
ZTE, Sanechips

R1-2001946
Draft Reply LS on random access procedure in NR-U
LG Electronics

R1-2002310
Discussion on NR-U PRACH root sequence and random access procedure for 2-step RACH


Apple

R1-2001507
LS on DCP
RAN2, Huawei

R1-2001580
Draft reply LS on DCP
ZTE

R1-2001642
Discussion on MAC-PHY interactions for DCP and CSI reporting
vivo

R1-2002189
TP to address RAN2 LS on DCP
NEC

R1-2002663
Draft reply LS on the configuration of ps-TransmitPeriodicCSI and ps-TransmitPeriodicL1-RSRP


Huawei, HiSilicon

R1-2001508
LS to RAN1 on preamble-to-PRU mapping for 2-step CFRA
RAN2, Ericsson

R1-2001639
Discussion on preamble-to-PRU mapping for 2-step CFRA
vivo

R1-2001948
Draft Reply LS on preamble-to-PRU mapping for 2-step CFRA
LG Electronics

R1-2002102
Draft reply LS on preamble-to-PRU mapping for 2-step CFRA
Samsung

R1-2002311
Discussion on preamble-to-PRU mapping for 2-step CFRA
Apple

R1-2002374
[DRAFT] LS Response on preamble-to-PRU mapping for 2-step CFRA
Ericsson

R1-2002659
Draft LS reply to RAN2 on preamble-to-PRU mapping for 2-step CFRA
Huawei, HiSilicon

R1-2001509
LS on the applicability of UE capability for NE-DC
RAN2, ZTE

R1-2001628
[DRAFT] Reply LS on the applicability of UE capabilities for NE-DC
ZTE

R1-2002678
draft reply LS on the applicability of UE capability for NE-DC
Huawei, HiSilicon

R1-2001510
LS to RAN1 on T-delta in IAB
RAN2, Samsung

R1-2002101
Discussion on T_delta in IAB
Samsung

R1-2002187
Draft reply LS on T_delta in IAB
LG Electronics

//Note: there are also contributions under 7.2.3.4

R1-2001511
LS to RAN1 on the starting point of MSGB window
RAN2, ZTE

R1-2001640
Discussion on the starting point of MSGB window
vivo

R1-2001716
[Draft] Reply LS on the starting point of MsgB window
ZTE, Sanechips

R1-2001947
Draft Reply LS on the starting point of MSGB window
LG Electronics

R1-2002103
Draft reply LS on the starting point of MSGB window
Samsung

R1-2002309
Discussion on the starting point of MsgB window
Apple

R1-2002375
[DRAFT] LS Response on the starting point of MSGB window
Ericsson

R1-2002658
Draft LS reply to RAN2 on the starting point of MSGB window
Huawei, HiSilicon

R1-2001512
Reply LS on signaling of Q for a serving cell in NR-U
RAN2, Qualcomm

R1-2001931
Discussion on signaling of Q for a serving cell in NR-U
LG Electronics

R1-2001513
Guidelines for UE capability definitions
RAN2, Ericsson, Intel

R1-2001514
LS on dormant BWP configuration and related operation
RAN2, OPPO

R1-2001629
[DRAFT] Reply LS on dormant BWP configuration and related operation
ZTE

R1-2001630
Discussion on dormant BWP configuration and related operation
ZTE

R1-2001638
Draft Reply LS on dormant BWP configuration and related operation
vivo

R1-2001771
Draft reply LS on dormant BWP configuration and related operation
OPPO

R1-2001838
Draft LS reply on dormant BWP configuration and related operation
MediaTek Inc.

R1-2002051
Discussion on RAN2 LS on dormant BWP configuration and related operation
Futurewei

R1-2002055
Discussion on RAN2 LS on dormant BWP configuration and related operation
LG Electronics

R1-2002057
Draft reply LS on dormant BWP configuration and related operation
CATT

R1-2002298
[DRAFT] Reply LS on dormant BWP configuration and related operation
Nokia, Nokia Shanghai Bell

R1-2002515
Draft response to LS on dormant BWP configuration and related operation
Qualcomm Incorporated

R1-2002664
Draft LS response to RAN2 LS on dormancy behavior
Huawei, HiSilicon

R1-2002680
Discussion on the reply LS for SCell dormancy
Huawei, HiSilicon

R1-2001519
Reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
RAN2, NTT DOCOMO

R1-2001590
Draft reply LS on UE capabilities of CSI-RS
ZTE

R1-2001901
Draft reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
vivo

R1-2001980
Draft reply LS on CSI-RS capabilities
Intel Corporation

R1-2002100
Draft reply to RAN2 LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
Samsung

R1-2002427
[Draft] Reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
NTT DOCOMO, INC

R1-2002514
Draft response to Reply LS on CSI-RS capabilities
Qualcomm Incorporated

R1-2002673
Discussion on Reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
Huawei, HiSilicon

R1-2002681
[Draft] Reply LS on CSI-RS capabilities (FG 2-33/36/40/41/43)
Huawei, HiSilicon

R1-2001522
LS on Tx switching between two uplink carriers
RAN4, Apple

R1-2001627
[DRAFT] Reply LS on Tx switching between two uplink carriers
ZTE

R1-2002308
[Draft] Reply LS on UE Tx switching period delay and DL interruption
Apple

R1-2002394
Discussion on RAN1 specification impact of DL interruption
CATT

R1-2002516
Dicussion on 1Tx-2Tx switching impact in RAN1
Qualcomm Incorporated

R1-2002615
On UE Tx switching period delay and DL interruption
Nokia, Nokia Shanghai Bell

//Related to R1-2000165 (Secondary DRX)

R1-2001581
Discussion on secondary DRX group
ZTE

R1-2001582
Draft reply LS on secondary DRX group
ZTE

R1-2001693
Discussion on 2nd DRX group
vivo

R1-2001845
Discussion on impact of secondary DRX group
MediaTek Inc.

R1-2002056
Discussion on the RAN1 impacts on Secondary DRX group
CATT

R1-2002492
Draft LS response on secondary DRX group
Ericsson

R1-2002493
On secondary DRX group
Ericsson

R1-2002578
RAN1 impact analysis due to the introduction of secondary DRX cycle
Huawei, HiSilicon

R1-2002662
Draft reply LS on secondary DRX cycle
Huawei, HiSilicon

//Related to R1-2001236 (2-Step)

R1-2001717
[Draft] Reply LS on the support of 2-step CFRA
ZTE, Sanechips

R1-2002376
[DRAFT] LS Response on Support of CSI-RS in 2-step CFRA
Ericsson

R1-2002660
Draft LS reply to RAN2 on support of 2-step CFRA
Huawei, HiSilicon

R1-2001966
LS/o on synchronization of Y.DNI-fr “Framework and Requirements of Decentralized Trustworthy Network Infrastructure” in Q2/13
ITU-T SG13, China Telecom, Huawei

//Related to R1-2001237 (2-step)

R1-2002373
[DRAFT] LS Response on NR-U PRACH root sequence for 2-step RA
Ericsson

//Related to R1-1909950 (received interference power measurement)
R1-2002670
Discussion on the feasibility of received interference power measurement in NR
Huawei, HiSilicon

R1-2002671
Draft Reply LS on the feasibility of received interference power measurement
Huawei, HiSilicon

//Related to V2X

R1-2002677
[Draft] Reply LS on UL-SL prioritization
Huawei

// The following LSs are noted (RAN1 is cc-ed)

R1-2001503
LS on power control for NR-DC
RAN2, vivo

R1-2001504
LS to RAN4 on measurement range and granularity
RAN2, Intel

R1-2001515
Reply LS on updates for TS 36.300 and TS 38.300
RAN3, Ericsson

R1-2001516
Reply LS on UAV positioning
SA1, InterDigital

R1-2001520
LS on gNB measurements report mapping for NR Positioning
RAN4, Ericsson

R1-2001521
LS on UL LBT failure recovery for the target cell
RAN4, Ericsson

R1-2001523
Reply LS on CLI measurement capability
RAN4, Huawei

R1-2002657
On resouce reservation in NB-IoT and eMTC
Futurewei

Withdrawn
5.1 RAN1 Aspects for RF requirements for NR frequency range 1 (FR1) (3)
Refer to RP-193266 for details regarding RAN4-led WID on RF requirements for NR frequency range 1 (FR1)
//From the LS section

R1-2002191
Summary of uplink Tx switching
Moderator (China Telecom)

[100b-e-LS-TxSwitching-01] Email approval of the reply LS for R1-2001522 by 4/24 under AI 5.1 (CT/Apple, Jianchi/Chunhai)
· Discussion on potential RAN1 specification impact.

· Reply LS to RAN4
[100b-e-LS-TxSwitching-02] Email discussion/approval of the remaining issues inter-band UL CA

· Mapping between UL transmission ports and Tx chain

· Switching mechanism

· Support of codebook based PUSCH transmission.

till 4/27, and the corresponding TPs if any by 4/30 (CT, Jianchi)
[100b-e-LS-TxSwitching-03] Email discussion/approval of the remaining issues for SUL, EN-DC and other general issues
· How to capture the additional time for PUSCH preparation procedure

· Whether additional preparation time is needed for other UL channels/signals

· UE behaviour in switching period

· Twisted-order scheduling/observation period

· For EN-DC, mapping between UL transmission ports and Tx chain, TDM pattern, switching mechanism, handling of transmission collision between 1Tx transmission in LTE and 2Tx transmission in NR.

· Whether more than two uplink carriers can be supported.

till 4/27, and the corresponding TPs if any by 4/30 (CT, Jianchi)
[100b-e-LS-TxSwitching-04] Email approval of TP capturing RAN1#100-e’s agreements regarding UL Tx switching (04/27-04/29) – Jianchi (CT)
R1-2001626
Remaining Issues on Support of Tx Switching between Two Uplink Carriers
ZTE

R1-2001643
Potential RAN1 impact to support TDM switching between the 1Tx and 2Tx UL carriers
vivo

R1-2001743
Discussion on Tx Switching between Two Uplink Carriers
OPPO

R1-2001821
Remaining Details on UE Tx Switching between Two UL Carriers
MediaTek Inc.

R1-2002059
Discussion on UL switching
CATT

R1-2002104
DL interruption during switching period between 1Tx and 2Tx for two uplink carriers
Samsung

R1-2002190
Remaining issues on uplink Tx switching
China Telecom

R1-2002222
On Tx switching between two uplink carriers
Nokia, Nokia Shanghai Bell

R1-2002413
RAN1 aspects of UL Tx switching
Ericsson

R1-2002661
Discussion on the remaining problems of supporting Tx switching between two uplink carriers


Huawei, HiSilicon

5.2 RAN1 Aspects for RF Requirement Enhancements for FR2
Refer to RP-192653 for details regarding RAN4-led WID on NR RF Requirement Enhancements for FR2
Note: although contributions are allowed, RAN1’s work is “reactive” as follows:

· RAN4 will provide further details on the RAN4 agreed solution(s) to RAN1/RAN2 before RAN1/RAN2 start their work if RAN1/RAN2 help is needed. 

· This objective does not aim to propose the same alternatives which were not agreed (i.e. Alt1, Alt2 and Alt3 not agreed in RAN1#98 under Rel-16 NR eMIMO work item)
R1-2001592
Enhancement on FR2 MPE mitigation
ZTE

6 E-UTRA
6.1 Maintenance of E-UTRA Releases 8 – 15

R1-2002163
[Draft] Correction on WUS related procedure for NB-IoT
ZTE

R1-2002164
[Draft] Correction on WUS related procedure for BL/CE UE
ZTE

R1-2002165
[Draft] Inclusion of WUS related procedures in TS 36.201
ZTE

R1-2002166
Discussion on WUS transmission in TDD special subframe
ZTE

R1-2002167
Draft CR on WUS transmission in TDD special subframe
ZTE

R1-2002171
Clarification on set of values for HARQ delay
Qualcomm Incorporated

R1-2002172
Clarification on PUCCH collision for HARQ-ACK bundling
Qualcomm Incorporated

R1-2002598
Correction on Msg3 NPUSCH retransmission for NB-IoT EDT
Huawei, HiSilicon

R1-2002599
Correction on NRS presence in non-anchor carriers for NB-IoT
Huawei, HiSilicon

R1-2002600
Correction on NPDCCH monitoring for NB-IoT
Huawei, HiSilicon

6.2 Maintenance of E-UTRA Release 16

Only essential corrections 
For contributions (except UE features), only text proposals are allowed (no individual draft CRs, please!). 

· Endorsed TPs will be produced as a result of email discussion/approval

· The finally endorsed TPs should be accompanied with analysis of “reasons for change”, “summary of changes”, “Specs/Sections impacted”, and “consequences if not approved” and individually documented with a tdoc #

· Editors’ CRs are to be prepared and endorsed by consolidating the endorsed TPs with a complete CR cover page

Limit to maximum 1 contribution per 1 company/organization/university for any agenda item, including “Others”. 

[100b-e-LTE-Editor-WICode-Spec#] Ediors’ CRs to consolidate all the endorsed TPs to be prepared by 5/4 and to be endorsed by 5/7 (Editors)
· Per Rel-16 work item and per spec
· On the cover page, please summarize the changes (reasons, the changes, impect) for each endorsed TP incorporated

· If there is any editorial update not resulting from any email threads, please duly state them on the CR cover page

[100b-e-LTE-Rel-16-RRC] Updated RRC parameters consolidating new/updated RRC parameters (if any) to be prepared by 5/4 and to be endorsed by 5/7 – Fred (Qualcomm)

· Any new RRC parameters are highly discouraged!
6.2.1 Maintenance of Additional MTC Enhancements
6.2.1.1 UE-group wake-up signal
R1-2001576
Corrections on UE-group wake-up signal
Huawei, HiSilicon

6.2.1.2 Support for transmission in preconfigured UL resources
R1-2002357
Preconfigured UL resources feature lead summary RAN1 #100bis-e
Sierra Wireless, S.A.

R1-2001567
Corrections on transmission in preconfigured UL resources
Huawei, HiSilicon

R1-2001850
Remaining issues for transmission in preconfigured UL resources for MTC
ZTE

R1-2002173
Support for transmission in preconfigured UL resources
Qualcomm Incorporated

R1-2002356
LTE-M Pre-configured UL Resources Design Considerations 
Sierra Wireless, S.A.

R1-2002503
Corrections for Preconfigured UL resources for LTE-MTC
Ericsson

R1-2002641
Remaining issues for preconfigured UL resources
Nokia, Nokia Shanghai Bell

6.2.1.3 Scheduling of multiple DL/UL transport blocks
R1-2002512
Feature lead summary for Multi-TB scheduling for LTE-MTC
Ericsson

R1-2001568
Corrections on scheduling of multiple transport blocks
Huawei, HiSilicon

R1-2001852
Remaining issues on scheduling enhancement for MTC
ZTE

R1-2001928
Remaining issues on multiple transport blocks scheduling in MTC
LG Electronics

R1-2002174
Scheduling of multiple DL/UL transport blocks
Qualcomm Incorporated

R1-2002504
Corrections for Multi-TB scheduling for LTE-MTC
Ericsson

R1-2002642
Remaining issues for scheduling of multiple TBs
Nokia, Nokia Shanghai Bell

R1-2002654
HARQ-ACK bundling for Multi-TB scheduling
Futurewei

6.2.1.4 Coexistence of LTE-MTC with NR

R1-2002513
Feature lead summary for NR coexistence performance improvements for LTE-MTC
Ericsson

R1-2001569
Corrections on eMTC co-existence with NR
Huawei, HiSilicon

R1-2001855
Remaining issues on LTE-MTC resource reservation
ZTE

R1-2002175
Coexistence of LTE-MTC with NR
Qualcomm Incorporated

R1-2002505
Corrections for NR coexistence performance improvements for LTE-MTC
Ericsson

R1-2002643
Remaining issues for co-existence of eMTC with NR
Nokia, Nokia Shanghai Bell

6.2.1.5 Support of quality report in Msg3 and Connected Mode

6.2.1.6 MPDCCH performance improvement

R1-2001574
Corrections on MPDCCH performance improvement
Huawei, HiSilicon

R1-2001854
Clarification on MPDCCH performance improvement
ZTE

6.2.1.7 CE mode A and B improvements for non-BL UEs

R1-2001930
FL summary of CE mode A and B improvements for non-BL UEs
Moderator (LG Electronics)

R1-2001859
Correction on CSI-RS based CSI feedback for non-BL UE
ZTE

R1-2001929
Corrections on CE mode A and B improvements for non-BL UEs
LG Electronics

6.2.1.8 Use of LTE Control Channel Region for DL Transmission

R1-2002644
Feature lead summary on use of LTE control channel region
Moderator (Nokia)

R1-2001860
Clarification on use of LTE control region for PUR related DL transmission
ZTE

R1-2002506
Corrections for Use of LTE control channel region for LTE-MTC
Ericsson

6.2.1.9 Use of RSS for measurement improvements
6.2.1.10 Others

6.2.2 Maintenance of Additional Enhancements for NB-IoT
6.2.2.1 UE-group wake-up signal
R1-2001575
Corrections on UE-group wake-up signal
Huawei, HiSilicon

6.2.2.2 Support for transmission in preconfigured UL resources

R1-2002640
Feature lead summary #1 on transmission in pre-configured UL resources for NB-IoT
Moderator (Huawei)

R1-2001570
Corrections on transmission in preconfigured UL resources
Huawei, HiSilicon

R1-2001851
Remaining issues for transmission in preconfigured UL resources for NB-IoT
ZTE

R1-2002176
Support for transmission in preconfigured UL resources
Qualcomm Incorporated

R1-2002507
Corrections for Preconfigured UL resources for NB-IoT
Ericsson

R1-2002645
Remaining issues for preconfigured UL resources
Nokia, Nokia Shanghai Bell

6.2.2.3 Scheduling of multiple DL/UL transport blocks
R1-2001571
Corrections on scheduling of multiple DL/UL transport blocks
Huawei, HiSilicon

R1-2001853
Remaining issues on scheduling enhancement for NB-IoT
ZTE

R1-2002186
Scheduling of multiple DL/UL transport blocks
Qualcomm Incorporated

R1-2002508
Corrections for Multi-TB scheduling for NB-IoT
Ericsson

R1-2002646
Remaining issues for scheduling of multiple TBs
Nokia, Nokia Shanghai Bell

6.2.2.4 Coexistence of NB-IoT with NR

R1-2001572
Corrections on coexistence of NB-IoT with NR
Huawei, HiSilicon

R1-2001856
Remaining issues on NB-IoT resource reservation
ZTE

R1-2002177
Coexistence of NB-IoT with NR
Qualcomm Incorporated

R1-2002509
Corrections for NR coexistence performance improvements for NB-IoT
Ericsson

R1-2002647
Remaining issues for co-existence of NB-IoT with NR
Nokia, Nokia Shanghai Bell

6.2.2.5 Support of Quality report in Msg3 and Connected Mode
6.2.2.6 Presence of NRS on a non-anchor carrier for paging

6.2.2.7 Others

6.2.3 Maintenance of DL MIMO efficiency enhancements for LTE
6.2.3.1 Support of enhancing SRS capacity and coverage
6.2.3.1.1 Additional SRS symbols

R1-2001573
Corrections on additional SRS symbols
Huawei, HiSilicon

R1-2001593
Maintenance of additional SRS symbols
ZTE

R1-2001910
Text proposals on additional SRS symbols
LG Electronics

R1-2001945
Remaining details of additional SRS symbols
Nokia, Nokia Shanghai Bell

R1-2001981
Corrections to additional SRS
Intel Corporation

R1-2002178
Additional SRS symbols
Qualcomm Incorporated

R1-2002486
Maintenance on additional SRS symbols
Ericsson

6.2.3.1.2 Others

6.2.3.2 Others

6.2.4 Maintenance of LTE-based 5G Terrestrial Broadcast

R1-2002601
Text proposal for new numerology in TS 36.201
Huawei, HiSilicon

6.2.4.1 New numerology(ies) for PMCH for support of rooftop reception
R1-2001635
Corrections to TS 36.211 for the 0.37kHz subcarrier spacing MBSFN
ZTE

R1-2002179
Support of longer numerologies for rooftop reception
Qualcomm Incorporated

R1-2002626
UE assumptions of MBSFN-RS for new PMCH numerology for support of rooftop reception


EBU, BBC, IRT

6.2.4.2 Necessity and detailed enhancements to the physical channels and signals in the CAS
R1-2002180
Corrections for CAS reception
Qualcomm Incorporated
6.2.4.3 New numerology for support of mobility of up to 250km/h
6.2.4.4 Others

R1-2001636
Text proposal for new numerology in TS 36.214
ZTE

6.2.5 LTE Rel-16 UE Features (8)
[100e-b-LTE-UEFeatures-Remaining] Email discussion/approval of remaining issues (especially the one identified as low priority items in FL’s summaries) starting no earlier than 4/30 till next meeting – Hiroki (DCM)/Ralf (ATT)
6.2.5.1 UE features for additional MTC enhancements (2)
R1-2002428
Summary on UE features for additional MTC enhancements
Moderator (NTT DOCOMO, INC.)

[100b-e-LTE-UEFeatures-eMTC-01] Email discussion/approval on feature group structure for both additional MTC enhancements and additional enhancements for NB-IoT (20th-24th April) – (DCM, Hiroki)

· Discuss whether FG[1-2] is a separate FG (i.e., remove bracket) or FG[1-2] is removed and added as a component in FG1-1

· This issue is jointly discussed with FG[2-2]

· Discuss whether FG[1-7] is a separate FG (i.e., remove bracket) or FG[1-7] is removed and added as a component in FG1-3/1-4/1-5/1-6

· This issue is jointly discussed with FG[2-4]

· Discuss whether FG1-9 is kept as a separate FG or FG1-9 is removed and added as a component in FG1-3/1-4/1-5/1-6

· This issue is jointly discussed with FG2-5

· Discuss whether or not to introduce separate FGs for slot/symbol level resource reservation for FG1-23 to 1-26

· This issue is jointly discussed with FG2-12 and 2-13

[100b-e-LTE-UEFeatures-eMTC-02] Email discussion/approval on feature group structure for additional MTC enhancements (20th-24th April) – (DCM, Hiroki)

· Discuss whether to introduce a separate FG for the combination of FG1-3 and the larger UL TBS or to reuse the legacy capability signaling to support the combination

· Discuss whether to introduce separate FGs for the combinations of FG1-10/11/12/13 with each legacy feature or to reuse the legacy capability signaling to support the combinations

· Discuss whether to introduce a separate FG for multi-TB unicast HARQ multiplexing or to add multi-TB unicast HARQ multiplexing as component in FG1-11

· Discuss whether or not to introduce a separate FG for CSI-RS-based feedback with codebook subset restriction

· Confirm to keep FG1-1/3/4/5/6/8/10~19/21~41

· Confirm to remove FG1-20

· Confirm to remove FG1-42 to leave this FG to RAN4

R1-2001857
Discussion on UE features for additional MTC enhancements
ZTE

R1-2002181
UE features for eMTC
Qualcomm Incorporated

R1-2002510
On the RAN1 UE feature list for Rel-16 LTE-MTC
Ericsson

R1-2002604
Rel-16 UE features for LTE-MTC
Huawei, HiSilicon

6.2.5.2 UE features for additional enhancements for NB-IoT (1)
R1-2002429
Summary on UE features for additional enhancements for NB-IoT
Moderator (NTT DOCOMO, INC.)

[100b-e-LTE-UEFeatures-NBIoT-01]  Email discussion/approval on feature group structure for additional enhancements for NB-IoT (20th-24th April) – Hiroki (DCM)

· Confirm to keep FG2-1/3/6~16

R1-2001858
Discussion on   UE features for additional enhancements for NB-IoT
ZTE

R1-2002182
UE features for NB-IoT
Qualcomm Incorporated

R1-2002511
On the RAN1 UE feature list for Rel-16 NB-IoT
Ericsson

R1-2002605
Rel-16 UE features for NB-IoT
Huawei, HiSilicon

6.2.5.3 UE features for DL MIMO efficiency enhancements for LTE (3)
R1-2001864
Summary on UE features for DL MIMO efficiency enhancements for LTE
Moderator (AT&T)

[100b-e-LTE-UEFeatures-MIMO-01] Email discussion/approval of “Proposal 1 (3-1)” in R1-2001864 till 4/24 – Ralf (ATT)
· Note: “FL Proposal 4 (3-3)“ in R1-2001860 can be discussed within the scope of this email discussion 
[100b-e-LTE-UEFeatures-MIMO-02] Email discussion/approval of “Proposal 2 (3-1B)” in R1-2001864 till 4/24 – Ralf (ATT)

· Note: “FL Proposal 4 (3-3)“ in R1-2001860 can be discussed within the scope of this email discussion
[100b-e-LTE-UEFeatures-MIMO-03] Email discussion/approval of “Proposal 3 (3-2)” in R1-2001864 till 4/24 – Ralf (ATT)
R1-2001594
UE features of DL MIMO efficiency enhancements for LTE
ZTE

R1-2002160
Discussion on RAN1 UE feature for LTE eMIMO
LG Electronics

R1-2002183
UE features for MIMO
Qualcomm Incorporated

R1-2002474
On UE features for LTE DL MIMO efficiency enhancements
Ericsson

R1-2002606
Rel-16 UE features for LTE MIMO
Huawei, HiSilicon

6.2.5.4 UE features for LTE-based 5G terrestrial broadcast (1)
R1-2001865
Summary on UE features for LTE-based 5G terrestrial broadcast
Moderator (AT&T)

[100b-e-LTE-UEFeatures-TERR_BCAST-01] Email approval of the table for feature groups 4-2 and 4-3 as in R1-2001865 till 4/22 – Ralf (ATT)
6.2.5.5 Others

Including interactions among UE features across WIs

R1-2002430
Summary on LTE UE features for others
Moderator (NTT DOCOMO, INC.)

R1-2001755
Disconssion on LTE feature list for 5G_V2X_NRSL
OPPO

R1-2002242
LTE UE features for V2X
Ericsson

6.2.6 Others

7 NR Maintenance

7.1 Maintenance of Release 15 NR
Only essential corrections – a rejected draft CR will be marked in red
· Note: no intention to use editors’ alignment CRs any more
R1-2002200
On capturing features that are mandatory without capability signaling in specifications
Ericsson

7.1.1 Maintenance for Initial access and mobility
R1-2001608
Correction on RRC parameter of failureDetectionResources
ZTE

R1-2001609
Correction on configuration of ra-searchspace
ZTE

R1-2002105
38.213 DRAFT CR (Rel-15, F) on PRACH power ramping suspension
Samsung

R1-2002106
38.213 DRAFT CR (Rel-16, A) on PRACH power ramping suspension
Samsung

R1-2002107
38.211 DRAFT CR (Rel-15, F) on the calculation of k_SSB
Samsung

R1-2002108
38.211 DRAFT CR (Rel-16, A) on the calculation of k_SSB
Samsung

R1-2002109
38.211 DRAFT CR (Rel-15, F) on the reference for L_max
Samsung

R1-2002168
Discussion on the QCL assumption for paging reception
OPPO

R1-2002169
Draft CR on the QCL assumption for paging reception
OPPO

R1-2002231
Correction on SS-RSRPB measurement
Huawei, HiSilicon

7.1.2 Maintenance for MIMO
R1-2001644
CR on PDSCH EPRE to CSIRS EPRE assumption for CQI feedback
vivo

R1-2001911
TP for the maximum number of CSI-RS resources for BM
LG Electronics

R1-2002110
38.214 DRAFT CR (Rel-15, F) on CSI reporting for BWP size < 24 PRBs
Samsung

R1-2002111
38.214 DRAFT CR (Rel-16, A) on CSI reporting for BWP size < 24 PRBs
Samsung

R1-2002195
Draft CR on SRS carrier switching for 38.212
ZTE

R1-2002196
Draft CR on SRS carrier switching for 38.213
ZTE

R1-2002197
Clarification on minimum number of supported PDSCH Resource Element mapping patterns


Ericsson

R1-2002202
Correction on TRS configuration to TS38.214
CMCC

R1-2002290
PUCCH Spatial Relation after CBRA BFR
Nokia, Nokia Shanghai Bell

R1-2002291
Clarification on the number of occupied CPUs and delay requirements for CSI reports using PUSCH

Nokia, Nokia Shanghai Bell

R1-2002292
Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations N_CPU
Nokia, Nokia Shanghai Bell

R1-2002312
Clarifications on TRS configuration with duplexing direction change
Apple

R1-2002313
Discussion on applicable time for PDSCH and PUSCH beam indication
Apple

R1-2002314
Draft CR on L1-RSRP report on PUSCH
Apple

R1-2002315
Draft CR on applicable time for PDSCH beam indication
Apple

R1-2002316
Draft CR on applicable time for PUSCH beam indication
Apple

R1-2002317
Draft CR on QCL assumption for DL PT-RS and DMRS
Apple

R1-2002318
Draft CR on QCL indication for CSI-RS
Apple

R1-2002491
Correction on CSI-RS resource set configuration
Ericsson

R1-2002497
Correction to contention-based beam failure recovery
Ericsson

R1-2002517
Draft CR on resource and port occupation of duplicate CSI-RS resource
Qualcomm Incorporated

R1-2002518
Discussion on CR on resource and port occupation of duplicate CSI-RS resource
Qualcomm Incorporated

R1-2002519
Draft CR on CSI report based on deactivated or deconfigured resources
Qualcomm Incorporated

R1-2002520
Discussion on CSI report based on deactivated or deconfigured resources
Qualcomm Incorporated

R1-2002521
Draft CR on type-A and Type-B Carrier Switching
Qualcomm Incorporated

R1-2002522
Discussion paper on Type-A and Type-B Carrier Switching
Qualcomm Incorporated

7.1.3 Maintenance for Scheduling/HARQ aspects
R1-2001610
Correction on PDCCH blind detection for NR-DC
ZTE

R1-2001645
Clarification on UE behavior with empty list for SFI combination per cell
vivo

R1-2001646
CR on OCC length for PUCCH format 4
vivo

R1-2001780
Correction on out-of-order HARQ of SPS PDSCH release
OPPO

R1-2001781
Correction on multiplexing HARQ-ACK/SR/CSI
OPPO

R1-2001982
Clarification on UE features on BWP operations
Intel Corporation

R1-2002060
Clarification for processing order of UL multiplexing and cancellation
CATT

R1-2002061
Correction on the definition for timeline condition
CATT

R1-2002062
Correction for PUCCH repetition transmission
CATT

R1-2002063
Clarification on PUSCH frequency hopping
CATT

R1-2002223
Draft 38.212 CR - Specification alignment on SFI configuration
Nokia, Nokia Shanghai Bell

R1-2002230
Correction on rate-matching for PDSCH/PDSCH with SPS in TS38.214
Huawei, HiSilicon

R1-2002379
Correction on rate matching output sequence length
Sharp

R1-2002380
Correction on removal of padding bits in DCI size alignment procedure
Sharp

R1-2002462
UL grant overridden between configured grant and RAR grant
ASUSTeK

R1-2002463
Draft LS on UL grant overridden between configured grant and RAR grant
ASUSTeK

R1-2002464
CR on multi-slot PUCCH repetition overlap with a single PUSCH
ASUSTeK

R1-2002523
Draft CR on 38.214 PDSCH resource mapping
Qualcomm Incorporated

R1-2002581
Clarification on UCI multiplexing on PUSCH
Huawei, HiSilicon

R1-2002582
Correction on PDCCH candidate colliding with RB-symbol level RMR in TS38.213
Huawei, HiSilicon

R1-2002616
Draft 38.212 CR Correction to dynamic frequency domain resource allocation type selection


Nokia, Nokia Shanghai Bell

R1-2002617
Draft 38.214 CR Corrections to PDSCH PRB bundling
Nokia, Nokia Shanghai Bell

R1-2002629
Clarification on PUSCH slot-aggregation and A-CSI multiplexing
Ericsson

R1-2002668
Corrections on multiplexing UCI from multiple PUCCHs on PUSCH.
Huawei, HiSilicon

R1-2002669
Corrections on dynamic PUSCH skipping with overlapping with PUCCH
Huawei, HiSilicon

R1-2002675
Correction on PDCCH blind detection capability for CA in TS38.213
Huawei, HiSilicon

R1-2002682
Corrections on dynamic PUSCH skipping without overlapping with PUCCH
Huawei, HiSilicon

7.1.4 Maintenance for NR-LTE co-existence
R1-2002524
Draft CR on 38.213 ENDC single-Tx opeation
Qualcomm Incorporated

R1-2002525
Draft CR on 38.213 ENDC single-Tx operation
Qualcomm Incorporated

R1-2002619
Draft 36.213 CR Correction on PUSCH and PDSCH scheduling with CSS
Nokia, Nokia Shanghai Bell

7.1.5 Maintenance for UL power control
R1-2002665
Correction on PUSCH with a Type 1 configured grant
Huawei, HiSilicon

7.1.6 Others

R1-2001960
Correction for inconsistent shadow fading parameters in NTN rural scenario
Nokia, Nokia Shanghai Bell

R1-2001961
Correction for inconsistent shadow fading parameters in NTN rural scenario
Nokia, Nokia Shanghai Bell

7.2 Maintenance of Release 16 NR

Only essential corrections 
For contributions (except UE features), only text proposals are allowed (no individual draft CRs, please!). 

· Endorsed TPs will be produced as a result of email discussion/approval

· The finally endorsed TPs should be accompanied with analysis of “reasons for change”, “summary of changes”, “Specs/Sections impacted”, and “consequences if not approved” and individually documented with a tdoc #

· Editors’ CRs are to be prepared and endorsed by consolidating the endorsed TPs with a complete CR cover page

Limit to maximum 1 contribution per 1 company/organization/university for any agenda item, including “Others”. 

[100b-e-NR-Editor-WICode-Spec#] Ediors’ CRs to consolidate all the endorsed TPs to be prepared by 5/4 and to be endorsed by 5/7 (Editors)
· Per Rel-16 work item and per spec
· On the cover page, please summarize the changes (reasons, the changes, impect) for each endorsed TP incorporated

· If there is any editorial update not resulting from any email threads, please duly state them on the CR cover page

[100b-e-NR-Rel-16-RRC] Updated RRC parameters consolidating new/updated RRC parameters (if any) to be prepared by 5/4 and to be endorsed by 5/7 – Fred (Qualcomm)

· Any new RRC parameters are highly discouraged!
[100b-e-NR-38.215-RSRPB-ATF] Email discussion/approval of 38.213 CRs based on the following contributions

· R1-2002648 (agenda item 7.2.6)

· R1-2002649 (agenda item 7.2.6)

· R1-2002639 (agenda item 7.2)

· R1-2002231 (agenda item 7.1)

till 4/23 (Javier, Keysight)
R1-2002618
Draft 38.214 CR Correction to TBS determination when 3824<Ninfo<3825
Nokia, Nokia Shanghai Bell

R1-2002639
Addition of UE measurements to enable OTA conformance testing
Keysight Technologies UK Ltd

7.2.1 Maintenance of Two step RACH for NR (4)
[100b-e-NR-2step-RACH-01] Email discussion/approval of the issue of validation of MsgA PRACH/PUSCH

· Additional PUSCH validation rule

· PRACH occasions not associated with SSB are considered as invalid

till 4/23, with corresponding TP (if any) endorsed by 4/29 (ZTE, Li)
[100b-e-NR-2step-RACH-02] Email discussion/approval of the issue of PRACH/PUSCH conflicting with slot format 
· Define the UE behavior when MsgA PRACH or PUSCH is conflicting with slot format, and the UE behavior if transmission of MsgA PRACH is cancelled.

· N symbols before valid PO should be protected from being indicated as downlink

till 4/23, with corresponding TP (if any) endorsed by 4/29 (ZTE, Li)
[100b-e-NR-2step-RACH-03] Email discussion/approval of the issue of intra-slot frequency hopping

· Mapping order is based on the first hop
· Guard band between hops

· Align the description of first and second hop in the specs

till 4/24, with corresponding TP (if any) endorsed by 4/30 (ZTE, Li)
[100b-e-NR-2step-RACH-04] Email discussion/approval of the issue of issue left from the last meeting

· Reference point of slot offset

· DMRS configuration

till 4/24, with corresponding TP (if any) endorsed by 4/30 (ZTE, Li)
7.2.1.1 Channel Structure for Two-Step RACH

R1-2001712
FL summary on the maintenance of the 2-step RACH channel strucutre
Moderator  (ZTE)

R1-2002728
FL summary on the maintenance of 2-step RACH channel structure Moderator  (ZTE)

R1-2001524
Corrections on channel structure of 2-step RACH
Huawei, HiSilicon

R1-2001647
Remaining issues on channel structure for 2-step RACH
vivo

R1-2001710
Remaining issues of the channel structure for 2-step RACH
ZTE, Sanechips

R1-2001766
Discussion on Channel Structure for Two-step RACH
OPPO

R1-2001949
Remaining details of Channel Structure for 2-step RACH
LG Electronics

R1-2001958
Discussion on Two-step RACH LS received from RAN2
Nokia, Nokia Shanghai Bell

Withdrawn

R1-2001976
Discussion on Two-step RACH related LS received from RAN2
Nokia, Nokia Shanghai Bell

R1-2001983
Remaining details of channel structure for 2-step RACH
Intel Corporation

R1-2002064
Remaining issues on 2-step RACH channel structure
CATT

R1-2002112
Channel Structure for Two-Step RACH
Samsung

R1-2002259
Remaining issue on preamble and PRU mapping for 2-step CFRA
Spreadtrum Communications

R1-2002319
Remaining issues on channel structure for 2-step RACH
Apple

R1-2002369
Channel Structure Related Corrections For 2-Step RACH
Ericsson

R1-2002431
Maintenance for Channel Structure for Two-Step RACH
NTT DOCOMO, INC.

R1-2002526
Remaining issues and clarification on channel structure for Two-Step RACH
Qualcomm Incorporated

7.2.1.2 Procedure for Two-step RACH

R1-2001713
FL summary on the maintenance of the 2-step RACH procedures
Moderator  (ZTE)

R1-2002729
FL summary on the maintenance of 2-step RACH procedures 
Moderator  (ZTE)

R1-2001525
Corrections on procedure of 2-step RACH
Huawei, HiSilicon

R1-2001648
Remaining issues on procedure for 2-step RACH
vivo

R1-2001711
Remaining issues of the 2-step RACH procedures
ZTE, Sanechips

R1-2001767
Remaining issues for procedure of 2-step RACH
OPPO

R1-2001950
Remaining details of Procedure for 2-step RACH
LG Electronics

R1-2001959
RAN1 terminology alignment for two-step RACH
Nokia, Nokia Shanghai Bell

R1-2001984
Remaining details of procedure for 2-step RACH
Intel Corporation

R1-2002065
Remaining issues on 2-step RACH procedure
CATT

R1-2002113
Procedure for Two-step RACH
Samsung

R1-2002370
Procedure Related Corrections for 2-Step RACH
Ericsson

R1-2002432
Maintenance for Procedure for Two-step RACH
NTT DOCOMO, INC.

R1-2002527
Remaining issues on procedures for Two-Step RACH
Qualcomm Incorporated

7.2.1.3 Others

R1-2001951
Remaining details of designing 2-step RACH for NR-U
LG Electronics

R1-2002371
MsgA PUSCH Configuration in 2-Step CFRA
Ericsson

R1-2002574
Further considerations on 2-step RACH
Huawei, HiSilicon

7.2.2 Maintenance of NR-based Access to Unlicensed Spectrum
7.2.2.1 Physical Layer Signals and Channels

7.2.2.1.1 Initial access signals/channels 

R1-2001649
Remaining issues on initial access signals and channles
vivo

R1-2001702
Remaining issues on the initial access signals for NR-U
ZTE, Sanechips

R1-2001756
Discussion on the remaining issues of initial access signal/channel
OPPO

R1-2001932
Remaining issues of initial access signals and channels for NR-U
LG Electronics

R1-2002028
Initial access signals and channels
Ericsson

R1-2002114
Initial access signals and channels for NR-U
Samsung

R1-2002224
Remaining issues on Initial Access Signals and Channels for NR-U
Nokia, Nokia Shanghai Bell

R1-2002262
Remaining issues on initial access signals/channels
Spreadtrum Communications

R1-2002575
Maintainance on the initial access signals and channels
Huawei, HiSilicon

7.2.2.1.2 DL signals and channels

R1-2002685
Feature lead summary for NR-U DL Signals and Channels
Moderator (Lenovo)

R1-2001532
Maintainance on DL signals and channels
Huawei, HiSilicon

R1-2001650
Remaining issues on physical DL channel design in unlicensed spectrum
vivo

R1-2001703
Remaining issues on the DL channels for NR-U
ZTE, Sanechips

R1-2001757
Discussion on the remaining issues of DL signals and channels
OPPO

R1-2001902
Remaining issues on DL signals and channels for NR-U
MediaTek Inc.

R1-2001933
Remaining issues of DL signals and channels for NR-U
LG Electronics

R1-2001985
DL signals and channels for NR-unlicensed
Intel Corporation

R1-2002029
DL signals and channels
Ericsson

R1-2002054
Maintenance for DL signals and channels for NR-U
Panasonic Corporation

R1-2002115
DL signals and channels for NR-U
Samsung

R1-2002225
Remaining issues on DL signals and channels
Nokia, Nokia Shanghai Bell

R1-2002275
Remaining issues in DL signals and channels
Spreadtrum Communications

R1-2002320
Remaining issues of DL signals and channels
Apple

R1-2002381
Remaining issues and corrections on DL signals and channels for NR-U
Sharp

R1-2002528
TP for DL signals and channels for NR-U
Qualcomm Incorporated

R1-2002630
Text proposal for search space group switching
Google Inc.

7.2.2.1.3 UL signals and channels

R1-2002036
Feature Lead Summary for UL Signals and Channels
Ericsson

R1-2001533
Maintainance on uplink signals and channels
Huawei, HiSilicon

R1-2001651
Remaining issues on physical UL channel design in unlicensed spectrum
vivo

R1-2001704
Remaining issues on the UL channels for NR-U
ZTE, Sanechips

R1-2001758
Discussion on the remaining issues of UL signals and channels
OPPO

R1-2001875
Remaining issues on UL signals and channels for NR-U
Fujitsu

R1-2001903
Remaining issues on UL signals and channels for NR-U
MediaTek Inc.

R1-2001934
Remaining issues of UL signals and channels for NR-U
LG Electronics

R1-2001973
Remaining issues for UL signals and channels for NR-U
Lenovo, Motorola Mobility

R1-2001986
UL signals and channels for NR-unlicensed
Intel Corporation

R1-2002030
UL signals and channels
Ericsson

R1-2002075
TP for SRS configuration
CATT

R1-2002116
UL signals and channels for NR-U
Samsung

R1-2002192
Remaining Issues on UL Signals and Channels for NR-U
Nokia, Nokia Shanghai Bell

R1-2002246
UL signals and channels
ETRI

R1-2002276
Remaining issues in UL signals and channels
Spreadtrum Communications

R1-2002321
Remaining issues of UL signals and channels
Apple

R1-2002365
TPs on uplink signals in NRU
NEC

R1-2002382
Remaining issues on UL signals/channels for NR-U
Sharp

R1-2002433
Remaining issues on UL signals and channels for NR-U
NTT DOCOMO, INC.

R1-2002529
TP for UL signals and channels for NR-U
Qualcomm Incorporated

7.2.2.2 Physical Layer Procedure

7.2.2.2.1 Channel access procedures 

R1-2002284
Feature lead summary on NR-U channel access procedures
Moderator (Nokia)

R1-2001534
Maintainance on the channel access procedure
Huawei, HiSilicon

R1-2001652
Remaining issues on the channel access procedures
vivo

R1-2001705
Remaining issues on the channel access procedure for NR-U
ZTE, Sanechips

R1-2001759
Discussion on the remaining issues of channel access procedure
OPPO

R1-2001935
Remaining issues of channel access procedure for NR-U
LG Electronics

R1-2001987
Channel access mechanism for NR-unlicensed
Intel Corporation

R1-2002031
Channel access procedures
Ericsson

R1-2002117
Channel access procedures for NR-U
Samsung

R1-2002193
Remaining Issues on Channel Access Procedures for NR-U
Nokia, Nokia Shanghai Bell

R1-2002247
Remaining issues on channel access procedures for NR-U
ETRI

R1-2002383
Remaining issues and corrections on channel access procedure for NR-U
Sharp

R1-2002405
Remaining issues on channel access for NR-U operation
MediaTek Inc.

R1-2002434
Remaining issues on channel access procedures for NR-U
NTT DOCOMO, INC.

R1-2002465
TP on shared spectrum in NR-U
NEC

R1-2002530
TP for Channel access procedures for NR unlicensed
Qualcomm Incorporated

R1-2002632
Remaining issues on channel access procedure for NR-U
WILUS Inc.

R1-2002684
COT sharing information in CG-UCI
Lenovo, Motorola Mobility

7.2.2.2.2 Enhancements to initial access procedure

R1-2001701
Feature lead summary on NR-U enhancement to initial access procedures
Moderator (Charter Communications, Inc)

R1-2001535
Maintainance on the initial access procedures
Huawei, HiSilicon

R1-2001653
Remaining issues on initial access procedure for NR-U
vivo

R1-2001706
Remaining issues on the initial access procedure for NR-U
ZTE, Sanechips

R1-2001760
Discussion on the remaining issues of enhancements to initial access procedure
OPPO

R1-2001936
Remaining issues of initial access and mobility for NR-U
LG Electronics

R1-2001988
Enhancements to initial access and mobility for NR-unlicensed
Intel Corporation

R1-2002032
Enhancements to initial access procedures
Ericsson

R1-2002118
Initial access procedures for NR-U
Samsung

R1-2002248
Remaining issues on initial access procedure for NR-U
ETRI

R1-2002263
Remaining issues on initial access procedure
Spreadtrum Communications

R1-2002278
On Enhancements to Initial Access Procedures for NR-U
Nokia, Nokia Shanghai Bell

R1-2002407
Remaining issues on initial access procedure for NR-U operation
MediaTek Inc.

R1-2002531
TP for Initial access and mobility procedures for NR-U
Qualcomm Incorporated

7.2.2.2.3 HARQ enhancement

R1-2001536
Maintainance on HARQ-ACK enhancement
Huawei, HiSilicon

R1-2001654
Remaining issues on HARQ operation for NR-U
vivo

R1-2001707
Remaining issues on the HARQ for NR-U
ZTE, Sanechips

R1-2001761
Discussion on the remaining issues of HARQ enhancements
OPPO

R1-2001904
Remaining issues on HARQ operation for NR-U
MediaTek Inc.

R1-2001937
Remaining issues of HARQ procedure for NR-U
LG Electronics

R1-2001974
Remaining issues for HARQ enhancement for NR-U
Lenovo, Motorola Mobility

R1-2001989
Enhancements to HARQ for NR-unlicensed
Intel Corporation

R1-2002033
HARQ enhancement
Ericsson

R1-2002119
HARQ enhancement for NR-U
Samsung

R1-2002227
Remaining issues on NR-U HARQ scheduling and feedback
Nokia, Nokia Shanghai Bell

R1-2002249
HARQ enhancement
ETRI

R1-2002306
One shot HARQ ACK feedback
InterDigital, Inc.

R1-2002384
Remaining issues and corrections on HARQ enhancement for NR-U
Sharp

R1-2002532
TP for Enhancements to Scheduling and HARQ Operation for NR-U
Qualcomm Incorporated

R1-2002631
Text proposal for enhanced dynamic HARQ procedures
Google Inc.

7.2.2.2.4 Configured grant enhancement 
R1-2001657
Feature lead summary on NRU configured grant enhancement
Moderator (vivo)

R1-2001537
Maintainance on the configured grant procedures
Huawei, HiSilicon

R1-2001655
Remaining issues on the enhancements to configured grant
vivo

R1-2001708
Remaining issues on the configured grant for NR-U
ZTE, Sanechips

R1-2001762
Discussion on the remaining issues of configured grant enhancements
OPPO

R1-2001814
Remaining issues on configured grant enhancement for NR-U
Sony

R1-2001938
Remaining issues of configured grant for NR-U
LG Electronics

R1-2001975
Remaining issues for configured grant enhancement for NR-U
Lenovo, Motorola Mobility

R1-2001990
Enhancements to configured grants for NR-unlicensed
Intel Corporation

R1-2002034
Configured grant enhancement
Ericsson

R1-2002120
Configured grant enhancement for NR-U
Samsung

R1-2002162
Clarification on NR-U configured grant enhancement
Panasonic Corporation

R1-2002194
Remaining Issues on Configured Grant Enhancement for NR-U
Nokia, Nokia Shanghai Bell

R1-2002435
Remaining issues on configured grant enhancement for NR-U
NTT DOCOMO, INC.

R1-2002533
TP for Enhancements to configured grants for NR-U
Qualcomm Incorporated

R1-2002633
Remaining issue on configured grant for NR-U
WILUS Inc.

7.2.2.2.5 Wide-band operation

R1-2001940
Summary on maintenance of wide-band operation for NR-U
Moderator (LG Electronics)

R1-2001538
Maintainance on the wideband operation procedures
Huawei, HiSilicon

R1-2001656
Remaining issues on wideband operation in NR-U
vivo

R1-2001709
Remaining issues on the wideband operation for NR-U
ZTE, Sanechips

R1-2001763
Discussion on the remaining issues of wide-band operations
OPPO

R1-2001905
Remaining issues on wideband operation for NR-U
MediaTek Inc.

R1-2001939
Remaining issues of wide-band operation for NR-U
LG Electronics

R1-2001991
Wideband operation for NR-unlicensed
Intel Corporation

R1-2002035
Wideband operation
Ericsson

R1-2002121
Wide-band operation for NR-U
Samsung

R1-2002198
Remaining issues on Rel-16 NR-U wideband operations 
Panasonic

R1-2002226
Remaining issues on Wideband operation in NR-U
Nokia, Nokia Shanghai Bell

R1-2002277
Remaining issues in wide-band operation
Spreadtrum Communications

R1-2002322
Remaining issues of wideband operation
Apple

R1-2002385
Remaining issues on wide-band operation for NR-U
Sharp

R1-2002534
TP for Wideband operation for NR-U operation
Qualcomm Incorporated

7.2.2.3 Others

R1-2001764
Discussion on the other remaining issues
OPPO

R1-2002576
Other Maintainance for NR-U
Huawei, HiSilicon

7.2.3 Maintenance of Integrated Access and Backhaul for NR (4)
[100b-e-NR-IAB-01] Email approval regarding the following TP for section 6.3.3.2 of 38.211:

	<Unchanged text is omitted>

For the IAB-MT part of an IAB node, the following applies:

<Unchanged text is omitted>

-     if the higher-layer parameter prach-ConfigurationSOffset is configured, the subframe number sn sn  from in Tables 6.3.3.2-2 to 6.3.3.2-3 and the slot number from in Table 6.3.3.2-4 shall be replaced by  (sn +(s) mod L where sn is the slot or subframe number, [image: image5.png]


(s({0, 1, …, L-1} is given by the higher-layer parameter prach-ConfigurationSOffset,  and  is the number of subframes in a frame when using in Tables 6.3.3.2-2 to 6.3.3.2-3 and the number of slots in a frame for SCS of 60 KHz when using in Table 6.3.3.2-4.
<Unchanged text is omitted>


Till 4/22 –(Qualcomm, Luca)
[100b-e-NR-IAB-02] Email discussion/approval regarding IAB-MT Resource/Control Channel Configuration

· Usage of tdd-UL-DL-ConfigDedicated-IAB-MT 

· IAB-MT Common Search Space 

· IAB-MT Specific Search Space 

· Max AI DCI Payload Size 

By 4/24, with potential TP/LS by 4/29 (ATT, Thomas)
[100b-e-NR-IAB-03] Email discussion/approval regarding IAB Operation in Paired Spectrum

· DU Resource Configuration 

· Soft resource availability indication in paired spectrum 

By 4/24, with potential TP/LS by 4/29 (ATT, Thomas)
[100b-e-NR-IAB-04] Email discussion w.r.t. LS in R1-2001510, included a potential reply LS and TP, covering
· #1: Whether/how the T_delta mapping depends on FR (granularity) and/or SCS (range), including how to make IAB node and its parent to use the same FR and the same SCS in the mapping.

· #2: How to deal with Rel-15 UE behavior of NTA rounding in the context of IAB case-1 timing.  

With LS dissucssion/approval till 4/24 and potential TPs till 4/29. (ZTE, Wenfeng)
7.2.3.1 Extensions of SSBs for inter-IAB-node discovery and measurements
7.2.3.2 Extensions of RACH occasions and periodicities for backhaul RACH resources

R1-2001874
Remaining issues in IAB PRACH
ZTE, Sanechips

7.2.3.3 Mechanisms for resource multiplexing among backhaul and access links
R1-2001863
Summary on Mechanisms for resource multiplexing among backhaul and access links
Moderator (AT&T)

R1-2001526
Remaining issues on resource multiplexing among backhaul and access links
Huawei, HiSilicon

R1-2001658
Remaining issue on mechanisms for resource multiplexing among backhaul and access links


vivo

R1-2001862
Remaining maintenance issues for IAB resource multiplexing
AT&T

R1-2001882
Remaining issues in IAB resource multiplexing
ZTE, Sanechips

R1-2001952
Remaining details on IAB resource multiplexing
LG Electronics

R1-2002203
Remaining issues on mechanisms for resource multiplexing among backhaul and access links


CMCC

R1-2002535
Remaining issues and clarifications on IAB resource management
Qualcomm Incorporated

R1-2002650
Definition of IAB half-duplex operation in paired spectrum
Ericsson

7.2.3.4 Mechanism to support the “case-1” OTA timing alignment
R1-2001893
FL summary on remaining issues in IAB case-1 timing
Moderator (ZTE)

R1-2001527
Remaining issues on the "case-1" OTA timing alignment
Huawei, HiSilicon

R1-2001792
Specifications for T_delta signalling
Nokia, Nokia Shanghai Bell

R1-2001883
Remaining issues in IAB case-1 timing
ZTE, Sanechips

R1-2001953
Text proposals on case 1 timing alignment
LG Electronics

R1-2002436
Discussion on the RAN2 LS on T_delta in IAB
NTT DOCOMO, INC

R1-2002536
Remaining issues on OTA timing
Qualcomm Incorporated

R1-2002651
On T_delta Signaling in MAC CE
Ericsson

7.2.3.5 Others

R1-2002652
On max number of commonSearchSpaceListIAB
Ericsson

7.2.4 Maintenance for 5G V2X with NR sidelink (1+4+11+4+1+4+2+1=28)
R1-2002053
TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”, v1.2.0
Huawei

[100b-e-NR-5G_V2X_NRSL-37.985] Email approval of R1-2002053 till 04/22 (Huawei, Matthew)
7.2.4.1 Physical layer structure for sidelink (4)
R1-2002123
Feature lead summary#1 for agenda item 7.2.4.1 Physical layer structure for sidelink
Moderator (Samsung)

R1-2002124
Feature lead summary#2 for agenda item 7.2.4.1 Physical layer structure for sidelink
Moderator (Samsung)

[100b-e-NR-5G_V2X_NRSL-PHYstructure-01] Email discussion/approval regarding TBS determination for PSSCH 
   - A. How to deal with PSFCH overhead
   - B. How to deal with PSSCH DMRS overhead
   - C. How to deal with 2nd SCI overhead
   - D. How to deal with CSI-RS/PT-RS
   - E. Whether/how to use indication of sl_xOverhead
till 4/27, with potential TP till 4/30 – Jeongho (SS)
 

[100b-e-NR-5G_V2X_NRSL-PHYstructure-02] Email discussion/approval on resource pool configuration - subchannel size and resource pool size

   - A. Whether/how to introduce additional subchannel size and whether the number of PRBs configured for a resource pool should be integer multiple of sub-channel size     
 till 4/24, with potential TP till 4/29 – Jeongho (SS)
[100b-e-NR-5G_V2X_NRSL-PHYstructure-03] Email discussion/approval regarding signaling of resource pool
   - A. Periodicity of resource pool bitmap, length of the bitmap, excluded slots, reserved slots

till 4/24, with potential TP till 4/29 – Jeongho (SS)
[100b-e-NR-5G_V2X_NRSL-PHYstructure-04] Email discussion/approval regarding SCI related 
   - A. Indication of MCS tables (if necessary) and MCS table determination 

till 4/24, with potential TP till 4/29 – Jeongho (SS)
R1-2001550
Remaining details of sidelink physical layer structure
Huawei, HiSilicon

R1-2001577
Remaining issues of NR sidelink physical layer structure
ZTE, Sanechips

R1-2001659
Remaining issues on physical layer structure for NR sidelink
vivo

R1-2001745
Draft text proposals on physical layer structure for NR-V2X
OPPO

R1-2001803
Remaining details of Physical layer structure for sidelink
Nokia, Nokia Shanghai Bell

R1-2001815
Remaining issues on Physical layer structure for sidelink
Sony

R1-2001846
Discussion on sidelink physical layer structure
MediaTek Inc.

R1-2001884
Discussion on physical layer structure for NR sidelink
LG Electronics

R1-2001962
Physical layer structure for sidelink
TCL Communication Ltd.

R1-2001967
Discussion on physical layer structure for NR sidelink
Lenovo, Motorola Mobility

R1-2001977
Remaining Issues in Physical Layer Structures for NR V2X
Fraunhofer HHI, Fraunhofer IIS

R1-2001992
Solutions to remaining opens of physical structure for NR V2X sidelink design
Intel Corporation

R1-2002039
Remaining details on physical layer structure for the sidelink
Futurewei

R1-2002076
Remaining issues on physical layer structure for NR sidelink
CATT

R1-2002689
On Physical Layer Structures for NR Sidelink
Samsung

Revision of R1-2002122


R1-2002204
Remaining issues on physical layer structure for sidelink
CMCC

R1-2002232
Physical layer structure for NR sidelink
Ericsson

R1-2002265
Remaining issues of physical layer structure for sidelink
Spreadtrum Communications

R1-2002299
Remaining Issues on Physical Layer Structure for NR V2X
InterDigital, Inc.

R1-2002323
On Remaining Details of NR V2X Physical Layer Structure
Apple

R1-2002358
Remaining issue on physical layer structure for sidelink in NR V2X
Panasonic Corporation

R1-2002361
Remaining issues on physical layer structure
NEC

R1-2002386
Remaining issues on physical layer structure for NR sidelink
Sharp

R1-2002400
Discussion on SCI content for 5G V2X
Xiaomi Communications

R1-2002437
Remaining issues on sidelink physical layer structure
NTT DOCOMO, INC.

R1-2002537
Considerations on Physical Layer aspects of NR V2X
Qualcomm Incorporated

R1-2002627
Remaining issues of V2X PHY layer structure
Mitsubishi Electric RCE

7.2.4.2 Resource allocation for NR sidelink (11)
7.2.4.2.1 Mode 1 (5)
R1-2002244
Feature lead summary#1 on Resource allocation for NR sidelink Mode 1
Moderator (Ericsson)

R1-2002245
Feature lead summary#2 on Resource allocation for NR sidelink Mode 1
Moderator (Ericsson)

[100b-e-NR-5G_V2X_NRSL-Mode-1-01] Email discussiona/approval w.r.t. issues for CG:
· Determination of slots for sidelink transmission for configured grant type 1

· Including LTE Uu scheduling NR SL

· Remaining details on HPN determination, including whether there is an issue with collision of HPNs and, if so, how to solve it.

· Largest number of periods that the transmission of a TB can span

· Remaining details on periodicity and configuration of (re-)transmission within a period, including values in RRC spec.

till 4/24, with potential TPs till 4/29 (Ericsson, Ricardo)
[100b-e-NR-5G_V2X_NRSL-Mode-1-02] Email discussion/approval on some aspects for HARQ reporting to gNB:
· Determination of the UL slot used for SL HARQ-ACK reporting to the gNB.
· Including how to ensure sufficient PSFCH-PUCCH processing time, any restrictions or assumptions on offset between SL and Uu timing in mode-1，potential scheduling collision in async case

· The two WAs from RAN1#100-e, including confirming/revoking them and subsequent details.

· Termination of transmission (including early termination).

till 4/24, with potential TPs till 4/29 (Ericsson, Ricardo)
[100b-e-NR-5G_V2X_NRSL-Mode-1-03] Email discussion/approval on some aspects of reports on PUCCH for type 1 codebook: 
· Required changes to the Rel-15 procedures (as agreed) – TS 38.213 Section 9.1.2.1:

· Identify the parameters that are applicable for sidelink and which parameters are not, including: PDSCH occasion, K_1 values, dl-DataToUL-ACK, set of row indices R, ratio between SCS, TDD configuration, number of cells, etc.

· Changes to the determination of the set of occasion for candidate PDSCH receptions (first pseudo code block).

till 4/24, with potential TPs till 4/29 (Ericsson, Ricardo)
[100b-e-NR-5G_V2X_NRSL-Mode-1-04] Email discussion/approval on some aspects of reports on PUCCH for type 2 codebook: 
· Required changes to the Rel-15 procedures (as agreed) – TS 38.213 Section 9.1.3.1:

· Identify the parameters that are applicable for sidelink and which parameters are not, including PDSCH-to-HARQ_feedback timing indicator, K0, PSFCH resource periodicity N, number of cells, etc.

· Support of sidelink assignment indices (SAI): cSAI and/or tSAI

· Changes to the determination of the HARQ-ACK information bits 
[image: image6.wmf]ACK

O

ACK

ACK

ACK

o

o

o

1

1

0

~

,...,

~

 

,

~

-


till 4/24, with potential TPs till 4/29 (Ericsson, Ricardo)
[100b-e-NR-5G_V2X_NRSL-Mode-1-05] Email approval for the TPs relaed to Q2 in R1-2001367
· The TP in R1-2001410 targeted TS 38.213 but it should have targeted TS 38.214 (changing SCI format 0_1 to SCI format 0_2)

till 4/22 (Ericsson, Ricardo)
R1-2001551
Remaining details of sidelink resource allocation mode 1
Huawei, HiSilicon

R1-2001660
Remaining issues on mode 1 resource allocation mechanism
vivo

R1-2001746
Remaining issues of mode 1 resource allocation for NR-V2X
OPPO

R1-2001804
Remaining details of Resource allocation for sidelink - Mode 1
Nokia, Nokia Shanghai Bell

R1-2001876
Remaining issues on configured grant for NR V2X
Fujitsu

R1-2001885
Discussion on resource allocation for Mode 1
LG Electronics

R1-2001895
Remaining issues of mode 1 operation on sidelink
ZTE, Sanechips

R1-2001906
Sidelink mode-1 resource allocation
MediaTek Inc.

R1-2001963
Resource allocation for NR sidelink Mode 1
TCL Communication Ltd.

R1-2001968
Discussion on resource allocation for NR sidelink Mode 1
Lenovo, Motorola Mobility

R1-2001993
Solutions to remaining opens of resource allocation mode-1 for NR V2X sidelink design
Intel Corporation

R1-2002040
Remaining details on mode-1 resource allocation
Futurewei

R1-2002077
Remaining issues on  Mode 1 resource allocation in NR V2X
CATT

R1-2002125
On Mode 1 for NR Sidelink
Samsung

R1-2002184
Mode-1 resource allocation for NR V2X
ITL

R1-2002205
Remaining issues on mode 1 resource allocation mechanism
CMCC

R1-2002233
Resource Allocation Mode 1 for NR SL
Ericsson

R1-2002266
Remaining issues in NR sidelink mode 1 resource allocation
Spreadtrum Communications

R1-2002300
Remaining Issues on NR Sidelink Mode 1 Resource Allocation
InterDigital, Inc.

R1-2002324
On Remaining Details of Mode 1 Resource Allocation
Apple

R1-2002377
On Resource Allocation Mode 1 for NR V2X
Convida Wireless

R1-2002387
Remaining issues on resource allocation mode 1 for NR sidelink
Sharp

R1-2002401
On Sidelink HARQ report of V2x communications
Xiaomi Communications

R1-2002438
Remaining issues on resource allocation mechanism mode 1
NTT DOCOMO, INC.

R1-2002466
Remaining issues on sidelink resource allocation mode 1
ASUSTeK

R1-2002538
Sidelink Resource Allocation Mode 1
Qualcomm Incorporated

7.2.4.2.2 Mode 2 (6)
[100b-e-NR-5G_V2X_NRSL-Mode-2-01] Email discussion/approval w.r.t. re-evaulation including aspects:

· Whether/how to ensure the timing restrictions in re-evaluation, including potential change of pre-selected resources

· Whether to mandate every slot re-evaluation

till 4/27, with potential TPs till 4/30 (Intel, Sergey)
[100b-e-NR-5G_V2X_NRSL-Mode-2-02] Email discussion/approval w.r.t. pre-emption including aspects:

· Finalization of the RRC parameter for pre-emption configuration per resource pool (still TBD in the RRC list)

· Relation of pre-emption RSRP threshold and Step 1 checking

· Which resources can be re-selected after pre-emption condition – only ones to be transmitted or to be signalled

till 4/27, with potential TPs till 4/30 (Intel, Sergey)
[100b-e-NR-5G_V2X_NRSL-Mode-2-03] Email discussion/approval w.r.t. step2 including aspects:

· Whether to support HARQ retransmissions on unreserved resources

· Early in time initial resource selection

· Relation of selected number of resources and signalled number of resources, including number of outstanding reservations

till 4/27, with potential TPs till 4/30 (Intel, Sergey)
[100b-e-NR-5G_V2X_NRSL-Mode-2-04] Email discussion/approval w.r.t. periodic reservation including aspects:

· Backward signalling option

· Periods for exclusion if a slot is not monitored in a sensing window

till 4/24, with potential TPs till 4/29 (Intel, Sergey)
[100b-e-NR-5G_V2X_NRSL-Mode-2-05] Email approval of TP capturing previous meeting’s agreement regadingTime and frequency resource indication based on the submitted contributions till 4/22 (Intel, Sergey)
[100b-e-NR-5G_V2X_NRSL-Mode-2-06] Email approval of TPs based on the following contributions:
a) PSSCH DMRS

· Alignment correction: ZTE/Sanechips, NEC

b) Capturing exclusion of TTIs in the same period / aperiodic reservations

· NTT DOCOMO, ASUSTeK, Qualcomm

c) Configurable T2min parameter value should actually be T2min - T1

· Ericsson 

d) L1 priority for pre-emption, not L2 priority 

· Ericsson

till 4/23 (Intel, Sergey)
R1-2001552
Remaining details of sidelink resource allocation mode 2
Huawei, HiSilicon

R1-2001661
Remaining issues on mode 2 resource allocation mechanism
vivo

R1-2001749
Discussion on remaining open issue for mode 2
OPPO

R1-2001793
Remaining Issues on Sidelink Mode 2 Resource Allocation
Panasonic Corporation

R1-2001805
Remaining details of Resource allocation for sidelink - Mode 2
Nokia, Nokia Shanghai Bell

R1-2001877
Remaining details on mode 2 resource allocation for NR V2X
Fujitsu

R1-2001886
Discussion on resource allocation for Mode 2
LG Electronics

R1-2001896
Remaining issues of mode 2 operation on sidelink
ZTE, Sanechips

R1-2001907
Sidelink mode-2 resource allocation
MediaTek Inc.

R1-2001964
Resource allocation for NR sidelink Mode 2
TCL Communication Ltd.

R1-2001969
Discussion on resource allocation for NR sidelink Mode 2
Lenovo, Motorola Mobility

R1-2001978
Remaining Issues in Resource Allocation for Mode 2 NR V2X
Fraunhofer HHI, Fraunhofer IIS

R1-2001994
Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design
Intel Corporation

R1-2002041
Remianing details on mode-2 resource allocation
Futurewei

R1-2002078
Remaining issues on Mode 2 resource allocation in NR V2X
CATT

R1-2002126
On Mode 2 for NR Sidelink
Samsung

R1-2002234
Resource allocation Mode 2 for NR SL
Ericsson

R1-2002267
Remaining issues in NR sidelink mode 2 resource allocation
Spreadtrum Communications

R1-2002301
Remaining Issues on NR Sidelink Mode 2 Resource Allocation
InterDigital, Inc.

R1-2002325
On Remaining Details of Mode 2 Resource Allocation
Apple

R1-2002362
Remaining issues on resource allocation Mode 2
NEC

R1-2002388
Remaining issues on resource allocation mode 2 for NR sidelink
Sharp

R1-2002402
On resource reservation in Mode 2 resource allocation
Xiaomi Communications

R1-2002439
Remaining issues on resource allocation mechanism mode 2
NTT DOCOMO, INC.

R1-2002487
Remain details on mode-2 resource allocation for NR V2X
ITL

R1-2002489
Remaining issue for Mode 2 resource allocation in NR V2X
ASUSTeK

R1-2002539
Sidelink Resource Allocation Mechanism for NR V2X
Qualcomm Incorporated

7.2.4.2.3 Others

R1-2001662
Remaining issues for resource allocation
vivo

R1-2001750
On other aspects of mode 2 resource allocation
OPPO

R1-2002235
Other outstanding resource allocation issues
Ericsson

R1-2002585
Remaining details of pre-emption
Huawei, HiSilicon

7.2.4.3 Sidelink synchronization mechanism (4)
R1-2002395
Feature lead summary#1 on AI 7.2.4.3 Sidelink synchronization mechanism
CATT

R1-2002396
Feature lead summary#2 on AI 7.2.4.3 Sidelink synchronization mechanism
CATT

R1-2002397
Feature lead summary#3 on AI 7.2.4.3 Sidelink synchronization mechanism
CATT

[100b-e-NR-5G_V2X_NRSL-SYNC-01] Email discussion/approval related to PSBCH contents - Indication of TDD configuration (a,k.a. issue 1-1) by 4/24, with potential TPs by 4/29 (CATT, Teng)
[100b-e-NR-5G_V2X_NRSL-SYNC-02] Email discussion/approval related to DM-RS sequence initialization for PSBCH and QCL for S-SSB (a,k.a. issues 2 and 3) by 4/24, with potential TPs by 4/29 (CATT, Teng)
[100b-e-NR-5G_V2X_NRSL-SYNC-03] Email discussion/approval related to sync procedure - SL SSIDs/sync resources for each priority and Lower SLSS ID with higher priority for P6/P6’ UE (a,k.a. issues 4-1 & 4-2) by 4/24, with potential TPs by 4/29 (CATT, Teng)
[100b-e-NR-5G_V2X_NRSL-SYNC-04] Email discussion/approval related to

· Slot number/sidelink timing derived from GNSS

· Resource sets for S-SSB transmission

· Timing determination of S-SSB

 (a,k.a. issues 5,6,7) by 4/24, with potential TPs by 4/29 (CATT, Teng)
R1-2001553
Remaining details of sidelink synchronization mechanisms
Huawei, HiSilicon

R1-2001578
Remaining issues of Synchronization
ZTE, Sanechips

R1-2001663
Remaining issues on sidelink synchronization mechanism
vivo

R1-2001747
Remaining issues of synchronization for NR-V2X
OPPO

R1-2001806
Remaining details of Sidelink synchronization mechanism
Nokia, Nokia Shanghai Bell

R1-2001847
Discussion on sidelink based synchronization mechanism
MediaTek Inc.

R1-2001878
Remaining issues on sidelink synchronization procedure
Fujitsu

R1-2001887
Discussion on NR sidelink synchronization mechanism
LG Electronics

R1-2001995
Solutions to remaining opens of sidelink synchronization for NR V2X design
Intel Corporation

R1-2002042
Remaining details on synchronization for sidelink
Futurewei

R1-2002079
Remaining issues on sidelink synchronization mechanism in NR V2X
CATT

R1-2002127
On Synchronization Mechanisms for NR Sidelink
Samsung

R1-2002206
Remaining issues on sidelink synchronization mechanisms
CMCC

R1-2002236
Synchronization mechanism for NR SL
Ericsson

R1-2002268
Remaining issues in synchronization mechanism for NR V2X
Spreadtrum Communications

R1-2002326
On PSBCH TDD Configuration Indication
Apple

R1-2002389
Remaining issues on synchronization mechanism for NR sidelink
Sharp

R1-2002440
Remaining issues on sidelink synchronization mechanism
NTT DOCOMO, INC.

R1-2002540
Synchronization mechanism enhancment for cluster merge
Qualcomm Incorporated

7.2.4.4 In-device coexistence between LTE and NR sidelinks (1)
R1-2002732
FL Summary #2 of In-device Coexistence Aspects in NR-V2X

Moderator (Qualcomm)

[100b-e-NR-5G_V2X_NRSL-InDevice-Coex-01] Email approval for TPs capturing the previous agreements regarding the following issues:
· PSFCH priority is set to the priority of the corresponding PSSCH (Issue 2 in the summary in R1-2002732).
· Prioritization is performed across RATs, not within a RAT, for in-device coexistence (Issue 2 in the summary in R1-2002732).
· When multiple transmissions using NR sidelink are overlapped with transmission/reception using LTE sidelink, and the priorities of multiple transmissions are different, the highest priority of multiple transmissions using NR sidelink is used for compared with transmission/reception using LTE sidelink (Issue 9 in the summary in R1-2002732)
· Behavior is left up to UE implementation if packet priority of LTE SL and/or NR SL is not known sufficiently in advance (Issue 2 in the summary in R1-2002732).
· Note: A TP will be proposed for this point only if needed to resolve any ambiguity after preparing the other TPs.
by 4/27 (Gabi, Qualcomm)
R1-2001554
Remaining details of in-device coexistence between LTE and NR sidelinks
Huawei, HiSilicon

R1-2001664
Remaining issues on In-device coexistence between NR and LTE sidelinks
vivo

R1-2001751
Remaining open issues on in-device coexistence
OPPO

R1-2001807
Remaining details of In-device coexistence between LTE and NR sidelinks
Nokia, Nokia Shanghai Bell

R1-2001888
Discussion on in-device coexistence between LTE and NR sidelinks
LG Electronics

R1-2001897
Remaining issues of in-device coexistence
ZTE, Sanechips

R1-2002050
Remaining details on in-device coexistence between LTE and NR sidelinks
Futurewei

R1-2002128
On In-device Coexistence between LTE and NR Sidelink
Samsung

R1-2002237
In-device coexistence between LTE and NR sidelinks
Ericsson

R1-2002541
Corrections for in-device coexistence between LTE and NR sidelink
Qualcomm Incorporated

7.2.4.5 Physical layer procedures for sidelink (4)
R1-2001892
Feature lead summary#1 for AI 7.2.4.5 Physical layer procedures for sidelink
Moderator (LG Electronics)

[100b-e-NR-5G_V2X_NRSL-PHY-Procedure-01] Email discussion/approval regarding handling TX and RX of multiple PSFCHs
· PSD of each PSFCH when transmitting multiple PSFCH TX
· Prioritization between TX and RX when the UE is required to TX/RX multiple PSFCH

till 4/23, with potential TPs by 4/28 (Hanbyul, LGE)
[100b-e-NR-5G_V2X_NRSL-PHY-Procedure-02] Email discussion/approval regarding SL/UL prioritization and UL/SL power sharing
· Prioritization in the cases mentioned in RAN2 LS (R1-2000161), i.e., “how to handle all other physical channels in UL/SL prioritization”

· Prioritization between UL TX and SL TX in case of simultaneous TXs of UL and SL across difference carriers

till 4/23, with potential TPs by 4/28 (Hanbyul, LGE)
[100b-e-NR-5G_V2X_NRSL-PHY-Procedure-03] Email discussion/approval regarding indicating SL HARQ feedback related information
· How to indicate HARQ feedback Option to RX UE
· How to indicate whether SL HARQ feedback is enabled or disabled to RX UE
· Whether to support mixing blind and feedback-based retransmissions of a TB

till 4/24, with potential TPs by 4/29 (Hanbyul, LGE)
[100b-e-NR-5G_V2X_NRSL-PHY-Procedure-04] Email discussion/approval regarding CSI reporting latency bound and association with CSI trigger
· Introduction and time/frequency location of SL CSI reference resource
· How to determine the latency bound of SL CSI reporting
· Whether/how to associate the reported CSI with the CSI trigger
till 4/24, with potential TPs by 4/29 (Hanbyul, LGE)
R1-2001555
Remaining details of physical layer procedures for sidelink
Huawei, HiSilicon

R1-2001665
Remaining issues on physical layer procedure for NR sidelink
vivo

R1-2001748
Remaining issues of physical layer procedure for NR-V2X
OPPO

R1-2001808
Remaining details of Physical layer procedures for sidelink
Nokia, Nokia Shanghai Bell

R1-2001879
Remaining issues on power reduction rule for NR V2X
Fujitsu

R1-2001889
Discussion on physical layer procedure for NR sidelink
LG Electronics

R1-2001898
Remaining issues on PHY procedures for Rel-16 sidelink
ZTE, Sanechips

R1-2001965
Physical layer procedures for sidelink
TCL Communication Ltd.

R1-2001970
Discussion on physical layer procedures for NR sidelink
Lenovo, Motorola Mobility

R1-2001979
Remaining Issues in Physical Layer Procedures for NR V2X
Fraunhofer HHI, Fraunhofer IIS

R1-2001996
Solutions to remaining opens of physical layer procedures for NR V2X sidelink design
Intel Corporation

R1-2002043
Remaining details on physical procedures for sidelink
Futurewei

R1-2002080
Remaining issues on physical layer procedures for NR V2X
CATT

R1-2002129
On Physical Layer Procedures for NR Sidelink
Samsung

R1-2002207
Remaining issues on physical layer procedures for sidelink
CMCC

R1-2002238
Physical layer procedures for NR sidelink
Ericsson

R1-2002269
Remaining issues in physical layer procedures for sidelink
Spreadtrum Communications

R1-2002302
Remaining Issues on Physical Layer Procedures for NR V2X
InterDigital, Inc.

R1-2002327
On Remaining Details of Physical Layer Procedures for NR V2X Sidelink
Apple

R1-2002359
Remaining issue on physical layer procedures for sidelink in NR V2X 
Panasonic Corporation

R1-2002363
Remaining issues on physical layer procedure
NEC

R1-2002390
Remaining issues on physical layer procedures for NR sidelink
Sharp

R1-2002441
Remaining issues on sidelink physical layer procedure
NTT DOCOMO, INC.

R1-2002473
Remaining issues on sidelink physical layer procedure on NR V2X
ASUSTeK

R1-2002542
Physical layer procedures for sidelink
Qualcomm Incorporated

7.2.4.6 QoS management for sidelink (2)
R1-2002253
Summary#1 for AI 7.2.4.6 QoS management for sidelink
Moderator (Nokia)

R1-2002254
Summary#2 for AI 7.2.4.6 QoS management for sidelink
Moderator (Nokia)

[100b-e-NR-5G_V2X_NRSL-QoS-01] Email discussion/approval on how to meet CR limits – specify or leave it up to UE implementation? till 4/23 and if a TP is necessary, TP approval 4/28 (Nokia, Torsten)
[100b-e-NR-5G_V2X_NRSL-QoS-02] Email discussion/approval on TX Parameter restriction based on speed” till 4/24 and if a TP is necessary, TP approval 4/29 (Nokia, Torsten)
R1-2001556
Remaining details of QoS management for NR sidelink
Huawei, HiSilicon

R1-2001666
Remaining issues on QoS management for sidelink
vivo

R1-2001752
Remaining open issues on QoS
OPPO

R1-2001809
Remaining details of QoS management for sidelink
Nokia, Nokia Shanghai Bell

R1-2001890
Discussion on QoS management for NR sidelink
LG Electronics

R1-2001899
Remaining issues on QoS
ZTE, Sanechips

R1-2001997
Remaining details of QoS and congestion control for NR V2X design
Intel Corporation

R1-2002044
Remaining details of QoS for sidelink
Futurewei

R1-2002081
Remaining issues on QoS management in NR V2X
CATT

R1-2002130
On QoS Management for NR Sidelink
Samsung

R1-2002239
QoS management for NR sidelink
Ericsson

R1-2002303
Remaining Issues on Congestion control and QoS Management for NR-V2X
InterDigital, Inc.

R1-2002328
On Remaining Details of Sidelink Channel Occupancy Ratio
Apple

7.2.4.7 Support of NR Uu controlling LTE sidelink (1)
R1-2002707
FL summary #1 on NR Uu scheduling LTE sidelink
Moderator (FUTUREWEI)
[100b-e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-01] Email discussion/approval on issue 5 for DL path loss only: whether open loop power control should be supported on the sidelink when NR Uu schedules LTE sidelink, till 4/23; if a TP is necessary, TP approval till 4/28 (Futurewei, Phillippe)
R1-2001557
Remaining details of NR Uu control for LTE sidelink
Huawei, HiSilicon

R1-2001667
Remaining issues on support of NR Uu controlling LTE sidelink
vivo

R1-2001810
Remaining details of Support of NR Uu controlling LTE sidelink
Nokia, Nokia Shanghai Bell

R1-2001900
Remaining issues on NR Uu control LTE sidelink
ZTE, Sanechips

R1-2002240
NR UU controlling LTE sidelink transmissions
Ericsson

7.2.4.8 Others

R1-2001668
Remaining aspects for NR V2X
vivo

R1-2001754
Discussion on priority setting for higher layer contents
OPPO

R1-2002241
Remaining details on resource pool determination
Ericsson

R1-2002586
Remaining details of sidelink RRC Parameter List
Huawei, HiSilicon

R1-2002543
Syncronisation details for NR-V2X
Qualcomm Incorporated

7.2.5 Maintenance of Physical Layer Enhancements for NR URLLC (3+3+4+2+3+2+3=20)
Including maintenance for IIoT
7.2.5.1 PDCCH enhancements (3)
R1-2002688
Feature lead summary#1 on PDCCH enhancements
Moderator (Huawei)
[100b-e-NR-L1enh-URLLC-PDCCH enhancements-01] Email discussion/approval on remaining issues on enhanced PDCCH monitoring capability: 

· The per-CC limit on the maximum number of non-overlapping CCEs per monitoring span for (2, 2) and (4, 3)

· The per-CC limit on the maximum number of monitored PDCCH candidates per monitoring span

· Capability on the number of CCs with Rel-16 monitoring capability

by 4/24; if there is a spec impact, endorsing the corresponding TP by 4/29
[100b-e-NR-L1enh-URLLC-PDCCH enhancements-02] Email discussion/approval on remaining issues on scaling PDCCH monitoring capability: 

· Definition of “aligned spans” and “non-aligned spans” 

· Scale the monitoring capability for “non-aligned spans” case 

by 4/24; if there is a spec impact, endorsing the corresponding TP by 4/29
[100b-e-NR-L1enh-URLLC-PDCCH enhancements-03] Email discussion/approval on remaining issues on PDCCH overbooking/dropping and corrections on DCI format 0_2/1_2: 

· Span(s) for PDCCH overbooking/dropping

· How to perform PDCCH dropping in a span 

· Corrections on DCI format 0_2/1_2 
by 4/24; if there is a spec impact, endorsing the corresponding TP by 4/29
R1-2001545
Corrections on PDCCH enhancement for URLLC
Huawei, HiSilicon

R1-2001611
Remaining issues on PDCCH enhancements for NR URLLC
ZTE

R1-2001669
PDCCH enhancements for URLLC
vivo

R1-2001694
Maintenance of Rel-16 URLLC PDCCH enhancements
Nokia, Nokia Shanghai Bell

R1-2001773
PDCCH enhancements for URLLC
OPPO

R1-2001784
Remaining Issue of PDCCH Enhancements for NR URLLC
Ericsson

R1-2001813
Remaining Issues on PDCCH Enhancements for URLLC
Quectel

R1-2001839
Remaining issues on PDCCH enhancements
MediaTek Inc.

R1-2001919
Remaining issues of PDCCH enhancements for NR URLLC
LG Electronics

R1-2001998
Remaining issues on PDCCH enhancements for URLLC
Intel Corporation

R1-2002082
Remaining issues on PDCCH enhancements
CATT

R1-2002131
Remaining issues for PDCCH enhancements
Samsung

R1-2002255
Remaining issues of PDCCH enhancements for URLLC
Spreadtrum Communications

R1-2002329
Remaining Issues on PDCCH Enhancements for eURLLC
Apple

R1-2002360
Remaining issues on PDCCH enhancements for NR URLLC 
Panasonic Corporation

R1-2002391
Remaining issues on PDCCH enhancements for NR URLLC
Sharp

R1-2002408
Remaining details of PDCCH Enhancements
Motorola Mobility, Lenovo

R1-2002442
Remaining issues for PDCCH enhancements for Rel.16 URLLC
NTT DOCOMO, INC.

R1-2002484
Remaining issue for TCI field
ASUSTeK

R1-2002544
Remaining issues on PDCCH Enhancements for URLLC
Qualcomm Incorporated

R1-2002634
Remaining issues on PDCCH enhancement for NR URLLC
WILUS Inc.

R1-2002655
DCI size alignment
Futurewei

7.2.5.2 UCI enhancements (3)
[100b-e-NR-L1enh-URLLC-UCI_Enh-01] Email discussion/approval on interaction between SR, CSI configuration and PUCCH-Config, incl.
· Interaction between SR priority and PUCCH-Config priority (3.3.1)

· Configurable PUCCH resource length for SR and CSI (3.3.2)

· Sub-slot length for ECP (2.3.3) -- Seems agreeable already. Quickly check and agree.

Till 4/24, and potential TP for endorsement by 4/29 

[100b-e-NR-L1enh-URLLC-UCI_Enh-02]: Email discussion/approval on PUCCH configuration related to DCI format/bitfield, incl.
· PHY priority in some cases (i.e. both DCI format 0_1/1_1 and 0_2/1_2 are configured, priority field is not configured, only one PUCCH-config is configured) (4.3.1, 4,3.2, 3.4.2)

· HARQ-ACK codebook construction related to DCI format 1_1/1_2 (for Type1 ) and DAI bit width (for Type 2) (2.3.1a, 2.3.2)

· PUCCH resource determination for reduced size of PRI field (6.1.1)

· Correction related to [intraUEPrioritization] (4.4.1) -- -- Quickly check if agreeable. No long-time discussions.

 till 4/27, and potential TP for endorsement by 4/30 

[100b-e-NR-L1enh-URLLC-UCI_Enh-03] Email discussion/approval on intra-UE cancellation timeline (and order if related) till 4/23, and potential TP for endorsement by 4/28 

R1-2001548
Corrections on UCI enhancement
Huawei, HiSilicon

R1-2001607
Discussion on remaining issues on UCI enhancement for URLLC
Beijing Xiaomi Mobile Software

R1-2001612
Remaining issues on UL control enhancements for NR URLLC
ZTE

R1-2001670
UCI enhancements for URLLC
vivo

R1-2001695
Maintenance of Rel-16 URLLC UCI enhancements
Nokia, Nokia Shanghai Bell

R1-2001774
UCI enhancements for URLLC
OPPO

R1-2001785
Remaining Issue of UCI Enhancements for NR URLLC
Ericsson

R1-2001840
Remaining issues on UCI enhancements
MediaTek Inc.

R1-2001871
Remaining issue on UCI enhancement
Panasonic Corporation

R1-2001880
Remaining issues on UCI enhancements for URLLC
Fujitsu

R1-2001920
Remaining issues of UCI enhancements for NR URLLC
LG Electronics

R1-2001999
Remaining issues on UCI enhancements for URLLC
Intel Corporation

R1-2002083
Remaining issues on UCI enhancements
CATT

R1-2002132
Remaining issues for UCI enhancements
Samsung

R1-2002208
Remaining issues on UCI enhancements for URLLC
CMCC

R1-2002250
UCI enhancements
ETRI

R1-2002256
Remaining Issues of UCI Enhancements for URLLC
Spreadtrum Communications

R1-2002304
UCI enhancements for URLLC
InterDigital, Inc.

R1-2002330
Remaining Issues on UCI Enhancements for eURLLC
Apple

R1-2002411
Remaining issue on the priority of A-SRS
ITRI

R1-2002443
Remaining issues for UCI enhancement for Rel-16 URLLC
NTT DOCOMO, INC.

R1-2002500
Discussion on multiplexing UCI of same type on a PUSCH
ASUSTeK

R1-2002545
Remaining issues on UCI Enhancements for URLLC
Qualcomm Incorporated

R1-2002635
Remaining issues on UCI enhancement for NR URLLC
WILUS Inc.

7.2.5.3 PUSCH enhancements (4)
R1-2002332
Feature lead summary #1 on PUSCH enhancements for NR eURLLC (AI 7.2.5.3)



Moderator (Apple)
R1-2002731
Outcome of the preparation email discussion on PUSCH enhancements for NR eURLLC (AI 7.2.5.3) Moderator (Apple)

[100b-e-NR-L1enh-URLLC-PUSCH-01] Email approval of TP for the previous agreement:

· For PUSCH repetition Type B, PUSCH transmit power is determined based on the nominal repetition duration.

based on the submitted contributions till 4/23 (Apple, Sigen)
[100b-e-NR-L1enh-URLLC-PUSCH-02] Email discussion/approval regarding the issue identified w.r.t.  UCI multiplexing on PUSCH till 4/24, with potential TP for approval till 4/29 (Apple, Sigen)
[100b-e-NR-L1enh-URLLC-PUSCH-03] Email discussion/approval regarding the issue identified w.r.t. interaction with DL/UL directions till 4/24, with potential TP for approval till 4/29 (Apple, Sigen)
[100b-e-NR-L1enh-URLLC-PUSCH-04] Email discussion/approval regarding the issue identified w.r.t. 
· A-CSI/SP-CSI on PUSCH

· Extended CP

· TP for segmentation

· whether/how to clarify DMRS overhead for TBS determination

multiplexing on PUSCH till 4/27, with potential TP for approval till 4/30 (Apple, Sigen)
R1-2001549
Corrections on PUSCH enhancement
Huawei, HiSilicon

R1-2001613
Remaining issues on PUSCH enhancements for NR URLLC
ZTE

R1-2001671
PUSCH enhancements for URLLC
vivo

R1-2001696
Maintenance of PUSCH enhancements for Rel-16 NR URLLC
Nokia, Nokia Shanghai Bell

R1-2001775
PUSCH enhancements for URLLC
OPPO

R1-2001786
Remaining Issue of PUSCH Enhancements for NR URLLC
Ericsson

R1-2001816
Remaining issues on PUSCH enhancements for URLLC
Sony

R1-2001872
Remaining issues on URLLC PUSCH enhancement
Panasonic Corporation

R1-2001881
Remaining issues on PUSCH enhancements for URLLC
Fujitsu

R1-2001921
Remaining issues of PUSCH enhancements for NR URLLC
LG Electronics

R1-2002000
Corrections on PUSCH enhancements for URLLC
Intel Corporation

R1-2002084
Remaining issues on PUSCH enhancements
CATT

R1-2002133
Remaining issues for PUSCH enhancements
Samsung

R1-2002209
Remaining issues on PUSCH enhancements for URLLC
CMCC

R1-2002251
PUSCH enhancements
ETRI

R1-2002264
Discussion on  PUSCH enhancements for URLLC
Spreadtrum Communications

R1-2002305
PUSCH enhancements for URLLC
InterDigital, Inc.

R1-2002331
Remaining Issues in PUSCH enhancements
Apple

R1-2002392
Remaining issues on PUSCH enhancements for NR URLLC
Sharp

R1-2002403
Discussion on multiplexing UCI of same type on a PUSCH
ASUSTeK

Withdrawn

R1-2002409
UCI multiplexing in PUSCH repetition type B
Motorola Mobility, Lenovo

R1-2002444
Remaining issues for PUSCH enhancements for Rel.16 URLLC
NTT DOCOMO, INC.

R1-2002546
Remaining issues on PUSCH enhancements for URLLC
Qualcomm Incorporated

R1-2002636
Remaining issues on PUSCH enhancement for NR URLLC
WILUS Inc.

7.2.5.4 Enhancements to scheduling/HARQ (2)
R1-2002720 
Summary of the Remaining Issues on HARQ and Scheduling Enhancements for URLLC
Moderator (Qualcomm)
[100b-e-NR-L1enh-URLLC-Scheduling and HARQ-01] Email discussion/approval regarding handling of collision and cancellation for PUCCH carrying HARQ-ACK with different priorities based on the following proposal as a starting point: 

Proposal:

· If a UE is scheduled with a first PDSCH and a second PDSCH which is starting later than the first PDSCH, if the PUCCH carrying HARQ-ACK for the first and second PDSCHs overlap in time: 

· If the two PUCCHs correspond to same HARQ-ACK CB, 

· Two PUCCHs are said to “overlap” when they are mapped to the same slot or sub-slot

· Multiplexing according to Rel. 15 should be followed.

· If the two PUCCHs correspond to HARQ-ACK CBs with different priorities, 

· Two PUCCHs are said to “overlap” when they have time-domain overlap of at least one symbol

· The low priority PUCCH should be cancelled.

· If the two PUCCHs correspond to HARQ-ACK CBs with different priorities, the UE separately reports the capability for cancellation in the following cases: 
· Transmission of the PUCCH carrying HARQ-ACK for the second PDSCH starts at the same symbol or earlier than that of the first PDSCH.

· Transmission of the PUCCH carrying HARQ-ACK for the second PDSCH starts later than that of the first PDSCH.

· Handling the collision between two PUCCHs carrying HARQ-ACK with different priorities is reported separately by the UE for the following cases: 
· The corresponding PDSCHs are scheduled on the same carrier.
· The corresponding PDSCHs are scheduled on different carriers.

Discussion until 04/24. If agreement is made, TP to be finalized by 04/29 (Qualcomm, Kianoush)
[100b-e-NR-L1enh-URLLC-Scheduling and HARQ-02] Eamil approval on spec update to avoid out-of-order HARQ-ACK for sub-slot based codebook using the proposed TPs captured under Issue #2 in R1-20027202 until 04/24 (Qualcomm, Kianoush)
R1-2001614
Remaining issues on scheduling/HARQ enhancements for NR URLLC
ZTE

R1-2001672
Enhancement for Scheduling/HARQ
vivo

R1-2001700
Maintenance of Rel-16 URLLC scheduling/HARQ enhancements
Nokia, Nokia Shanghai Bell

R1-2001776
Enhancements to scheduling and HARQ
OPPO

R1-2001787
Remaining Issue of Scheduling/HARQ for NR URLLC
Ericsson

R1-2001922
Remaining issues of Enhancements to scheduling/HARQ for NR URLLC
LG Electronics

R1-2002001
Remaining issues on scheduling and HARQ enhancements for URLLC
Intel Corporation

R1-2002085
Remaining issues on out-of-order scheduling/HARQ
CATT

R1-2002210
Remaining issues on scheduling and HARQ
CMCC

R1-2002445
Remaining issue on scheduling and HARQ enhancements for URLLC
NTT DOCOMO, INC.

R1-2002547
Remaining issues on scheduling/HARQ enhancements for URLLC
Qualcomm Incorporated

R1-2002583
Corrections on operation of out-of-order
Huawei, HiSilicon

7.2.5.5 Enhanced inter UE Tx prioritization/multiplexing (3)
R1-2001676
Summary of UL inter UE Tx prioritization for URLLC
Moderator (vivo)

R1-2002738
Summary#2 of UL inter UE Tx prioritization for URLLC
Moderator (vivo)

[100b-e-NR-L1enh-URLLC-InterUE-01] Email discussion/approval regarding interaction between intra-UE prioritization/multiplexing and inter-UE cancellation (issues #1/2/3/4 in R1-2002738) till 4/24, with potentil TP for approval till 4/29 – (vivo, Xueming)
[100b-e-NR-L1enh-URLLC-InterUE-02] Email discussion/approval regarding issues #6/7/8/9 in R1-2002738 till 4/24, with potentil TP for approval till 4/29 – (vivo, Xueming)
[100b-e-NR-L1enh-URLLC-InterUE-03] Email discussion/approval regarding issues #10/11/12 in R1-2002738 till 4/23, with potentil TP for approval till 4/28 – (vivo, Xueming)
R1-2001547
Corrections on UL inter-UE multiplexing
Huawei, HiSilicon

R1-2001615
Remaining issues on UL inter-UE multiplexing between eMBB and URLLC
ZTE

R1-2001673
UL inter UE Tx prioritization for URLLC
vivo

R1-2001697
Maintenance of Rel-16 URLLC Enhanced inter UE Tx prioritization/multiplexing
Nokia, Nokia Shanghai Bell

R1-2001777
Inter UE Tx prioritization and multiplexing
OPPO

R1-2001788
Remaining Issue of Inter-UE Prioritization and Multiplexing of  UL Transmissions
Ericsson

R1-2001817
Remaining issues on inter-UE multiplexing
Sony

R1-2001841
Remaining issues on enhanced inter UE Tx prioritization/multiplexing
MediaTek Inc.

R1-2001873
Remaining issue on inter-UE prioritization
Panasonic Corporation

R1-2001923
Remaining issues of UL inter UE Tx prioritization
LG Electronics

R1-2002002
Remaining issues on enhanced inter-UE multiplexing
Intel Corporation

R1-2002086
Remaining issues on inter-UE UL multiplexing
CATT

R1-2002134
Remaining issues for inter-UE multiplexing
Samsung

R1-2002211
Remaining issues on inter UE Tx prioritization/multiplexing
CMCC

R1-2002252
Enhanced inter UE Tx prioritization/multiplexing
ETRI

R1-2002257
Remaining issues of enhanced inter UE Tx prioritization/multiplexing
Spreadtrum Communications

R1-2002307
Inter-UE prioritization/multiplexing
InterDigital, Inc.

R1-2002333
Remaining Issues on Inter-UE Cancellation for eURLLC
Apple

R1-2002548
Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization
Qualcomm Incorporated

R1-2002637
Remaining issues on inter-UE multiplexing for NR URLLC
WILUS Inc.

7.2.5.6 Enhanced UL configured grant transmission (2)
R1-2001796
Summary of enhanced UL configured grant transmission for URLLC
Moderator (vivo)

R1-2001797
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001798
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001799
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001800
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001801
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001802
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

R1-2001811
Summary of enhanced UL configured grant transmission for URLLC
vivo Mobile Communication Co.,

Withdrawn

[100b-e-NR-L1enh-URLLC-eCG-01] Email discussion/approval regarding DMRS and PTRS operation for dynamic PUSCH and configured grant PUSCH (section 3.1.1. 3.1.2 and 3.1.3 in R1-2001796) till 4/24, and potential TPs for approval by 4/29 (vivo, Lihui)
[100b-e-NR-L1enh-URLLC-eCG-02] Email discussion/approval regarding corrections as in section 3.2.1, 3.2.2 and 4.1 in R1-2001796 till 4/24, and potential TPs for approval by 4/29 (vivo, Lihui)
R1-2001546
Corrections on configured grant transmission
Huawei, HiSilicon

R1-2001616
Remaining issues on enhancements for UL configured grant transmission
ZTE

R1-2001674
Enhanced UL configured grant transmissions  for URLLC
vivo

R1-2001698
Maintenance of Rel-16 URLLC Configured Grant enhancements
Nokia, Nokia Shanghai Bell

R1-2001778
Configured grant enhancements for URLLC
OPPO

R1-2001789
Remaining Issue of Enhancements to UL Configured Grant Transmission for NR URLLC


Ericsson

R1-2001924
Remaining issues of Enhanced UL configured grant transmission for NR URLLC
LG Electronics

R1-2002087
Remaining issues on enhanced UL configured grant transmission
CATT

R1-2002334
Remaining Issues in Enhanced Configured Grant Transmission
Apple

R1-2002412
Maintenance of UL grant Type 2 scheduling activation
ITRI

R1-2002446
Remaining issues for enhanced Configured grant for Rel.16 URLLC
NTT DOCOMO, INC.

7.2.5.7 Others (3)
R1-2001926
Summary on maintenance of other aspects for URLLC/IIOT
Moderator (LG Electronics)

R1-2002721
Summary#2 on maintenance of other aspects for URLLC/IIOT
Moderator (LG Electronics)
[100b-e-NR-L1enh-URLLC-IIoTenh-01] Email discussion/approval regarding SPS PDSCH collision handling including sections 2.1 and 2.2 of R1-2002721 till 4/24, with potential TP for approval till 4/29 (LGE, Hyunho)
[100b-e-NR-L1enh-URLLC-IIoTenh-02] Email discussion/approval regarding HARQ-ACK feedback related issues for SPS including sections 3.1, 3.2 and 3.4 of R1-2002721 till 4/24, with potential TP for approval till 4/29 (LGE, Hyunho)
[100b-e-NR-L1enh-URLLC-IIoTenh-03] Email discussion/approval regarding other remaining issues including section 4.1/4.3/3.3 of R1-2002721 till 4/23, with potential TP for approval till 4/28 (LGE, Hyunho)
R1-2001606
Discussion on remaining issues on DL SPS enhancement for URLLC
Beijing Xiaomi Mobile Software

R1-2001617
Remaining issues on SPS enhancements
ZTE

R1-2001675
Other issues for URLLC
vivo

R1-2001699
Maintenance of Rel-16 URLLC/IIoT SPS enhancements
Nokia, Nokia Shanghai Bell

R1-2001779
DL SPS enhancement and Intra-UE multiplexing/prioritization
OPPO

R1-2001790
Remaining Issue of DL SPS Enhancements for NR URLLC
Ericsson

R1-2001842
Remaining issues on multiple SPS configurations
MediaTek Inc.

R1-2001925
Remaining issues of other aspects for URLLC/IIOT
LG Electronics

R1-2002003
Remaining issues on enhanced DL SPS for IIoT
Intel Corporation

R1-2002088
Remaining issues on IIoT
CATT

R1-2002135
Remaining issues for DL SPS and PHR
Samsung

R1-2002335
Remaining Issues in SPS enhancements
Apple

R1-2002447
Remaining issues for SPS enhancement for Rel-16 URLLC
NTT DOCOMO, INC.

R1-2002485
Remaining issues on DL SPS enhancements
Asia Pacific Telecom co. Ltd

R1-2002549
Remaining issues on DL SPS for URLLC
Qualcomm Incorporated

R1-2002584
Corrections on other aspects for URLLC/IIOT enhancements
Huawei, HiSilicon

R1-2002638
Remaining issues on DL SPS for NR URLLC
WILUS Inc.

7.2.6 Maintenance of Enhancements on MIMO for NR
R1-2002648
Introduction of NR ATF measurements
Keysight Technologies UK Ltd

R1-2002649
Correction on SS-RSRPB measurement
Keysight Technologies UK Ltd

7.2.6.1 CSI Enhancement for MU-MIMO Support

R1-2002138
Feature lead summary for MU-MIMO CSI
Moderator (Samsung)

R1-2001562
Remaining issues on MU-CSI in R16
Huawei, HiSilicon

R1-2001595
Maintenance of CSI enhancement for MU-MIMO
ZTE

R1-2001677
Discussion on remaining issues on MU CSI
vivo

R1-2001725
Text proposals for CSI enhancements for MU-MIMO support
OPPO

R1-2001912
Correction on enhanced Type II port selection codebook
LG Electronics

R1-2002089
Remaining issues on CSI Enhancement for MU-MIMO
CATT

R1-2002136
On maintenance of Rel.16 MU CSI enhancements
Samsung

R1-2002137
Summary of offline email discussion on [100e-NR-eMIMO-MUCSI-01] follow up
Moderator (Samsung)

R1-2002293
Maintenance on Rel-16 CSI enhancements
Nokia, Nokia Shanghai Bell

R1-2002336
Remaining issues for Rel-16 Type II CSI enhancement
Apple

R1-2002410
Remaining issues on MU-CSI Enhancements
Motorola Mobility, Lenovo

R1-2002550
Remaining issues on CSI enhancement for MU-MIMO support
Qualcomm Incorporated

7.2.6.2 Enhancements on Multi-TRP/Panel Transmission 
R1-2002406
FL summary for Multi-TRP/Panel Transmission 
Moderator (OPPO)

R1-2001563
Remaining issues on multi-TRP in R16
Huawei, HiSilicon

R1-2001596
Maintenance of multi-TRP enhancements
ZTE

R1-2001678
Discussion on remaining issues on multi TRP operation
vivo

R1-2001726
Text proposals for enhancements on multi-TRP and panel Transmission
OPPO

R1-2001822
Enhancements on Multi-TRP/Panel Transmission
MediaTek Inc.

R1-2001913
Text proposals on enhancements on multi-TRP/panel transmission
LG Electronics

R1-2001971
Remaining issues on multi-TRP/panel transmission
Lenovo, Motorola Mobility

R1-2002004
Corrections to multi-TRP
Intel Corporation

R1-2002052
TP on Multi-TRP/Panel Transmission
Futurewei

R1-2002090
Remaining issues on multi-TRP/panel transmission
CATT

R1-2002139
On Rel.16 multi-TRP/panel transmission
Samsung

R1-2002212
Remaining issues on multi-TRP/panel transmission
CMCC

R1-2002270
Discussion on remaining issues for multi-TRP operation
Spreadtrum Communications

R1-2002294
Maintenance of Rel-16 Multi-TRP operation
Nokia, Nokia Shanghai Bell

R1-2002337
Remaining issues for Multi-TRP enhancement
Apple

R1-2002364
Remaining issues on multi-TRP transmission
NEC

R1-2002378
Clarification on Multi-TRP URLLC Scheme 4
Convida Wireless

R1-2002448
Remaining issues on multi-TRP/panel transmission
NTT DOCOMO, INC

R1-2002472
Remaining issues on Multi-TRP/Panel Transmission
Ericsson

R1-2002551
Multi-TRP Enhancements
Qualcomm Incorporated

7.2.6.3 Enhancements on Multi-beam Operation
R1-2001915
FL summary#1 on Rel-16 MB1 maintenance
Moderator (LG Electronics)

R1-2002339
Feature Lead Summary #1 on L1-SINR and SCell BFR
Apple

R1-2002340
Feature Lead Summary #2 on L1-SINR and SCell BFR
Apple

R1-2001564
Remaining issues on multi-beam enhancements in R16
Huawei, HiSilicon

R1-2001597
Maintenance of multi-beam operation
ZTE

R1-2001679
Discussion on remaining issues on multi beam operation
vivo

R1-2001727
Remaining issues on Multi-beam Operation Enhancement
OPPO

R1-2001818
Remaining issues on multi-beam operation
Sony

R1-2001823
Remaining issues on multi-beam operation
MediaTek Inc.

R1-2001914
Remaining issues on multi beam operation
LG Electronics

R1-2001972
Remaining issues on Scell beam failure recovery
Lenovo, Motorola Mobility

R1-2002005
 Corrections to Multi-beam Operation 
Intel Corporation

R1-2002091
On Rel.16 support for multi-beam operation
CATT

R1-2002140
On Rel.16 support for multibeam operation
Samsung

R1-2002213
Remaining issues on multi-beam operation
CMCC

R1-2002271
Discussion on remaining issues on multi-beam operation
Spreadtrum Communications

R1-2002283
Enhancements on multi-beam operation
Fraunhofer IIS, Fraunhofer HHI

R1-2002295
Maintenance of Rel-16 Beam Management
Nokia, Nokia Shanghai Bell

R1-2002338
Remaining issues on beam management enhancement
Apple

R1-2002449
Remaining issues on multi-beam operation
NTT DOCOMO, INC

R1-2002498
On the timing for pathloss RS activation
Ericsson, Nokia

R1-2002552
Enhancements on Multi-beam Operation
Qualcomm Incorporated

7.2.6.4 Full TX Power UL transmission
R1-2001719
Feature lead summary on ULFPTx
Moderator (vivo)

R1-2001565
Remaining issues on UL full power transmission in R16
Huawei, HiSilicon

R1-2001598
Maintenance of full power UL transmission
ZTE

R1-2001680
Discussion on remaining issues on UL full power transmission
vivo

R1-2001728
Text proposals for full TX power UL transmission
OPPO

R1-2001824
Remaining issues on UL full power transmission
MediaTek Inc.

R1-2001916
Text proposals on full Tx power UL transmission
LG Electronics

R1-2002006
Correction to Full Tx Power UL Transmission
Intel Corporation

R1-2002092
Remaining issues on UL full power transmission
CATT

R1-2002141
On UL full power transmission
Samsung

R1-2002214
Discussion on full TX power UL transmission
CMCC

R1-2002272
Discussion on remaining issues on full TX power for UL transmission
Spreadtrum Communications

R1-2002296
Maintenance of Rel-16 Full TX Power UL transmission
Nokia, Nokia Shanghai Bell

R1-2002341
Remaining issues for UL Full Power transmission enhancement
Apple

R1-2002368
Corrections for Full Power UL Transmission
Ericsson

R1-2002450
Remaining issue on full Tx power UL transmission
NTT DOCOMO, INC

R1-2002553
Remaining issues on full power UL transmission
Qualcomm Incorporated

7.2.6.5 Low PAPR RS

R1-2001566
Discussion on low PAPR sequence in R16
Huawei, HiSilicon

R1-2001599
Maintenance of low PAPR RS
ZTE

R1-2001681
Discussion on remaining issues on low PAPR RS
vivo

R1-2001729
Text proposals for low PAPR RS
OPPO

R1-2001812
Corrections in low PAPR RS for pi/2 BPSK PUSCH
Indian Institute of Tech (H)

Withdrawn

R1-2001825
Maintenance of low PAPR RS
MediaTek Inc.

R1-2001908
Remaining issues on low PAPR RS
Ericsson

R1-2002007
Low PAPR RS
Intel Corporation

R1-2002273
Discussion on remaining issues on Low PAPR DMRS sequence
Spreadtrum Communications

R1-2002297
Maintenance on Rel-16 Low PAPR RS
Nokia, Nokia Shanghai Bell

R1-2002554
Remaining issues on low PAPR RS
Qualcomm Incorporated

7.2.6.6 Others

R1-2001730
Remaining issues on multi-TRP and panel Transmission
OPPO

R1-2001917
TP for the maximum number of CSI-RS resources for BM
LG Electronics

R1-2001918
Critical MIMO issue from observations in the field
Ericsson

R1-2002342
On further MIMO enhancement
Apple

R1-2002467
On limitations of PDSCH reliable transmission
vivo

R1-2002468
On limitations of PDSCH reliable transmission
vivo

Withdrawn

R1-2002469
On limitations of PDSCH reliable transmission
vivo

Withdrawn

R1-2002470
On limitations of PDSCH reliable transmission
vivo

Withdrawn

R1-2002471
On limitations of PDSCH reliable transmission
vivo

Withdrawn

R1-2002667
Other open issues for Rel-16 MIMO
Huawei, HiSilicon

7.2.7 Maintenance of UE Power Saving for NR (5)
//From the LS section:

[100b-e-NR- pow_sav-fromLS] Email approvel of the corresponding TP for R1-2001507 by 4/23 under 7.2.7.1 (CATT, Fangchen)
7.2.7.1 PDCCH-based power saving signal/channel (2)
R1-2002698
Summary of PDCCH-based Power Saving Signal/Channel
Moderator (CATT)
[100b-e-NR-UE_pow_sav-WUS-01] Email discussion/approval on DCI format 2_6 Monitoring and Related Procedures in section 1.1 and subsections of R1-2002698 till 4/24, with potential TP for endorsement till 4/29 (CATT, Fangchen)

[100b-e-NR-UE_pow_sav-WUS-02] Email discussion/approval on RAN1 and RAN2 alignment in section 1.2 and subsections in R1-2002698 till 4/24, with potential TP for endorsement till 4/29 (CATT, Fangchen)
R1-2001539
Remaining issues on PDCCH based power saving
Huawei, HiSilicon

R1-2001583
Remaining issues on WUS PDCCH
ZTE

R1-2001682
Maintenance of PDCCH-based power saving signal
vivo

R1-2001768
Remaining issues for Power saving signal
OPPO

R1-2001819
Remaining issues on PDCCH-based WUS
Sony

R1-2001843
Remaining issues on PDCCH-based power saving signal
MediaTek Inc.

R1-2001943
Remaining issues on PDCCH-based power saving signal/channel
LG Electronics

R1-2002008
Remaining details of PDCCH-based power saving signal/channel
Intel Corporation

R1-2002093
Remaining issues on the Power Saving Signals/Channels
CATT

R1-2002142
Remaining issues for PDCCH-based power saving signal
Samsung

R1-2002215
Remaining issues on minimum time gap for PDCCH-based power saving signal/channel
CMCC

R1-2002218
On open issues related to DCI format 2_6
Nokia, Nokia Shanghai Bell

R1-2002261
Clarification on power saving signal
Spreadtrum Communications

R1-2002366
Remaining Issues for PDCCH-based Power Saving Signal/Channel
InterDigital

R1-2002414
Remaining issues for WUS
Ericsson

R1-2002451
Maintenance for PDCCH-based power saving signal/channel
NTT DOCOMO, INC.

R1-2002555
Remaining issues for PDCCH-based power saving channel
Qualcomm Incorporated

7.2.7.2 Procedure of cross-slot scheduling power saving techniques (2)
[100b-e-NR-UE_pow_sav-Cross_Slot-01] Email discussion/approval to resolve Issues related to active BWP change

· Issue #1: Whether and how to apply the currently active minimum scheduling offset restriction in the case of cross-BWP scheduling?

· Issue #2: Whether and how to decide the applied minimum scheduling offset restriction for the slots after BWP switch and before the application delay is ended?

· Issue #3: Clarification of numerology conversion in case of active BWP change 

· Note: Consider proposal(s) to jointly resolve the above three issues 

Till 4/24, with potential TP for approval till 4/29 (MTK, Weide)
[100b-e-NR-UE_pow_sav-Cross_Slot-02] Email discussion/approval to resolve Issues related to exception handling:

· Issue #1: Additional exceptional cases, including BFR, MsgB-RNTI, C/CS/MCS-RNTI monitored in CSS type 3 if default TDRA table is applied, SP-CSI-RNTI, etc.

· Issue #2: Error handling if UE receives both DCI format 1_1 and format 0_1 with inconsistent values in the 1-bit indications 

· Issue #3: Error handling when the UE detects an invalid TDRA entry by DCI format 0_0/1_0

· Issue #4: Whether and how resolve the inconsistency between the range of A-CSI triggering offset and that of K0min

Till 4/24, with potential TP for approval till 4/29 (MTK, Weide)
R1-2001540
Remaining issues on cross-slot scheduling based power saving
Huawei, HiSilicon

R1-2001584
Remaining issues on cross-slot scheduling power saving techniques
ZTE

R1-2001683
Maintenance of procedure of cross-slot scheduling power saving techniques
vivo

R1-2001769
Remaining issues for cross-slot scheduling
OPPO

R1-2001820
Remaining issues on cross-slot scheduling for UE power saving
Sony

R1-2001844
Remaining issues on cross-slot scheduling adaptation
MediaTek Inc.

R1-2001944
Remaining issues on procedure of cross-slot scheduling power saving techniques
LG Electronics

R1-2002009
Remaining details of cross-slot scheduling power saving techniques
Intel Corporation

R1-2002094
Remaining issues on Power saving scheme with cross-slot scheduling
CATT

R1-2002143
Remaining issues for cross-slot scheduling power saving techniques
Samsung

R1-2002216
Remaining issues on cross-slot scheduling adaptation in cross-BWP scheduling
CMCC

R1-2002219
Procedure of cross-slot scheduling power saving techniques
Nokia, Nokia Shanghai Bell

R1-2002258
Remaining issues on cross-slot scheduling
Spreadtrum Communications

R1-2002343
Remaining Issues on Cross-Slot Power Save
Apple

R1-2002367
Remaining Issues for Cross-Slot Scheduling for UE Power Saving
InterDigital

R1-2002415
Remaining issues for cross-slot scheduling
Ericsson

R1-2002452
Maintenance for procedure of cross-slot scheduling power saving techniques
NTT DOCOMO, INC.

R1-2002556
Remaining issues in cross-slot scheduling power saving
Qualcomm Incorporated

7.2.7.3 UE adaptation to maximum number of MIMO layers
R1-2001541
Remaining issues on UE adaptation to maximum MIMO layers
Huawei, HiSilicon

R1-2001585
On UE adaptation to maximum number of MIMO layers
ZTE

R1-2002220
On open issues for per-BWP DL MIMO layers
Nokia, Nokia Shanghai Bell

R1-2002416
Remaining issues for MIMO layer signaling per BWP
Ericsson

7.2.7.4 Others

R1-2001586
Views on  power saving enhancement
ZTE

R1-2001684
Other issues for R16 UE power saving
vivo

R1-2002417
UE power saving using search space set switching
Ericsson

R1-2002577
Other Remaining issues for Rel-16 Power saving
Huawei, HiSilicon

7.2.8 Maintenance of NR positioning support
7.2.8.1 DL Reference Signals for NR Positioning

R1-2001558
Maintenance of DL PRS for NR positioning
Huawei, HiSilicon

R1-2001600
Maintenance of DL reference signals for NR positioning
ZTE

R1-2001685
Discussion on remaining issues on DL RS for NR positioning
vivo

R1-2001731
Remaining Issues on DL Positioning Reference Signal
OPPO

R1-2001954
Remaining details of DL Reference signals for NR positioning
LG Electronics

R1-2002046
Remaining details on DL Reference Signals
Futurewei

R1-2002095
Remaining issues on DL PRS for NR Positioning
CATT

R1-2002144
DL Reference Signals for NR Positioning
Samsung

R1-2002288
Corrections to DL reference signals for NR positioning
Intel Corporation

R1-2002557
Discussion on DL PRS processing & related UE capabilities
Qualcomm Incorporated

R1-2002620
Maintenance of rel16 DL reference signals for NR positioning
Ericsson

7.2.8.2 UL Reference Signals for NR Positioning

R1-2001559
Maintenance of SRS for NR positioning
Huawei, HiSilicon

R1-2001601
Maintenance of UL reference signals for NR positioning
ZTE

R1-2001686
Discussion on remaining issues on UL RS for NR positioning
vivo

R1-2001732
Remaining Issues on UL Positioning Reference Signal
OPPO

R1-2002038
Maintenance on UL reference signals for NR Positioning
Nokia, Nokia Shanghai Bell

R1-2002047
Remaining details on UL Reference Signals
Futurewei

R1-2002096
Remaining issues on UL SRS for NR Positioning
CATT

R1-2002145
UL Reference Signals for NR Positioning
Samsung

R1-2002199
Discussion on staggered SRS for NR Positioning 
Fraunhofer IIS, Fraunhofer HHI

R1-2002286
Corrections to UL reference signals for NR positioning
Intel Corporation

R1-2002621
Maintenance of rel16 UL reference signals for NR positioning
Ericsson

7.2.8.3 UE and gNB measurements for NR Positioning

R1-2001560
Maintenance of NR positioning measurements
Huawei, HiSilicon

R1-2001602
Maintenance of UE and gNB measurements for NR positioning
ZTE

R1-2001733
Remaining Issues on Measurements for NR Positioning
OPPO

R1-2002048
Remaining details on Measurements
Futurewei

R1-2002097
Remaining issues on NR Positioning Measurements
CATT

R1-2002146
UE and gNB measurements for NR Positioning
Samsung

R1-2002622
Maintenance of rel16 UE and gNB measurements for NR Positioning
Ericsson

7.2.8.4 Physical-layer procedures to support UE/gNB measurements
R1-2001561
Maintenance of physical layer procedures to support positioning measurements
Huawei, HiSilicon

R1-2001603
Maintenance of physical-layer procedure for NR positioning
ZTE

R1-2001687
Discussion on remaining issues on physical-layer procedures for NR positioning
vivo

R1-2001734
Remaining Issues on Physical Layer Procedures for NR Positioning
OPPO

R1-2001955
Remaining details of physical-layer procedure to support UE/gNB measurements
LG Electronics

R1-2002049
Remaining details on Measurement Procedures
Futurewei

R1-2002098
Remaining issues on NR Positioning Procedures
CATT

R1-2002147
Physical-layer procedures to support UE/gNB measurements
Samsung

R1-2002217
Remaining issues on physical layer procedure for UL SRS transmission
CMCC

R1-2002287
Corrections to physical layer procedures for NR positioning
Intel Corporation

R1-2002623
Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements
Ericsson

7.2.8.5 Others

R1-2001735
Discussion on RE mapping in reference signal for NR positioning
OPPO

R1-2002676
On positioning latency and aperiodic SRS
Huawei, HiSilicon

7.2.9 Maintenance of NR Mobility Enhancements 

R1-2002010
Issue Summary for NR Mobility Enhancements
Moderator (Intel Corporation)

7.2.9.1 Physical Layer Aspects for Mobility Enhancements during HO and SCG Change

R1-2001530
Remaining issues on DAPS-HO
Huawei, HiSilicon

R1-2001624
Remaining issues on NR mobility enhancements in physical layer
ZTE

R1-2002011
Corrections to Physical layer aspects of NR mobility enhancement
Intel Corporation

R1-2002148
Remaining issues for NR Mobility Enhancement
Samsung

R1-2002221
Remaining physical layer aspects of dual active protocol stack based HO
Nokia, Nokia Shanghai Bell

R1-2002344
On remaining issues on NR mobility enhancements
Apple

R1-2002490
Correction to UL power sharing for DAPS HO
Ericsson

R1-2002558
Maintenance for NR mobility enhancements
Qualcomm Incorporated

7.2.9.2 Others
R1-2001531
Remaining PHY aspects for CHO
Huawei, HiSilicon

R1-2001625
Discussion on FR2 mobility interruption enhancements
ZTE
7.2.10 Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR) (7)
7.2.10.1 Uplink Power Control for Supporting NR-NR Dual-Connectivity (1)
R1-2002346
Feature lead summary #1 on UL Power Control for NN-DC
Moderator (Apple)
R1-2002743
Outcome of preparation discussion on UL Power Control for NN-DC
Moderator (Apple)

[100b-e-NR- LTE_NR_DC_CA-ULPC-01] Email discussion/approval of issues 1/2/3/4/5 in R1-2002346 till 4/24, with potential TPs for approval till 4/29 (Apple, Hong)
R1-2001528
UL power control for NR-NR dual connectivity
Huawei, HiSilicon

R1-2001618
Remaining Issues of Dynamic Power Sharing for NR-DC
ZTE

R1-2001688
Remaining issues on uplink power control for NR-NR DC
vivo

R1-2001736
Text proposals for UL Power Sharing for NR-DC
OPPO

R1-2002012
Remaining issues on uplink power control for NN-DC
Intel Corporation

R1-2002345
Remaining issues of UL Power Control for NN-DC
Apple

R1-2002418
Remaining issues for NR-DC UL Power Control
Ericsson

R1-2002607
Remaining details of Rel-16 DC uplink power control
Nokia, Nokia Shanghai Bell
7.2.10.2 Potential Enhancements to Single Tx Switched Uplink Solution for EN-DC (1)
R1-2002348
Feature lead summary #1 on single Tx operation for EN-DC
Moderator (Apple)
[100b-e-NR- LTE_NR_DC_CA-SingleTx-01] Email discussion/approval regarding the following issues:

· Issue 1 on type 1 UE hailing of colliding UL in single Tx operation, as described in section 2 of R1-2002348
· Issue 2 on type 1 UE capability with semi-static UL transmission, as described in section 3 of R1-2002348
· Discussion of the TP is delayed until the corresponding UE feature discussions finish.

· Issue 3a and 3b on clarification of Type 2 UE behavior, as described in sections 4.1 and 4.2 of R1-2002348, respectively.

till 4/23, with potential TPs for approval till 4/28 – (Apple, Wei)

R1-2001529
Enhancements for single UL operation for EN-DC
Huawei, HiSilicon

R1-2001619
Remaining Issues on Single Tx Switched Uplink Solution for EN-DC
ZTE

R1-2001835
Remaining issues on single Tx switched UL solution for EN-DC
MediaTek Inc.

R1-2002347
Remaining issues on single Tx operation for EN-DC
Apple

R1-2002419
Remaining issues for single Tx UL enhancements
Ericsson

R1-2002559
Remaining issues for EN-DC Single-Tx TDM Operation
Qualcomm Incorporated

R1-2002608
Remaining details on Enhancements to Single Tx Switched Uplink Solution for EN-DC
Nokia, Nokia Shanghai Bell

Late submission

7.2.10.3 Support of efficient and low latency serving cell configuration/activation/setup (2)
R1-2002421
Summary of efficient and low latency serving cell configuration/activation/setup
Moderator (Ericsson)

R1-2002739
Summary of efficient and low latency serving cell configuration/activation/setup Moderator (Ericsson)
[100b-e-NR- LTE_NR_DC_CA-ScellDormancy-01] Email approval of the TPs based on:

· TP(from R1-2001542) to 38.212 on FDRA for Case2 dormancy indication for dynamic RA type indication

· TP(from R1-2002149) to 38.213 on DCI indication for configured SCells with dormant BWP

· TP(from R1-2002228) to 38.213 on aligning with RAN2 agreements on UL dormancy handling 

· TP(from R1-2001542) to 38.213 on wording for SCell dormancy “without scheduling PDSCH”

Till 4/23 (Ericsson, Ravi)
[100b-e-NR- LTE_NR_DC_CA-ScellDormancy-02] Email discussion/approval for the following issues (issues 1-1/2/3/4/5 as in R1-2002739)
· Processing time and HARQ timing for Case 2 dormancy indication 

· UE behavior for SCells configured with dormant BWP when DCI 2-6 is not detected

· Handling of CIF≠0 for Case 2 dormancy indication 

· Handling of “BWP indicator field” in DCI of SCell with dormant BWP

· DCI format 1_1/0_1/2_6 with dormancy indication only in first 3 symbols of a slot

Till 4/24, with potential TPs for approval till 4/29 (Ericsson, Ravi)
R1-2001542
Remaining issues on SCell dormancy indication
Huawei, HiSilicon

R1-2001620
Remaining Issues on Fast SCell Activation
ZTE

R1-2001689
Remaining issues on Scell dormancy like behavior
vivo

R1-2001770
Discussion on remaing issues of Scell dormancy
OPPO

R1-2002013
Remaining issues on SCell dormancy behavior
Intel Corporation

R1-2002066
Discussion on SCell Dormancy when DCI format 2_6 is not detected
CATT

R1-2002149
Remaining issues on dormancy Scell
Samsung

R1-2002185
Remaining issues on SCell dormancy behavior
MediaTek Inc.

R1-2002228
Remaining issues on Efficient CA design
Nokia, Nokia Shanghai Bell

R1-2002420
Remaining issues for reduced latency Scell management for NR CA
Ericsson

R1-2002560
Remaining issues for SCell dormancy
Qualcomm Incorporated

7.2.10.4 Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH (1)
R1-2002611
FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH
Moderator (Nokia)

R1-2002612
FL summary #2 on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH
Moderator (Nokia)

[100b-e-NR- LTE_NR_DC_CA-X-CC A-CSI-RS-01] Email approvals of the TPs based on the following issues #1/2/3/4 in R1-2002611 till 4/23 (Nokia, Karri)
R1-2001543
Remaining issues on the A-CSI RS triggering with different numerology
Huawei, HiSilicon

R1-2001690
Remaining issues on aperiodic CSI-RS triggering
vivo

R1-2001737
Text proposals for aperiodic CSI-RS triggering with different numerologies
OPPO

R1-2001836
Remaining issues on X-carrier ACSI-RS triggering with different SCS
MediaTek Inc.

R1-2002496
Correction to aperiodic CSI-RS triggering with mixed numerology
Ericsson

R1-2002561
Remaining issues for aperiodic CSI-RS triggering with different numerology
Qualcomm Incorporated

7.2.10.5 Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA (1)
R1-2002201
FL summary on support of unaligned frame boundary for R16 NR inter-band CA
Moderator (CMCC)

[100b-e-NR- LTE_NR_DC_CA-Unaligned_CA-01] Email approval of the TPs as summarized as issues #1 and #2 in R1-2002201 till 4/23 (CMCC, Xiaodong)
R1-2001621
Remaining Issues on ASync CA
ZTE

R1-2001691
Remaining issues on unaligned frame boundary for CA
vivo

R1-2002422
Remaining issues for inter-band CA with unaligned frame boundary
Ericsson

R1-2002475
Remaining issues on support of unaligned frame boundary with slot alignment 
ASUSTeK

R1-2002579
Remaining issues on intra-band CA with unaligned frame boundary
Huawei, HiSilicon

R1-2002609
Maintanence of unaligned frame boundary with slot alignment and partial SFN alignment for inter-band CA
Nokia, Nokia Shanghai Bell

7.2.10.6 Support of cross-carrier scheduling with different numerology (1)
R1-2002613
FL summary on cross-carrier scheduling with different numerology
Moderator (Nokia)

R1-2002614
FL summary #2 on cross-carrier scheduling with different numerology
Moderator (Nokia)

[100b-e-NR- LTE_NR_DC_CA-X-CC scheduling-01] Email approval the TPs based on the following:
· Issue#1 of R1-R2002613

· Spec-improvement #1 of R1-R2002613

· TP of Proposal#4 of R1-2001692
· TP of Proposal#2 of R1-2002561
till 4/23 (Nokia, Karri)
R1-2001622
Remaining Issues on Cross-carrier Scheduling with Mixed Numerologies
ZTE

R1-2001692
Remaining issues on cross-carrier scheduling with mix numerologies
vivo

R1-2001837
Remaining issues on X-carrier scheduling with different SCS
MediaTek Inc.

R1-2001942
Remaining issue on cross-carrier scheduling with different numerology
LG Electronics

R1-2002014
Remaining issues on cross-carrier scheduling with different numerology
Intel Corporation

R1-2002067
Discussion on HARQ-ACK feedback for SPS PDSCH release with cross-carrier scheduling


CATT

R1-2002423
Remaining issues for cross-carrier scheduling with different numerologies
Ericsson

R1-2002580
Remaining issues on cross-carrier scheduling with different numerology
Huawei, HiSilicon

R1-2002610
Maintenance of cross-carrier scheduling with different numerology
Nokia, Nokia Shanghai Bell

Late submission

7.2.10.7 Others

R1-2001544
Discussion on Cross-carrier triggering PDCCH order
Huawei, HiSilicon

R1-2001623
Discussion on the PUCCH group configuration for NR-DC
ZTE

R1-2002424
Data rate handling for intra-FR NR-DC
Ericsson

7.2.11 NR Rel-16 UE Features (1+2+6+5+4+10+6+5+5+4+5+1+2+2=58)
[100e-b-NR-UEFeatures-Remaining] Email discussion/approval of remaining issues (especially the one identified as low priority items in FL’s summaries) starting no earlier than 4/30 till next meeting – Hiroki (DCM)/Ralf (ATT)
7.2.11.1 UE features for two-step RACH (2)
R1-2002453
Summary on UE features for two-step RACH
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-2Step-01] Email discussion/approval on feature group structure for two step RACH (20th-24th April) – (DCM, Hiroki)
· Confirm to keep FG9-1 as single FG

· Confirm to keep FG9-2 as single FG

· Confirm to keep FG9-3 as single FG

· Discuss whether or not to keep FG9-4 as single FG

· Discuss whether or not to add following new feature groups. If there is no consensus to add a new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

· 60 kHz SCS for msgA PUSCH as an optional UE feature in FR1

· PUSCH transmission with frequency hopping

· max number msgB to be monitored/decoded per slot

[100b-e-NR-UEFeatures-2Step-02] Email discussion/approval on Basic channel structure and procedure for two step RACH (dates TBD) – (DCM, Hiroki)
· Down select following alternatives, and discuss whether/how to change the description of components.

· Alt 1: detailed feature group

· Alt 2: simplified basic feature group

R1-2001714
Discussion on the UE features for two-step RACH
ZTE, Sanechips

R1-2002015
Discussion on UE features for two-step RACH
Intel Corporation

R1-2002068
Discussion of NR Rel-16 UE features for two-step RACH
CATT

R1-2002150
UE features for two-step RACH
Samsung

R1-2002349
Views on NR 2-step RACH UE feature
Apple

R1-2002372
UE Features for Two-Step RACH
Ericsson

R1-2002562
Discussion on two step RACH UE features
Qualcomm Incorporated

R1-2002588
Rel-16 UE features for 2-step RACH
Huawei, HiSilicon

7.2.11.2 UE features for NR-U (6)
R1-2002458
Summary on UE features for NR-U
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-NRU-01] Email discussion/approval on the basic feature groups structure for NR-U (20th-24th April) – (DCM, Hiroki)

· Discuss on how to define basic FGs to cover all deployment scenarios
· Options 1 and 2 stated below are the starting point

· Option 1: define new basic FGs in addition to current basic FGs to cover all deployment scenarios (e.g., [3] and [5])

· Option 2: define new basic FGs with components that have tightly related functionality to replace current basic FGs (e.g., [8])

· Discuss whether/how following FGs can be included in basic FGs

· 10-3, 10-11, 10-14, 10-15, 10-16, 10-16a, 10-17, 10-18, 10-19, 10-20, 10-20a, 10-24, 10-25, 10-28, 10-29, 10-30

· Whether or not “Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0” can be separate capability from basic FGs

· Whether or not “Type 2B channel access” can be separate capability from basic FGs

[100b-e-NR-UEFeatures-NRU-02] Email discussion/approval on feature groups structure related to DL operation for NR-U (20th-24th April) – (DCM, Hiroki)

· Discuss on whether or not “Support fixed frame period of 5ms and 10ms” and “Support fixed frame periods shorter than 5ms” can be separate capabilities.

· Discuss whether or not the capability FG10-8 are separate for each length or some groups are formed to signal the capability together

· Discuss whether or not 10-9/9a/9b/9c can be combined into a single FG

· Discuss whether or not 10-14/10-15/10-16/10-16a/10-17 can be combined into a single FG

· If no, whether or not 10-15 can be further split under other group DAI/NFI configured or not

· Discuss whether or not 10-19/10-19a/10-19b/10-19c are needed

· Discuss whether or not 10-26 is needed

· Discuss whether or not 10-31 is needed

[100b-e-NR-UEFeatures-NRU-03] Email discussion/approval on feature groups structure related to DL+UL operation for NR-U (20th-24th April) – (DCM, Hiroki)

· Discuss on followings regarding 10-3 to 10-3b and [10-12]

· Whether or not 10-3/10-3a/10-3b/10-3c can be combined into a single FG

· Whether or not 10-12 can be combined into 10-3b/10-3c

· If not, whether or not 10-12 can be split for PF2 and PF3 separately

· Discuss whether or not 10-13a is needed as a FG

· Discuss whether or not 10-18/10-24/10-28 can be combined into a single FG

· Discuss whether or not following update can be applied

	10-21a
	Support using ED threshold for UL to DL COT sharing
	1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL

2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL

3. Indicate in CG-UCI the COT sharing information



	10-21b
	Support UL to DL COT sharing
	1. Support cat 4 LBT for scheduled UL to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16

2. Support cat 4 LBT for CG-PUSCH to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16

3. Indicate in CG-UCI the COT sharing information




· Discuss whether or not 10-22 is needed

[100b-e-NR-UEFeatures-NRU-04] Email discussion/approval on issues with capability signaling impacts on basic FGs for NR-U (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 1

[100b-e-NR-UEFeatures-NRU-05] Email discussion/approval on issues with capability signaling impacts on FGs related to DL operation for NR-U (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 2

[100b-e-NR-UEFeatures-NRU-06] Email discussion/approval on issues with capability signaling impacts on FGs related to DL+UL operation for NR-U (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 3

R1-2001715
Discussion on the UE features for NR-U
ZTE, Sanechips

R1-2001720
Discussion on Rel-16 NRU UE features
vivo

R1-2001765
Discussion on UE feature for NRU
OPPO

R1-2001826
Views on Rel-16 UE features for NR-U
MediaTek Inc.

R1-2001941
Discussion on UE features for NR-U
LG Electronics

R1-2002016
UE features for NR-U
Intel Corporation

R1-2002037
UE features for NR-U
Ericsson

R1-2002151
UE features for NR-U
Samsung

R1-2002350
Discussions on NR-U UE features
Apple

R1-2002393
Discussion on UE feature for NR-U
Sharp

R1-2002480
On UE features NR Unlicensed
Nokia, Nokia Shanghai Bell

R1-2002563
Discussion on NR-U UE features
Qualcomm Incorporated

R1-2002589
Rel-16 UE features for NR-U
Huawei, HiSilicon

R1-2002683
UE Features for NR-U
TCL Communications

7.2.11.3 UE features for IAB (5)
R1-2001866
Summary on UE features for IAB
Moderator (AT&T)

[100b-e-NR-UEFeatures-IAB-01] Email discussion/approval of proposal 1 in R1-2001866 till 4/24 – Ralf (ATT)
[100b-e-NR-UEFeatures-IAB-02] Email discussion/approval of proposal 3 in R1-2001866 till 4/24 – Ralf (ATT)
[100b-e-NR-UEFeatures-IAB-03] Email discussion/approval of proposal 4 in R1-2001866 till 4/24 – Ralf (ATT)
[100b-e-NR-UEFeatures-IAB-04] Email discussion/approval:
· Discuss which Rel. 15 layer-1 mandatory UE features (as defined in TR 38.822) are supported by IAB-MTs
(dates TBD)– Ralf (ATT)
[100b-e-NR-UEFeatures-IAB-05] Email discussion/approval of proposal 5 (medium priority part) in R1-2001866 (dates TBD) – Ralf (ATT)
R1-2001861
Views on NR Rel-16 IAB Features
AT&T

R1-2001894
Discussion on NR Rel-16 IAB MT features
ZTE, Sanechips

R1-2002017
Discussion on UE features for IAB
Intel Corporation

R1-2002152
UE features for IAB
Samsung

R1-2002188
Discussion on RAN1 UE feature for NR IAB
LG Electronics

R1-2002564
Discussion on IAB UE features
Qualcomm Incorporated

R1-2002590
Rel-16 UE features for IAB
Huawei, HiSilicon

R1-2002653
Needed features for operator deployed IAB nodes
Ericsson

7.2.11.4 UE features for 5G V2X (4)
R1-2001867
Summary on UE features for 5G V2X
Moderator (AT&T)

[100b-e-NR-UEFeatures-V2X-01] Email discussion/approval of proposal conclusion in R1-2001867(It is RAN1’s understanding that LTE SL capability can be acquired by UE-EUTRA-Capability specified in TS 36.331)  by 4/22 – Ralf (ATT)
[100b-e-NR-UEFeatures-V2X-02] Email discussion/approval of proposal 2 in R1-2001867 by 4/24 – Ralf (ATT)
[100b-e-NR-UEFeatures-V2X-03] Email discussion/approval of Proposals {1, 3, 5, 6, 10, 11, 13, 14, 16, 17} (stabilizing the overall number of rows and separate FGs) – focusing on the high priority parts -  in R1-2001867 by 4/24 – Ralf (ATT)
[100b-e-NR-UEFeatures-V2X-04] Email discussion/approval of Proposals {3, 4, 5, 8, 11, 12, 13, 14, 15, 16, 17}– focusing on the medium priority parts -  in R1-2001867, dates TBD – Ralf (ATT)
R1-2001579
Discussion on Rel-16 5G_V2X_NRSL UE Features
ZTE, Sanechips

R1-2001753
Discussion on UE feature list for Release16 5G-V2X
OPPO

R1-2001783
Discussion on 5G V2X UE features
vivo

R1-2001827
Views on Rel-16 UE features for NR V2X
MediaTek Inc.

R1-2001891
Discussion on NR Rel-16 UE features for 5G V2X
LG Electronics

R1-2002018
Input to discussion on UE features for NR-V2X
Intel Corporation

R1-2002045
Features for Rel-16 V2X
Futurewei

R1-2002069
Discussion of UE features for NR V2X
CATT

R1-2002153
UE features for NR V2X
Samsung

R1-2002243
UE features for 5G V2X
Ericsson

R1-2002351
On NR UE V2X Features
Apple

R1-2002398
UE features for 5G V2X 
Panasonic Corporation

R1-2002483
On UE features for 5G V2X
Nokia, Nokia Shanghai Bell

R1-2002565
Discussion on V2X UE features
Qualcomm Incorporated

R1-2002666
Rel-16 UE features for 5G V2X
Huawei, HiSilicon

7.2.11.5 UE features for URLLC/IIoT (10)
R1-2002459
Summary on UE features for URLLC/IIoT
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-URLLC/IIoT-01] Email discussion/approval on the feature groups structure related to PDCCH enhancements for URLLC (20th-24th April) – Hiroki (DCM)

· Confirm to keep 11-1/1a/2/2b

· Discuss following on 11-1a

· Whether or not to introduce separate capabilities for DL and UL DCI format

· Discuss following on 11-2b

· Confirm to introduce separate capabilities for support of mixed Rel-16 PDCCH monitoring capability and Rel-15 PDCCH monitoring capability on different serving cells.

· Whether to introduce separate capabilities for mixed Rel-16 capability with Rel-15 PDCCH monitoring capability FG 3-1, FG 3-2, FG 3-5b on different serving cells.

· If the separate capabilities are introduced,

· Candidate values for capability on number of CCs with FG 3-1, FG3-2, and FG3-5b

[100b-e-NR-UEFeatures-URLLC/IIoT-02] Email discussion/approval on the feature groups structure related to UCI enhancements for URLLC (20th-24th April) – Hiroki (DCM)

· Confirm to keep 11-3/4

· Discuss following on 11-3

· Whether to introduce separate FGs for the simultaneous use of CBG-based UL transmission and minimum processing capability 2 (e.g., 11-3a/3b/3c/3d/3e)

· Discuss following on 11-4

· Whether or not to merge 11-4 with 12-1

· If not merged, whether or not to introduce separate UE capabilities for DCI format 0_1 and DCI format 0_2 based on 12-1

· Whether or not to introduce separate UE capabilities for DL priority and UL priority for component 4)

· Whether or not to introduce separate UE capabilities for DCI format 0_1/1_1 and DCI format 0_2/1_2 for component 4)
· Discuss whether or not to keep FG11-4x for two sub-slot based HARQ-ACK codebook construction

· Discuss whether or not to keep FG11-4a, and if yes, whether to split it into two rows, one for DL and one for UL

[100b-e-NR-UEFeatures-URLLC/IIoT-03] Email discussion/approval on the feature groups structure related to enhanced UL configured grant transmission for URLLC (20th-24th April) – Hiroki (DCM)

· Confirm to keep 11-9/10

· Discuss whether or not to merge FG11-9a with FG12-2a
· Discuss whether or not to FG11-10 is merged with FG11-11

· Discuss whether or not to further merge FG11-10 with FG12-3

[100b-e-NR-UEFeatures-URLLC/IIoT-04] Email discussion/approval on the feature groups structure related to other enhancements for URLLC (20th-24th April) – Hiroki (DCM)

· Confirm to keep 11-5/6/7/8

· Discuss whether to introduce a separate capability signaling for the support of InvalidSymbolPattern for 11-5

· Discuss followings on 11-7

· Whether or not to introduce separate UE capability for cross-carrier UL CI indication

· Whether or not to introduce separate UE capability for the support of more than one monitoring occasion for DCI 2_4 per slot

· Whether to introduce a FG (e.g. 11-7b) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of inter-UE cancelation
· Discuss whether or not to keep FG11-7a

· If removed, if UE reports to support FG11-7 and 6-23, it should automatically support the feature of FG 11-7a.

· Discuss whether or not to introduce following as new FG. If there is no consensus to add a new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

· “UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer”

· “Fixed TB CRC for interrupted initial PUSCH”

[100b-e-NR-UEFeatures-URLLC/IIoT-05] Email discussion/approval on the feature groups structure for NR IIoT (20th-24th April) – Hiroki (DCM)

· Confirm to keep 12-2/2a/5

· Discuss whether to introduce a FG (e.g. 12-1a) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of intra-UE prioritization

· Discuss whether or not to define following FGs:

· Support of SPS periodicity shorter than 10 ms

· Support of SPS activation by DCI format 1_2

· Discuss whether or not to introduce separate UE capabilities for support of SPS release by DCI format 1_1 and 1_2
[100b-e-NR-UEFeatures-URLLC/IIoT-06]  Email discussion/approval on issues with capability signaling impacts on FGs related to PDCCH enhancements for URLLC (dates TBD) – Hiroki (DCM)
· Discuss whether or not FG11-1 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether or not FG11-1a needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss followings on 11-2

· Whether or not reporting type is FSPC for FG11-2

· For component 5), whether different reporting type should be defined for component 5)

· Whether or not to merge component 1) and 3), and component 2) and 4)

· Whether or not to add a new component to indicate if the UE can support non-aligned spans

· Discuss whether or not report type of 11-2b is per UE per FSPC

· If it is per UE, 
· Whether or not FG11-2b needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 1

[100b-e-NR-UEFeatures-URLLC/IIoT-07]  Email discussion/approval on issues with capability signaling impacts on FGs related to UCI enhancements for URLLC (dates TBD) – Hiroki (DCM)
· Discuss followings on 11-3

· Confirm to remove component 3) and accordingly the note for component 3)

· Whether report type should be per UE or per FSPC or per band

· If it is per UE, 
· It can be confirmed that FG11-3 does not need “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· Discuss followings on 11-4

· Whether or not to include component 6) in FG 11-4

· Whether or not report type should be per UE or per FS

· If it is per UE, 
· Confirm FG11-4 does not need “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· Discuss whether report type of 11-4x should be per UE or per FSPC
· If it is per UE, 
· Confirm that FG11-4x does not need “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· Discuss whether FG11-4a needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 2

[100b-e-NR-UEFeatures-URLLC/IIoT-08]  Email discussion/approval on issues with capability signaling impacts on FGs related to enhanced UL configured grant transmission for URLLC (dates TBD) – Hiroki (DCM)
· Discuss followings on 11-9

· Whether the brackets of component 2 and 3 can be removed.

· Candidate value for component 2) and 3).

· Whether or not UE capability signaling should be on the number of ‘active’ CG configurations rather than ‘configured’ CG configurations.

· Whether or not report type should be per UE or per FSPC

· If it is per UE, 
· Whether FG11-9 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether or not FG11-9a needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether FG11-10 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 3

[100b-e-NR-UEFeatures-URLLC/IIoT-09]  Email discussion/approval on issues with capability signaling impacts on FGs related to other enhancements for URLLC (dates TBD) – Hiroki (DCM)
· Discuss followings on 11-5

· Whether or not FG11-5 includes component 3, 6, 8, and 9

· Whether report type should be per UE or per band

· If it is per UE, 
· Confirm FG11-5 does not need “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· Discuss followings on 11-6

· Whether to add “a component for the supported maximum number of PUSCH repetitions” or to remove it

· Whether or not report type should be per UE or per band

· If it is per UE, 
· Whether FG11-6 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss followings on 11-7

· Whether report type should be per UE or per FS

· If it is per UE, 
· whether FG11-7 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Confirm to remove the following FFS “FFS: Whether to add new FG with FG11-7 as prerequisite for the support of more than one monitoring occasion for DCI 2_4 per slot? Can we just add the following note to address the concern?”

· Discuss followings on 11-8

· Whether or not FG11-8 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 4

[100b-e-NR-UEFeatures-URLLC/IIoT-10]  Email discussion/approval on issues with capability signaling impacts on FGs for NR IIoT (dates TBD) – Hiroki (DCM)
· Discuss followings on 12-1

· Whether or not report type is per FSPC or per UE

· Whether or not to modify the description of note “A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]).”

· Discuss followings on 12-2

· Clarify that the component 3 is about all serving cells within a cell group or across different cell groups

· Candidate value for component 3)

· Whether report type should be per UE or per FSPC

· If it is per UE, 
· whether FG11-1 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether FG12-2a needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”
· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether or not FG12-3 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Discuss whether or not FG12-5 needs “FDD/TDD differentiation” and “FR1/FR2 differentiation”

· If differentiation is needed for both,
· Whether/how to clarify capability interpretation for “support mixture of FDD/TDD and/or FR1/FR2”

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussion in FL proposal 5

R1-2001632
Discussion on UE feature for URLLC/IIoT
ZTE

R1-2001721
Discussion on Rel-16 URLLC/IIOT UE features
vivo

R1-2001782
Discussion on UE features for URLLC/IIoT
OPPO

R1-2001791
On UE Features for URLLC and IIoT
Ericsson

R1-2001795
UE features for URLLC
China Unicom

R1-2001828
Views on Rel-16 UE features for NR URLLC/IIoT
MediaTek Inc.

R1-2001927
Discussion on UE features for URLLC/IIoT
LG Electronics

R1-2002019
On UE features for Rel-16 eURLLC and IIoT
Intel Corporation

R1-2002070
Discussion of UE features for NR URLLC/IIoT
CATT

R1-2002154
UE features for URLLC/IIoT
Samsung

R1-2002352
Discussions on UE Features for URLLC/IIoT
Apple

R1-2002399
UE features for URLLC/IIoT 
Panasonic Corporation

R1-2002482
On UE features for URLLC/IIOT
Nokia, Nokia Shanghai Bell

R1-2002566
Discussion on eURLLC and IIOT UE features
Qualcomm Incorporated

R1-2002591
Rel-16 UE features for URLLC
Huawei, HiSilicon

7.2.11.6 UE features for eMIMO (6)
R1-2001868
Summary on UE features for eMIMO
Moderator (AT&T)

[100b-e-NR-UEFeatures-eMIMO-01] Email discussion/approval till 4/24 – Ralf (ATT), (16-1 family)

· Decide the number of FGs for “L1-SINR reporting” by down-selecting between

· Alt. 1-1: one FG 

· Alt. 1-2: two FGs 

· Alt. 1-3: three FGs 

· Decide the number of FGs for “TCI state activation and spatial relation update” by down-selecting between 

· Alt. 2-1: one FG 

· Alt. 2-2: three FGs 

· Decide if “Resources for beam management, pathloss measurement, BFD, and BFR” is a feature group

· Alt. 3-1: no dedicated FG

· Alt. 3-2: one FG

[100b-e-NR-UEFeatures-eMIMO-02] Email discussion/approval till 4/24 – Ralf (ATT), (16-2 family)

· Decide how many new FGs to add as a result of separating basic from optional components in 16-2a “Multi-DCI based multi-TRP”

· Decide how many new FGs to add as a result of separating basic from optional components in 16-2b “Single-DCI based multi-TRP”

[100b-e-NR-UEFeatures-eMIMO-03] Email discussion/approval till 4/24 – Ralf (ATT),  (16-3 family)

· Decide how many new FGs to add as a result of separating basic from optional components in 16-3a “Regular eType-II”

· Decide how many new FGs to add as a result of separating basic from optional components in 16-3b “Port selection eType-II”

[100b-e-NR-UEFeatures-eMIMO-04] Email discussion/approval till 4/24 – Ralf (ATT), (16-5 family)

· Whether 16-5a “UL full power transmission mode 0” is deleted

· Whether 16-5c “UL full power transmission mode 2” is split into two FG

[100b-e-NR-UEFeatures-eMIMO-05] Email discussion/approval till 4/24 – Ralf (ATT), (16-6a)

· Whether 16-6a “Low PAPR DMRS for PUSCH” is split into two FG

[100b-e-NR-UEFeatures-eMIMO-06] Email discussion/approval of medium priority items as in Section 3 of R1-2001868 (dates TBD) – Ralf (ATT)
R1-2001604
NR eMIMO UE features
ZTE

R1-2001722
Discussion on Rel-16 eMIMO UE features
vivo

R1-2001738
Discussion on Rel-16 eMIMO UE features
OPPO

R1-2001794
UE features for MIMO
China Unicom

R1-2001829
Views on Rel-16 UE features for NR eMIMO
MediaTek Inc.

R1-2002020
UE features for NR eMIMO
Intel Corporation

R1-2002071
Discussion of UE features for NR MIMO
CATT

R1-2002155
UE features for eMIMO
Samsung

R1-2002161
Discussion on RAN1 UE feature for NR eMIMO
LG Electronics

R1-2002274
Discussions on UE features for eMIMO
Spreadtrum Communications

R1-2002353
Views on Rel-16 eMIMO UE feature list
Apple

R1-2002476
On UE features for eMIMO
Nokia, Nokia Shanghai Bell

R1-2002494
eMIMO UE features
Ericsson

R1-2002499
Discussion on UE features for eMIMO
CMCC

R1-2002567
Discussion on eMIMO UE features
Qualcomm Incorporated

R1-2002592
Rel-16 UE features for MIMO
Huawei, HiSilicon

R1-2002628
Discussion on UE capability issues  
Fraunhofer IIS, Fraunhofer HHI

7.2.11.7 UE features for UE power savings (5)
R1-2001869
Summary on UE features for UE power savings
Moderator (AT&T)

[100b-e-NR-UEFeatures-PowSav-01] Email  discussion/approval to resolve FFS in proposal 4 (19-4) as in R1-2001869 by 4/22 – Ralf (ATT)
[100b-e-NR-UEFeatures-PowSav-02] Email discussion/approval on the medium priority item as in proposal 1 as in R1-2001869 (4/24-4/28) – Ralf (ATT)
[100b-e-NR-UEFeatures-PowSav-03] Email discussion/approval on the medium priority item as in proposal 2 as in R1-2001869 by (4/24-4/28)– Ralf (ATT)
[100b-e-NR-UEFeatures-PowSav-04] Email discussion/approval on the medium priority item as in proposal 3  as in R1-2001869 by (4/24-4/28)– Ralf (ATT)
[100b-e-NR-UEFeatures-PowSav-05] Email discussion/approval on the medium priority item as in proposal 4 as in R1-2001869 by (4/24-4/28)– Ralf (ATT)
R1-2001587
Discussion on UE feature for UE  power saving
ZTE

R1-2001772
Discussion on Power Saving related UE features
OPPO

R1-2001830
Views on Rel-16 UE features for NR UE power savings
MediaTek Inc.

R1-2002021
On UE features for Rel-16 UE Power Saving
Intel Corporation

R1-2002072
Discussion of UE features for NR UE power savings
CATT

R1-2002425
Discussion on UE features for UEPS
Ericsson

R1-2002481
On UE features for UE Power Savings
Nokia, Nokia Shanghai Bell

R1-2002568
Discussion on power saving UE features
Qualcomm Incorporated

R1-2002593
Rel-16 UE features for UE power saving
Huawei, HiSilicon

R1-2002625
Discussion on Rel-16 UE power saving UE features
vivo

7.2.11.8 UE features for NR positioning (5)
R1-2002460
Summary on UE features for NR positioning
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-Positioning-01] Email discussion/approval on reconstructing the feature group structure for NR positioning (20th-24th April) – Hiroki (DCM)
· Reconstruct the features list to align with agreed capability signaling in RAN2

· Starting from rapporteur’s updated features list below

· In this email discussion, the target is to confirm reconstructed FGs that cover all the feature groups in R1-2001484
	13-1
	NR E-CID DL SSB RRM measurements for NR Positioning

	13-2
	NR E-CID DL CSI-RS RRM measurements for NR Positioning

	13-3
	Basic DL PRS Processing Capability

	13-4
	DL PRS QCL Processing Capability

	13-5
	DL PRS Resources for DL AoD

	13-6
	DL PRS Measurement Report for DL-AoD

	13-7
	DL PRS Resources for DL-TDOA

	13-8
	DL PRS RSTD Measurement Report for DL-TDOA

	13-9
	SRS Resources for Positioning

	13-10
	OLPC for SRS for Positioning

	13-11
	Spatial Relation for SRS for Positioning

	13-12
	DL PRS Resources for Multi-RTT

	13-13
	UE Rx-Tx Measurement Report for Multi-RTT


[100b-e-NR-UEFeatures-Positioning-02] Email discussion/approval on the need of possible new FGs for NR positioning (20th-24th April) – Hiroki (DCM)
· Discuss whether or not to introduce following new FGs. If there is no consensus to add a new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

· Support of the dedicated transmission of SRS for positioning

· Support of simultaneous processing of LTE PRS and NR PRS

· Support of simultaneous transmissions of SRS for positioning on a symbol

· Support of concurrent measurements (DL RSRP, DL RSTD, UE Rx-Tx Time Difference)

· Indication of concurrent configuration of list of measurements in supported CA Band Combination in the BandCombinationList

[100b-e-NR-UEFeatures-Positioning-03] Email discussion/approval on issues with capability signaling impacts for NR positioning based on DL PRS (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussions (e.g., FG13-3~8 in FL proposal 1)

[100b-e-NR-UEFeatures-Positioning-04]  Email discussion/approval on issues with capability signaling impacts for NR positioning based on SRS (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussions (e.g., FG13-9~11 in FL proposal 1)

[100b-e-NR-UEFeatures-Positioning-05] Email discussion/approval on issues with capability signaling impacts for NR positioning based on ECID and for multi-RTT (dates TBD) – Hiroki (DCM)
· Discuss on component(s) of each FG that need to be reported and candidate values for the component(s)

· Discuss on reporting type of each FG

· Discuss on the need of xDD and/or FRx differentiation for each FG of per-UE type

· Note that discussed FGs in this email discussion are derived by outcome of high priority email discussions (e.g., FG13-1/2/12/13 in FL proposal 1)

R1-2001605
NR positioning UE features
ZTE

R1-2001723
Discussion on UE features for Rel-16 NR positioning
vivo

R1-2001739
Discussion on UE features for NR Positioning
OPPO

R1-2001831
Views on Rel-16 UE features for NR positioning
MediaTek Inc.

R1-2001956
Discussion on UE features for NR positioning
LG Electronics

R1-2002022
Input to discussion on UE features for NR Positioning
Intel Corporation

R1-2002073
Discussion of UE features for NR positioning
CATT

R1-2002156
UE features for NR positioning
Samsung

R1-2002479
On UE features for NR Positioning
Nokia, Nokia Shanghai Bell

R1-2002569
Discussion on NR Positionign UE features
Qualcomm Incorporated

R1-2002712
Rel-16 UE features for NR positioning
Huawei, HiSilicon

Revision of R1-2002587
R1-2002624
View on UE feature description for NR positioning
Ericsson

7.2.11.9 UE features for NR mobility enhancements (4)
R1-2001870
Summary on UE features for NR mobility enhancements
Moderator (AT&T)

[100b-e-NR-UEFeatures-Mobility-01] Email discussion/approval proposal 1 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT)
· Note: “FL Proposal 5“ in R1-2001870 can be discussed within the scope of this email discussion
[100b-e-NR-UEFeatures-Mobility-02] Email discussion/approval proposal  2 and proposal 5 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT)

[100b-e-NR-UEFeatures-Mobility-03] Email discussion/approval proposal 3 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT)

[100b-e-NR-UEFeatures-Mobility-04] Email discussion/approval proposal 4 (high priority item) in R1-2001870 by 4/24 – Ralf (ATT)
R1-2001633
Discussion on UE feature for NR mobility enhancements
ZTE

R1-2001832
Views on Rel-16 UE features for NR mobility enhancements
MediaTek Inc.

R1-2002023
UE feature for NR Mobility Enhancement
Intel Corporation

R1-2002157
UE features for NR mobility enhancement
Samsung

R1-2002354
Views on NR mobility ehancement UE feature
Apple

R1-2002478
On UE features for NR Mobility Enhancements
Nokia, Nokia Shanghai Bell

R1-2002495
Mobility UE features
Ericsson

R1-2002570
Discussion on Mobility Enhancements UE features
Qualcomm Incorporated

R1-2002594
Rel-16 UE features for mobility enhancement
Huawei, HiSilicon

7.2.11.10 UE features for MR-DC/CA (5)
R1-2002454
Summary on UE features for MR-DC/CA
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-MRDCCA-01] Email discussion/approval on feature group structure for UL power sharing for NR-DC (20th-24th April) – Hiroki (DCM)
· Discuss whether to adopt FG18-1/18-1a/18-1b or FG[18-1]/[18-1a]/[18-1b]

· Alt.1: Adopt FG18-1/18-1a/18-1b (i.e., remove FG[18-1]/[18-1a]/[18-1b])

· It is clarified that FG18-1 is for both synchronous and asynchronous NR-DC scenarios

· It is clarified that FG18-1a is for synchronous NR-DC scenario only

· It is clarified for FG18-1b that T_offset is only used for dynamic power sharing with look-ahead 

· Alt.2: Adopt FG[18-1]/[18-1a]/[18-1b] (i.e., remove FG18-1/18-1a/18-1b)

· Whether [18-1] is removed or not, and whether it should be discussed in RAN or RAN1

· Alt.3: Other if any

[100b-e-NR-UEFeatures-MRDCCA-02] Email discussion/approval on feature group structure for cross-carrier operation with different SCS (20th-24th April) – Hiroki (DCM)
· Confirm that FG[18-5a] for “Default QCL assumption for cross-carrier scheduling” is kept (i.e., remove bracket)

· It is clarified that FG18-5a is only for same SCS

· Discuss whether new FG for “UL CA with mixed numerologies” is added or not

· Discuss whether new FG for “Cross-carrier scheduling with different SCS for URLLC” is added or not

· Discuss whether FG[18-6a] for “Default QCL assumption for cross-carrier A-CSI-RS triggering” is kept (i.e., remove bracket) or removed (i.e., added in 18-6)

· Discuss whether new FG for “Cross-carrier A-CSI-RS triggering with different SCS for URLLC” is added or not

· Confirm to keep FG18-5/6

[100b-e-NR-UEFeatures-MRDCCA-03] Email discussion/approval on feature group structure for other MR-DC/CA enhancements than UL power sharing for NR-DC and cross-carrier operation with different SCS (20th-24th April) – Hiroki (DCM)
· For 18-2 component 3, discuss whether or not to clarify HARQ subframe offset is optional for EN-DC with LTE TDD PCell

· Discuss whether new FG18-4b for “SCell dormancy indication without data scheduling within active time” is added or not

· Confirm to keep FG18-2/2a/3/3a/4/4a/7/8

[100b-e-NR-UEFeatures-MRDCCA-04]  Email discussion/approval on issues with capability signaling impacts for Enhancements to Single Tx Switched Uplink Solution for EN-DC (dates TBD) – Hiroki (DCM)
· Discuss followings on FG18-2/2a/3/3a

· Whether 18-3a is per band combination or per UE

· Whether the component 5 “if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using single UL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE)” for FG18-2/2a is kept (i.e., remove bracket) or removed

· Whether/how the FG18-2a/3a are modified so that the capabilities generally allow LTE UL transmissions outside the HARQ-ACK designated subframes for single-Tx operation

· Whether/how to clarify some details for FG18-2/2a/3/3a as below

· Whether or not to clarify that EN-DC single-Tx operation is for synchronous EN-DC

· For 18-2, whether or not “intra-band” in the name of FG should be removed

· For 18-2a, whether “TDD” should be removed from “Applicable to in FDD-LTE TDD-NR EN-DC” or not

· What is prerequisite for 18-3a

[100b-e-NR-UEFeatures-MRDCCA-05] Email discussion/approval on issues with capability signaling impacts for other MR-DC/CA enhancements than single Tx switched uplink solution for EN-DC (dates TBD) – Hiroki (DCM)
· Discuss whether FG18-4/4a/[4b] are per band combination or per UE

· Discuss followings on FG18-5

· Whether per band is added for FG18-5/[5a] or not

· Whether the component 2 of 18-5 “Processing up to X unicast DCI scheduling (DL and UL) per scheduled CC” is kept or removed

· Whether the component 3 of 18-5 is added to FG[18-5a] or not

· Whether or not to define the maximum number of unicast DCIs in one scheduling slot/span across all scheduled cells

· Discuss whether per band is added for FG18-6/[6a] or not

· Discuss whether/how to define a signaling structure where the UE can indicate the grouping of cells across which the UE is capable of applying time offsets

R1-2001631
Discussion on UE feature for MR-DC CA
ZTE

R1-2001833
Views on Rel-16 UE features for MR-DC/CA
MediaTek Inc.

R1-2002024
UE feature for MR-DC
Intel Corporation

R1-2002426
Discussion on UE features for MR-DC
Ericsson

R1-2002477
On UE features for MR-DC/CA
Nokia, Nokia Shanghai Bell

R1-2002571
Discussion on UE features for MR-DC/CA
Qualcomm Incorporated

R1-2002595
Rel-16 UE features for MR-DC/CA
Huawei, HiSilicon

7.2.11.11 UE features for CLI/RIM (1)
R1-2002455
Summary on UE features for CLI/RIM
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-CLIRIM-01] Email discussion/approval on issues with capability signaling impacts for CLI/RIM (20th-24th April) – Hiroki (DCM)
· Confirm to keep FG17-1/2/3/4

· Discuss following on FG17-1

· Whether the maximum number of measurement resources configured for CLI-RSSI measurement is reported or not

· Alt.1: UE reports both maximum number of measurement resources configured for CLI-RSSI measurement and maximum number of measurement resources configured for CLI-RSSI measurement within one slot

· Alt.2: UE reports maximum number of measurement resources configured for CLI-RSSI measurement

· Alt.3: UE has to support 64 CLI-RSSI measurement resource in order to support CLI-RSSI

· If the maximum number of measurement resources configured for CLI-RSSI measurement is reported in FG17-1, what are candidate values

· Whether the component 2 “Subcarrier spacing for CLI-RSSI measurement is same as subcarrier spacing for active BWP” is necessary or not

· Whether FG17-1 is reported per band or per UE

· Discuss followings on FG17-2

· Whether the maximum number of measurement resources configured for SRS-RSRP measurement is reported or not

· Alt.1: UE reports both maximum number of measurement resources configured for SRS-RSRP measurement and maximum number of measurement resources configured for SRS-RSRP measurement within one slot

· Alt.2: UE reports maximum number of measurement resources configured for SRS-RSRP measurement

· Alt.3: UE has to support 32 SRS-RSRP measurement resource in order to support SRS-RSRP

· If the maximum number of measurement resources configured for SRS-RSRP measurement is reported in FG17-2, what are candidate values

· Whether FG17-2 is reported per band or per UE

· Whether a joint limit for CLI SRS, SSB and CSI-RS for RSRP measurement in a slot is necessary or not

· Whether NR supports multi-port SRS-RSRP measurement or not

R1-2001588
Discussion on UE feature for CLI
ZTE

R1-2001740
Discussion on UE features for CLI/RIM
OPPO

R1-2002158
UE features for CLI/RIM
Samsung

R1-2002279
UE features for CLI/RIM
Ericsson

R1-2002404
Discussion on Rel-16 CLI UE features
vivo

R1-2002488
On UE features for CLI/RIM
Nokia, Nokia Shanghai Bell

R1-2002572
Discussion on UE features for CLI
Qualcomm Incorporated

R1-2002596
Rel-16 UE features for CLI/RIM
Huawei, HiSilicon

R1-2002686
UE features for CLI/RIM
Intel Corporation

7.2.11.12 UE features for TEIs (2)
R1-2002456
Summary on UE features for TEIs
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-TEIs-01] Email discussion/approval on feature group structure for NR TEI (20th-24th April) – Hiroki (DCM)
· Confirm to keep FG14-1/2/3/4/5/6

· Discuss whether the bracket for FG14-1a is removed or FG14-1a is removed

· Discuss whether FG14-5 includes component 2 “Support for directional collision handling between reference and other cell(s) for half-duplex operation in CA with different SCS” (i.e., 14-5a is removed) or 14-5a is separately defined (i.e., component 2 for FG14-5 is removed)

· Discuss whether the bracket for FG14-7 is removed or FG14-7 is removed

· Discuss whether the FG for CSI trigger states containing non-active BWP is introduced or not. If there is no consensus to add the new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

[100b-e-NR-UEFeatures-TEIs-02] Email discussion/approval on issues with capability signaling impacts for NR TEI (dates TBD) – Hiroki (DCM)
· Discuss whether/how to report capability signaling for the component 2 “Up to 3 LTE-CRS non-overlapping rate matching patterns within a NR carrier” of FG14-1

· Alt.1: UE reporting component 1 for 14-1 also reports component 2 from {1, 2, 3}.

· Alt.2: UE reporting component 1 for 14-1 also reports component 2 from {2, 3}.

· Alt.3: UE does not report component 2, i.e., UE reporting component 1 larger than 2 supports component 2 (up to 3 patterns within a NR carrier), and UE reporting component 1 as 2 supports up to 2 patterns within a NR carrier

· Discuss whether FG14-2 is reported per band or per UE

· Discuss whether FG14-3 is reported per UE or per band

· Discuss followings for FG14-4

· Whether or not FG14-4 includes component 2 and 3

· Whether or not component 1 for FG14-4 i.e., signaling of xTyR configuration in Rel-16, only supports downgraded xTyR configurations which are decoupled from highest xTyR reported in Rel-15

· Discuss whether FG14-5 (and FG14-5a if defined) is reported per band combination or per UE

· Discuss followings for FG14-7 (if the bracket for FG14-7 is removed)

· What is the component(s) of FG14-7

· Whether the FG14-7 is per band or per UE

· Confirm following updates

· For 14-1 and [14-1a]

· The bracket for the note “The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot” can be removed.

· For “Need of FR1/FR2 differentiation”, it can be clarified that FG14-1 is only for FR1, i.e., “N/A (FR1 only)”.

· The FG14-4 is reported per band combination.

· For 14-5 and [14-5a]

· For “Need of FDD/TDD differentiation”, it can be clarified that FG14-5 is only for TDD, i.e., “N/A (TDD only)”.

· For “Need of FR1/FR2 differentiation”, it can be “N/A” in case that “type” is per band or can be “No” in case that “type” is per UE.

· For “Need of FR1/FR2 differentiation” of FG14-6, “N/A (FR1only]” should be revised to “N/A (FR1 only)”.  

R1-2001724
Discussion on UE TEI feature 14-7
vivo

R1-2001741
Discussion on Rel-16 UE features for TEIs
OPPO

R1-2001834
Views on Rel-16 UE features for NR TEIs
MediaTek Inc.

R1-2002025
UE features for NR TEI
Intel Corporation

R1-2002280
UE features for TEIs
Ericsson

R1-2002573
Discussion on UE features for TEI
Qualcomm Incorporated

R1-2002597
Rel-16 UE features for TEIs
Huawei, HiSilicon

7.2.11.13 Others (2)
Including interactions among UE features across WIs
R1-2002457
Summary on NR UE features for others
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-UEFeatures-Others-01] Email discussion/approval on updates for Rel-15 capabilities (20th-24th April) – Hiroki (DCM)
· Confirm the updated FG8-1

· Discuss whether the FGs [5-11c]/[5-12c]/[5-13g]/[5-13h] for up to 3 unicast PDSCHs (PUSCHs) per slot per CC for different TBs are introduced or removed. If there is no consensus to add a new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

· Discuss whether the FG [5-35] for simultaneously enabling CBG and multiple PDSCHs per slot is introduced or removed. If there is no consensus to add a new feature group at the end of this email discussion, the new feature group is not introduced in Rel-16.

[100b-e-NR-UEFeatures-Others-02]: Email discussion/approval on general issues having capability signaling impact (dates TBD) – Hiroki (DCM)
· Whether to convert most per-UE capability to per-band capability
R1-2001634
Remaining issues on Rel-16 NR UE features
ZTE

R1-2001742
Discussion on the support of SRS transmission in all symbols of a slot
OPPO

R1-2002026
On UE feature list
Intel Corporation

R1-2002159
UE features for other aspects
Samsung

R1-2002281
Potential change/update on existing UE features for Rel-16 UE
Ericsson

R1-2002656
High-level discussion on Rel-16 UE features
Futurewei

R1-2002674
Other aspects of Rel-16 UE features
Huawei, HiSilicon

R1-2002687
Discussion on UE features
Qualcomm Incorporated

7.2.12 Others (3)
Including any essential TPs related to the endorsed Rel-16 NR TEIs and CLI/RIM.

R1-2002461
Summary on Rel-16 NR TEI related discussion
Moderator (NTT DOCOMO, INC.)

[100b-e-NR-TEIs-01]: Email discussion/approval of TPs on remaining issues for CLI measurement and reporting 

· Whether/how to capture agreed UE behavior on SRS-RSRP measurement in TS38.214

· TPs in R1-2001589 and R1-2001957 are starting points for the discussion

· Whether/how to capture agreed UE behavior on CLI-RSSI measurement in TS38.331

· TP in R1-2001957 is a starting point for the discussion

till 4/23, and if LS is necessary (for the 2nd sub-bullet), LS approval till 4/28 (DCM, Hiroki)
[100b-e-NR-TEIs-02]: Email discussion/approval of TPs on remaining issues for half-duplex operation in CA

· Whether/how to capture the agreement which is not correctly reflected in TS38.213

· TPs in R1-2002074 and R1-2002229 are starting points for the discussion

· Whether/how to cover mixed numerology case

· TP in R1-2002229 is a starting point for the discussion

till 4/23 (DCM, Hiroki)
100b-e-NR-TEIs-03]: Email discussion/approval on whether or not (and if yes, how) the text below in UE feature 6-1 needs to be updated:
· This feature should be mandatory without capability signaling for at least BWPs which is the same as the set of specified channel BW. RAN4 may discuss other BW requirements.

Note: this is due to the motivation that an operator may need to deploy NR with reduced BW between 5MHz and 10MHz due to necessity to transmit TRS within whole 10MHz BWP, i.e. 52PRBs. 

Till 4/24 – Hiroki (DCM)
R1-2001589
Discussion on CLI and TBS ambiguity
ZTE

R1-2001957
Remaining details of CLI measurement and reporting at a UE
LG Electronics

R1-2002027
Maintenance of aperiodic CSI-RS triggering with beam switching timing of 224 and 336
Intel Corporation

R1-2002074
Remaining issues of half-duplex operation in CA
CATT

R1-2002170
On TRS muting for NR coexistence with a narrow band system
MediaTek Inc.

R1-2002229
On remaining NR TEI issues
Nokia, Nokia Shanghai Bell

R1-2002282
Remaining issues for Rel-16 maintenance and TEI
Ericsson

R1-2002355
Considerations on HARQ/CSI enhancements
Apple

R1-2002679
Discussion on conditions of rate matching pattern overlapping with PDSCH DMRS symbols


Huawei, HiSilicon

_1648726717.unknown

