3GPP TSG RAN WG1 #100bis		R1- 200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda item:	6.2.2.4

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Email discussion [100b-e-LTE-NB_IoTenh3-Coex-NR-04]

Document for:	Discussion and Decision

# Background

This document contains the discussion for the following:

[100b-e-LTE-NB_IoTenh3-Coex-NR-04]  Email approval of the reply LS for R1-2001518 for both eMTC and NB-IoT by 4/23 – Alberto (Qualcomm)

The following tdocs have been submitted to this agenda item regarding this LS:

- R1-2001848	Discussion on RAN2 LS on NR coexistence	ZTE

- R1-2002502	On the LS on NR coexistence for NB-IoT/eMTC	Ericsson

- R1-2002602	Draft reply LS on NR coexistence	Huawei, HiSilicon

- R1-2002175	Coexistence of LTE-MTC with NR	Qualcomm Incorporated

- R1-2002177	Coexistence of NB-IoT with NR	Qualcomm Incorporated

# Question 1

Question 1: For NB-IoT, which, if any, resource reservation parameters for NR are likely to be the same across different carriers?

Question 1.1: If some resource reservation parameters for NR can be same across different NB-IoT carriers then can all the parameters be the same for all the NB-IoT carriers configured via dedicated signalling?

- Ericsson: Not all carriers have the same configuration, but not every carrier will have a different configuration – RAN2 can configure each carrier selected from a small set (e.g. two) of different configurations. Additionally, it should be possible to disable resource reservation for a given carrier.

- ZTE: None of the resource reservation parameters are likely to be the same for different carriers (regardless of these being configured by dedicated signaling or not).

- Huawei: It should be allowed that all the resource reservation parameters are different.

- Qualcomm: Periodicity/startPosition/subframe(slot) level resource/time domain reservation can be cell common. Bitmaps may be different for different carriers – a default parameter can be provided in SIB, which may be overridden by dedicated signaling.

Q1-1: Is it acceptable to have common values for periodicity / startPosition / time domain reservation across different carriers?

  - Option 1: They are likely to be common, but they may be different.

  - Option 2: RAN2 can assume that these values are common.

  - Option 3: None of these values are likely to be common.

| Company name | Comment |
| --- | --- |

Q1-2: Is it acceptable to have any of the following signaling optimizations?

  - Option 1: Override SIB parameters by unicast RRC.

  - Option 2: Signal a small set of configurations in SIB. Each carrier includes a pointer to one of these configurations

| Company name | Comment |
| --- | --- |

# Question 2

Question 2: For NB-IoT and eMTC, which, if any, parameters are likely to be the same for uplink and downlink?

- Ericsson, ZTE: It cannot be assumed that any parameters are likely to be the same.

- Huawei:

  - For NB-IoT and eMTC, periodicity / startPosition/subframeBitmap/slotBitmap may be common for uplink and downlink.

  - For eMTC, symbolBitmap may be the same for downlink and uplink.

- Qualcomm: It is desirable to configure uplink and downlink separately.

Q2-1: On common values for periodicity / startPosition / subframeBitmap/slotBitmap for uplink and downlink, plus additionally symbolBitmap for eMTC:

  - Option 1: They are likely to be common, but they may be different.

  - Option 2: RAN2 can assume that these values are common.

  - Option 3: None of these values are likely to be common.

| Company name | Comment |
| --- | --- |

# Question 3

Question 3: For NB-IoT and eMTC, which, if any, parameters are likely to be the same in neighbour cells on the same carrier frequency?

- Ericsson, ZTE: It cannot be assumed that any parameters are likely to be the same.

- Huawei: If the same SSB pattern is configured in neighbor cells, resource reservation are the same in the same carrier frequency.

- Qualcomm: In some cases, the same resource reservation may be present in the same carrier frequency, but it should be possible to override (e.g. with delta configuration)

Q3-1: Can parameters be common across neighbor cells?

  - Option 1: They are likely to be common, but they may be different.

  - Option 2: They may be common depending on the deployment and configurations.

  - Option 3: None of these values are likely to be common.

In all cases, RAN2 should provide signaling support to indicate different patterns.

| Company name | Comment |
| --- | --- |

# Draft LS response <To be drafted after receiving responses>