3GPP TSG RAN WG1 #100bis	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.10.4

Source:	Moderator (Nokia)

Title:	Discussion summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:	Discussion and Decision

1	Introduction

This document is used to collect companies views for the issues identified for AI 7.2.10.4 email discussion thread:

[100b-e-NR- LTE_NR_DC_CA-X-CC A-CSI-RS-01] Email approvals of the TPs based on the following issues #1/2/3/4 in R1-2002611 till 4/23 (Nokia, Karri).

2	Companies’ views on discussion topics

2.1	Issue #1

| Description | Source |
| --- | --- |
| Scaling of the beam switching timing d to the applied SCS is missing from three places | Huawei, vivo |

FL proposal: Adopt the TP proposed in R1-2001543 (same TP in both Huawei and Vivo doc) to TS38.214 subclause 5.2.1.5.1a

Companies’ comments:

| Company | Comment |
| --- | --- |
| MTK | We are fine to adopt the FL proposal. |

2.2	Issue #2

| Description | Source |
| --- | --- |
| In agreements of RAN1 #100-e meeting, the value of beam switching timing d is added for cross-carrier aperiodic CSI-RS triggering. But the description of d value only is for the case that the scheduling offset is smaller than the threshold. The same description of d value can be supplemented for the case that the scheduling offset is equal to or greater than the threshold. | vivo |

FL proposal: Adopt the TP proposed in R1-2001690/Proposal 2 to TS38.214 subclause 5.2.1.5.1a

Companies’ comments:

| Company | Comment |
| --- | --- |
| MTK | We are fine to adopt the FL proposal. |

2.3	Issue #3

| Description | Source |
| --- | --- |
| Incorrect location for the RAN1#100e TP “including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states”, introduced to 5.2.1.5.1 instead of the intended 5.2.1.5.1a. | Oppo, MTek, Ericsson |

FL proposal: Confirm that the text was introduced in a wrong location and move it according to the proposal in R1-2001737 from 5.2.1.5.1 to 5.2.1.5.1a of TS38.214 (essentially the same TP in all three Tdocs)

Companies’ comments:

| Company | Comment |
| --- | --- |
| MTK | We are fine to adopt the FL proposal. |

2.4	Issue #4

| Description | Source |
| --- | --- |
| The agreement for default QCL assumption without configured CORESET is only captured in 5.2.1.5.1a for the case that PDCCH and the triggered aperiodic CSI-RS have different SCSs but not in 5.2.1.5.1 for the case that PDCCH and the triggered aperiodic CSI-RS have the same SCS. | Qualcomm |

FL proposal: Discuss if the Proposal 1 of R1-2002561 (below) is agreeable, and if so, if the corresponding TP to TS38.214 subclause 5.2.1.5.1 can be agreed:

Proposal 1: In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification. Adopt the proposed text proposal in 5.2.1.5.1 in TS 38.214

- If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID.

- else if the active BWP of the serving cell for receiving the aperiodic CSI-RS has configured ControlResourceSet, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.;

- else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

Companies’ comments:

| Company | Comment |
| --- | --- |
| MTK | In MR DC/CA UE features discussion, it is discussing - “whether FG[18-6a] for “Default QCL assumption for cross-carrier A-CSI-RS triggering” is kept (i.e., remove bracket) or removed (i.e., added in 18-6)“ If FG[18-6a] is kept, then we can agree on the Proposal. Else, we can not agree on the proposal for now. |

References

- R1-2001543	Remaining issues on the A-CSI RS triggering with different numerology	Huawei, HiSilicon

- R1-2001690	Remaining issues on aperiodic CSI-RS triggering	vivo

- R1-2001737	Text proposals for aperiodic CSI-RS triggering with different numerologies	OPPO

- R1-2001836	Remaining issues on X-carrier ACSI-RS triggering with different SCS	MediaTek Inc.

- R1-2002496	Correction to aperiodic CSI-RS triggering with mixed numerology	Ericsson

- R1-2002561	Remaining issues for aperiodic CSI-RS triggering with different numerology	Qualcomm Incorporated

- R1-2002611	FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH	Moderator (Nokia)

- R1-2002612	FL summary #2 on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH	Moderator (Nokia)