3GPP TSG RAN WG1 #100bis-e	         	 R1-200XXXX

e-meeting, April 20-30, 2020

Agenda item:		7.2.2.2.1

Source:	Moderator (Nokia)

Title:	TPs related to the email discussion [100b-e-NR-unlic-NRU-ChAcc-02]

Document for:		Discussion and Decision

# 1	Introduction

This document captures the text proposals in the following RAN1#100bis-e email thread:

[100b-e-NR-unlic-NRU-ChAcc-02] Email discussion/approval on clarifications to UL to DL COT sharing by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Timo (Nokia)

During the preparation phase it was identified that the following TDocs and proposals relate to corrections and clarifications to UL to DL COT sharing:

Issue #3 Clarifications to UL to DL COT sharing

| Clarifications to UL to DL COT sharing | R1-2001652 (2.2) R1-2001705 (2.1) R1-2001759 (2.3, 2.4) R1-2001935 (p5, p6) R1-2002247 (p1) R1-2002530 (p3) R1-2002632 (p1) R1-2002684 (p1, p2, p3) |
| --- | --- |

This contribution provides text proposals for some of the related aspects.

# 2. TPs

## 2.3 Clarification of the max duration of a UL-DL shared COT

From the agreement in [100b-e-NR-unlic-NRU-ChAcc-02]:

Further discussion on TPs for Sections 4.2.1 and 4.1.3 of 37.213 based on the inputs until 4/29

TS 37.213 --- Beginning of Text proposal #1 ---- 4.2.1 Channel access procedures for uplink transmission(s) <unchanged text omitted> A UE shall use Type 1 channel access procedure for transmissions related to random access procedure that initiate a channel occupancy with UL channel access priority class p=1 in Table 4.2.1. A UE shall not transmit on a channel for a Channel Occupancy Time that exceeds Tmcot,p where the channel access procedures are performed based on a channel access priority class p associated with the UE transmissions, as given in Table 4.2.1-1. The total duration of autonomous uplink transmission(s) obtained by the channel access procedure in this clause, including the following DL transmission if the UE sets 'COT sharing indication' in AUL-UCI to '1' in a subframe within the autonomous uplink transmission(s) as described in Subclause 4.1.3, shall not exceed Tulmcot,p, where Tulmcot,p is given in Table 4.2.1-1. <unchanged text omitted> --- End of the Text proposal #1 ----

TS 37.213 --- Beginning of Text proposal #2 ---- ### 4.1.3 DL channel access procedures in a shared channel occupancy <unchanged text omitted> If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a UL transmission on scheduled resources or a PUSCH transmission on configured resources by the UE after a gap as follows: <unchanged text omitted> --- End of the Text proposal #2 ----

Companies are asked to provide their views on the TPs using the Table below:

| Company / Org. | View on FL proposal #4 |
| --- | --- |

## 2.4 COT sharing indication in CG-UCI

From the agreement in [100b-e-NR-unlic-NRU-ChAcc-02]:

Finalize TP for the editorial corrections for COT sharing indication in CG-UCI until 4/29

TS 37.213 --- Beginning of Text proposal #3 ---- ### 4.1.3 DL channel access procedures in a shared channel occupancy <unchanged text omitted> For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows: - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is provided, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer. Each row of the table provides a channel occupancy sharing information given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing is not available. <unchanged text omitted> --- End of the Text proposal #3 ----

Companies are asked to provide their views on the TP using the Table below:

| Company / Org. | View on FL proposal #4 |
| --- | --- |

## 2.6 Clarifications

From the agreement in [100b-e-NR-unlic-NRU-ChAcc-02]:

Further discussion on TPs based on TP1 in R1-2002247 and the comments in the email discussion until 4/29

TS 37.213 --- Beginning of Text proposal #4 ---- ### 4.1.3 DL channel access procedures in a shared channel occupancy <unchanged text omitted> If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a UL transmission on scheduled resources or a PUSCH transmission on configured resources by the UE after a gap as follows: <unchanged text omitted> --- End of the Text proposal #4 ----

Companies are asked to provide their views on the TP using the Table below:

| Company / Org. | View on FL proposal #4 |
| --- | --- |

# 3. Conclusions

TBA

# References

| 1 | R1-2001534 | Maintainance on the channel access procedure | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2001652 | Remaining issues on the channel access procedures | vivo |
| 3 | R1-2001705 | Remaining issues on the channel access procedure for NR-U | ZTE, Sanechips |
| 4 | R1-2001759 | Discussion on the remaining issues of channel access procedure | OPPO |
| 5 | R1-2001935 | Remaining issues of channel access procedure for NR-U | LG Electronics |
| 6 | R1-2001987 | Channel access mechanism for NR-unlicensed | Intel Corporation |
| 7 | R1-2002031 | Channel access procedures | Ericsson |
| 8 | R1-2002117 | Channel access procedures for NR-U | Samsung |
| 9 | R1-2002193 | Remaining Issues on Channel Access Procedures for NR-U | Nokia, Nokia Shanghai Bell |
| 10 | R1-2002247 | Remaining issues on channel access procedures for NR-U | ETRI |
| 11 | R1-2002383 | Remaining issues and corrections on channel access procedure for NR-U | Sharp |
| 12 | R1-2002405 | Remaining issues on channel access for NR-U operation | MediaTek Inc. |
| 13 | R1-2002434 | Remaining issues on channel access procedures for NR-U | NTT DOCOMO, INC. |
| 14 | R1-2002465 | TP on shared spectrum in NR-U | NEC |
| 15 | R1-2002530 | TP for Channel access procedures for NR unlicensed | Qualcomm Incorporated |
| 16 | R1-2002632 | Remaining issues on channel access procedure for NR-U | WILUS Inc. |
| 17 | R1-2002684 | COT sharing information in CG-UCI | Lenovo, Motorola Mobility |