3GPP TSG RAN WG1 #100bis-e	         	 R1-200XXXX

e-meeting, April 20-30, 2020

Agenda item:		7.2.2.2.1

Source:	Moderator (Nokia)

Title:	Summary of [100b-e-NR-unlic-NRU-ChAcc-02] Email discussion/approval on clarifications to UL to DL COT sharing

Document for:		Discussion and Decision

# 1	Introduction

This document captures the discussion in the following RAN1#100bis-e email thread:

[100b-e-NR-unlic-NRU-ChAcc-02] Email discussion/approval on clarifications to UL to DL COT sharing by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Timo (Nokia)

During the preparation phase it was identified that the following TDocs and proposals relate to corrections and clarifications to UL to DL COT sharing:

Issue #3 Clarifications to UL to DL COT sharing

| Clarifications to UL to DL COT sharing | R1-2001652 (2.2) R1-2001705 (2.1) R1-2001759 (2.3, 2.4) R1-2001935 (p5, p6) R1-2002247 (p1) R1-2002530 (p3) R1-2002632 (p1) R1-2002684 (p1, p2, p3) |
| --- | --- |

Note: Proposals p2 and p3 in R1-2002632 will be discussed in a later meeting together with other CWS update related CRs.

# 2. Issues identified in the contributions

To organize the email discussion, the issues have been grouped according to the chairman’s guidance.

## 2.1 ED Threshold for COT sharing

R1-2001652 Proposal 2: The UL to DL COT sharing ED threshold should be dynamically indicated to the UE or gNB.

R1- 2001935 Proposal #5: For a UE configured with ED threshold to be used for UE-initiated channel occupancy, the UE is allowed to select between configured ED threshold and ED threshold calculated based on UE’s configured maximum transmission power. If the UE does not choose the configured ED threshold, the UE indicates the row index corresponding to no COT sharing information in CG-UCI. Proposal #6: For a UE configured with ED threshold to be used for UE-initiated channel occupancy, UL grant indicates which ED threshold between configured ED threshold and ED threshold calculated based on UE’s configured maximum transmission power is applied to channel access procedure for the scheduled PUSCH.

FL Proposal #1: Discuss whether the above two proposals are agreeable.

| Company / Org. | View on FL proposal #1 |
| --- | --- |

## 2.2 UL-DL gap > 25 us

R1-2001705 Proposal 1: For the COT sharing, the gNB behavior of the gap between a DL transmission and the last PUSCH transmission is larger than 25us shall be clarified. It is preferred to follow the same behavior with the case that the gap is equal to 25us. 4.1.3 DL channel access procedures in a shared channel occupancy --------------------------------------------------------- Start of TP #1---------------------------------------------------------------- If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: - The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. - If the higher layer parameters ul-toDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. - If the gap is at least 25us or equal to 16us, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. <unchanged part omitted> For the case where a gNB uses channel access procedures as described in clause 4.1.1 to initiate a transmission and shares the corresponding channel occupancy with a UE that transmits a transmission as described in clause 4.2.1.2, the gNB may transmit a transmission within its channel occupancy that follows the UE's transmission if any gap between any two transmissions in the gNB channel occupancy is at least 25us or up to 16us. In this case the following applies: - If the gap is at least 25us or equal to 16us, the gNB can transmit the transmission on the channel after performing Type 2A or 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. <unchanged part omitted> --------------------------------------------------------- End of TP #1-----------------------------------------------------------------

R1-2002530 Proposal 3. For UL to DL COT sharing, if the gap is more than 25 us, Type 2A channel access can be used by gNB to transmit ==============TP for 37.213 4.1.3=================================== ----------------unchanged text removed----------------------- If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: - The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. - If the higher layer parameters ul-toDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. - If the gap is no less than 25us or is 16us, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively.

R1-2001537 Proposal 1：Some rows should be added to the cg-COT-SharingList-r16 table such that each additional row indicates a possible UL-burst-end symbol within a slot. - If the gNB receives a CG-UCI indicating a row index providing a UL-burst-end before the start of a slot-based DL transmission opportunity sharing the UL COT, the gNB may transmit the DL transmission after a gap from the symbol indicated by the UL-burst-end in the immediately preceding slot. ## *** <Beginning of Text Proposal 1> *** ### 4.1.3 DL channel access procedures in a shared channel occupancy ## *** Unchanged text is omitted *** For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows: - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is provided, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer. Each row of the table provides a channel occupancy sharing information given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing information is not available. - If the 'COT sharing information' in CG-UCI indicates a row index that corresponds to a CG-COT-Sharing-r16 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy assuming a channel access priority class p=channelAccessPriority-r16, starting from O=offset-r16 slots from the end of the slot where CG-UCI is detected, for a duration of D=duration-r16 slots where duration-r16, offset-r16 and channelAccessPriority-r16 are higher layer parameters provided by CG-COT-Sharing-r16. - If additional rows are configured in the table provided by cg-COT-SharingList-r16 wherein each additional row indicates a possible ‘UL-burst-end’ symbol within a slot and if the gNB receives a 'COT sharing information' in a CG-UCI indicating a row index that corresponds to a ulBurstEnd-r16 before the start of a DL transmission opportunity sharing the UL COT, the gNB assumes that the CG UL burst will end at the symbol indicated by the ulBurstEnd-r16 in the slot immediately preceding the slot determined by the parameter offset-r16 in another CG-UCI received before the start of the DL transmission opportunity. In that case, the gNB may transmit the DL transmission after a gap duration specified earlier in this subclause from the symbol indicated by the ulBurstEnd-r16. - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is not provided, and if 'COT sharing information' in CG-UCI indicates '1', the gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16 symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer. The transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. ## *** Unchanged text is omitted *** ## *** <End of Text Proposal 1> ***

FL Proposal #2: Discuss whether and how to capture the support for UL-DL gap larger than 25 us in a UE-initiated COT

| Company / Org. | View on FL proposal #2 |
| --- | --- |

## 2.3 Clarification of the max duration of a UL-DL shared COT

R1-2001759 Proposal 4: For uplink COT sharing, clarify that the total duration of UL and DL transmissions should not exceed the MCOT which initiated by the UE. - Adopt TP4 into section 4.1.3 of TS 37.213. ----------------------------------- TP4: Start of TP 37.213 section 4.1.3 --------------------------------------------- 4.1.3 DL channel access procedures in a shared channel occupancy <Unchanged parts are omitted> If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in subclause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: - The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. - If the higher layer parameters ul-toDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. - The total duration of a PUSCH transmission on scheduled or configured resources including the following DL transmission obtained by Type 1 channel access procedure with UL channel access priority class p, shall not exceed Tulmcot,p, where Tulmcot,p is given in Table 4.2.1-1. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in subclause 4.1.2.3. - If the gap is 25us or 16us, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in subclause 4.1.2.1 and 4.1.2.2, respectively. <Unchanged parts are omitted> ----------------------------------------End of TP 37.213 section 4.1.3 ----------------------------------------------

FL Proposal #3: Discuss whether and how to capture the above clarification into 37.213

| Company / Org. | View on FL proposal #3 |
| --- | --- |

## 2.4 COT sharing indication in CG-UCI

R1-2001759 Proposal 5: The gNB shall ignore the COT sharing indication in CG-UCI if the CG-UCI and the corresponding CG-PUSCH is transmitted within the gNB’s COT. - Adopt TP5 into section 4.2.1.0.0 of TS 37.213. -----------------------------------TP5: Start of TP 37.213 section 4.2.1.0.0------------------------------------ 4.2.1.0.0 Channel access procedures upon detection of a common DCI <Unchanged parts are omitted> If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in subclause 11.1.1 of [7], the following is applicable: - The UE may switch from Type 1 channel access procedures as described in subclause 4.2.1.1 to Type 2A channel access procedures as described in subclause 4.2.1.2.1 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy. In this case, if the UL transmissions are PUSCH transmissions on configured resources, the UE may assume any priority class for the channel occupancy shared with the gNB. The gNB shall ignore the ‘COT sharing information’ in CG-UCI. <Unchanged parts are omitted> ----------------------------------------End of TP 37.213 section 4.2.1.0.0------------------------------------------

R1-2002684 Proposal 1: COT sharing information in CG-UCI for indicating the shared resources is updated slot by slot; the indication is consistent. Proposal 2: Upon reception of COT sharing information in CG-UCI, only DL unicast data with CAPC value not larger than the UL CAPC value indicated by the COT sharing information is allowed to be transmitted in the shared resources. Proposal 3: Adopt the TP to reflect the above two proposals in TS37.213: -----------------------------------------------< BEGIN TEXT PROPOSAL >------------------------------------------------- ### 4.1.3 DL channel access procedures in a shared channel occupancy For the case where an eNB shares a channel occupancy initiated by a UE, the eNB may transmit a transmission that follows an autonomous PUSCH transmission by the UE as follows: - If 'COT sharing indication' in AUL-UCI in subframe n indicates '1', an eNB may transmit a transmission in subframe n+X, where X is subframeOffsetCOT-Sharing, including PDCCH but not including PDSCH on the same channel immediately after performing Type 2A DL channel access procedures in clause 4.1.2.1, if the duration of the PDCCH is less than or equal to duration of two OFDM symbols and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received. If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: - The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. - If the higher layer parameters ul-toDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. - If the gap is up to 16 µs, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. - If the gap is 25 µs or 16 µs, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows: - If both the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 and the higher layer parameter cg-COT-SharingList-r16 are provided to the UE, the UE transmits CG-UCI with log2C bits for COT sharing information, where C is the number of rows in a table provided in cg-COT-SharingList-r16. Each row of the table provides a channel occupancy sharing information given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing is not available. - If the 'COT sharing information' in CG-UCI indicates a row index that corresponds to a CG-COT-Sharing-r16 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy assuming a channel access priority class p= channelAccessPriority-r16, starting from O=offset-r16 slots from the end of the slot where CG-UCI is detected, for a duration of D=duration-r16 slots where duration-r16, offset-r16, and channelAccessPriority-r16 are higher layer parameters provided by CG-COT-Sharing-r16. Only the DL unicast transmission with user plane data having CAPC value not larger than the channelAccessPriority-r16 is allowed to be transmitted in the UE channel occupancy. The ‘COT sharing information’ in CG-UCI is updated by the UE slot by slot. - If the 'COT sharing information' in CG-UCI indicates a row index that corresponds to a CG-COT-Sharing-r16 that indicates channel occupancy sharing is not available, there is no channel occupancy sharing to gNB. - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is not provided and the higher layer parameter cg-COT-SharingOffset-r16 is provided to the UE, the UE transmits CG-UCI with 1 bit COT sharing information. If 'COT sharing information' in CG-UCI indicates '1', the gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16 symbols from the end of the slot where CG-UCI is detected. The transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. For the case where a gNB uses channel access procedures as described in clause 4.1.1 to initiate a transmission and shares the corresponding channel occupancy with a UE that transmits a transmission as described in clause 4.2.1.2, the gNB may transmit a transmission within its channel occupancy that follows the UE's transmission if any gap between any two transmissions in the gNB channel occupancy is at most 25 µs. In this case the following applies: - If the gap is 25 µs or 16 µs, the gNB can transmit the transmission on the channel after performing Type 2A or 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. - If the gap is up to 16 µs, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. -----------------------------------------------< END TEXT PROPOSAL >-------------------------------------------------

FL Proposal #4: Discuss whether and how to capture the above into 37.213

| Company / Org. | View on FL proposal #4 |
| --- | --- |

## 2.5 Correction on DL channel access in a shared COT initiated by a UE

R1-2002632 - Proposal 1: Adopt the following text proposal on TS 37.213. ===========================Start of Text Proposal for TS37.213============================ ### 4.1.3 DL channel access procedures in a shared channel occupancy For the case where an eNB shares a channel occupancy initiated by a UE, the eNB may transmit a transmission that follows an autonomous PUSCH transmission by the UE as follows: - If 'COT sharing indication' in AUL-UCI in subframe n indicates '1', an eNB may transmit a transmission in subframe n+X, where X is subframeOffsetCOT-Sharing, including PDCCH but not including PDSCH on the same channel immediately after performing Type 2A DL channel access procedures in clause 4.1.2.1, if the duration of the PDCCH is less than or equal to duration of two OFDM symbols and it shall contain at least AUL-DFI or UL grant to the UE from which the PUSCH transmission indicating COT sharing was received. If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: - The transmission shall contain transmission to the UE that initiated the channel occupancy and can include non-unicast and/or unicast transmissions where any unicast transmission is only transmitted to the UE that initiated the channel occupancy. - If the higher layer parameters ul-toDL-CO-SharingED-Threshold-r16 is not provided, the transmission shall not include any unicast transmissions and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. - If the gap is up to 16us, the gNB can transmit the transmission on the channel after performing Type 2C DL channel access as described in clause 4.1.2.3. - If the gap is 25us or 16us, the gNB can transmit the transmission on the channel after performing Type 2A or Type 2B DL channel access procedures as described in clause 4.1.2.1 and 4.1.2.2, respectively. For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows: - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is provided, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer. Each row of the table provides a channel occupancy sharing information given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing information is not available. - If the 'COT sharing information' in CG-UCI indicates a row index that corresponds to a CG-COT-Sharing-r16 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy assuming a channel access priority class p= channelAccessPriority-r16, starting from O=offset-r16 slots from the end of the slot where CG-UCI is detected, for a duration of D=duration-r16 slots where duration-r16, offset-r16, and channelAccessPriority-r16 are higher layer parameters provided by CG-COT-Sharing-r16. - If the higher layer parameter ul-toDL-CO-SharingED-Threshold-r16 is not provided, and if 'COT sharing information' in CG-UCI indicates '1', the gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16 symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer. The transmission shall not include any unicast transmissions and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively. <unchanged parts are omitted> =========================== End of Text Proposal for TS37.213============================.

FL Proposal #5: Discuss whether and how to capture the above change into 37.213

| Company / Org. | View on FL proposal #5 |
| --- | --- |

## 2.6 Clarifications

R1-2002247 In the DL channel access procedures in a shared COT, from our understanding, gNB can perform a DL transmission following any UL transmission if the prior UL burst includes a PUSCH. However, in TS 37.213 Subclause 4.1.3, it may be misread that a DL transmission in a shared COT should directly follow a PUSCH after a gap with the PUSCH [1]. To be clearer, the following change is proposed. Proposal 1: Update TS 37.213 Subclause 4.1.3 based on the following TP. -------------------------------------------------------- Start of TP #1 ---------------------------------------------------- If a gNB shares a channel occupancy initiated by a UE using the channel access procedures described in clause 4.2.1.1 on a channel, the gNB may transmit a transmission that follows a UL transmission including a PUSCH transmission on scheduled or configured resources by the UE after a gap as follows: <unchanged part omitted> For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows a UL transmission including the configured grant PUSCH transmission by the UE as follows: -------------------------------------------------------- End of TP #1 -----------------------------------------------------

FL Proposal #6: Discuss whether and how to capture the above clarification into 37.213

| Company / Org. | View on FL proposal #6 |
| --- | --- |

# 3. Conclusions

TBA

# References

| 1 | R1-2001534 | Maintainance on the channel access procedure | Huawei, HiSilicon |
| --- | --- | --- | --- |
| 2 | R1-2001652 | Remaining issues on the channel access procedures | vivo |
| 3 | R1-2001705 | Remaining issues on the channel access procedure for NR-U | ZTE, Sanechips |
| 4 | R1-2001759 | Discussion on the remaining issues of channel access procedure | OPPO |
| 5 | R1-2001935 | Remaining issues of channel access procedure for NR-U | LG Electronics |
| 6 | R1-2001987 | Channel access mechanism for NR-unlicensed | Intel Corporation |
| 7 | R1-2002031 | Channel access procedures | Ericsson |
| 8 | R1-2002117 | Channel access procedures for NR-U | Samsung |
| 9 | R1-2002193 | Remaining Issues on Channel Access Procedures for NR-U | Nokia, Nokia Shanghai Bell |
| 10 | R1-2002247 | Remaining issues on channel access procedures for NR-U | ETRI |
| 11 | R1-2002383 | Remaining issues and corrections on channel access procedure for NR-U | Sharp |
| 12 | R1-2002405 | Remaining issues on channel access for NR-U operation | MediaTek Inc. |
| 13 | R1-2002434 | Remaining issues on channel access procedures for NR-U | NTT DOCOMO, INC. |
| 14 | R1-2002465 | TP on shared spectrum in NR-U | NEC |
| 15 | R1-2002530 | TP for Channel access procedures for NR unlicensed | Qualcomm Incorporated |
| 16 | R1-2002632 | Remaining issues on channel access procedure for NR-U | WILUS Inc. |
| 17 | R1-2002684 | COT sharing information in CG-UCI | Lenovo, Motorola Mobility |