3GPP TSG RAN WG1 Meeting #100b-e                    	R1-200xxxx

eMeeting, April 20 - 30, 2020

Agenda Item:	7.2.2.2.2

Source:	Moderator (Charter Communications)

Title:	Draft [100b-e-NR-unlic-NRU-InitAccessProc-01]

Document for:	Discussion and Decision

# Introduction

Three email discussions have been sanctioned in RAN1#100b-e on initial access procedures for NR-U. This first discussion that aims to converge by 4/24 has the following scope:

[100b-e-NR-unlic-NRU-InitAccessProc-01] Email discussion/approval on following issues related to SS/PBCH blocks by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Amitav (Charter)

- Finalize remaining details of parameter Q signalling and interpretation

- Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of L_max

These issues have been selected based on the preparatory discussion summarized in [14].

# Company views

## Signaling of Q in MIB based on RAN2 LS response

The issue is summarized in [14] with the following proposal:

Proposal. Based on LS response from RAN2, the UE interprets ssbSubcarrierSpacingCommon (1 bit) and LSB of ssb-SubcarrierOffset (1 bit) of the Rel-15 MIB for providing the value of ssbPositionQCL-Relationship-r16. These changes are reflected in TS 38.213 Clause 4.1.

For example, TP for section 4.1 in 38.213:

Table 4-1: Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to NSSBQCL subCarrierSpacingCommon; LSB of ssb-SubcarrierOffset; NSSBQCL. scs15or60; 0; 1. scs15or60; 1; 2. scs30or120; 0; 4. scs30or120; 1; 8.

| Company | Views |
| --- | --- |

## Whether configuration of Q for RRM measurements and SCell/SCG (re)config is mandatory, or a default value of Q=8 can be assumed by UE.

Summary: For RRM measurement configuration from MeasObjectNR and SIB2/SIB4, down-select one of the following:

- Option 1: Network always provides a common Q value (ssb-PositionQCL-Common-r16) per frequency to UE.

- Option 2: If no Q value is provided, UE assumes Q=8.

For SCell addition, SCG addition, and reconfiguration with sync, down-select one of the following:

- Option 1: The Q value of the cell to be added is always provided to UE via dedicated RRC signaling, i.e. ssb-PositionQCL-r16 in ServingCellConfigCommon.

- Option 2: If no Q value is provided, UE assumes Q=8.

FL proposal is to discuss the principle before bringing in a TP.

| Company | Views |
| --- | --- |

## Whether the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block to the last transmitted SS/PBCH block should not be greater than Q.

Detailed discussion is given in Sec. 4 of [12].

FL proposal is to discuss the principle before bringing in a TP.

| Company | Views |
| --- | --- |

## Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of L_max

Summary: Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of Lmax (applies to both licensed and shared spectrum operation)

Alt. 1: Remove citation

Alt. 2: Point to TS 38.133 instead and notify RAN4 that the word “candidate” should be removed in the paragraph above Table 8.1.1-2 in 38.133 to be consistent with Rel-16 notation.

| Company | Views |
| --- | --- |

# References

R1-2001535	Maintainance on the initial access procedures	Huawei, HiSilicon

R1-2001653	Remaining issues on initial access procedure for NR-U	vivo

R1-2001706	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2001760	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2001936	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2001988	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2002032	Enhancements to initial access procedures	Ericsson

R1-2002118	Initial access procedures for NR-U	Samsung

R1-2002248	Remaining issues on initial access procedure for NR-U	ETRI

R1-2002263	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2002278	On Enhancements to Initial Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2002407	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2002531	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

R1-2001701   FL summary 72222 NRU 	Charter Communications