3GPP TSG RAN WG1 Meeting #100b-e                    	R1-200xxxx

eMeeting, April 20 - 30, 2020

Agenda Item:	7.2.2.2.2

Source:	Moderator (Charter Communications)

Title:	Draft [100b-e-NR-unlic-NRU-InitAccessProc-01]

Document for:	Discussion and Decision

# Introduction

Three email discussions have been sanctioned in RAN1#100b-e on initial access procedures for NR-U. This first discussion that aims to converge by 4/24 has the following scope:

[100b-e-NR-unlic-NRU-InitAccessProc-01] Email discussion/approval on following issues related to SS/PBCH blocks by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Amitav (Charter)

- Finalize remaining details of parameter Q signalling and interpretation

- Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of L_max

These issues have been selected based on the preparatory discussion summarized in [14].

# Company views

## Signaling of Q in MIB based on RAN2 LS response

The issue is summarized in [14] with the following proposal:

Proposal. Based on LS response from RAN2, the UE interprets ssbSubcarrierSpacingCommon (1 bit) and LSB of ssb-SubcarrierOffset (1 bit) of the Rel-15 MIB for providing the value of ssbPositionQCL-Relationship-r16. These changes are reflected in TS 38.213 Clause 4.1.

For example, TP for section 4.1 in 38.213:

Table 4-1: Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to NSSBQCL subCarrierSpacingCommon; LSB of ssb-SubcarrierOffset; NSSBQCL. scs15or60; 0; 1. scs15or60; 1; 2. scs30or120; 0; 4. scs30or120; 1; 8.

| Company | Views |
| --- | --- |
| Samsung | Support the TP. |

## Whether configuration of Q for RRM measurements and SCell/SCG (re)config is mandatory, or a default value of Q=8 can be assumed by UE.

Summary: For RRM measurement configuration from MeasObjectNR and SIB2/SIB4, down-select one of the following:

- Option 1: Network always provides a common Q value (ssb-PositionQCL-Common-r16) per frequency to UE.

- Option 2: If no Q value is provided, UE assumes Q=8.

For SCell addition, SCG addition, and reconfiguration with sync, down-select one of the following:

- Option 1: The Q value of the cell to be added is always provided to UE via dedicated RRC signaling, i.e. ssb-PositionQCL-r16 in ServingCellConfigCommon.

- Option 2: If no Q value is provided, UE assumes Q=8.

FL proposal is to discuss the principle before bringing in a TP.

| Company | Views |
| --- | --- |
| Samsung | For both questions, prefer Option 1. No matter which option is agreed, an LS to RAN2 is needed. If we remember correctly, this issue was discussed offline before, and several companies mentioned Option 1 such that RAN1/RAN2 don’t need to discuss the default RRC parameter value. |

## Whether the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block to the last transmitted SS/PBCH block should not be greater than Q.

Detailed discussion is given in Sec. 4 of [12].

FL proposal is to discuss the principle before bringing in a TP.

| Company | Views |
| --- | --- |
| Samsung | We are OK with the proposal as a clarification. |

## Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of L_max

Summary: Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of Lmax (applies to both licensed and shared spectrum operation)

Alt. 1: Remove citation

Alt. 2: Point to TS 38.133 instead and notify RAN4 that the word “candidate” should be removed in the paragraph above Table 8.1.1-2 in 38.133 to be consistent with Rel-16 notation.

| Company | Views |
| --- | --- |
| Samsung | Alt 1. TS 38.133 has no intention to define L_max in our understanding. It even refers back to TS 38.213 for the value of L_max, so we don’t think Alt 2 could work. However, in deed the word “candidate” should be removed in TS 38.133. |

# References

R1-2001535	Maintainance on the initial access procedures	Huawei, HiSilicon

R1-2001653	Remaining issues on initial access procedure for NR-U	vivo

R1-2001706	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2001760	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2001936	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2001988	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2002032	Enhancements to initial access procedures	Ericsson

R1-2002118	Initial access procedures for NR-U	Samsung

R1-2002248	Remaining issues on initial access procedure for NR-U	ETRI

R1-2002263	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2002278	On Enhancements to Initial Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2002407	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2002531	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

R1-2001701   FL summary 72222 NRU 	Charter Communications