3GPP TSG RAN WG1 #100bis-e			R1-200XXXX

e-Meeting, April 20th – April 30th, 2020

Title:	LS on removing the word “candidate” in Subclause 8.1.1 of TS 38.133 v16.3.0

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	RAN WG1

To:	RAN WG4

Cc:	-

Contact Person:

Name:	Hongbo Si, Amitav Mukherjee

E-mail Address:	Hongbo_dot_Si_at_samsung_dot_com, Amitav_dot_Mukherjee_at_charter_dot_com

1. Overall Description:

RAN WG1 has agreed to utilize two sets of indexes for SS/PBCH block in NR-U, i.e., SS/PBCH block index and candidate SS/PBCH block index. A candidate SS/PBCH block is utilized for indexing the candidate SS/PBCH blocks in a half frame, and the UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL, where NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, i is the candidate SS/PBCH block index, and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block with kSSB<24.

According to above, RAN WG1 expects a corresponding change of the index of SS/PBCH block in the Subclause 8.1.1 of TS 38.133 v16.3.0, and RAN WG1 kindly asks RAN WG4 to take the following TP into account for developing their specification for operation with shared spectrum channel access.

====================== Start of TP for Subcaluse 8.1.1 of TS 38.133 v16.3.0 ===================

================================== Text Omitted =====================================

UE shall be able to monitor up to NRLM RLM-RS resources of the same or different types in each corresponding carrier frequency range, depending on a maximum number Lmax of candidate SSBs per half frame according to TS 38.213 [3], where NRLM is specified in Table 8.1.1-2, and meet the requirements as specified in clause 8.1. UE is not required to meet the requirements in clause 8.1 if RLM-RS is not configured and no TCI state for PDCCH is activated.

================================== Text Omitted =====================================

====================== End of TP for Subcaluse 8.1.1 of TS 38.133 v16.3.0 ====================

2. Actions:

To RAN WG4.

ACTION: RAN WG1 kindly asks RAN WG4 to take above TP into account for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #101-e		25th May – 5th June 2020		e-Meeting

TSG-WG1 Meeting #102			24th – 28th August 2020		e-Meeting