3GPP TSG RAN WG1 #100b-e			R1-201XXXX

e-Meeting, April 20th – 30th, 2020

Title:	[DRAFT] LS on Signaling of Q Parameter for NR-U

Release:	Rel-16

Work Item:	NR_unlic-core

Source:	Charter Communications [RAN WG1]

To:	RAN WG2, RAN WG4

Cc:

Contact Person:

Name:	Amitav Mukherjee

E-mail Address:	Amitav_dot_Mukherjee_at_charter_dot_com

1. Overall Description:

During RAN1#100b-e, RAN WG1 has made the following agreements related to initial access procedure enhancements for NR-U:

Agreement:

- For RRM measurement configuration from MeasObjectNR and SIB2/SIB4, network always provides a common Q value (ssb-PositionQCL-Common-r16) per frequency to UE.

- For SCell addition, SCG addition, and reconfiguration with sync, the Q value of the cell to be added is always provided to UE via dedicated RRC signaling, i.e. ssb-PositionQCL-r16 in ServingCellConfigCommon.

2. Actions:

To RAN WG2, RAN WG4

ACTION: RAN WG1 kindly asks RAN WG2 and RAN WG4 to take above agreements into account for developing their specifications.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-WG1 Meeting #101-e		25th May – 5th June 2020		Online

TSG-WG1 Meeting #102 		24th – 28th August 2020		Toulouse