3GPP TSG RAN WG1 Meeting #100b-e                    	R1-200xxxx

eMeeting, April 20 - 30, 2020

Agenda Item:	7.2.2.2.2

Source:	Moderator (Charter Communications)

Title:	Draft-100b-e-NR-unlic-NRU-InitAccessProc-02

Document for:	Discussion and Decision

Introduction

Three email discussions have been sanctioned in RAN1#100b-e on initial access procedures for NR-U. This second discussion that aims to converge by 4/24 has the following scope:

[100b-e-NR-unlic-NRU-InitAccessProc-02] Email discussion/approval on following issues related to RA procedure by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/29 – Amitav (Charter)

- MsgA PRACH-PUSCH gap for NR-U

- Remaining details of RACH occasion validation for FBE access

These issues have been selected based on the preparatory discussion summarized in [14].

Company views

MsgA PRACH-PUSCH gap for NR-U

Proposal 1: Apply the same PRACH-PUSCH gap defined in R16 to msgA PRACH for NR-U.

Proposal 2: Apply CP extension to PUSCH to enable no-gap msgA.

Proposal 3: Support a zero symbol gap (N = 0) between the PRACH and PUSCH parts of MsgA

| Company | Views |
| --- | --- |
| ZTE | Among the 3 proposals, we prefer Proposal 3. We agree that symbol gap N=0 is beneficial to NRU LBT, but we think this has already been supported in 38.213, i.e. N should be 2 or 4 for licensed band, and it can be derived that N could be 0 or any other value for NRU, thus we don’t need to make any further modifications. I don’t think we should limit N=0 for NRU to decrease the configuration flexibility. As for proposal 1, it’s against the agreement made in 2-step RACH WI. Proposal 2 is an optimization issue. In 38.213: For a Type-2 random access procedure, a UE transmits a PUSCH, when applicable, after transmitting a PRACH. The UE encodes a transport block provided for the PUSCH transmission using redundancy version number 0. If useInterlacePUSCH-Common is not provided, the PUSCH transmission is after the PRACH transmission by at least N symbols where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. |

RACH occasion validation for FBE access #1

Proposal: To accommodate DL processing time at UE, a PRACH resource is considered invalid if it overlaps with the first X symbols at the front of each FFP when FBE operation is indicated, where X could be configured or fixed in spec.

FL suggestion is to first discuss the above proposal before moving to potential TP.

| Company | Views |
| --- | --- |
| ZTE | We don’t agree with this proposal, it’s not reasonable to define a ‘X’ to indicate the possible SSB or other DL signal position which shouldn’t be limited at the front of a FFP. |

RACH occasion validation for FBE access #2

Update TS 38.213 for RACH occasion validation in FBE mode when UE is not provided tdd-UL-DL-ConfigurationCommon, and for Type-2 RA procedure.

| Company | Views |
| --- | --- |
| ZTE | Support. We suggest to adopt the TP7 in [3]. |

Proposal: A PRACH resource in the channel occupancy of a Fixed Frame Period is valid only if a UE detects any DL transmission in the serving cell before the PRACH resource in the same FFP.

| Company | Views |
| --- | --- |
| ZTE | Support. |

- References

R1-2001535	Maintainance on the initial access procedures	Huawei, HiSilicon

R1-2001653	Remaining issues on initial access procedure for NR-U	vivo

R1-2001706	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2001760	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2001936	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2001988	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2002032	Enhancements to initial access procedures	Ericsson

R1-2002118	Initial access procedures for NR-U	Samsung

R1-2002248	Remaining issues on initial access procedure for NR-U	ETRI

R1-2002263	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2002278	On Enhancements to Initial Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2002407	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2002531	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

R1-2001701   FL summary 72222 NRU 	Charter Communications