3GPP TSG RAN WG1 Meeting #100-BIS-e                    	R1-200170x

eMeeting, April 20 - 30, 2020

Agenda Item:	7.2.2.2.2

Source:	Moderator (Charter Communications)

Title:	Feature lead summary#1 on NR-U enhancement to initial access procedures

Document for:	Discussion and Decision

# Introduction

A number of proposed corrections to Rel-16 specifications have been submitted to RAN1#100-BIS-e on initial access procedures for NR-U [1]-[13]. This first summary provides a list of the submitted corrections/clarifications and a proposal for 3 email discussions to resolve the corrections identified as higher priority.

The outcomes of email discussions in RAN1#100-e [14]-[16]are provided in the Appendix.

# Corrections for SS/PBCH Block

| Issue # | Description | Tdoc | Email discussion |
| --- | --- | --- | --- |
| 2.1 | Based on LS response from RAN2, the UE interprets ssbSubcarrierSpacingCommon (1 bit) and LSB of ssb-SubcarrierOffset (1 bit) of the Rel-15 MIB for providing the value of ssbPositionQCL-Relationship-r16. Misc: (How to set LSB of k_SSB to determine PRB grid can be discussed under agenda item 7.2.2.1.1) Misc: Correct “ssbSubcarrierSpacingCommon” to “SubcarrierSpacingCommon” in TS 38.213 Subclause 4.1. | R1-2001936 R1-2001988 R1-2001706 R1-2001760 R1-2001535 R1-2002118 | Y |
| 2.2 | UE performs rate-matching for all of SS/PBCH block candidate position indices (within configured DRS transmission window) QCLed with actually transmitted SS/PBCH block indices that are provided by ssb-PositionsInBurst in RMSI. When DCI format 1_1 contains rate matching indication and at least one configured rate-match pattern overlaps the REs of candidate SSB, UE shall perform the rate-matching around candidate SSB for PDSCH reception based on rate matching indication. | R1-2001936 R1-2001760 | N |
| 2.3 | Discuss/clarify whether the transmission of repetition of SSBs with the same beam within DRS transmission window is allowed or not | R1-2002248 | N |
| 2.4 | Missing instances of ‘candidate SS/PBCH block index’ terminology in: TS 38.214 Subclause 5.1.4 relating to PDSCH rate matching. TS 38.213 Subclause 8.1 relating to RO validation. TS 38.213 Subclause 11.1.1 relating to UL validation in SFI. TS 38.213 Subclause 5 relating to RLM. | R1-2002248 R1-2002263 R1-2002118 | N (propose to resolve in next meeting) |
| 2.5 | For RRM measurement configuration from MeasObjectNR and SIB2/SIB4, down-select one of the following: - Option 1: Network always provides a common Q value (ssb-PositionQCL-Common-r16) per frequency to UE. - Option 2: If no Q value is provided, UE assumes Q=8. For SCell addition, SCG addition, and reconfiguration with sync, down-select one of the following: - Option 1: The Q value of the cell to be added is always provided to UE via dedicated RRC signaling, i.e. ssb-PositionQCL-r16 in ServingCellConfigCommon. - Option 2: If no Q value is provided, UE assumes Q=8. Send LS to RAN2 based on outcome. | R1-2002407 R1-2001706 R1-2002118 R1-2002032 | Y |
| 2.6 | Proposal: Within a discovery burst transmission window, the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block to the last transmitted SS/PBCH block should not be greater than Q | R1-2002407 | Y |
| 2.7 | UE behavior in case GSCN offset refers to a GSCN which is not allowed in band n46. If the GSCN offset refers to a GSCN which is not allowed in [8-1, TS 38.101-1] for FR1, UE may ignore the information related to GSCN of SS/PBCH locations in performing cell search. | R1-2001760 | N (propose to resolve in next meeting) |
| 2.8 | Proposal: “For the CSI-RS outside DRS window, the associated SSB index should be the SSB index, for the CSI-RS inside DRS window, the associated SSB should be the candidate SSB index.” | R1-2001760 |  |
| 2.9 | Replace terminology “discovery burst transmission window” with “SS/PBCH block transmission window” in TS 38.213 | R1-2002263 | N (editorial) |
| 2.10 | Merge the determination process of QCL and SSB index in Clause 4.1 in TS 38.213. | R1-2001706 | N (editorial) |
| 2.11 | Update the reference [38.104] provided within 38.211 subclauses 7.3.3.1 and 7.4.1.4.1 as [38.213] within a related CR. | R1-2002278 | N (this change is already there in 38.211 V16.0.0) |
| 2.12 | Clarify in TS 38.213 Subclause 4.1 that “when the candidate SS/PBCH index is known by the UE” is required when i modNSSBQCL is computed. | R1-2002278 | N (close to being a editorial change) |
| 2.13 | Reflect RAN4 agreement that UE is allowed to take any active SCell in the cell group as timing reference cell in TS 38.213 Subclause 4.1. | R1-2001706 | N (this is valid but email thread is already overloaded) |
| 2.14 | Support Lmax=8 for semi-static channel access mode. Update TS 38.213 Subclause 4.1 to reflect RAN1#100-e conclusion that the SSB candidate positions in idle period is invalid | R1-2001535 R1-2002531 | N (no consensus on these proposals in RAN1#100-e) |
| 2.15 | Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of Lmax (either remove citation, or point to TS 38.133 and notify RAN4 that the word “candidate” should be removed in the paragraph above Table 8.1.1-2 in 38.133 to be consistent with Rel-16 notation). Applies to both licensed and shared spectrum operation. | R1-2002118 R1-2002032 R1-2002278 | Y due to possible dependence with RAN4 spec |

# Corrections for RACH

| Issue # | Description | Tdoc | Email discussion |
| --- | --- | --- | --- |
| 3.1 | To accommodate DL processing time at UE, a PRACH resource is considered invalid if it overlaps with the first X symbols at the front of each FFP when FBE operation is indicated, where X could be configured or fixed in spec. | R1-2001653 | Y |
| 3.2 | MsgA PRACH-PUSCH gap: Proposal 1: Apply the same PRACH-PUSCH gap defined in R16 to msgA PRACH for NR-U. Proposal 2: Apply CP extension to PUSCH to enable no-gap msgA. | R1-2002407 R1-2001936 R1-2002032 | Y |
| 3.3 | New PRACH sequences introduced in TS 38.211 should be applicable only to NR-U. L_RA = 839 is not supported for NR-U. | R1-2002278 | N (not essential correction for NR-U itself) |
| 3.4 | Reply to RAN2 whether the new NR-U long ZC sequences are applicable for NR-U 2-step RA as well. | R1-2002278 R1- 2002373 R1-2002310 (Not applicable) | Y |
| 3.5 | In FBE mode, COT detection for IDLE UEs should be based on PDCCH monitoring, where DCI payload is configured in SIB1. UE assumes that FFP is acquired by gNB if detects PDCCH of configured payload size scrambled with SI-RNTI in the lowest PDCCH candidate of TYPE0 CSS in the first slot of the FFP containing at least one valid RO. | R1-2002278 | N (No consensus in RAN1#100-e) |
| 3.6 | Capture in TS 38.213 Subclauses 8.2 and 8.2A the RAN2 agreement that downlink assignment is valid for successful RAR reception if the two LSB bits of the SFN indicated in DCI format 1_0 scrambled by RA-RNTI or msgB-RNTI correspond to the PRACH occasion used to transmit the Random Access Preamble. | R1-2001988 R1-2001706 R1-2002531 R1-2002032 | Y |
| 3.7 | Update TS 38.213 for RACH occasion validation in FBE mode when UE is not provided tdd-UL-DL-ConfigurationCommon, and for Type-2 RA procedure. Related proposal: A PRACH resource in the channel occupancy of a Fixed Frame Period is valid only if a UE detects any DL transmission in the serving cell before the PRACH resource in the same FFP. | R1-2001706 R1-2001535 R1-2002032 | Y |
| 3.8 | Capture 12 bits for PUSCH frequency resource allocation in RAR in Section 8.3 of TS 38.213 | R1-2001706 | N (propose to discuss in next meeting) |
| 3.9 | If a PRACH occasion is overlapped (fully or partially) with a slot which contains RMSI, the PRACH occasion should be treated as invalid PRACH occasion. | R1-2001760 | N (no consensus in last meeting to prioritize this issue) |

# Corrections for RRM/RLM

| Issue # | Description | Tdoc | Email discussion |
| --- | --- | --- | --- |
| 4.1 | If SSB is configured as RLM-RS, UE will only use one SSB sample with the largest RSRP among the set of candidate SSBs indicated by ssb-Index. | R1-2001653 | Y |
| 4.2 | If one CSI-RS resource is configured as RLM-RS, UE should skip invalid CSI-RS when performing IS and OOS evaluation. Besides, UE will stop IS and OOS evaluation and not report any state when there is no any valid CSI-RS sample in the latest indication period between current indication time and previous indication time. | R1-2001653 | N (no consensus in WI phase) |
| 4.3 | Converge on TP to 38.215 for agreement on RSSI definition and configuration in RAN1#100-e (TP alternatives provided below this table). | R1-2001988 R1-2001706 R1-2002032 | Y |
| 4.4 | Alt. 1: The number of OFDM symbols for RSSI measurement duration should be scale with configured reference SCS. i.e. · For 15 kHz: {sym1, sym14, sym28, sym42, sym70} · For 30 kHz: {sym2, sym28, sym54, sym84, sym140} · For 60 kHz+NCP: {sym4, sym56, sym108, sym168, sym280} · For 60 kHz+ECP: {sym4, sym48, sym96, sym144, sym240} Alt. 2: Add extra symbols or modify supported symbols of baseline set {sym1, sym14, sym28, sym42, sym70} to account for ECP. | R1-2001535 R1-2002118 | Y |
| 4.5 | One CSI-RS resource shall have multiple potential transmission locations within the discovery burst transmission window, similar to SSB. The group of CSI-RS sequences corresponding to the group of QCLed SS/PBCH blocks shall utilize the same CSI-RS sequence, similar to LTE LAA. | R1-2002118 | N (propose to discuss in next meeting) |

TP1 for issue 4.3 [3][6]:

| Definition | Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) measured by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. The UE measures observed only in configured OFDM symbols and in the configured measurement bandwidth over N number of resource blocks corresponding to the LBT channel bandwidth [TS 37.213 §4.0] where the channel has center frequency of configured by ARFCN-ValueNR, by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. Higher layers configure the measurement bandwidth, ARFCN-ValueNR, reference subcarrier spacing and the measurement duration and, i.e. which OFDM symbol(s) should be measured by the UE. For frequency range 1, the reference point for the RSSI shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported RSSI value shall not be lower than the corresponding RSSI of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

TP2 for issue 4.3 [7]:

| Definition | Received Signal Strength Indicator (RSSI), comprises the linear average of the total received power (in [W]) observed only in the configured OFDM symbols and in the configured measurement bandwidth over N number of resource blocks corresponding to LBT bandwidth with the center frequency of configured ARFCN, by the UE from all sources, including co-channel serving and non-serving cells, adjacent channel interference, thermal noise etc. Higher layers configure the reference numerology, measurement bandwidth, measurement duration and which OFDM symbol(s) should be measured by the UE. For frequency range 1, the reference point for the RSSI shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported RSSI value shall not be lower than the corresponding RSSI of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

# Other submitted TPs/proposals not for email discussion in this agenda

Multiple editorial changes listed in Section 4 of [11], Section 2.7 of [7], and Section 2.7 of [3] can be addressed in the next meeting.

# Proposed email discussions for phase 1 of RAN1#100-BIS-e

The following three email discussions and sub-topics are proposed for further discussion in AI 7.2.2.2.2 during the RAN1#100-BIS-e preparation phase until 4/17:

- [Email discussion A on SS/PBCH blocks]

  - (#2.1) Signaling of Q in MIB based on RAN2 LS response.

  - (#2.5) Whether configuration of Q for RRM measurements and SCell/SCG (re)config is mandatory, or a default value of Q=8 can be assumed by UE.

  - (#2.6) Whether the number of candidate SS/PBCH blocks from the first transmitted SS/PBCH block to the last transmitted SS/PBCH block should not be greater than Q.

  - (#2.15) Correct the citation of TS 38.104 in TS 38.213 Subclause 4.1 in relation to the definition of L_max (either remove citation, or point to TS 38.133 and notify RAN4 that the word “candidate” should be removed in the paragraph above Table 8.1.1-2 in 38.133 to be consistent with Rel-16 notation).

- [Email discussion B on RA procedure]

  - (#3.1, #3.7) Remaining details of RACH occasion validation for FBE access

  - (#3.2) MsgA PRACH-PUSCH gap for NR-U.

  - (#3.4) Reply to RAN2 LS whether the new NR-U long ZC sequences are applicable for NR-U 2-step RA as well.

  - (#3.6) Capture in TS 38.213 Subclauses 8.2 and 8.2A the RAN2 agreement that downlink assignment is valid for successful RAR reception if the two LSB bits of the SFN indicated in DCI format 1_0 scrambled by RA-RNTI or msgB-RNTI correspond to the PRACH occasion used to transmit the RAR.

- [Email discussion C on RRM/RLM]

  - (#4.1) If SSB is configured as RLM-RS, UE will only use one SSB sample with the largest RSRP among the set of candidate SSBs indicated by ssb-Index.

  - (#4.3) Converge on TP to 38.215 for agreement on RSSI definition and configuration in RAN1#100-e.

  - (#4.4) Finalize the number of OFDM symbols for RSSI measurement duration configuration.

For issues not included above, it does not mean that corrections are not needed; rather, these issues can be discussed at the next meeting.

| Company | Views |
| --- | --- |

# Appendix

[100e-NR-unlic-NRU-InitAccessProc-01] Email discussion/approval regarding SSB position validation for FBE scenario, clarification on interpretation of ssb-PositionsInBurst for UE procedure for receiving control information, and LS to RAN4 on candidate SS/PBCH block index and SS/PBCH block index terminology for alignment by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Conclusions

Agreement:

Adopt the following TP for TS 38.213:

--------------------------------- Begin TP ---------------------------------

10       UE procedure for receiving control information

<Unchanged part omitted>

For monitoring of a PDCCH candidate by a UE in a slot or in a span, if the UE

-     has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-     does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-     at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to candidate SS/PBCH blocks corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE in a slot, if the UE

-     has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

-     does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

-     at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to candidate SS/PBCH blocks corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

<Unchanged part omitted>

--------------------------------- End TP ----------------------------------------

Conclusion:

For semi-static channel access, SSBs that (partially) fall in the idle region of a fixed frame period should be considered as invalid. No PDSCH rate matching and no RLM/RRM measurement will be done for those candidate SSB positions.

Agreement:

LS to RAN2/4 on SSB index and candidate SSB index for NR-U is endorsed in R1-2001357

[100e-NR-unlic-NRU-InitAccessProc-02] Email discussion/approval regarding issues related to RA procedure including: HARQ-ACK LBT parameter signaling in success RAR for 2-step RACH procedure and Configuration of RNTI and DCI payload for IDLE UE on FBE cell to permit PRACH within FFP by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Conclusions

The following agreements were made:

Agreement:

LBT parameter applied for HARQ-ACK transmission corresponding to Msg. B reception is provided via each success RAR in Msg. B PDSCH.

Agreement:

The following text proposal for TS 38.213 is adopted.

------------------------------ Text Proposal --------------------------------

8.2A  Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding RA-RNTI, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-    an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-    transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-     a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-     a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

-     the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-     a channel access type and CP extension for PUCCH transmission is indicated by ChannelAccess-CPext field of 2 bits in the successRAR for operation with shared spectrum channel access [15, TS 37.213]

-     the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

---------------------------- Unchanged text omitted---------------------------

------------------------------------------------- End of TP-------------------------------------------------------------------------

Agreement:

The following text proposal for TS 37.213 is adopted

------- Start of Text proposal for TS37.213 --------

4.2.1     Channel access procedures for uplink transmission(s)

[Unchanged text omitted]

A UE shall use Type 1 channel access procedures for PUCCH transmissions unless stated otherwise in this subclause. If a DL grant or a RAR message for successRAR scheduling a PUCCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures.

[Unchanged text omitted]

------- End of Text proposal for TS37.213 --------

[100e-NR-unlic-NRU-InitAccessProc-03] Email discussion/approval regarding issues related to RRM/RLM including:

- RSSI measurement duration

- RMTC configuration in RRC signalling and FFS points for value ranges compared to LAA LTE

- The need to replace Lmax with Lmax and add values 10, 20 for RLM in TS 38.213 subclause 5

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Conclusions

The following agreements were made:

Agreement:

The RMTC-Config can indicate a reference SCS and CP as one of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}

Agreement:

The L1 averaging duration of RSSI measurements (within a configured measurement duration) is limited to 1 OFDM symbol of a configured reference subcarrier spacing.

Agreement:

Keep value range for rmtc-Period-r16 and rmtc-SubframeOffset-r16 the same as for LTE-LAA.

Agreement:

Keep value range for rssi-Result-r16 and channelOccupancyThreshold-r16 the same as for LTE-LAA and inform RAN2 of this decision.

Agreement:

For RSSI (and channel occupancy measurements) layer 3 filtering is not applied.

LS to RAN2 on RRM and Random Access is approved in R1-2001375

# References

R1-2001535	Maintainance on the initial access procedures	Huawei, HiSilicon

R1-2001653	Remaining issues on initial access procedure for NR-U	vivo

R1-2001706	Remaining issues on the initial access procedure for NR-U	ZTE, Sanechips

R1-2001760	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2001936	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2001988	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2002032	Enhancements to initial access procedures	Ericsson

R1-2002118	Initial access procedures for NR-U	Samsung

R1-2002248	Remaining issues on initial access procedure for NR-U	ETRI

R1-2002263	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2002278	On Enhancements to Initial Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2002407	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2002531	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

R1-2001315 Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-01] Charter Communications

R1-2001316 Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-02] Charter Communications

R1-2001317 Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-03] Charter Communications