3GPP TSG-RAN WG1 Meeting #100bis-e	                 R1-200xxxx

e-Meeting, April 20th – April 30th, 2020

Agenda Item:	7.2.3.3

Source:	AT&T

Title:	Summary on [100b-e-NR-IAB-02] Email discussion/approval regarding IAB-MT Resource/Control Channel Configuration

Document for:	Discussion/Approval

# Introduction

This contribution provides a summary on maintenance issues for IAB-MT Resource/Control Channel Configuration.

# Resource multiplexing among backhaul and access links

The following issues for maintenance of Rel-16 IAB were identified to be discussed via email in RAN1#100bis-e:

[100b-e-NR-IAB-02] Email discussion/approval regarding IAB-MT Resource/Control Channel Configuration

- Usage of tdd-UL-DL-ConfigDedicated-IAB-MT

- IAB-MT Common Search Space

- IAB-MT Specific Search Space

- Max AI DCI Payload Size

By 4/24, with potential TP/LS by 4/29 (ATT, Thomas)

## Usage of tdd-UL-DL-ConfigDedicated-IAB-MT

Source: R1-2001526

Background: The higher layer parameter tdd-UL-DL-ConfigDedicated-IAB-MT for an IAB-node MT is similar to the higher layer parameter tdd-UL-DL-ConfigurationDedicated for a UE in Section 11 of TS 38.213 except that new slot formats for an IAB-node MT can be indicated by tdd-UL-DL-ConfigDedicated-IAB-MT.

In Section 11 of TS 38.213, the UE behaviors related to slot format determination with tdd-UL-DL-ConfigurationDedicated are described. However, though it may be straightforward, the IAB-node MT behaviors related to slot format determination with tdd-UL-DL-ConfigDedicated-IAB-MT have not been captured. Consequently, when tdd-UL-DL-ConfigDedicated-IAB-MT is provided, the IAB node MT cannot determine how to use the parameter in order to determine the slot format.

FL Proposal 2.1: Adopt the following text proposal for TS 38.213 Section 14:

< Unchanged parts are omitted > #### For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration. #### For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigDedicated-IAB-MT. If the IAB-node MT is provided tdd-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon. The tdd-UL-DL-ConfigDedicated-IAB-MT provides - a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT - for each slot configuration from the set of slot configurations - a slot index for a slot provided by slotIndex - a set of symbols for a slot by symbols where - if symbols = allDownlink, all symbols in the slot are downlink - if symbols = allUplink, all symbols in the slot are uplink - if symbols = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible. - if symbols = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible. For each slot having a corresponding index provided by slotIndex, the IAB-MT applies a format provided by a corresponding symbols. The statements using the term "tdd-UL-DL-ConfigDedicated" in clauses 11.1 is replaced with "tdd-UL-DL-ConfigDedicated-IAB-MT" for the IAB-node MT of an IAB node. < Unchanged parts are omitted >

Discussion:

| Company | Do you agree with Proposal 2.1? | Comments |
| --- | --- | --- |
| Huawei | Yes | None |
| Intel | Yes | None |

## IAB-MT Common Search Space

Source: R1-2001862, R1-2002652

Background: In the latest version of the 38.331 running CR capturing RRC impacts of IAB, the following parameter is defined: commonSearchSpaceListIAB-v16xy which is a list of additional common search spaces for IAB-MT. While RAN1 agreed there would be a Search Space configuration applicable for IAB-MTs, the size of this parameter was not defined. However, given the expectation that the PDCCH formats would be largely reused for the backhaul link as well as the access link, it is reasonable to that the same number of Search Spaces can be configured for UEs and MTs.  Furthermore, an LS should be sent to RAN2 with the agreed value.

- FL Proposal 2.2: The length of the list for commonSearchSpaceListIAB-v16xy is 4. Send a LS to RAN2 regarding the agreed value.

Discussion:

| Company | Do you agree with Proposal 2.2? | Comments |
| --- | --- | --- |
| Huawei | Yes | We observed that commonSearchSpaceListIAB-v16xy in the 38.331 running CR is in PDCCH-ServingCellConfig but not in PDCCHConfigCommon unlike commonSearchSpaceList is in PDCCHConfigCommon for the UE. We suggest some update below The length of the list for commonSearchSpaceListIAB-v16xy is 4. Send a LS to RAN2 regarding the agreed value and inform RAN2 that the signaling details are up to RAN2. |
| Intel | Yes | None |

## IAB-MT Specific Search Space

Source: R1-2001862

Background: In the latest version of the 38.331 running CR capturing RRC impacts of IAB, the parameter mt-Specific-v16xy  lists the DCI formats which can be configured specifically for an IAB-MT. While RAN1 agreed that DCI Format 2_0 may indicate UL-Flexible-DL slot formats specifically for IAB-MTs and DCI Format 2_5 carries the IAB-MT specific soft resource availability indicator, it was not explicitly agreed in RAN1 that these two DCI Formats should be configured via a different search space compared to the common and UE-specific Search Spaces.

FL Proposal 2.3.1: Confirm DCI Format 2_5 can be configured in an IAB-MT specific manner (i.e. via RRC parameter mt-Specific-v16xy in SearchSpace) with the same number of aggregation levels and candidates as DCI format 2_0.

Discussion:

| Company | Do you agree with Proposal 2.3? | Comments |
| --- | --- | --- |
| Huawei | We think some discussions are needed here. | We think the following issues should be discussed: - The search space for DCI format 2_0 - [Huawei]: DCI format 2_0 can be monitored by IAB-MT in both common search space and UE-specific search space. Several child IAB nodes supporting the specific UL-Flexible-DL slot formats can monitor DCI format 2_0 in common search space while it can be monitored only by IAB node. - The search space for DCI format 2_5 - [Huawei]: DCI format 2_5 can be monitored by IAB-MT in UE-specific search space. There seems no need for IAB-MT to monitor DCI format 2_5 in common search space since the DU-IA indication is intended for each IAB node - The need of IAB-MT specific search space - [Huawei]: Since the IAB-MT functions like a UE, we think the IAB-MT can monitor DCI format 2_0 and DCI format 2_5 in CSS and USS or USS respectively (not necessarily at the same time) instead of a newly introduced search space. Even without introducing the new IAB-MT specific search space, it is still possible to configure separate monitoring periodicity for DCI format 2_0 and DCI format 2_5. Hence it is not clear whether IAB-MT specific search space is really needed. - The ALs and number of candidates for DCI format 2_5. - [Huawei]: The ALs and number of candidates for DCI format 2_5 can be configured separately from DCI format 2_0 in UE-specific SS |
| Intel | More discussion needed. | We think DCI format 2_5 can be monitored by IAB-MT in both common search space and UE-specific search space as DCI format 2_0. For example, when cell-specific soft resources becomes available to multiple IAB nodes with the same parent, common search space can be applied. |

FL Proposal 2.3.1: Confirm DCI Format 2_0 can be monitored by an IAB-MT in a common and IAB-MT specific (signaling details up to RAN2) search space.

Discussion:

| Company | Do you agree with Proposal 2.3.1? | Comments |
| --- | --- | --- |

FL Proposal 2.3.2: Confirm DCI Format 2_5 can be monitored by an IAB-MT in a common and IAB-MT specific (signaling details up to RAN2) search space, with the same number of aggregation levels and candidates as DCI format 2_0.

Discussion:

| Company | Do you agree with Proposal 2.3.2? | Comments |
| --- | --- | --- |

## Max AI DCI Payload Size

Source: R1-2001862

Background: In the RAN1 RRC parameter spreadsheet the following parameter positionInDCI-AI was defined as the (starting) position (bit) of the availabilityCombinationId (AI-Index) for the indicated IAB-DU cell (iabDuCellId-AI) within the DCI payload. It has a value range of INTEGER(0..maxAI-DCI-PayloadSize-1), however maxAI-DCI-PayloadSize was not defined. Given that the design of the availability indicator (DCI Format 2_5) followed the SFI design (DCI Format 2_0), it should be straightforward that they have the same value.

FL Proposal 2.4: Confirm maxAI-DCI-PayloadSize = maxSFI-DCI-PayloadSize = 128.

Discussion:

| Company | Do you agree with Proposal 2.4? | Comments |
| --- | --- | --- |
| Huawei | Yes | None |
| Intel | Yes | None |

# Summary

TBD

Additional editorial corrections for RAN1 specifications are summarized in Appendix A.

# Appendix A Editorial corrections to existing specifications

A.1 MT Slot Formats

From 38.213:

“An IAB-node MT can be provided, by SlotFormatCombinationsPerCell-IAB-MT, a list of slot format combinations applicable for one serving cell and, by SlotFormatIndicator-IAB-MT, a configuration for monitor a DCI format 2_0 indicating a slot format combination, from the list of slot format combinations, over a number of slots as described in Subclause 11.1.1.”

The two parameters, SlotFormatCombinationsPerCell-IAB-MT and SlotFormatIndicator-IAB-MT, are not yet agreed to be included in the higher layer parameters list for RAN2.

A.2 DU Resource Configuration

The DU resource configuration is given by higher layer parameter gNB-DU Resource Configuration while the parameter name in 38.213 does not match:

For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-ConfigurationgNB-DU Cell Resource Configuration

The IAB-node DU can assume a same SCS configuration for availabilityCombinations for a serving cell as an SCS configuration provided by IAB-DU-Resource-Configuration-TDD-ConfigNB-DU Cell Resource Configuration for the serving cell

A.3 DCI Format 2_5: resourceAvailability

One issue about DU-IA configuration is that the 2nd sub-bullet below is used to describe availability combination, while the parameter used for direct mapping the soft symbol availability combination is resourceAvailability in AvailabilityCombination. The current text was modified from the Rel-15 text for DCI 2_0, but the change of “slotFormats” to “resourceAvailability” is not consistently applied in the 2nd sub-bullet.

-------------------------------------part of 38.213 text for DCI 2_5 ----------------------------------

For each serving cell of an IAB-node DU in a set of serving cells of the IAB-node DU, the IAB-node DU can be provided:

-	an identity of the IAB-node DU serving cell by iabDuCellId-AI

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI

-	a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes

-	resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-node DU serving cell, and

-    a mapping for the soft symbol availability combination provided by AvailabilityCombination to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId

--------------------------------------------------------------------------------------------------------

-------------------------------------part of 38.213 text for DCI 2_0 ----------------------------------

-	a set of slot format combinations by slotFormatCombinations, where each slot format combination in the set of slot format combinations includes

-	one or more slot formats indicated by a respective slotFormats for the slot format combination, and

-	a mapping for the slot format combination provided by slotFormats to a corresponding SFI-index field value in DCI format 2_0 provided by slotFormatCombinationId

--------------------------------------------------------------------------------------------------------