3GPP TSG RAN WG1 #100bis-e	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.4.2.1

Source:	Moderator (Ericsson)

Title:	Text proposal for TS 38.213 related to the agreements in [100e-NR-5G_V2X_NRSL-RA_Mode1-02]

Document for:	Endorsement

# 1	Introduction

This document contains the text proposal reverting the changes made to TS 38.213 resulting from the endorsement of the TP in R1-2002233, which was based on the following agreements from [100e-NR-5G_V2X_NRSL-RA_Mode1-02]:

Agreements:

- The mapping between the values of HPN signaled in DCI and HPN signaled in SCI is fixed for a TB, and is up to UE implementation.

- For dynamic grant, the toggling of NDI in DCI is used as the toggling of NDI in SCI for the first SL transmission scheduled by the DCI. The SCI for the remaining transmissions scheduled by the DCI, if any, have the NDI untoggled with respect to the first SL transmission.

- FFS NDI in SCI for PUCCH ACK-NACK error cases

Agreements:

- The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants.

  - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation.

Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made.

These agreements have impact on SCI format 0_2 which is transmitted by PSSCH. The corresponding changes have been made to TS 38.214 in R1-2002822.

The motivation for the TP is as follows:

- Reasons for change: the corresponding specification text is related to SCI format 0_2 which is carried by PSSCH (to be described in TS 38.214)

- Summary of changes: specification text describing how to set HARQ process ID and NDI is removed.

- Specs/Sections impacted: TS 38.213 Section 16.4

- Consequences if not approved: Specification refers to the wrong SCI format.

# 2	Text Proposal for TS 38.213

TP for TS 38.213:

---------------------------------- Start of Text Proposal ---------------------------------

<Unchanged parts omitted>

## 16.4	UE procedure for transmitting PSCCH

A UE can be provided a number of symbols in a resource pool, by timeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by frequencyResourcePSCCH, for a PSCCH transmission with a SCI format 0_1.

<Unchanged parts omitted>

---------------------------------- End of Text Proposal ---------------------------------