3GPP TSG RAN WG1 Meeting #100bis-E					     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	Email Discussion #2 [100b-e-NR-5G_V2X_NRSL-Mode-2-02]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This document provides discussion on issues in the second email discussion on V2X Mode-2 during RAN1#100bis-e.

[100b-e-NR-5G_V2X_NRSL-Mode-2-02] Email discussion/approval w.r.t. pre-emption including aspects:

- Finalization of the RRC parameter for pre-emption configuration per resource pool (still TBD in the RRC list)

- Relation of pre-emption RSRP threshold and Step 1 checking

- Which resources can be re-selected after pre-emption condition – only ones to be transmitted or to be signalled

till 4/27, with potential TPs till 4/30 (Intel, Sergey)

Agreements: - Support a resource pre-emption mechanism for Mode-2 - A UE triggers reselection of already signaled resource(s) as a resource reservation in case of overlap with resource(s) of a higher priority reservation from a different UE and, SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold - Only the overlapped resource(s) is/are reselected - FFS - the timeline for reselection - other details - FFS whether or not to support other potential UE behaviour (e.g, power boosting/reduction) - This mechanism can be enabled or disabled, per resource pool - FFS details Agreements: - For pre-emption, both full and partial frequency domain overlap in the same slot are considered as the overlapping condition to trigger resource reselection, wherein the whole resource is reselected even if the partial overlap happened - (Re-)selection procedure for an already reserved but pre-empted resource to be used for transmission in a slot ‘m’ is not required to be triggered at moment > ‘m – T3’ - T3 here is identical to T3 introduced for the re-evaluation - FFS whether re-selection of the already-reserved, but pre-empted resource applies only to the resource transmitted in slot ‘m’ or to other already-reserved and pre-empted resource(s) signaled in the SCI in slot ’m’ as well

Discussion

The first aspect relates to the open question of RRC configuration. The agreement says that pre-emption can be enabled or disabled, per resource pool, and FFS details. In the last meeting, no consensus was reached to conclude whether the activation in the pool applies to any priority level or to a sub-set of priorities.

There are different options observed in contributions:

- The per resource pool pre-emption configuration is priority unaware, i.e. it is a flag {enabled, disabled/not provided}. Companies in favour of this option usually refer to sufficiency of such configuration option and allow any higher priority transmission to pre-empt any lower priority transmission.

- The per resource pool pre-emption configuration is priority aware, a scalar priority value {0…7} controls activation of pre-emption. Companies in favour of this option refer to better controlled pre-emption rate, also can realize the scenarios where only the highest priority can pre-empt other transmissions. There are two contributions showing pre-emption SLS evaluations [13][27] motivating to introduce priority dependent activation for pre-emption rate control.

Q1: Which of the following options is preferred?

- Option 1

  - Finalize the RRC parameter for pre-emption activation per resource pool by confirming that it is {enabled, disabled}, and no separate priority value provided

- Option 2

  - Finalize the RRC parameter for pre-emption activation per resource pool by introducing a priority level p_preemption {0…7}, and if priority p_SCI associated with the resource indicated in SCI is higher than p_preemption and prioTX, then pre-emption can be triggered

| Source | Option | Comment |  |
| --- | --- | --- | --- |
| Ericsson | 2 | We think it is important to limit the use of pre-emption to those cases when it really makes sense to use it. ON/OFF configuration is not sufficient. Nonetheless, it should be possible to prevent pre-emption between some priority pairs. |  |
| Intel | 2 | Pre-emption is more important for high priority transmissions. Other mechanisms can be used for medium/low priorities |  |
| Futurewei | 2 | It is important that the highest priority traffic can always sent. |  |
| NTT DOCOMO | 1 | Traffic with higher priority should always be prioritized than the one with lower priority |  |
| Apple | 2 | Pre-emption is triggered only for very high priority traffic. This avoids the frequent pre-emption scenario in a system. |  |
| Panasonic | 1 | Option 1 is sufficient for simplicity. |  |
| Option 1 | 1 | Since it can be ON/OFF, no need to further restrict the usage of pre-emption operation. |  |
| Sharp | 2 | Agree with other companies that pre-emption should not be a frequent event. |  |
| MediaTek | 2 | Unnecessary pre-emption triggering should be minimized. Option-2 provides greater configuration flexibility. |  |
| TCL | 2 | Pre-emption should be configurable per priority to avoid high priority packet loss. |  |
| NEC | 1 | Option 1 is simple. We think the associated SL-RSRP threshold in Q2 outcome will also limit the frequency of pre-emption in the case where both traffics have low priorities. |  |
| Xiaomi | 1 | If pre-emption is enabled, it should be applied to all the priorities so higher priority transmissions can be first transmitted. |  |
| Samsung | 1 | Option 1 is sufficient |  |
| Bosch | 2 | Option 2 to avoid frequent pre-emption. |  |
| Qualcomm | 2 | Our evaluations show that excessive pre-emption degrades system performance and a finer granularity mechanism is needed to enable/disable. The behaviour when both priorities are above the threshold should also be clarified, but that a secondary detail. |  |
| Fraunhofer | 1 | If pre-emption is enabled, higher priority transmissions should always be prioritized over lower priority ones. |  |
| Huawei/HiSilicon | 1 | The basic idea of pre-emption is to guarantee the successful delivery of higher priority traffic. For Option2, we think it’s very hard to set an “accurate” threshold, and thus maybe inaccurate in some cases and impact the system performance. If pre-emption happens to be a frequent event in some cases, it actually means there are many higher priority traffic. And if Option 2 is adopted in these cases, then those higher priority traffic maybe cannot be delivered successfully due to the “inaccurate” threshold, which clearly impacts the system performance. |  |
| ZTE, Sanechips | 1 | In our view, pre-emption can be realized through normal step1 procedure. In the normal step1, - The RSRP threshold is configured based on both Tx priority and Rx priority, i.e., to judge whether a resource can be used or not, the Tx priority has already been considered. - For each Tx priority, the resources with which priority levels should be excluded (can be preempted) can be accomplished by proper RSRP threshold configuration. For example: If we want that a priority level a of a transmitting UE may preempt the resources whose priority of decoded SCI is b, a lower RSRP threshold for resources exclusion is configured for the priority level pair (a, b). In option 1, the flag of {enabled, disabled} is used to turn on/off re-evaluation due to preemption is allowed or not. |  |

The second aspect is the open question which RSRP threshold is used to decide about pre-emption triggering when priority condition is met. There are the following alternatives:

- RSRP threshold is one the used for resource identification in Step 1 including all increments

  - In one alternative, this is the threshold in current resource selection window when checking for pre-emption is triggered. This option is very similar to re-evaluation and can be considered with higher priority to ensure same mechanism between features.

  - In another alternative, this is the threshold in the resource selection window when the resource was selected previously. In this case, the RSRP threshold may not reflect changed loading.

- RSRP threshold is the one configured per-priority for resource identification, before any increments during Step 1

- RSRP threshold is separately configured for pre-emption purpose, it is not subject to increments

Q2: Which option of RSRP threshold for pre-emption checking is preferred

- Option 1

  - The RSRP threshold after Step 1 checking, i.e. including any 3 dB increments

    - Option 1a

      - The threshold after Step 1 checking on a current resource selection window (triggered specifically for pre-emption)

    - Option 1b

      - The threshold after Step 1 checking on the resource selection window during latest re-evaluation for this resource

- Option 2

  - The RSRP threshold (pre-)configured for regular Step 1 checking, before any increments

- Option 3

  - A separately configured per priority pair RSRP threshold

| Source | Option | Comment |  |
| --- | --- | --- | --- |
| Ericsson | 2 | The configuration must be based on some fixed parameters. Not on something internal to the UE |  |
| Intel | Option 1a | UE executes Step-1 and checks if pre-empted resource is within candidate resource set |  |
| Futurewei | 3 or 2 | Agree with Ericsson that a UE-internal value cannot be used. Option 3 provides a little bit more flexibility than option 2, thus is slightly preferred to 2 |  |
| NTT DOCOMO | 1a | RSRP threshold specific to pre-emption is not necessary. Similar to re-evaluation is sufficient. |  |
| Apple | Similar to Option 1a | Only configure the maximum RSRP threshold per priority pair. If the maximum RSRP threshold is reached and less than X% of candidate resources are identified, then pre-emption is triggered. |  |
| Panasonic | 2 |  |  |
| vivo | Option 1a-like | We prefer a solution which has commonality with re-evaluation procedure, i.e., the following agreed behavior - Step 1 of the resource (re-)selection procedure is performed at least at the moment ‘m-T3’, and if the pre-selected resource is not in the identified candidate resource set, Step 2 is triggered for reselection of the resource |  |
| Sharp | 2 | The RSRP threshold for regular Step 1 checking is used for exclusion of resources. If the measured RSRP is high enough according to option 1a, the resources indicated in the corresponding SCI are already excluded even without the need to check the (priority-related) pre-emption specific conditions. |  |
| MediaTek | 2 |  |  |
| TCL | 3 or 2 | Agree with Futurewei. |  |
| NEC | 2 | The RSRP threshold including any 3dBm is used to make sure X% matters which is not proper to trigger pre-emption/re-selection |  |
| Xiaomi | 1a | Similar behaviour as resource selection is preferred. |  |
| Samsung | 3 | Option 3 provides more flexibility than Option 2. For pre-emption purpose, RSRP threshold can be configured separately |  |
| Bosch | 1a | Option 1a is our preferred option. |  |
| Qualcomm | 1a | This simplifies UE implementation and specification. We don’t see the benefit of forcing the UE to redo the RSRP increase process (Option 2). |  |
| Fraunhofer | 2 or 3 | The RSRP threshold used to trigger pre-emption has to be based on the priority of the TB to be transmitted and the priority of the received SCI. |  |
| Huawei/HiSilicon | Option 1a-like | Prefer to have a solution similar as re-evaluation procedure. |  |
| ZTE, Sanechips | Option 1a | During Step-1, to check if a resource is within candidate resource set, same RSRP comparing mechanism is used, including RSRP thresholds and 3 dB increments. |  |

The third aspect in this discussion is related to the FFS whether re-selection of the already-reserved, but pre-empted resource applies only to the resource transmitted in slot ‘m’ or to other already-reserved and pre-empted resource(s) signalled in the SCI in slot ’m’ as well.

Based on the discussion, the following options were identified:

- Re-selection is performed only for the upcoming resource to be used for transmission

- Re-selection is performed for any resource to be used for transmission or signaled, if those fulfil pre-emption triggering condition

- Re-selection is performed for all resources to be used for transmission or signaled, if at least one of these resources fulfil the pre-emption triggering condition

  - Note, this may violate prior RAN1 agreement that only overlapped resource(s) are reselected

Q3.1: Which option for resource re-selection due to pre-emption is preferred

- Option 1

  - Re-selection is performed only for the upcoming resource to be used for transmission

- Option 2

  - Re-selection is performed for any resource to be used for transmission or signaled, if those fulfil pre-emption triggering condition

- Option 3

  - Re-selection is performed for all resources to be used for transmission or signaled, if at least one of these resources fulfil the pre-emption triggering condition

| Source | Option | Comment |  |
| --- | --- | --- | --- |
| Ericsson | Option 2 | Option 3 unnecessarily reselects resources that have no issue. Option 1 is not justified, as it does not reselect resources for which the pre-emption condition holds, even if these are further apart in time. The earlier resources are re-selected and reserved, the better. |  |
| Intel | Option 2 is preferred | Option 3 - there may be no need to reselect all resources. Option 1 – it is better to reselect resource signalled in slot ‘m’ as well |  |
| Futurewei | 2 | Reselect only for the resource affected |  |
| NTT DOCOMO | 2 | Agree with Ericsson |  |
| Apple | Option 2 | Reselect the resources to be transmitted or to be signaled |  |
| Panasonic | Option 2 | Option 2 is a good candidate as signalled resource could also be pre-empted, while option 1 is too limited and option 3 is too much |  |
| vivo | Option 1 | We slightly prefer option 1. For pre-emption operation, we should be carefully to avoid to trigger unnecessary re-selection. Option 2 means double check for a given transmission resource, the first check occurs at the time before it’s reserved, the second check occurs just before using the resource, the two check may incur different judgement of resource re-selection triggering, which depends on outcome of Q2. Regarding option 2, we have a question. The so-called ‘signaled resource’ include both aperiodic and periodic reserved resource, correct?. |  |
| Sharp | Option 2 | Only affected resources should be considered. |  |
| MediaTek | Option 2 | Agree with FL’s comment that Option-3 violates prior RAN1 agreement. |  |
| TCL | Option 2 | Reselection of pre-empted resources whenever known to be pre-empted. Side question : What if a pre-empted resource can not be replaced by another one without breaking regular resource selection constrains with other ongoing reservations (e.g. inter-resource distance, HARQ…) should it be discarded or apply a limited option 3 on a need basis (which would be hard to control in Ue implementations)? |  |
| NEC | Option 1 | We think vivo's comments make sense. Besides, do we still need "<m-T3" restriction for "signaled in the SCI in slot ’m’" case? |  |
| Xiaomi | Option 2 | Only the resource that be occupied by pre-emption should be considered for re-selection. |  |
| Samsung | Option 2 |  |  |
| Bosch | Option 2 | Only resources satisfying pre-emption conditions are reselected. |  |
| Qualcomm | Option 2 or 3 | We prefer Option 2, but RAN2 already specified Option 3 and we’d be ok with that as well. |  |
| Fraunhofer | Option 2 | Agree with Ericsson. |  |
| Huawei/HiSilicon | Option 2 | The earlier the re-selection is performed, the smaller latency can be achieved, and the more retransmission chances can be guaranteed which can ensure the successful delivery of the packet in a given PDB. So Option 2 is preferred rather than Option 1. Considering the SCI with the reservation has already been transmitted, there is no need to change the resources which are not pre-empted. So Option 3 is not necessary. |  |
| ZTE, Sanechips | Option 3 | For more flexible resource selection, it should be allowed that resource re-selection is performed for all resources. Whether to reselect all resources is up to UE implementation. |  |

There is one more sub-aspect, where like re-evaluation, the reselection of a pre-empted resource may need to fulfil any introduced timing restrictions.

Q3.2: When re-selection of the pre-empted resource(s) is performed, whether to allow violation of timing restrictions?

| Source | Comment |  |
| --- | --- | --- |
| Ericsson | No. Timing restrictions cannot be violated. They reflect considerations on processing times, etc. that cannot be altered. |  |
| Intel | In our view at least HARQ RTT should be ensured, while SCI chain integrity(signaling window) may not be ensured. |  |
| Futurewei | No |  |
| NTT DOCOMO | No |  |
| Apple | No. Timing restriction should be kept. |  |
| Panasonic | No violation of timing restriction should be allowed. A UE could treat the transmission as failure if no suitable resource can be found. |  |
| vivo | No |  |
| Sharp | Similar view as Intel. “Timing restrictions” in the proposal should be clarified. |  |
| MediaTek | No |  |
| TCL | No |  |
| NEC | No. |  |
| Samsung | No |  |
| Bosch | No, in general. Any unattainable transmission needs to be declared as failure. |  |
| Qualcomm | No. Timing restrictions cannot be violated. Except for the initial transmission after pre-emption, where no timing restriction applies. |  |
| Fraunhofer | No |  |
| Huawei/HiSilicon | No. The timing restriction cannot be violated. Other UE behaviour can also be considered such as power reduction to fulfil the timing requirement. |  |
| ZTE, Sanechips | No. At least HARQ RTT should be ensured |  |

Summary of proposals on the relevant issues

- Finalization of the RRC parameter for pre-emption configuration per resource pool (still TBD in the RRC list)

  - Priority dependent configuration: [3][13][20][27]

    - [13][27] show SLS evaluation in support of it

  - Not priority dependent configuration: [6][10][12][15][16][24]

- Relation of pre-emption RSRP threshold and Step 1 checking

  - [13][16][27]

- Which resources can be re-selected – only ones to be transmitted or to be signalled

  - [2][5][7][9][13][16][17][21][22]

- References

- R1-2001552	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2001661	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2001749	Discussion on remaining open issue for mode 2	OPPO

- R1-2001793	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2001805	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-2001877	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2001886	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2001896	Remaining issues of mode 2 operation on sidelink	ZTE, Sanechips

- R1-2001907	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2001964	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2001969	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-2001978	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001994	Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design	Intel Corporation

- R1-2002041	Remianing details on mode-2 resource allocation	Futurewei

- R1-2002078	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2002126	On Mode 2 for NR Sidelink	Samsung

- R1-2002234	Resource allocation Mode 2 for NR SL	Ericsson

- R1-2002267	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2002301	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2002325	On Remaining Details of Mode 2 Resource Allocation	Apple

- R1-2002362	Remaining issues on resource allocation Mode 2	NEC

- R1-2002388	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2002402	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

- R1-2002439	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2002487	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2002489	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2002539	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated