3GPP TSG RAN WG1 Meeting #100bis-E					     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Intel Corporation)

Title:	Email Discussion #4 [100b-e-NR-5G_V2X_NRSL-Mode-2-04]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This document provides discussion on issues in the fourth email discussion on V2X Mode-2 during RAN1#100bis-e.

[100b-e-NR-5G_V2X_NRSL-Mode-2-04] Email discussion/approval w.r.t. periodic reservation including aspects:

- Backward signalling option

- Periods for exclusion if a slot is not monitored in a sensing window

till 4/24, with potential TPs till 4/29 (Intel, Sergey)

Agreements: - Down-select in the next meeting one of the following options - Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported - Option 2: When periodic reservations are enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a resource index for the purpose of backward indication - Option 3: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a resource index for the purpose of backward indication Agreements: - On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled: - A set of possible period values additionally includes all integer values from 1 to 99 ms Conclusion Evaluate till the next meeting whether given the agreed set of configurable reservation periodicities, the change to the exclusion procedure is necessary, wherein currently all configured period values are used for exclusion as inherited from LTE

Discussion

The first aspect relates to the discussion in the previous meeting on support of backward indication (resource index indication) similar to LTE, in case of periodic reservations. Different level of support was expressed.

- Option 1: no backward indication

  - Companies supporting this option argue that other mechanisms are sufficient

- Option 2: 1-bit backward indication

  - Companies supporting this option argue that it is a good compromise between no indication and full indication, i.e. in case of Nmax = 3, only one resource can be reserved in past.

- Option 3: full backward indication

  - Companies supporting this option argue that it was present in LTE, and if NR does not support similar mechanism, the performance SPS sidelink transmission in LTE may not be achieved by NR. There are SLS evaluation results in [13] showing that backward indication provides gains in periodic traffic environment.

Q1: Which of the following options related to backward indication is preferred? General support can be expressed, while the detailed proposal can be worked out in the second phase.

- Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported

- Option 2: When periodic reservations are enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a resource index for the purpose of backward indication

- Option 3: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a resource index for the purpose of backward indication

| Source | Option | Comment |  |
| --- | --- | --- | --- |
| Ericsson | Option 2 | OK with option 3 as well but as second preference |  |
| Intel | Option 3 is preferred | OK with option 2 as a second preference |  |
| NTT DOCOMO | Option 2 or 3 | Support backward indication |  |
| Xiaomi | Option 1 | Do not see the benefit to support backward indication |  |
| TCL | Option 3 | Ok with option 2 as second preference |  |
| Qualcomm | Option 1 | Since additional retransmissions within a period are supported, it is not clear how indicating those reservations from the last periodic resource SCI would be reconciled with backward indication. |  |
| Bosch | Option 3 or 2 | Option 3 is preferred. If not agreeable, then at least option 2. |  |
| Huawei/HiSilicon | Option 3 | Sensing UE may miss SCI, Option 3 can solve this issue and makes sensing procedure more accurate. Option 3 can cover Option 1 and Option 2. Compared to Option 2, Option 3 provides full flexibility, and there is only 1 extra bit in case of Nmax = 3, so the cost is very small. |  |

The second aspect relates to the conclusion in the last meeting to evaluate the case of periodic resource exclusion for the slots not monitored in the sensing window. The following points are discussed:

- Due to support of very fine granularity of period configuration, exclusion of the full set of up to 15 different periods when a UE skipped monitoring of a slot in the sensing window may lead to excessive exclusions

- At least one contribution [27] shows performance differences between cases when LTE procedure is followed or when the set of periods for exclusion is updated

- Some companies argue that exclusion by a single period used by the TX UE may also be inappropriate due to existence of aperiodic traffic. I.e., the aperiodic traffic should not be accounted in this case.

- In general, there are the following alternatives identified

  - Keep excluding by all configured period values

  - Exclude only using TX UE period value

  - Do not apply this exclusion at all

  - Separately configure a set of periods for exclusion

  - Apply other mechanisms for reduced exclusion rate

    - Probabilistic exclusion

    - Reduced number of excluded resources

    - A subset of periods used for exclusion

Q2: Which of the following options related to exclusion of slots corresponding to the slots not monitored in the sensing window? General support can be expressed, while the detailed proposal can be worked out in the second phase.

- Option 1: Do not change current procedure and apply all the periods configured for a UE

- Option 2: Change current procedure

  - Option 2a: Exclude only based on the period used by the UE selecting the resources

  - Option 2b: Do not apply this exclusion step

  - Option 2c: Apply a different set of periods, e.g. a reduced sub-set or a separately configured set

| Source | Option | Comment |  |
| --- | --- | --- | --- |
| Ericsson |  | For periods of 16 or more slots (i.e., 32/2), we do not see the need to make any changes to the specification. |  |
| Intel | Option 2c | In our view the universal solution is to apply a different set of periods, e.g. a reduced sub-set or a separately configured set |  |
| vivo | Option 1 | Compared with LTE, we have some more values in 0-99ms. However, only a few value in 0~99 can be beneficial, e.g., 30, 50…, considering only a few value is configured per pool, we do not think such additional value will degrade the system performance. |  |
| NTT DOCOMO | See comments | Which option is better depends on situations. For shorter periodicity, option 1 is better than option 2 as excluding only the period used by the UE is not sufficient. For longer periodicity, option 2 is ok. |  |
| Samsung | Option 2b | For this issue, a simple solution is not to perform step 5 in NR resource (re-)selection procedure. |  |
| TCL | Option 1 | Agree with vivo. No strong opinion here. |  |
| Qualcomm | Option 2a | Over-exclusion is exacerbated by the additional possible period values supported in NR V2X. Our evaluation results show performance gains of using Option 2a instead of Option 1. We can also be ok with Option 2b as a second preference. |  |
| Bosch | Option 2a (or 2c) | If we can guarantee that there will be only few values selected between 1-99, then keeping current specification is possible. However, Option 2a (or 2c) may be more beneficial in all cases. |  |
| Huawei/HiSilicon | Option 1 | All configured periods should be excluded as in LTE, the ratio of candidate resource set reported to higher layer can be guaranteed by the configuration of X and the increasing on RSRP threshold. The over exclusion is not a critical issue in step 5). Option 2a is not preferred since the periods indicated on the non-monitored slots would be any period allowed by the higher layer. Option 2c is also not preferred since such sub-set may cause inaccuracy and thus collision. |  |

Summary of proposals on the relevant issues

Last time the backward indication issue and periods for exclusion of slots not monitored in the sensing window were discussed. The following issues are still open based on tdoc review:

- Backward signalling option

  - Option 1: [3][5][8][9][18][20][22]

  - Option 2: [17][24]

  - Option 3: [1][7][10][13][24]

    - Note: [13] shows results in support if it

- Periods for exclusion if a slot is not monitored in a sensing window

  - All configured, as in LTE: [17]

  - The one used for selection: [19][27][13]

    - Note: [27] shows results in support if it

  - No exclusion: [16]

  - Separate set / sub-set / handling

    - [13] (separate set)

    - [3][15][26] - (mechanisms to reduce exclusions for small periods)

- References

- R1-2001552	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2001661	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2001749	Discussion on remaining open issue for mode 2	OPPO

- R1-2001793	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2001805	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

- R1-2001877	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2001886	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2001896	Remaining issues of mode 2 operation on sidelink	ZTE, Sanechips

- R1-2001907	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2001964	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2001969	Discussion on resource allocation for NR sidelink Mode 2	Lenovo, Motorola Mobility

- R1-2001978	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001994	Solutions to remaining opens of resource allocation mode-2 for NR V2X sidelink design	Intel Corporation

- R1-2002041	Remianing details on mode-2 resource allocation	Futurewei

- R1-2002078	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2002126	On Mode 2 for NR Sidelink	Samsung

- R1-2002234	Resource allocation Mode 2 for NR SL	Ericsson

- R1-2002267	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2002301	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2002325	On Remaining Details of Mode 2 Resource Allocation	Apple

- R1-2002362	Remaining issues on resource allocation Mode 2	NEC

- R1-2002388	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2002402	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

- R1-2002439	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2002487	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2002489	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2002539	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated