# [100b-e-NR-5G_V2X_NRSL-SYNC-03]

# Email discussion/approval related to sync procedure

[100b-e-NR-5G_V2X_NRSL-SYNC-03] Email discussion/approval related to sync procedure –

SL SSIDs/sync resources for each priority and

Lower SLSS ID with higher priority for P6/P6’ UE

(a,k.a. issues 4-1 & 4-2) by 4/24, with potential TPs by 4/29 (CATT, Teng)

## Issue 4-1 SL SSIDs/sync resources for each priority

Proposal 5: the following corrections shall be applied in current synchronization mechanism to solve the issues of ambiguity:

- Reserve a SL-SSID for InC UE directly synch with GNSS, e.g. SL-SSID=1.

- Then the SL-SSID set for In_C UE directly synch with gNB/eNB is SL-SSID= [2,335].

- For UE OoC sync to UE OoC, distinguish 2 cases:

- If sync Ref UE is directly sync to GNSS (i.e., SL-SSID=0, and transmitting on resource 3).

- Resource 2: InC = 0. SL-SSID=0.

- Other cases:

- Resource 1 or 2 (different from Sync Ref):

- If the SL-SSID of Sync Ref UE is 0, SL-SSID = 336 and InC = 0;

- Else, SL-SSID is from Sync Ref and InC = 0.

| Company | Views |
| --- | --- |

## Issue 4-2 Lower SLSS ID with higher priority for P6/P6’ UE

Proposal 6:  RAN1 should discuss whether UE can be (pre-)configured with P6 sync source with SSID-based selection as the alternative of RSRP-based mechanism in Rel-16.

| Company | Views |
| --- | --- |
| Qualcomm | Yes, RAN1 should discuss and agree on the proposed mechanism. Merging independent synchronization clusters is a critical issue that needs to be addressed in NR V2X; otherwise, NR V2X performance can be significantly degraded. This view is shared by the primary users of NR V2X/Sidelink technology (the public-safety community, car manufacturers, and industrial OEMs) and the many companies who co-sourced R1-2002540. We tested alternative proposals that came up in prior RAN1 discussions and provided results in our companion contribution R1-2002543, where we observed that they do not address the issue. |