# [100b-e-NR-5G_V2X_NRSL-SYNC-04]

# Email discussion/approval related to sync timing

[100b-e-NR-5G_V2X_NRSL-SYNC-04] Email discussion/approval related to

- Slot number/sidelink timing derived from GNSS

- Resource sets for S-SSB transmission

- Timing determination of S-SSB

(a,k.a. issues 5,6,7) by 4/24, with potential TPs by 4/29 (CATT, Teng)

## Issue 5 Slot number/sidelink timing derived from GNSS

Proposal 7: When UE selects GNSS as the synchronization reference and offsetDFN is provided, the following TP is supported.

------------------------------------------------------ Start of Draft TP of 38.331--------------------------------------------------

5.8.12	DFN derivation form GNSS

When the UE selects GNSS as the synchronization reference source, the DFN used for NR sidelink communication is derived from the current UTC time, by the following formulae:

DFN= Floor (0.1*0.001* (Tcurrent – Tref – offsetDFN)) mod 1024

SubframeNumber= Floor (0.001*(Tcurrent –Tref – offsetDFN)) mod 10

SlotNumber= Floor (0.001*(Tcurrent – Tref – OffsetDFN)*2μ) mod 2μ

Where:

Tcurrent is the current UTC time that obtained from GNSS. This value is expressed in milliseconds microseconds;

Tref is the reference UTC time 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Thursday, December 31, 1899 and Friday, January 1, 1900). This value is expressed in milliseconds microseconds;

OffsetDFN is the value sl-OffsetDFN if configured, otherwise it is zero. This value is expressed in milliseconds microseconds;

μ=0/1/2/3 corresponding to the 15/30/60/120 khz SCS for SL respectively.

-------------------------------------------------------- End of Draft TP of 38.331-------------------------------------------------

| Company | Views |
| --- | --- |

## Issue 7 Resource sets for S-SSB transmission

Proposal 9: The number of synchronization resource set is same as LTE-V2X. How to use the sync resource set follows the same mechanism as LTE-V2X.

| Company | Views |
| --- | --- |

## Issue 8 Timing determination of S-SSB

Description: A UE should be able to determine the frame timing, slot timing, and symbol timing from a received S-SSB, and the specification reflecting this aspect is still not complete.

| Company | Views |
| --- | --- |