3GPP TSG RAN WG1 #100bis						R1-2001892

e-Meeting, April 20th – 30th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Feature lead summary#1 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Potential issues for email discussions

- Power control

  - Issue 1 : Handling TX and RX of multiple PSFCHs

    - PSD of each PSFCH when transmitting multiple PSFCH TX

    - Prioritization between TX and RX when the UE is required to TX/RX multiple PSFCH

  - Issue 2: Details of SL/UL prioritization

    - Prioritization in the cases mentioned in RAN2 LS (R1-2000161)

  - Issue 3: Details of UL and SL power sharing

    - Prioritization between UL TX and SL TX in case of simultaneous TXs of UL and SL across difference carriers

- HARQ feedback

  - Issue 4: Details of indicating SL HARQ feedback related information

    - How to indicate HARQ feedback Option to RX UE.

    - How to indicate whether SL HARQ feedback is enabled or disabled to RX UE

  - Issue 4A: Whether/how to support mixing blind and feedback-based retransmissions of a TB

  - Issue 5: Whether/how to capture UE procedure for receiving HARQ-ACK

  - Issue 6: Exact location of PSFCH slots in the time domain in a given resource pool

- SL CSI

  - Issue 7: How to determine the latency bound of SL CSI reporting

  - Issue 8: Definition of SL CSI reference resource

  - Issue 9: Whether/how to associate the reported CSI with the CSI trigger

Company input on the identification of email discussion topics:

| Company | Prioritized issues | Comments |
| --- | --- | --- |
| LGE | Issue 1, Issue 2, Issue 4, Issue 7 | Issue 9 can be discussed together with Issue 7 as we think they are can be accommodated in a single thread while they are related with each other (i.e., Issue 9 needs to be considered if the CSI reporting latency budget is not so small so that it is likely to trigger CSI report multiple times within the latency bound). |
| TCL | Issue 4, Issue 1, Issue 9, Issue 2 | We agree with LGE comment that issue 7 and 9 may potentially be treated under a single thread. |
| ZTE, Sanechips | Issue 1, Issue {2,3}, Issue 4, Issue 7 | Issues 2 and 3 are related to each other, and can be treated together. |
| Panasonic | Issue 1, Issue 2, Issue 4, Issue 7 | We are OK with LGE’s suggestion on prioritized issues. |
| OPPO | Issue 1, Issue {2,3}, Issue 4, Issue 7 | We agree with ZTE’s suggestion. |
| NEC | Issue 1, Issue {2,3}, issue 4, Issue 9, | We agree with ZTE’s suggestion. |
| CATT | Issue 1, Issue {2,3}, Issue 4, Issue 7 | We share the same views with ZTE and OPPO |
| Intel | 1, {2,3}, 4, {7,8,9} | 2 and 3 can be bundled. In CSI we should resolve 7 due to RAN2 impact, but we can also touch 8 and/or 9 to start resolving these issues, and potentially continue in the next meeting. At least issue 8 has many discussion points which may be hard to close in one meeting next time. |
| NTT DOCOMO | Issue 1, Issue 6, Issue 7, Issue 4 | We are OK to discuss issue 7 and issue 9 in a single thread. |
| Ericsson | Issue 1, Issue 2, Issue 4, Issue 7 |  |
| Fujitsu | Issue 1, Issue {2,3}, Issue 4, Issue 7 | We agree with ZTE’s suggestion. |
| vivo | Issue 1, Issue {2,3}, Issue 4, Issue {7,9} | Issue 2 and 3 can be bundled together. Issue 7, and 9 can also be combined. Although Issue 8 is important, it may become too large for a mail thread to combining 7, 8, and 9. |
| Huawei, HiSilicon | Issue 7, 8, 1, {2+3} | For issue {2+3}, we can likely split the handling between April and May meetings by the feature lead proposing a suitable sub-scope per meeting, since the impact should be almost confined to RAN1 given that RAN2 have made extensive agreements on their parts. |
| Apple | Issue 1, Issue 2, Issue 4, Issue 7 | We are also OK to discuss Issue 2 together with Issue 3. |
| Futurewei | 1, {2,3}, 4, {7, 8, 9} | We are also fine treating only one or two of {7, 8, 9}. However, given that they all relate to the same topic, they might be addressed in a single discussion |
| InterDigital | 1, {2, 3}, 4, {7, 9} | If we have to choose between 7 and 9, the issue 7 seems more urgent as RAN2 is waiting for more details. Otherwise, good to discuss them together. |
| Sharp | 1, 2, 4, 9 |  |
| QC | 4, {2, 3}, Mixed of blind and feedback retransmission, 1 | Mixed of blind and feedback retransmission Is suggested to be treated in PHY procedure instead of Mode2. |
| Samsung | 1, {2,3}, 4, 7 | We agree to discuss Issue 2 and Issue 3 together. Issue 7 needs to be resolved due to RAN2 impact. Combining issue 7 together with 8&9 will make the the scope too large and may impact the progress. |
| Fraunhofer | 1, {2,3}, 4, {7,9} | We agree with the FL’s latest proposals |
| Bosch | {4, 4A,5,6}, {7,9} | We agree with the FL suggested issues. We consider the ones indicated here are very essential to be discussed during this meeting. |
| Nokia, NSB | {2, 3}, 4, {7, 8, 9} | Regarding to the proposed [100b-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-04], we shall consider the details of SL-CSI-RS resources (Issue 8) as the top issue. We agree with other 3 email discussion proposals. |

Proposed email discussion topics:

- [100b-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-01] Handling TX and RX of multiple PSFCHs

  - PSD of each PSFCH when transmitting multiple PSFCH TX

  - Prioritization between TX and RX when the UE is required to TX/RX multiple PSFCH

- [100b-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-02] SL/UL prioritization and UL/SL power sharing

  - Prioritization in the cases mentioned in RAN2 LS (R1-2000161)

  - Prioritization between UL TX and SL TX in case of simultaneous TXs of UL and SL across difference carriers

- [100b-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03] Indicating SL HARQ feedback related information

  - How to indicate HARQ feedback Option to RX UE

  - How to indicate whether SL HARQ feedback is enabled or disabled to RX UE

  - [Whether to support mixing blind and feedback-based retransmissions of a TB]

- [100b-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-04] CSI reporting latency bound and association with CSI trigger

  - [Introduction and time/frequency location of SL CSI reference resource]

  - How to determine the latency bound of SL CSI reporting

  - Whether/how to associate the reported CSI with the CSI trigger

- Power control

  - Handling TX and RX of multiple PSFCHs

- Issue 2-1-1: How to perform power control for multiple PSFCH transmission in a PSFCH transmission occasion.

  - Upper bound for the transmit power of each PSFCH TX

    - P_CMAX

      - Support: [LG,6] [Apple,19]

    - P_CMAX – 10log10(N) where N is the number of PSFCH TXs

      - Support: [Huawei,1] [vivo,2] [Lenovo,9] [CATT,13] [CMCC,15] [Ericsson,16] [Spreadtrum,17]

  - For power-limited scenario, what is the number of PSFCH TXs by a UE in a PSFCH TX occasion

    - Up to UE implementation: [Huawei,1] [vivo,2] [ZTE,7] [CATT,13] [CMCC,15] [Spreadtrum,17]

    - The number is determined such that the total transmit power of PSFCH TXs is lower than P_CMAX: [LG,6] [Apple,19]

    - Maximum supportive value: [Samsung,14] [Ericsson,16]

      - Determined by UE capability: [Samsung,14]

  - For non-power-limited scenario, what is the number of PSFCH TXs by a UE in a PSFCH TX occasion

    - Up to UE implementation: [vivo,2] [ZTE,7] [CMCC,15]

    - Maximum supportive value: [Huawei,1] [LG,6] [CATT,13] [Samsung,14] [Ericsson,16] [Spreadtrum,17] [Apple,19]

      - Determined by UE capability: [Huawie,1] [Samsung,14]

      - 5: [LG,6]

- Issue 2-1-2: How to select between PSFCH TXs and PSFCH RXs

  - The highest priority in the set of PSFCH TXs and the highest priority in the set of PSFCH RXs are used

    - Support: [LG,6] [Samsung,14] [Sharp,22] [NTT,23]

  - Whether or not to drop a subset of PSFCH RXs when PSFCH RXs are prioritized.

    - Yes:

    - No: [LG,6]

- Others

<<<Feature lead proposal will be added.>>>

  - Handling SL and UL transmissions

- Issue 2-2-1: How to define priority of UL transmission

  - Reuse priorities defined in RAN2 for PUSCH with UL-SCH without SL HARQ feedback, PUCCH with SR for SL or UL, PRACH, MSG3

    - Support: [LG,6] [CATT,13]

    - Other views:

      - [Lenovo,9]: RACH on SCell, SRS, CSI are deprioritized over SL TX.

      - [Spreadtrum,17]: the priority of PRACH is configured.

  - A priority of PUCCH carrying SL HARQ reporting is the same as that of the corresponding PSFCH

    - Support: [Huawei,1] [OPPO,3] [LG,6] [CATT,13] [Spreadtrum,17] [Apple,19] [Panasonic,20]

  - A priority of PUSCH carrying SL HARQ reporting without UL-SCH is the same as that of the corresponding PSFCH

    - Support: [Huawei,1] [OPPO,3] [LG,6] [Apple,19]

  - PUSCH carrying SL HARQ reporting with UL-SCH has

    - both UL priority of UL-SCH and SL priority of SL HARQ reporting

      - Support: [Huawei,1] [Apple,19]

    - UL priority of UL-SCH only

      - Support: [LG,6]

  - A priority of PUCCH carrying UCI except for SR is

    - Prioritized over SL TX: [vivo,2]

    - Determined by LTE UL/SL prioritization: [LG,6]

    - Configured: [CATT,13] [Spreadtrum,17]

    - UE implementation: [Panasonic,20]

- Issue 2-2-2: How to define priority of SL transmission

  - A priority of PSFCH is the same as that of the corresponding PSSCH.

    - Support: [vivo,2] [OPPO,3] [LG,6] [Intel,11] [CATT,13] [Spreadtrum,17] [Panasonic,20]

  - A priority of S-SSB is

    - (Pre)configured

      - Support: [OPPO,3] [LG,6] [Intel,11] [CATT,13] [Spreadtrum,17]

    - Deprioritized over UL TX

      - Support: [Huawei,1]

- Issue 2-2-3: When to prioritize which TX in case of simultaneous TXs of UL and SL

  - UL TX for URLLC is always prioritized over SL TX

    - Support: [Huawei,1] [ZTE,7] [Ericsson,16] [Apple,19]

    - Depending QoS of SL TX, SL TX can be prioritized [CMCC,15]

  - Reuse UL-SL prioritization made in RAN2 for UL transmission with available priority information (Use UL priority, SL priority, UL threshold, and SL threshold)

    - Support: [vivo,2] [Nokia,4] [LG,6] [Lenovo,9] [CATT,13] [Spreadtrum,17]

  - Reuse LTE UL-SL prioritization (Use SL priority and SL threshold)

    - Support: [Futurewei,12] [Samsung,14] [Ericsson,16] [Apple,19] [NEC,21] [Qualcomm,25]

  - Configure whether UL TX is prioritized or not

    - Support: [NEC,21]

  - Up to UE implementation

    - Support: [Panasonic,20]

  - For more than one SL transmissions overlapping with a UL transmission, the highest priority of SL transmissions is used for the comparison

    - Support: [Huawei,1] [LG,6] [NEC,21]

  - For more than one UL transmissions overlapping with a SL transmission, the highest priority of UL transmissions is used for the comparison

    - Support: [LG,6]

- Others

  - Consideration on how to support power sharing between UL transmission and SL transmission for dual connectivity [LG,6]

    - Reuse power control for NE-DC and EN-DC for simultaneous transmission of SL transmission and UL transmission on different carriers

    - NR-DC with Semi-static-mode1 or Semi-static-mode2 is supported for NR sidelink

<<<Feature lead proposal will be added.>>>

- HARQ feedback

  - Details of indicating SL HARQ feedback related information

- Issue 3-1-1: How to indicate whether SL HARQ feedback is enabled or disabled  to RX UE

  - Different 2nd-stage SCI formats

    - Support: [InterDigital,18] [Qualcomm,25]

  - Feedback request field in the 2nd-stage SCI

    - Support: [LG,6] [ZTE,7] [Fraunhofer,10] [CATT,13] [Spreadtrum,17] [Apple,19]

- Issue 3-1-2: How to indicate GC HARQ feedback Option 1/2 to RX UE

  - Alt 1: Introduce two 2nd-SCI format (One includes Zone ID field and communication range requirement field, and the other does not include these fields)

    - Support: [OPPO,3] [Nokia,4] [LG,6] [ZTE,7] [TCL,8] [Lenovo,9] [Fraunhofer,10] [CATT,13] [CMCC,15] [Ericsson,16] [Spreadtrum,17] [Apple,19] [Qualcomm,25]

      - 2nd-SCI format without Zone ID field and communication range requirement field has indicator to indicate groupcast HARQ feedback Option

        - Support: [OPPO,3] [LG,6] [ZTE,7] [Fraunhofer,10] [CATT,13]

  - Alt 2: GC HARQ feedback Option 1/Option 2 always use the same 2nd-stage SCI format

    - Support: [vivo,2] [Futurewei,12] [Samsung,14]

      - One code point of Communication range requirement field is used for the indication: [vivo,2]

      - Feedback option indicator in the 2nd-stage SCI is used for the indication: [Futurewei,12] [Samsung,14]

- Issue 3-1-3: Whether or how to indicate groupcast and unicast to RX UE

    - Unicast HARQ feedback is separately indicated by SCI

      - Support: [Nokia,4] [LG,6] [Fraunhofer,10] [Spreadtrum,17] [Qualcomm,25]

        - Alt 1: Different 2nd-SCI formats indicated by the 1st-SCI

          - Support: [Nokia,4] [Qualcomm,25]

        - Alt 2: 2nd-SCI indicates whether the RX UE performs groupcast HARQ feedback or unicast HARQ feedback

          - Support: [LG,6] [Fraunhofer,10] [Spreadtrum,17]

      - Not support: [OPPO,3] [ZTE,7] [CATT,13] [Samsung,14]

<<<Feature lead proposal will be added.>>>

  - Mix of blind retransmission and feedback-based retransmission for a TB

- Issue 3-2-1: Whether or how to support mixing blind and feedback-based retransmissions of a TB

  - Mix of blind retransmission and HARQ based retransmission is supported [OPPO,3] [Intel,11]

  - Further consideration on the restriction on the gap between PSSCHs with HARQ feedback is enabled for a TB [OPPO,3] [Intel,11]

  - Further consideration on the restriction on the gap between PSSCH with HARQ feedback is enabled and PSSCH with HARQ feedback is disabled for a TB [Intel,11]

  - Further consideration on the restriction on the gap between PSSCHs with HARQ feedback is disabled for a TB

  - Further consideration on the restriction on the gap between PSSCH with HARQ feedback is disabled and PSSCH with HARQ feedback is enabled for a TB

<<<Feature lead proposal will be added.>>>

  - Reporting HARQ status of the received PSFCH to higher layer

- Issue 3-3-1: Whether or how to capture UE procedure for receiving HARQ-ACK feedback on sidelink in TS38.213

  - Depending on cast type, GC HARQ feedback Options, PHY layer determines HARQ-ACK status to report to MAC layer [LG,6]

    - Rationale: MAC will define the UE behavior such as commencing retransmission based on the SL HARQ status reported from PHY

<<<Feature lead proposal will be added.>>>

  - Exact location of PSFCH slots in the time domain in a given resource pool

- Issue 3-4-1: What is the slot offset for the PSFCH resource in time domain in a resource pool

  - (periodPSFCHresource - 1)-th slot in a (or the first) period of a resource pool is the initial PSFCH slot in the period [NTT,23]

- Issue 3-4-2: Whether or how to allocate PSFCH resource across different periods of a resource pool

  - Rx UE doesn’t transmit PSFCH corresponding to PSSCH on orphan slots.  PSFCH location is calculated form the start of a resource pool period [Panasonic,20]

  - the number of slots in a period of a resource pool shall be a multiple of periodPSFCHresource [NTT,23]

<<<Feature lead proposal will be added.>>>

- Sidelink CSI

  - Signaling details of SL CSI

- Issue 4-1-1: How to configure latency bound for SL CSI reporting MAC CE

  - Option 1: (Pre)configuration in a resource pool indicates the latency bound value.

    - Support: [ZTE,7] [Samsung,14]

  - Option 2: PC5-RRC signaling indicates the latency bound value.

    - Support: [Nokia,4] [LG,6] [Lenovo,9] [Intel,11] [Futurewei,12] [CATT,13] [Apple,19] [Panasonic,20] [NTT,23]

  - Option 3: Explicit SCI indication

    - Support: [Huawei,1]

  - Comment from [OPPO,3]

    - L1-priority as indicated by SCI triggering the SL CSI reporting MAC CE is associated with the latency bound value.

  - Comment from [InterDigital,18]

    - Latency bound is determined based on UE speed

<<<Feature lead proposal will be added.>>>

  - Definition of SL CSI reference resource

- Issue 4-2-1: How to define SL CSI reference resource

  - CSI reference resource slot is a slot where SCI triggering the sidelink CSI reporting is transmitted

    - Support: [Huawei,1] [vivo,2] [LG,6] [CATT,13] [Samsung,14] [Apple,19] [Panasonic,20]  [NTT,24]

  - The same bandwidth as allocated for the PSSCH reception scheduled by SCI triggering the sidelink CSI reporting

    - Support: [Huawei,1] [vivo,2] [LG,6] [Intel,11] [Futurewei,12] [CATT,13] [Samsung,14] [Apple,19] [Panasonic,20]

  - PSCCH overhead

    - Predefined overhead: [vivo,2]

    - (Pre)configured overhead: [LG,6] [Intel,11] [Panasonic,20]

  - PSSCH symbol duration

    - Predefined value: [vivo,2]

    - (Pre)configured overhead: [LG,6] [Intel,11] [Apple,19]

    - Actual overhead of PSSCH containing CSI-RS: [Panasonic,20]

  - No CSI-RS overhead is used

    - Support: [vivo,2] [LG,6] [Intel,11]

  - 2nd-stage SCI overhead

    - No 2nd-stage SCI overhead is used: [LG,6] [Intel,11]

    - Lowest overhead per (pre)configuration is used: [Intel,11]

    - Actual overhead of PSSCH containing CSI-RS: [Panasonic,20]

  - Assumption on the number of DMRS symbol

    - Actual overhead of PSSCH containing CSI-RS: [vivo,2] [Panasonic,20]

    - Lowest density per (pre)configuration: [LG,6] [Intel,11]

    - (Pre)configured overhead: [LG,6]

- Issue 4-2-2: How to configure CQI table used for CSI reporting

  - Option 1: PC5-RRC configuration

    - Support: [Huawei,1] [Futurewei,12]

  - Option 2: (Pre)configuration

    - Support: [vivo,2]

- Issue 4-2-3: How to associate SL CSI reference resource(s) and SL CSI reporting

  - SL CSI reporting is associated with the latest received CSI-RS [Huawei,1] [NTT,23]

  - SL-CSI feedback windows for different SL CSI-RS transmission occasions shall not be overlapped in time domain [CATT,13]

  - Each CSI feedback trigger and/or reporting should include an associated index [InterDigital,18]

<<<Feature lead proposal will be added.>>>

References

- R1-2001555	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

- R1-2001665	Remaining issues on physical layer procedure for NR sidelink	vivo

- R1-2001748	Remaining issues of physical layer procedure for NR-V2X	OPPO

- R1-2001808	Remaining details of Physical layer procedures for sidelink	Nokia, Nokia Shanghai Bell

- R1-2001879	Remaining issues on power reduction rule for NR V2X	Fujitsu

- R1-2001889	Discussion on physical layer procedure for NR sidelink	LG Electronics

- R1-2001898	Remaining issues on PHY procedures for Rel-16 sidelink	ZTE, Sanechips

- R1-2001965	Physical layer procedures for sidelink	TCL Communication Ltd.

- R1-2001970	Discussion on physical layer procedures for NR sidelink	Lenovo, Motorola Mobility

- R1-2001979	Remaining Issues in Physical Layer Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS

- R1-2001996	Solutions to remaining opens of physical layer procedures for NR V2X sidelink design	Intel Corporation

- R1-2002043	Remaining details on physical procedures for sidelink	Futurewei

- R1-2002080	Remaining issues on physical layer procedures for NR V2X	CATT

- R1-2002129	On Physical Layer Procedures for NR Sidelink	Samsung

- R1-2002207	Remaining issues on physical layer procedures for sidelink	CMCC

- R1-2002238	Physical layer procedures for NR sidelink	Ericsson

- R1-2002269	Remaining issues in physical layer procedures for sidelink	Spreadtrum Communications

- R1-2002302	Remaining Issues on Physical Layer Procedures for NR V2X	InterDigital, Inc.

- R1-2002327	On Remaining Details of Physical Layer Procedures for NR V2X Sidelink	Apple

- R1-2002359	Remaining issue on physical layer procedures for sidelink in NR V2X 	Panasonic Corporation

- R1-2002363	Remaining issues on physical layer procedure	NEC

- R1-2002390	Remaining issues on physical layer procedures for NR sidelink	Sharp

- R1-2002441	Remaining issues on sidelink physical layer procedure	NTT DOCOMO, INC.

- R1-2002473	Remaining issues on sidelink physical layer procedure on NR V2X	ASUSTeK

- R1-2002542	Physical layer procedures for sidelink	Qualcomm Incorporated