Issue 3&4

Proposal:

- A DCI format 2_4 is only applicable to an uplink grant scheduling PUSCH/SRS if the ending symbol of the PDCCH carrying the UL grant is earlier than the first symbol of the PDCCH carrying DCI format 2_4. If the UE does not cancel a transmission in resources indicated by DCI format 2_4, the UE can receive an UL grant scheduling a transmission in any resources and transmit accordingly, if the ending symbol the PDCCH carrying UL grant is no earlier than the first symbol of the PDCCH carrying DCI format 2_4.

- UE does not expect to receive a PDCCH carrying UL grant with its ending symbol no earlier than the first symbol of a PDCCH carrying DCI format 2_4 scheduling a transmission in resource colliding with the ones indicated by DCI formant 2_4 or in the cancelled symbols that do not overlap with the resource indicated by the DCI format 2_4, if the UE has to cancel another transmission in resource indicated by the DCI format 2_4

  - FFS under what condition (if there is such a condition), that UE can be scheduled by an UL grant to transmit in any resource if the UE has to cancel a transmission in the resources indicated by the DCI format 2_4, and if the UL grant is received after the DCI format 2_4

Issue 6: BD limitations for UL CI monitoring

Proposal 1:

- Up to X BDs can be configured per UL CI monitoring occasion

- Option 1: X=1

- Option 2: X=2

- Option 3: X = 1 or 2 reported as UE capability

- Suggest to agree on X=1

Issue 7: Misaligned RUR start due to different UL SCS used by UEs monitoring the same UL CI.

- Suggest to agree on Option 3

Option 1:

•       Update the value range of  RRC parameterdelta_offset_d  as {0,1,2,3,4,5,6,7,8}

•       Update the RRC parameter list allowing delta_offset_d to be configured per UL BWP

•       adopt the following text proposal for 38.213 section 11.2A

TP for 38.213 Section 11.2A:

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is afterTproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, whered is provided byXXX using the SCS configuration of the PUSCH transmission or SRS transmission on the serving cell.Tproc,2 corresponds to the PUSCH processing capability 2[6, TS 38.214] assumingd2,1=0 withμ beingthe smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of anSRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that isTproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

Option 2:

- Introduce an RRC parameter to be used as a reference SCS to determineTproc,2 and offset d for ULCI.

  - The parameter is of type subcarrierspacing.

  - The network configures the reference SCS not larger than any SCS of the configured UL and DL BWPs for the serving cell.

  - Adopt the following text proposal

TP for 38.213 Section 11.2A:

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is afterTproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, whered is provided byXXX assuming SCS configured by high layer parameterreferenceSubcarrierSpacing-ULCI. Tproc,2 corresponds to the PUSCH processing capability 2[6, TS 38.214] assumingd2,1=0 withμ beingthe SCS configured by high layer parameterreferenceSubcarrierSpacing-ULCIthe smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of anSRStransmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

Option 3

  - UE uses the smallest SCS configuration between the SCS configuration of the PDCCH for DCI format 2_4 detection and the SCS configurations in scs-SpecificCarrierList of UL carrier to determine the RUR starting symbol.

  - UE uses the smallest SCS configurations in scs-SpecificCarrierList of UL carrier to determine offset d.

TP for option 3 in 38.213 section 11.2A

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after from the end of a PDCCH reception where the UE detects the DCI format 2_4, where is provided by XXX with the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier. corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and the SCS configurations provided in scs-SpecificCarrierList of UL carrier of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is after a last symbol of a CORESET where the UE detects the DCI format 2_4. <---------------------------Other parts are omitted ------------------------------->