# 3GPP TSG RAN WG1 #100bis                                                                               R1-20xxxxx

e-Meeting, April 20th – 30th, 2020

Title:	Summary of enhanced UL configured grant transmission for URLLC

Source:	Moderator (vivo)

Agenda Item:	7.2.5.6

Document for: 	Discussion and Decision

- Introduction

Based on the phase 1 discussions and suggestions, Chairman allocates following two email discussions for eCG for URLLC. In this document, [100b-e-NR-L1enh_URLLC-eCG-01] will be the focus. It is noted that the deadline for agreements/conclusions is 4/24, and the deadline for the corresponding TP is by 4/29.

- [100b-e-NR-L1enh-URLLC-eCG-01] Email discussion/approval regarding DMRS and PTRS operation for dynamic PUSCH and configured grant PUSCH (section 3.1.1. 3.1.2 and 3.1.3 in R1-2001796) till 4/24, and potential TPs for approval by 4/29 (vivo, Lihui)

- [100b-e-NR-L1enh-URLLC-eCG-02] Email discussion/approval regarding corrections as in section 3.2.1, 3.2.2 and 4.1 in R1-2001796 till 4/24, and potential TPs for approval by 4/29 (vivo, Lihui)

- DMRS and PTRS operation for DG and CG PUSCH

  - DMRS operation for Type 2 CG activated by DCI format 0_2 in case Antenna ports field is not present in DCI format 0_2

In the RAN1#100 e-meeting, the third email discussion [R1-2001424, URLLC-eCG-03] was postponed since it depends on the decision made in PDCCH session. It was the common understanding during the mail discussion that the DMRS operation for Type 2 configured grant PUSCH activated by DCI format 0_2 should be the same as that for dynamic PUSCH scheduled by DCI format 0_2 in case the antenna ports field is not present in DCI format 0_2.

For DG PUSCH, it was agreed that antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of 38.212 when the antenna ports bit field is not present in DCI 0_2 [TS 38.212]. Therefore, following proposals are made:

Proposal 1:

- If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2,

- the UE shall not use the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI with NDI= 0; the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0 is zero.

- for the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of TS 38.212.

- FL suggestion: agree proposal 1.

Any comments?

| Company | View |
| --- | --- |
| Nokia, NSB | We are fine with the proposal. |
| CATT | We are fine to the proposal |
| Samsung | Fine with the proposal. |
| Panasonic | We are fine to the proposal. |
| LG | We are fine with the proposal. |
| QC | Agree with proposal |
| MediaTek | We are fine with the proposal. |
| ZTE | Fine with the proposal. |
| Sharp | We are OK with the proposal. |
| vivo | Support the proposal. |
| Huawei, HiSilicon | We support the proposal. |
| Apple | We are fine the proposal |
| OPPO | We are fine the proposal |
| Ericsson | Support the proposal |
| ITRI | Support the proposal |

If above proposal is agreeable, then we can discuss whether and how to capture above into spec.

- [R1-2001616, ZTE] proposed no text proposal is needed given the current text in TS 38.212 can also cover the case for Type 2 CG PUSCH.

- [R1-2001546, HW] proposed following TP for Clause 7.3.1.1.3 in TS 38.212:

## Text proposal for Clause 7.3.1.1.3 in TS 38.212 v16.1.0 --------------------------------------- Start of Text Proposal ---------------------------------------------- ##### 7.3.1.1.3 Format 0_2 < Unchanged parts are omitted > If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, and DCI format 0_2 is with CRC scrambled by C-RNTI or SP-CSI-RNTI or MSC-C-RNTI or CS-RNTI with NDI set to ‘1’, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2, and DCI format 0_2 is with CRC scrambled by CS-RNTI and with NDI set to ‘0’, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. < Unchanged parts are omitted > --------------------------------------------- End of Text Proposal -----------------------------------------

[R1-2001674, vivo] proposed following TP:

##### 7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with cg-DMRS-Configuration in configuredGrantConfig, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 for PUSCH transmission scheduled by DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0. <Unchanged parts are omitted> A UE does not expect that the bit width of a field in DCI format 0_2 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_2 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_2 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_2 with CRC scrambled by C-RNTI for the same serving cell. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2, the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI is zero. <Unchanged parts are omitted>

Any comments or preference for above TPs?

| Company | View |
| --- | --- |
| Nokia, NSB | Slight preference for the HW version – as it is more precise in the applicability. comments on the vivo TP: - cg-DM-RS config is a mandatory parameter for CG – so no need to say if configured. So we prefer the HW version in this respect as it is more precise for which NDI we do what - the second addition (at the end) is not needed, as already stated in the field description that the size is 0 if AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured. So no need to repeat this here. |
| CATT | Above proposed TP is unnecessary because current spec already has corresponding description as below in 7.3.1.1.3 in 38.213 g10. “If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.” |
| Samsung | We slightly prefer ZTE’s comments. It is not necessary to capture duplicated sentence in 214 since 212 already provided same sentence with following. 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: [omitted part is not changed] If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. |
| LG | We prefer second TP with some modifications. In 7.3.1.1.3, antenna port field already has the sentence, “0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured;”. Therefore, latter part of change is not necessary. And it would be better to specify RNTI and NDI on the first part of second TP, like first TP. |
| QC | TP is not needed since current specification already captures the case, as CATT mentioned. |
| MediaTek | It seems the TP is not needed as CATT highlighted. |
| ZTE | No text proposal is needed given the current text in TS 38.212 can also cover the case for Type 2 CG PUSCH. |
| Sharp | We share the same views with CATT and ZTE. No text proposal is needed. |
| vivo | We are fine not to capture above TP if everyone think it is already clear from current spec. |
| Huawei, HiSilicon | We think a TP is needed and we support the first TP. To our understanding, Proposal 1 covers following two cases for Type 2 CG: - Case 1: UE is not configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. - Case 2: UE is not configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, and also not configured with dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. However, the following description in current spec only covers Case 1. Therefore, TP is needed to also cover Case 2 for Type 2 CG. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.. |
| OPPO | The same opinion as CATT, TP is not needed. |
| Ericsson | To address Huawei’s concern over Case 2, the following TP is simpler: “If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23.” |
| ITRI | This TP seems not necessary. |

  - Correction to DMRS transmission procedure for PUSCH scheduled by DCI format 0_2

[Nokia, R1-2001698, R1-2001694] proposed to adopt the following text proposal for PUSCH DMRS transmission with DCI format 0_2 to Sec. 6.1.4.2 and 6.2.2 of TS 38.214, because:

- If dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat1_2 are not configured, the DMRS transmission procedure for PUSCH scheduled by DCI format 0_2 follows the fallback DCI operation (i.e. DCI format 0_0)

- Otherwise (i.e. if configured), the procedures of PUSCH scheduled by DCI format 0_1 apply also for PUSCH scheduled by DCI format 0_2 by using the separately configured DMRS parameters.

TP to TS 38.214, Sec. 6.1.4.2 & 6.2.2: UE DM-RS transmission procedure description for DCI format 0_2 ### 6.1.4.2 Transport block size determination For a PUSCH scheduled by RAR UL grant or for a PUSCH scheduled by fallbackRAR UL grant or for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or for a PUSCH transmission with configured grant, or for a MsgA PUSCH transmission, if - and transform precoding is disabled and Table 5.1.3.1-2 is used, or - and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or - and transform precoding is enabled, the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot: - A UE first determines the number of REs allocated for PUSCH within a PRB by - , where is the number of subcarriers in the frequency domain in a physical resource block, Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH of Clause 6.1.2.3 for configured PUSCH, is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 and DCI format 0_2 in Clause 6.2.2, and is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the is not configured (a value from 6, 12, or 18), the is assumed to be 0. For Msg3 transmission the is always set to 0. <Unchanged text is omitted> ### 6.2.2 UE DM-RS transmission procedure If both higher layer parameter dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and higher layer parameter dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 are not configured, the DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_0 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2; Otherwise, the DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure,. the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. <Unchanged text is omitted>

[CATT, R1-2002087, R1-2002082] proposed a similar TP for TS 38.214  as shown below:

-------------------------------------------------- Start of text proposal ------------------------------------------------------ ### 6.2.2 UE DM-RS transmission procedure The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this section equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1 0 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedureor by RAR UL grant. , the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and < -----------------------text omitted-----------------------> < -----------------------to be continued-----------------------> - and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Clause 6.4.1.1.3 of [4, TS 38.211]. A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 0_1 as described in Clause 7.3.1.1 of [5, TS 38.212]. If a UE transmitting PUSCH is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH < -----------------------text omitted----------------------->

From FL perceptive, the TP proposed by Nokia is more complete by including both cases where dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat1_2 are configured and mrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat1_2.  The second correction “When transmitted PUSCH is neither scheduled by DCI format 0_1 0 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedureor by RAR UL grant. ,”  proposed by CATT by changing the negative expression to positive description is unnecessary, especially it involves operation for 2-step RACH which is beyond our expertise.  Therefore,

- FL suggestion: adopt the TP from [Nokia, R1-2001698, R1-2001694] for PUSCH DMRS transmission with DCI format 0_2 to Sec. 6.1.4.2 and 6.2.2 of TS 38.214.

Any comments?

| Company | View |
| --- | --- |
| Nokia, NSB | We are fine with FL proposal. Update based on the inputs by LGE there seems to be no reason to not use the procedure from 0_1 also for 0_2, independently if the DM-RS config for DCI format 0_2 is provided or not. As a consequence, the version by Samsung below seems to be sufficient: The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this section equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure |
| CATT | - We would like to suggest two ways for full description as follows: - CATT proposed TP + Nokia’s proposal on the case where dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat1_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat1_2 aren’t configured as follows If both higher layer parameter dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and higher layer parameter dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 are not configured, the DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_0 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2; - Or Nokia’s proposal + CATT proposal on how to determine DMRS-port description as follows: A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 0_1 as described in Clause 7.3.1.1 of [5, TS 38.212]. - We support changing the negative expression to positive description on 2nd correction in FL proposal because this will make description on PUSCH transmission condition more clear |
| Samsung | We prefer CATT’s first TP proposal since there is no need to add UE behaviour when both configurations are not configured because UE behaviour has not been defined when both “dmrs-UplinkForPUSCH-MappingTypeA” and “dmrs-UplinkForPUSCH-MappingTypeB” are not configured for DCI format0_1. Following is our suggestion with some modifications. The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this section equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure, |
| Panasonic | We agree to the FL suggestion. |
| LG | We don’t have strong view, however, we think this change doesn’t make much difference especially for the case that dmrs-UplinkForXXX are not configured. Without this TP, UE shall use default parameter which is specified 38.331. Those are almost same as fallback procedure. For example, UE use maxlength=len1, dmrs-type=type1 and dmrs-AdditionalPosition= pos1 or pos2 according to whether FH is enabled or not. In our view, we think only following part are necessary. The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. |
| QC | We are fine with Nokia’s proposal but TP for 38.214 Sec. 6.2.2. needs a modification. Specifically, “If both higher layer parameter ….” Should be replaced with“if none of higher layer parameter dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and higher layer parameter dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is configured, …” |
| ZTE | We slightly prefer what Samsung proposed above. |
| Sharp | We support the FL suggestion. |
| vivo | Support FL suggestion. |
| Apple | Samsung’s proposal is reasonable. |
| OPPO | We are fine with FL suggestion. |
| Ericsson | TP suggested by Samsung is sufficient |
| CATT2 | Regarding the proposed TP on antenna port determination, we clarify only first TP proposal in our contribution isn’t enough because there is no any description on how to determine antenna port in case AP field is absent in the DCI while the [DMRS configuration parameters] are configured in 6.2.2 DMRS transmission procedure section of 38.214. In addition, there is already similar description as blow on how to determine antenna port in 5.1.6.2 of 38.214. “A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 1_1 as described in Clause 7.3.1.2 of [5, TS 38.212].” So below proposed TP is needed to make the UL procedure of determination antenna port more complete. A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 0_1 as described in Clause 7.3.1.1 of [5, TS 38.212]. |
| ITRI | Fine with FL suggestion. |

  - Missing PTRS transmission procedure for PUSCH scheduled by DCI format 0_1 & 0_2 (38.214 - Sec. 6.2.3 & 6.2.3.1, 38.212 – Sec. 7.3.1.1.2 & 7.3.1.1.3)

RAN1 had agreed to support independent PTRS configuration which is available due to the independent DMRS configurations for DCI format 0_2 and Rel-15 containing the RRC parameter phaseTrackingRS in DMRS-UplinkConfig.

Agreements:

Support new RRC configuration for “PTRS-DMRS association” in DCI format 0_2

Currently, the effect on PTRS is currently not at all addressed in Sec. 6.2.3 of TS 38.214. [Nokia, R1-2001694] proposes to adopt the following TP for PUSCH PTRS transmission with DCI format 0_2 to Sec. 6.2.3 & 6.2.3.1 of TS 38.214 with changes marked in red:

TP to TS 38.214, 6.2.3 and 6.2.3.1: UE PTRS transmission procedure description for DCI format 0_2 ### 6.2.3 UE PT-RS transmission procedure The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is configured, to PUSCH transmissions scheduled by DCI format 0_0 or format 0_1 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. #### 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled ## <Unchanged text is omitted> For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port. For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers which are indicated by Precoding information and number of layers field in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNnumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI as: - PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer [x] of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer [y] of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI, where [x] and/or [y] are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. ## <Unchanged text is omitted>

Similarly, [Nokia, R1-2001694] proposes to adopt the following text proposal on the PTRS-DMRS association field for DCI format 0_1 and 0_2 to Sec. 7.3.1.1.2 & 7.3.1.1.3 of TS 38.212 with changes marked in red:

TP to TS 38.212, 7.3.1.1.2 and 7.3.1.1.3: Correction to DCI field size determination for PTRS-DMRS association ##### 7.3.1.1.2 Format 0_1 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted> ##### 7.3.1.1.3 Format 0_2 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank-ForDCIFormat0_2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted>

- FL suggestion: adopt above TPs.

Any comments?

| Company | View |
| --- | --- |
| Nokia, NSB | We are fine to adopt these TPs. |
| CATT | We are fine to the proposed TP. |
| Samsung | Agree with motivation. It may need to change some words, for example, “a UE PUSCH transmission” seems not generally used in specification. |
| Panasonic | We agree to the FL suggestion. |
| LG | We are fine with the proposal. For the second TP, it would be better to clarify that UE is not configured with PTRS-UplinkConfig in any of respective DMRS-UplinkConfig for transmissions. 0 bit if PTRS-UplinkConfig in both dmrs-UplinkForPUSCH-MappingTypeA orand dmrs-UplinkForPUSCH-MappingTypeB |
| QC | We agree with FL proposal, for the second TP, text needs modification: 0 bit if PTRS-UplinkConfig in neither dmrs-UplinkForPUSCH-MappingTypeA nor dmrs-UplinkForPUSCH-MappingTypeB is configured |
| ZTE | We are fine with the TPs above. |
| Sharp | We are fine to adopt TPs above. |
| vivo | Support the TPs. |
| Huawei, HiSilicon | We are fine with the TPs above. |
| Apple | Agree with QC’s suggestion, can we also check whether a similar change “neither … nor …” is needed for 0_1. |
| OPPO | We are fine with TPs above. |
| Ericsson | Support the TPs |
| ITRI | Support the TPs. |

- Summary and Proposals

For section 2.1, DMRS operation for Type 2 CG activated by DCI format 0_2 in case Antenna ports field is not present in DCI format 0_2, all companies agree proposal 1. 8 companies think no TP is necessary. 1 company think TP is necessary; 3 companies give comments on the proposed TP.

For section 2.2, correction to DMRS transmission procedure for PUSCH scheduled by DCI format 0_2, all companies are fine to adopt the TP with some wording refinement.

For section 2.3, missing PTRS transmission procedure for PUSCH scheduled by DCI format 0_1 & 0_2 (38.214 - Sec. 6.2.3 & 6.2.3.1, 38.212 – Sec. 7.3.1.1.2 & 7.3.1.1.3), all companies are fine to adopt the TP with some wording refinement.

Following proposals are stable:

Proposal 1:

- If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2,

- the UE shall not use the parameter cg-DMRS-Configuration in configuredGrantConfig to derive the bit width of the Antenna ports field in DCI format 0_2 with CRC scrambled by CS-RNTI with NDI= 0; the UE determines that the bit width of the Antenna ports field in the DCI format 0_2 with CRC scrambled by CS-RNTI with NDI=0 is zero.

- for the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of TS 38.212.

Note: for proposal 1, check further whether TP is necessary or not.

Proposal 2-1: adopt following TP to TS 38.214 Section 6.2.2.

TP to TS 38.214, Sec. 6.2.2: UE DM-RS transmission procedure description for DCI format 0_2 ### 6.2.2 UE DM-RS transmission procedure The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB. When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure,. the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration. <Unchanged text is omitted>

| Company | Views on Proposal 2 |
| --- | --- |
| Nokia, NSB | First, as we now basically say we follow the operation of DCI format 0_1 here, there is no need for the change in 6.1.4.2 anymore (this would have only been needed in case we follow DCI format 0 when not configured with DMRS). Secondly, based on this agreement we would also need for the DCI format 0_2 some type of antenna port definition – so I guess we may need the following minor change in 38.212 for DCI format 0_2, namely: TP to 212: <Unchanged text is omitted> ##### 7.3.1.1.3 Format 0_2 <Unchanged text is omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, <Unchanged text is omitted> If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. <Unchanged text is omitted> |
| CATT | In order to guarantee the complete and more description of antenna port determination and void introducing R15 related correction as commented by FL, we suggest adding below sentence to 6.2.2 section in 38.214. A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 0_2 as described in Clause 7.3.1.1 of [5, TS 38.212]. |

- It is observed that HW, Ericsson and Nokia have a good alignment here on another TP to cover two cases:

  - Case 1: UE is not configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2.

  - Case 2: UE is not configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, and also not configured with dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2.

FL suggestion: adopt following TP for TS 38.212, section 7.3.1.1.3.

Proposal 2-2: adopt following TP for TS 38.212, section 7.3.1.1.3.

##### 7.3.1.1.3 Format 0_2 <Unchanged text is omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, <Unchanged text is omitted> If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 and is configured with AntennaPorts-FieldPresence-ForDCIFormat0_2, the bitwidth of this field equals maxxA,xB, where xA is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and xB is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of xA and xB. If a UE is not configured with higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 but configured with one or more of dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2, antenna port(s) are defined assuming bit field index value 0 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23. <Unchanged text is omitted>

Proposal 3: adopt following TP to TS 38.214, section 6.2.3 and section 6.2.3.1.

TP to TS 38.214, 6.2.3 and 6.2.3.1: UE PTRS transmission procedure description for DCI format 0_2 ### 6.2.3 UE PT-RS transmission procedure The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is configured, to PUSCH transmissions scheduled by DCI format 0_0 or format 0_1 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. #### 6.2.3.1 UE PT-RS transmission procedure when transform precoding is not enabled ## <Unchanged text is omitted> For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212]. For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0. For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port. For partial-coherent and non-coherent codebook based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers which are indicated by Precoding information and number of layers field in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNnumberOfLayers: - if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI as: - PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1. - UL PT-RS port 0 is associated with the UL layer [x] of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer [y] of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI, where [x] and/or [y] are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212]. ## <Unchanged text is omitted>

Proposal 4: adopt following TP to TS 38.212, section 7.3.1.1.2 and 7.3.1.1.3.

TP to TS 38.212, 7.3.1.1.2 and 7.3.1.1.3: Correction to DCI field size determination for PTRS-DMRS association ##### 7.3.1.1.2 Format 0_1 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in neither dmrs-UplinkForPUSCH-MappingTypeA nor dmrs-UplinkForPUSCH-MappingTypeB is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted> ##### 7.3.1.1.3 Format 0_2 ## <Unchanged text is omitted> - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig in neither dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 nor dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2 is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank-ForDCIFormat0_2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) for transmission of one PT-RS port and two PT-RS ports respectively, and the DMRS ports are indicated by the Antenna ports field. If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part. ## <Unchanged text is omitted>

- Reference

| R1-2001546 | Corrections on configured grant transmission | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2001616 | Remaining issues on enhancements for UL configured grant transmission | ZTE |
| R1-2001674 | Enhanced UL configured grant transmissions for URLLC | vivo |
| R1-2001698 | Maintenance of Rel-16 URLLC Configured Grant enhancements | Nokia, Nokia Shanghai Bell |
| R1-2001778 | Configured grant enhancements for URLLC | OPPO |
| R1-2001789 | Remaining Issue of Enhancements to UL Configured Grant Transmission for NR URLLC | Ericsson |
| R1-2001924 | Remaining issues of Enhanced UL configured grant transmission for NR URLLC | LG Electronics |
| R1-2002087 | Remaining issues on enhanced UL configured grant transmission | CATT |
| R1-2002334 | Remaining Issues in Enhanced Configured Grant Transmission | Apple |
| R1-2002412 | Maintenance of UL grant Type 2 scheduling activation | ITRI |
| R1-2002446 | Remaining issues for enhanced Configured grant for Rel.16 URLLC | NTT DOCOMO, INC. |