##### TP for TS 38.214 V16.1.0

##### 5.2.2.2.5			Enhanced Type II Codebook

<Unchanged text is omitted>

-	When R=2:

-	For each subband in csi-ReportingBand that is not the first or last subband of a BWP, two precoding matrices are indicated by the PMI: the first precoding matrix corresponds to the first NPRBSB/2 PRBs of the subband and the second precoding matrix corresponds to the last NPRBSB/2 PRBs of the subband.

-	For each subband in csi-ReportingBand that is the first or last subband of a BWP

-	If NBWP,istart mod NPRBSB≥NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the first subband. If NBWP,istart mod NPRBSB<NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the first subband: the first precoding matrix corresponds to the first NPRBSB2-NBWP,istart mod NPRBSB PRBs of the first subband and the second precoding matrix corresponds to the last NPRBSB2 PRBs of the first subband.

-	If NBWP,istart+NBWP,isize-1mod NPRBSB+1≤NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the last subband. If NBWP,istart+NBWP,isize-1mod NPRBSB+1>NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the last subband: the first precoding matrix corresponds to the first NPRBSB2 PRBs of the last subband and the second precoding matrix corresponds to the last NBWP,istart+NBWP,isize-1mod NPRBSB+1-NPRBSB2  PRBs of the last subband.

<Unchanged text is omitted>

Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where

Minitial∈-2Mυ+1,-2Mυ+2,…,0

n3,l=n3,l(0),…,n3,l(Mυ-1)

n3,lf∈0,1,…,N3-1

which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l (for Mυ>1 and l=1,…, υ), l=1,…, υ, where

i1,5∈0,1,…,2Mυ-1

i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2Mυ-1Mυ-1-1N3>19

<Unchanged text is omitted>

For all values of N3, n3,l(0)=0 for l=1,…,υ. If Mυ>1, the nonzero elements of n3,l, identified by  n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using  Cx,y as defined in Table 5.2.2.2.5-4 and the algorithm:

<Unchanged text is omitted>

When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows:

-	If  N3≤19, i1,5=0 and is not reported. If Mν=1, i1,6,l=0, for l=1,…,ν, and is not reported. If Mν>1, i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases.

<Unchanged text is omitted>

The codebooks for 1-4 layers are given in Table 5.2.2.2.5-5, where m1(i), m2(i), for i=0,1,…,L-1, vm1i,m2(i)un unvm,nare obtained as in clause 5.2.2.2.3, and the quantities φl,i,f and yt,l are given by

<Unchanged text is omitted>

Table 5.2.2.2.5-5: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

<Unchanged text is omitted>

Where Wq1,q2, n1, n2,n3,pl(1),pl2,i2,5,l,tl=1N1N2γt,li=0L-1vm1i,m2(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vm1i,m2(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, p1(1)p2(1)p3(1)p4(1) and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2.

<Unchanged text is omitted>

##### 5.2.2.2.6	Enhanced Type II Port Selection Codebook

<Unchanged text is omitted>

Table 5.2.2.2.6-2: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

<Unchanged text is omitted>

Where Wi1,1,n3,pl(1),pl2,i2,5,l,tl1N1N2γt,li=0L-1vi1,1d+ipl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vi1,1d+ipl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to i1,1, n3,1, n3,2, n3,3, n3,4, p1(1)p2(1)p3(1)p4(1) and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2.

<Unchanged text is omitted>