3GPP TSG RAN WG1#100bis			R1-2002717

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.6.1

Source: 	Moderator (Samsung)

Title: 	Feature lead summary#2 for MU-MIMO CSI

Document for:	Discussion and Decision

# Introduction

The FL summary of the proposals in the submitted contributions ([1]-[12]) for Rel.16 NR_eMIMO MU-CSI maintenance is given below and categorized under the following sections:

- High priority (essential)

- Editorial

- Low priority (non-essential)

- UE feature/capability related: This will be discussion as a part of UE feature session

Proposals on Rel.16 draft shadow CRs are not summarized here since they are to be discussed as a part of Rel.15 maintenance.

- Summary

# In reference to the categorization given in this section (2.1, 2.2, 2.3, and 2.4), it is proposed that:

- The following offline agreement and conclusion outlined in issue H.0 be endorsed

Offline agreement: On the support of CSI reporting for BWP size < 24 PRBs, only wideband CSI reporting is supported with Rel.15 Type I single-panel codebook - Note: a draft CR has been submitted to Rel.15 maintenance session for MIMO Offline conclusion: On the support of CSI reporting for BWP size < 24 PRBs, there is no consensus in supporting Rel.15 Type-I multi-panel, Rel.15 Type-II, and Rel.16 eType-II codebooks for wideband CSI reporting

- The combined TP in section 3  from issues H.1, H.2, E.1, E.2, and E.3, to be endorsed

  - High priority (essential)

The following issues pertain to some ambiguity in the current description of the specs and may have some significant impact on spec completeness and/or UE implementation. Some of these issues, however, are still subject to further assessment.

Table 1 High-priority (essential)

| Issue | Description/Proposal | Companies |
| --- | --- | --- |
| H.0: SB size is undefined when NPRB < 24 PRBs | Note: This item needs no further discussion since offline agreement has been reached. Action: Endorse offline agreement [1]: - The resulting draft CRs (Rel.15 and Rel.16 shadow) will be endorsed in Rel.15 maintenance session for MIMO | Moderator |
| H.1: For R=2, when NBWP,istart+NBWP,isizemod NPRBSB=0 the last sub-band size equals NPRBSB | TP for TS 38.214 for section 5.2.2.2.5 ##### 5.2.2.2.5 Enhanced Type II Codebook ## <Unchanged text is omitted> - If NBWP,istart+NBWP,isizemod NPRBSB≤NPRBSB2, NBWP,istart+NBWP,isize-1mod NPRBSB≤NPRBSB2,one precoding matrix is indicated by the PMI corresponding to the last subband. If NBWP,istart+NBWP,isizemod NPRBSB>NPRBSB2 , NBWP,istart+NBWP,isize-1mod NPRBSB>NPRBSB2 , two precoding matrices are indicated by the PMI corresponding to the last subband: the first precoding matrix corresponds to the first NPRBSB2 PRBs of the last subband and the second precoding matrix corresponds to the last NBWP,istart+NBWP,isizemod NPRBSB-NPRBSB2 PRBs of the last subband. ## <Unchanged text is omitted> | vivo |
|  | - IfNBWP,istart+NBWP,isize-1mod NPRBSB+1≤NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the last subband. IfNBWP,istart+NBWP,isize-1mod NPRBSB+1>NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the last subband: the first precoding matrix corresponds to the firstNPRBSB2 PRBs of the last subband and the second precoding matrix corresponds to the lastNBWP,istart+NBWP,isize-1mod NPRBSB+1-NPRBSB2 PRBs of the last subband. | Apple, Nokia/NSB, Samsung, CATT, OPPO, ZTE/Sanechips, Qualcomm, vivo, Ericsson |
| H.2: FD basis indicator is not reported for Mυ=1 | ##### TP for TS 38.214 section 5.2.2.2.5 ##### 5.2.2.2.5 Enhanced Type II Codebook ## <Unchanged text is omitted> Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where Minitial∈-2Mυ+1,-2Mυ+2,…,0 n3,l=n3,l(0),…,n3,l(Mυ-1) n3,lf∈0,1,…,N3-1 which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l (for Mυ>1 and l=1,…, υ), where i1,5∈0,1,…,2Mυ-1. i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2Mυ-1Mυ-1-1N3>19.. ## <Unchanged text is omitted> For all values of N3, n3,l(0)=0 for l=1,…,υ. If Mυ>1, the nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ) for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using Cx,y as defined in Table 5.2.2.2.5-4 and the algorithm: s0=0 ## <Unchanged text is omitted> When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows: - If N3≤19, i1,5=0 and is not reported. If Mν=1, i1,6,l=0, for l=1,…,ν, and is not reported. If Mν>1, i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. <Unchanged text is omitted> | CATT, Nokia/NSB, Apple, Samsung, OPPO, ZTE/Sanechips, Qualcomm, vivo, Huawei/HiSi, Ericsson |

  - Editorial

The following issues pertain to relative simple editorial corrections which are valid and not expected to be contentious. Some textual refinement may be fitting and can be discussed.

Table 2 Editorial

| Issue # | Description/Proposal | Companies |
| --- | --- | --- |
| E.1: typographical correction | Table 5.2.2.2.5-5 and Table 5.2.2.2.6-2 of TS 38.214 Where Wi1,1,n3,pl(1),pl2,i2,5,l,tl1N1N2γt,li=0L-1vi1,1d+ipl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vi1,1d+ipl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2. | Huawei/HiSi, Apple, Nokia/NSB, Samsung, CATT, OPPO, ZTE/Sanechips, Qualcomm, vivo, Ericsson |
| E.2: TP to improve readability | TS 38.214 V16.1.0 ##### 5.2.2.2.5 Enhanced Type II Codebook <omitted text> The codebooks for 1-4 layers are given in Table 5.2.2.2.5-5, where m1(i), m2(i), for i=0,1,…,L-1, vm1i,m2(i) are obtained as in clause 5.2.2.2.3, and the quantities φl,i,f and yt,l are given by | OPPO, Apple, Nokia/NSB, Samsung, CATT, OPPO, ZTE/Sanechips, Qualcomm, vivo, Ericsson |
| E.3: typographical correction | Table 5.2.2.2.6-2 of TS 38.214 Where Wi1,1,n3,pl(1),pl2,i2,5,l,tl=1N1N2γt,li=0L-1vi1,1d+ipl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vi1,1d+ipl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to i1,1, n3,1, n3,2, n3,3, n3,4, p1(1), p2(1), p3(1)and p4(1), and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2. | LGE, Apple, Nokia/NSB, Samsung, CATT, OPPO, ZTE/Sanechips, Qualcomm, vivo, Ericsson |

  - Non-essential

The following issues pertain to non-essential proposals with some potential specification impact which are not intended to address incomplete or faulty functions. Therefore, they will not be discussed during the eMeeting.

Table 3 Low-priority (non-essential)

| Issue # | Proposal(s) | Companies |
| --- | --- | --- |
| N.1: support of eT2 and eT2 PS for BWP size < 24 PRBs | Support wideband CSI reporting with the following restrictions - R=1 - Discuss any additional restriction (such as rank, number of ports, L values etc.) | Samsung |
| N.2: Additional restriction on parameter combinations (L,p,beta) or K0 | Modify the definition of K0=maxx,β2LM1, by introducing a minimum value, x, from one of the following alternatives 1a. x=2, to ensure that a UE can report one NZC per polarisation for rank ν≤2 1b. x=max⁡{2,ν}, to ensure that a UE can report one NZC per polarisation for rank ν≤4 2a. x=L, to ensure that a UE can report one NZC for each selected beam for rank ν≤2 2b. x=L2⋅max⁡{2,ν}, to ensure that a UE can report one NZC for each selected beam for rank for ν≤4 | Nokia/NSB |
|  | The UE is not expected to be configured with the number of CSI subbands less than the minimum number of subbands as the following, where the number of CSI subbands is the number of 1’s in csi-ReportingBand. - When paramCombination-r16 = 1, the minimum number of CSI subbands is 5 for up to rank 2 CSI reporting, and 9 for rank 3 CSI reporting, and 13 for rank 4 CSI reporting - When paramCombination-r16 = 2, the minimum number of CSI subbands is 5 for rank 3 and rank 4 CSI reporting - When paramCombination-r16 = 3, the minimum number of CSI subbands is 5 for rank 3 and rank 4 CSI reporting | Apple |
|  | For eType II and eType II port-selection, support β=1 if N3R≤2 | Qualcomm |
|  | No need to have explicit restriction the parameter combination (L,p,beta) in the specification (can be handled by UE implementation) | ZTE/Sanechips, OPPO |
| N.3: Optimization on amplitude restriction inequality | Proposal (TP): 1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL 1maxp∈0,1maxi'ϵLf=0Mυ-1kl,i'+pL,f(3)pl,p1pl,i'+pL,f22f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL for l=1,…,υ, and p=0,1. L⊆0,1,…, L-1 is the set of indices of the selected beams that are not associated with any of the sets of group indices g(k) for k=0,1,2,3 described in 5.2.2.3. | MotM/Lenovo |
|  | Not needed since this addresses case(s) that can be handled via UE implementation | Samsung, Fraunhofer/HHI, Nokia/NSB |
| N.4: Optimization for IntS | Replace the intermediate set size from 2Mυ to 2M1, i.e. when N3>19, the size of the intermediate set is give by N3'=2M1 for RI={1,2,3,4}, where M1 is the number of FD bases selected for RI={1,2}. Reason: Agreement does not seem to address rank 3-4 with two independent M values: Agreement: - For N3>19, IntS is window-based and fully parameterized with Minitial, indicating that the intermediate set consists of FD bases mod(Minitial + n, N3), n=0,1, …, - The value where a is higher-layer configured from two possible values | Qualcomm |
|  | Current spec with the intermediate set size of 2Mυ is sufficient for rank 1-4 | Samsung, CATT, Fraunhofer/HHI, Nokia/NSB |
| N.5: Optimization for NNZC indication | log2K1 bits are used to indicate the strongest coefficients for RI=1, in which K1=KNZKNZ≤2L2LKNZ>2L. | CATT |

  - UE feature/capability related

The following issues pertain to remaining issues on UE features which include UE capability signaling aspect and revision on UE feature group description.

A summary for UE capability related proposals is given in the following table.

Table 4 UE capability signaling

| Issue # | Proposal(s) | Companies |
| --- | --- | --- |
| U.1: FD compression unit | On the number of PMI subbands, UE signals the maximum N3 value, which shall be larger than 19. Support to report it jointly with the maximum number of CSI-RS ports per resource, the maximum number of active CSI-RS resources per band and the maximum number of active CSI-RS ports per band. | ZTE/Sanechips |
|  | R = 2 and N3<=19 is an optional capability with a separate indication | vivo, Samsung |
|  | R=1 is mandatory and R=2 is optional | OPPO, Qualcomm |
| U.2: Max # A-CSI settings | For FR2, the maximum number of configured aperiodic CSI reporting settings a UE can report is same as Rel-15 - note: max = 4 in Rel.15 | Vivo |
|  | No need for additional capability signalling in relation to eType-II and eType-II-PS (should be applicable to Rel.16 in general) | Samsung |
| U.3: L=6 for eType-II | Introduce additional capability signaling | Samsung |
| U.4: Rank 3-4 | Introduce additional capability signaling | Samsung |
| U.5: Amplitude restriction: reuse Rel.15 parameter amplitudeSubsetRestriction | Introduce additional capability signaling | Samsung |
| U.5: CPU occupation rule for R=2 | For R=2, support one of the following for CSI-RS resource and ports occupation - AltA: When R=2, the number of active resources and the number of active ports within the resources should be counted twice in both CSI-RS account and codebook capability accounting; - AltB: In UE capability signalling, include whether supporting R=2 in each SupportedCSI-RS-Resource, i.e., SupportedCSI-RS-Resource contains {max number of ports per resource, max number of resources, max number of total ports, max number of PMIs per subband CQI}. | Qualcomm |
|  | Not supported | Samsung |
| U.6: Support for concurrent codebooks | In Rel-16, for UE capability of supporting concurrent codebooks with mixed types, support the following solution: - Report {codebook1, codebook2, max number of ports per resource, max number of resources, max number of ports} for concurrent codebook with mixed types - Codebook1 is restricted to Type I (SP/MP), and codebook2 is restricted to Type II (any type, Rel-15/16, regular/port-selection) - Limit the total number of current capabilities to 4 - For other concurrent codebooks types, the combined capability of the concurrent codebooks shall be within the capability of each codebook; - For concurrent codebook 1 with p1,k1 and codebook 2 with p2,k2, where pi and ki denote the number of ports per resource and the number of resources for codebook i triggered by the gNB, the UE expects maxp1,p2, k1+k2,p1k1+p2k2 is within the capability report of both codebook 1 and codebook 2. - Combination of 3 codebooks is not allowed. - Any combination of type II codebooks is not allowed. Intel proposes to introduce one of the following: - The list of supported combinations of the maximum number of CSI-RS ports, the maximum number of resources, the total number of Tx ports active per band across all the supported codebook types - The maximum number of CSI-RS resources and maximum number CSI-RS ports active for each band combination and for each codebook type | Qualcomm, Intel |
|  | Not supported | Samsung |

In addition, the following proposals on the description and categorization of MU-CSI-related UE features are summarized below:

Table 5 View on UE feature group list

| Companies | Proposal(s) |
| --- | --- |
| ZTE/Sanechips | Update FG description as follows 16-3a; Regular eType-II; - Support of codebook parameter combinations 1-6 - Support of PMI sub-bands with value N3<=19 - Rank 1 and 2 - UCI omission; TBD. 16-3a-1; CSI-RS and number of PMI subbands for eType-II; For regular eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19; 16-3a, TBD. 16-3a-2; Rank for eType-II; Support of rank 3,4 for regular eType-II; 16-3a, TBD. 16-3a-3; CBSR for eType-II; Support of CBSR for eType II; 16-3a, TBD. 16-3a-4; Codebook parameter combination 7-8 for eType II; Support of codebook parameter combinations 7-8; 16-3a, TBD. 16-3b; Port selection eType-II; - 6 parameter combinations (combos with L=6 don’t apply) - Support of PMI sub-bands with value N3<=19 - Rank 1 and 2 - UCI omission; TBD. 16-3b-1; CSI-RS and number of PMI subbands for port selection eType-II; For port selection eType-II: A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources across all CCs simultaneously, total # of Tx ports across all CCs simultaneously, Max # of PMI subbands N3}, where N3>=19; 16-3b, TBD. 16-3b-2; Rank for port selection eType-II; Support of rank 3,4 for port selection eType-II; 16-3b, TBD. 16-3c; Number of AP-CSI report settings per BWP; Support up to 8 configured aperiodic CSI report setting per BWP; TBD. |
| Apple | FG 16-3a: Regular eType-II Basic components - We can add another component “Support of rank 1,2” - Component 5, “UCI omission”, we prefer this to be moved to “Optional components” Optional components - Component 3, “CBSR”. There are 2 mode of CBSR, one is just on and off, i.e. only 2 amplitude restrictions, the other is choice from 4 amplitude restrictions. The second mode is agreed to be optional even for UE supports CBSR. Therefore, we suggest the change "(1) Whether UE supports CBSR (2) If UE supports CBSR, whether UE supports 4 values of restriction, i.e. amplitudeSubsetRestriction as in 38.214" FG 16-3b: Port selection eType-II Basic components - We can add another component “Support of rank 1,2” - Component 5, “UCI omission”, we prefer this to be moved to “Optional components” |
| Qualcomm | Support following UE feature list for MUCSI enhancement. 16-3a; Regular eType-II; - CSI-RS capability: A list of supported combinations, each combination is of {Max # of Tx ports in one resource, max # of resources and total # of Tx ports} to support regular eType II - Support of parameter combinations {support L=6, not support L=6} - Number of PMI subbands {R=1, R=1-2} - Jointly reported with component 1, i.e., a list of supported combinations, each combination is of { Max # of Tx ports in one resource, max # of resources and total # of Tx ports, R=1 or R=1-2} - Supported rank: {1-2, 1-4} - Support amplitude subset restriction level {no amplitude subset restriction, support amplitude subset restriction}. 16-3b; Port selection eType-II; - CSI-RS capability: A list of supported combinations, each combination is of {Max # of Tx ports in one resource, max # of resources and total # of Tx ports} to support eType II port-selection - Number of PMI subbands {R=1, R=1-2} - Jointly reported with component 1, i.e., a list of supported combinations, each combination is of { Max # of Tx ports in one resource, max # of resources and total # of Tx ports, R=1 or R=1-2} - Supported rank: {1-2, 1-4}. 16-3c; CSI-RS capabilities for concurrent codebooks with mixed types; A list of supported combinations, each combination is of {Codebook A, Codebook B, Max # of Tx ports in one resource, max # of resources and total # of Tx ports} to support codebook combinations of codebook A and codebook B.. |
| Nokia/NSB | - 16-3a: - Component 1: reuse same numbers as in R15. - Component 2: Only the first 6 combinations are mandatory. The last 2 are optional and can be moved to the optional components with 1-bit capability (L=6). - Component 3: This should read: Support of PMI sub-bands with value R=1 [FFS: and, for N3≤19, with value R=2] - Optional component 2: Support of PMI sub-bands with R=2 [FFS: andN3>19] - Optional component 3: Distinction is needed between support of “amplitude subset restriction” and “no amplitude subset restriction” - 16-3b: - Component 1: reuse same numbers as in R15. - Component 3: This should read: Support of PMI sub-bands with value R=1 [FFS: and, for N3≤19, with value R=2] - Optional component 1: Support of PMI sub-bands with R=2 [FFS: andN3>19] |

- Combined TP for TS 38.214 V16.1.0

##### 5.2.2.2.5 Enhanced Type II Codebook <Unchanged text is omitted> - When R=2: - For each subband in csi-ReportingBand that is not the first or last subband of a BWP, two precoding matrices are indicated by the PMI: the first precoding matrix corresponds to the first NPRBSB/2 PRBs of the subband and the second precoding matrix corresponds to the last NPRBSB/2 PRBs of the subband. - For each subband in csi-ReportingBand that is the first or last subband of a BWP - If NBWP,istart mod NPRBSB≥NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the first subband. If NBWP,istart mod NPRBSB<NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the first subband: the first precoding matrix corresponds to the first NPRBSB2-NBWP,istart mod NPRBSB PRBs of the first subband and the second precoding matrix corresponds to the last NPRBSB2 PRBs of the first subband. - If NBWP,istart+NBWP,isize-1mod NPRBSB+1≤NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the last subband. If NBWP,istart+NBWP,isize-1mod NPRBSB+1>NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the last subband: the first precoding matrix corresponds to the first NPRBSB2 PRBs of the last subband and the second precoding matrix corresponds to the last NBWP,istart+NBWP,isize-1mod NPRBSB+1-NPRBSB2 PRBs of the last subband. <Unchanged text is omitted> Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where Minitial∈-2Mυ+1,-2Mυ+2,…,0 n3,l=n3,l(0),…,n3,l(Mυ-1) n3,lf∈0,1,…,N3-1 which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l (for Mυ>1 and l=1,…, υ), l=1,…, υ, where i1,5∈0,1,…,2Mυ-1 i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2Mυ-1Mυ-1-1N3>19 <Unchanged text is omitted> For all values of N3, n3,l(0)=0 for l=1,…,υ. If Mυ>1, the nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using Cx,y as defined in Table 5.2.2.2.5-4 and the algorithm: <Unchanged text is omitted> When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows: - If N3≤19, i1,5=0 and is not reported. If Mν=1, i1,6,l=0, for l=1,…,ν, and is not reported. If Mν>1, i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. <Unchanged text is omitted> The codebooks for 1-4 layers are given in Table 5.2.2.2.5-5, where m1(i), m2(i), for i=0,1,…,L-1, vm1i,m2(i)un unvm,nare obtained as in clause 5.2.2.2.3, and the quantities φl,i,f and yt,l are given by <Unchanged text is omitted> Table 5.2.2.2.5-5: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS <Unchanged text is omitted> Where Wq1,q2, n1, n2,n3,pl(1),pl2,i2,5,l,tl=1N1N2γt,li=0L-1vm1i,m2(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vm1i,m2(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, p1(1)p2(1)p3(1)p4(1) and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2.. <Unchanged text is omitted> ##### 5.2.2.2.6 Enhanced Type II Port Selection Codebook <Unchanged text is omitted> Table 5.2.2.2.6-2: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS <Unchanged text is omitted> Where Wi1,1,n3,pl(1),pl2,i2,5,l,tl1N1N2γt,l=i=0L-1vi1,1d+ipl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vi1,1d+ipl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to i1,1, n3,1, n3,2, n3,3, n3,4, p1(1)p2(1)p3(1)p4(1) and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2.. <Unchanged text is omitted>

# References

- R1-2002137	Summary of offline email discussion on [100e-NR-eMIMO-MUCSI-01] follow up				Moderator (Samsung)

- R1-2001562	Remaining issues on MU-CSI in R16	Huawei, HiSilicon

- R1-2001595	Maintenance of CSI enhancement for MU-MIMO	ZTE

- R1-2001677	Discussion on remaining issues on MU CSI	vivo

- R1-2001725	Text proposals for CSI enhancements for MU-MIMO support	OPPO

- R1-2001912	Correction on enhanced Type II port selection codebook	LG Electronics

- R1-2002089	Remaining issues on CSI Enhancement for MU-MIMO	CATT

- R1-2002136	On maintenance of Rel.16 MU CSI enhancements	Samsung

- R1-2002293	Maintenance on Rel-16 CSI enhancements	Nokia, Nokia Shanghai Bell

- R1-2002336	Remaining issues for Rel-16 Type II CSI enhancement	Apple

- R1-2002410	Remaining issues on MU-CSI Enhancements	Motorola Mobility, Lenovo

- R1-2002550	Remaining issues on CSI enhancement for MU-MIMO support	Qualcomm Incorporated