3GPP TSG-RAN WG1 Meeting #100b-e	Tdoc R1-200nnn

Online, April 20th –30th, 2020

Agenda Item:	7.2.6.5

Source:	Ericsson

Title:	TPs associated with [100b-e-NR-eMIMO-lowPAPR-02]

Document for:	Decision

# Text Proposal for TS 38.211

  - 6.4.1.1.1.2	 Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231

where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in  [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the DCI according to clause 7.3.1.1.3 in  [4, TS38.212] for a transmission scheduled by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSKscramblingID0 and pi2BPSKscramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or by a PUSCH transmission with a configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSKscramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is 0 bit;

-	NIDnSCID=NIDcell otherwise;

-	otherwise, ru,vα,δ(n) is given by clause 5.2.2.

unchanged text omitted

# Text Proposal for TS 38.212

  - 7.3.1.1.3 	Format 0_2

<Unchanged parts are omitted>

-	Antenna ports – number of bits determined by the following:

-	0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured;

-	2, 3, 4, or 5 bits otherwise,

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, unless DMRSuplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-    2 bits as defined by 7.3.1.1.2-6A, if transform precoder is enabled, and DMRSuplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1;

-     4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, unless DMRSuplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured and π/2 BPSK modulation is used;

-     4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled, and DMRSuplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.