3GPP TSG RAN WG1 Meeting #100bis-e                                     R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Source: 	ZTE

Title: 	Maintenance of low PAPR RS

Agenda item:	7.2.6.5

Document for:	Discussion/Decision

Introduction

In Rel-16, low PAPR RS is introduced including DMRS for CP-OFDM and DFT-S-OFDM. In this meeting, we provide our views on some issues of the latest endorsed specifications.

This contribution is the revised version of R1-2001599.

Discussion on maintenance

In Rel-16, DCI format 0_2 is introduced for URLLC. It should be clarified if the agreed π/2-BPSK DMRS can be used for PUSCH scheduled by DCI 0_2 or not. In our view, if the higher-layer parameter DMRSuplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is used for PUSCH, it is natural to support the π/2-BPSK DMRS to achieve low PAPR benefit.

So we suggest that the agreed π/2-BPSK DMRS can be used for PUSCH scheduled by DCI 0_2. The corresponding text proposals for both 38.211 and 38.212 are provided below. The analysis for each TP is provided as well.

  - Text proposal 1 for TS 38.211 [1]:

Reason for change:  DCI format 0_2 is introduced for URLLC in Rel-16. It should be clarified whether the low-PAPR DMRS agreed in Rel-16 can be used for a PUSCH with π/2-BPSK modulation scheduled by DCI format 0_2.

Summary of change:  Add description on sequence generation for a transmission scheduled by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit.

Consequences if not approved:  The low-PAPR DMRS agreed in Rel-16 cannot be used for a PUSCH with π/2-BPSK modulation scheduled by DCI format 0_2.

Clauses affected:   Section 6.4.1.1.1.2

- 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration: - if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231 where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the DCI according to clause 7.3.1.1.3 in [4, TS38.212] for a transmission scheduled by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSKscramblingID0 and pi2BPSKscramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is not 0 bit or by a PUSCH transmission with a configured grant; - NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSKscramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI or by DCI format 0_2 if Antenna ports field in the DCI format 0_2 is 0 bit; - NIDnSCID=NIDcell otherwise; - otherwise, ru,vα,δ(n) is given by clause 5.2.2. <Unchanged parts are omitted>

  - Text proposal 2 for TS 38.212 [2]:

Reason for change:  DCI format 0_2 is introduced for URLLC in Rel-16. It should be clarified whether the low-PAPR DMRS agreed in Rel-16 can be used for a PUSCH with π/2-BPSK modulation scheduled by DCI format 0_2..

Summary of change:  Add low-PAPR DMRS related descriptions for Antenna ports field of DCI format 0_2.

Consequences if not approved:  The low-PAPR DMRS agreed in Rel-16 cannot be used for a PUSCH with π/2-BPSK modulation scheduled by DCI format 0_2

Clauses affected:   Section 7.3.1.1.3

- 7.3.1.1.3 Format 0_2 <Unchanged parts are omitted> - Antenna ports – number of bits determined by the following: - 0 bit if higher layer parameter AntennaPorts-FieldPresence-ForDCIFormat0_2 is not configured; - 2, 3, 4, or 5 bits otherwise, - 2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1, except that DMRS-uplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured and π/2 BPSK modulation is used; - 2 bits as defined by 7.3.1.1.2-6A, if transform precoder is enabled, and DMRS-uplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=1, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211]; - 4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2, except that DMRS-uplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured and π/2 BPSK modulation is used; - 4 bits as defined by Tables 7.3.1.1.2-7A, if transform precoder is enabled, and DMRS-uplinkTransformPrecoding-r16 and tp-pi2BPSK are both configured, π/2 BPSK modulation is used, dmrs-Type=1, and maxLength=2, where nSCID is the scrambling identity for antenna ports defined in Clause 6.4.1.1.1.2, in [4, TS38.211]; - 3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook; - 5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook. <Unchanged parts are omitted>

Conclusion

In this contribution, we provide our TPs on low-PAPR PUSCH DMRS. The details of the TPs can be found in section 2.

Reference

- 38.211 g10, NR Physical channels and modulations (Release 16)

- 38.212 g10, NR Multiplexing and channel coding (Release 16)