3GPP TSG RAN WG1 Meeting #100bis-e	R1- 20xxxxx

e-Meeting, 20th  – 30th , April 2020

Title:	NR-UE-pow-sav-WUS-01

Agenda item:	7.2.7.1

Source:	CATT

Document for:	Discussion

- DCI format 2_6 monitoring and related procedures

The minimum time gap value

The minimum time gap values 1 and 2  are based on majority view from the proposal of each contribution.  The range is not derived from any specification or formula.

Proposal 1:  The value of minimum time gap is decoupled with SCell dormancy indication.   Two values of minimum time gap in terms of slots per SCS are as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

| Company | Support/Not | Comments |
| --- | --- | --- |
| Ericsson | Support |  |
| Vivo | Support | For value 1 and 2, although our preference in our contribution is to align with the value provided by BWP switching type 1 and 2 respectively, but we can compromise to the current proposal 1 for the sake of progress. |
| Huawei, HiSilicon | OK with the value 2. OK with the value 1 for 60kHz/120kHz. The value 1 for 15kHz and 30kHz should be 0. | We are supportive that the value of minimum time gap is decoupled with Scell dormancy indication, which aligns with the majority view. And we are fine with the value2 as the larger minimum gap value. However, as I explained in previous email reply, the minimum time gap for WUS DCI monitoring in the specification is defined from the end of the slot where the last monitoring occasion coinside to the begining of thes slot where the ON duaration starts. So, if we convert the value of minimum time gap to a time delay for UE processing defined from the beginning of the slot where the last monitoring occasion coinside to the beginning of the slot where ON duration starts, the time delay corresponding to the Value1 of minimum time gap are {2, 2, 2, 3} slots for 15kHz, 30kHz, 60kHz and 120kHz resprectively . According to the 38.133, the BWP switching delay is also defined from the beginning of the slot where DCI triggering BWP switch is transmitted to the beginning of the slot where PUSCH/PDSCH can be scheduled in. Therefore, the effective delay for the UE processing corresponding to the minimum WUS gap in column Value1 is even larger than the type1 BWP switching delay for 15kHz SCS and equals to the type 1 BWP switching delay for 30kHz SCS. We think this is not reasonable. We agree with Sony and MTK that for the smaller minimum gap, it should be as small as possible for better power saving and is important for the commercial deployment. Therefore, we propose to use 0, 0 time gap for 15kHz and 30kHz for the smaller minimum time gap. For the value 1 of 60kHz and 120kHz, we are fine with them. Another question I think may need to be discussed/clarified is, corresponding to the reported minimum time gap value, what the assumption is regarding the location where the PDCCH would be trasmitted within a slot. We know that the location of PDCCH may impact the delay for UE processing. I think we should follow the same way as we did for application delay of cross-slot scheduling, i.e. the application delay is calculated assuming PDCCH case 1-1, and in case PDCCH 1-2 or 2, the Z value in applciation delay is increased by 1 additional slot. This can provide the best performance in different scenarios. Therefore, a note is suggested to be added under the table. According to the above, we suggest the group to agree the table from Fang-Chen with the following revision. Proposal 1: The value of minimum time gap is decoupled with SCell dormancy indication. Two values of minimum time gap in terms of slots per SCS are as follows, SCS (kHz); Minimum Time Gap TminimumTimeGap (slots). Value 1; Value 2. 15; 1[0]; 3. 30; 1[0]; 6. 60; 1; 12. 120; 2; 24. Note: the reported values correponds to the case where the PDCCH carries the DCI format 2_6 is transmitted within the first three symbols of a slot. For the case where PDCCH would be transmitted in symbols other than the first three symbols, the applied minimum time gap is increased by 1 compared with the reported one. |
| Samsung | Support | For value 1, we are OK with the values. For value 2, we suggest to consider value no larger than 3/6/9/18 for SCS of 15/30/60/120 KHz. |
| CMCC | Support the two values | We support the two values, but the starting time of Scell dormant BWP and non-dormant BWP switching delay is needed to be aligned between gNB and UE, due to the multiple DCI format 2_6 monitoirng occasions. This issue can be discussed independent from the minimum time gap values. |
| MediaTek | Support value 2 only | Value 2 is consistent across different SCS‘s, but value 1 is not consistent. For 60kHz SCS, if UE can finish WUS detection and wake up within 0.5 ms, the same timeline can be achieved 15 kHz and 30kHz SCS. Then, the correponding time gap should be 0 slot since UE can finish WUS detection and wake up within the WUS slot time. Keeping constant timing across different SCS’s is preferred to manage the latency impact in a unified way, and we suggest to set 0-slot time gap for 15 kHz and 30kHz SCS. |
| Intel | OK with value 1 | For value 2, we suggest {3,5,9,18} for {15,30,60,120}KHz respectively. It is not clear why value 2 has to be scalable across SCS. Many hardware functions/processing timeline do not scale linearly with SCS. |
| Panasonic | Support | In general we support both Value 1 and Value 2 for progress. We also understand the motivation to take Value 1 as 0 for 15 and 30 kHz SCS, which is to align with the Type 1 BWP switch delay. But we think it is no harm and even beneficial to relax the UE sleeping mode processing timeline before DRX ON when preparing to wake up, compared with Type 1 BWP switch delay. Because in our understanding Type 1 BWP switch is under the assumption of UE is already awake. Longer gap leaves UE with some time budget to stay in a lower power saving mode than being prepared to wake up at the very next slot. Thus, we think it may not be necessary to be comparable and aligned with type 1 BWP switch case. But one may argue that if UE wants slower processing timeline it could report the Vaule 2 as capability. Thus overall, we do not strongly object that. But for the sake of progress, the current proposal is agreeable and technically friendly for implementation. The one more slot latency does not bring any substantial performance loss. |
| Spreadtrum | Support the value 2 | For value 1, we think it may not be so tight. Even if the packet arrives before DRX ON with gap of very short time, e.g. 0.25ms, can gNB decide to wake up a UE in time according to scheduling algorithm? We suspect. From UE perspective, leaving the UE some time, e.g. 1ms, to prepare the functions for active time is beneficial for power saving. In other words, the UE only open PDCCH detector for DCI format 2-6 before DRX ON and if the PHY obtains the wakeup indication bit “1”, the PHY can notify the MAC entity to open the PDCCH detectors for active time, otherwise, notify nothing. With current value 1, UE may always standby in some subcarrier spacing, e.g. 60/120kHz. |
| LG | Support |  |
| ZTE | NOT | For Value 1, we think it can be as small as possible. We are okay to have the candidates as 0 for SCS 15/30 KHz. For Value 2, we are not sure why it should be 3 ms for all the SCS. Our preference is type 2 BWP switch delay or no larger than type 2 BWP switch delay. Furthermore, if UE does not report the minimum gap, it can be assumed to be all zeros by network, i.e., the default value is 0. |

- Inconsistent information from more than on DCI format 2_6 decoding output

Proposal 2:

Question 1: How would the clarification be captured?

- Alt 1: In the spepcfication

- Alt 2: In the conclusion of RAN1#100b-e

| Company | Support/Not | Comments |
| --- | --- | --- |
| Ericsson | Neither | Handling of inconsistent information is already specified in 38.213(as shown below), so we don’t see a need to add more clarification. If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format. |
| Vivo | Alt 1 | As Sasaki-san said, the sentence “ If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format“ inconsistency is checked within a PDCCH. The inconsistency check among PDCCHs (or search spaces within a MO) looks out of scope of the agreement. Relevent agreement is from RAN1#94b. Moreover, I think UE will never (or very seldom) has a chance to ‘discard’ since UE has the freedom to stop decoding for the next. Since no one denies the spec shall provide the freedom for the UE to skip decoding the rest of the other MOs in other slots for the next DRX cycle, we think reasonable assumptions which are captured in the spec should be provided in order to allow UE to do so. |
| Huawei, HiSilicon | Alt.1 | As we discussed in the email discussion, the clarification enables UE to early terminate the detection of following DCI format(s) 2_6 when a DCI format 2_6 has been already detected. Without this clarification, the UE cannot do this and can reduce the power saving gain significantly. |
| Samsung | Alt 2 | The contents of DCI format 2_6 are a gNB implementation issue. The UE behavior does not need to be defined for gNB misconfigurations. |
| CMCC | Alt 1 |  |
| MediaTek | Alt 1 | As the majority view, we also think that current spec does not cover the case of inconsistent information of DCI format 2_6 in multiple monitoring occasions. |
| Intel | Neither/Alt 2 | In our view, spec is clear in what UE is supposed to do if inconsistent information is received. If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format. It seems different companies have different interpretation of the above statement, but in our view, DCI format 2_6 providing different content in multiple MOs is an example of inconsistent information in a DCI format and thus, seems to be covered by the above statement. If at all needed, we can compromise to support Alt 2 |
| Panasonic | Alt 1 | If the majority thinks there should be somewhere in the specification such assumption should be clarified, we are okay with that. |
| LG | Alt 2 | In our perspective, the current spec can be applied to this case. If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format. |
| ZTE | ALT1 | The sentence in “If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format” does not cover the case of inconsistent information in multiple monitoring occasions of DCI format 2_6. |

Question 2:  The preferred sentence

- Alt-1(a): UE is not required to continue decoding PDCCH with CRC scrambled by PS-RNTI once UE decodes a DCI Format 2-6 with successful CRC check.

- Alt-1(b): UE can stop monitoring PDCCH with DCI Format 2-6 once the UE has detected one

- Alt-2(a): UE expects each of the more than one DCI formats 2_6 to indicate a same information for the next DRX cycle

- Alt-2(b): UE does not expect to receive different wake-up indication(s) or different dormancy indication(s) from the DCI formats 2_6 detected on monitoring occasions for the next DRX cycle.

- Alt-3: UE wakes up at the next DRX cycle if at least one DCI format 2_6 indicates the UE to wake up

| Company | Support/Not | Comments |
| --- | --- | --- |
| Ericsson | None of the above | Alt-2(a) seems to add an expectation based on which UE could soft-combine across multiple DCIs. We don’t see a need for such behavior. Alt-1(a) or 1(b) seem to be plausible without needing such expectation. |
| Vivo | Alt 2(b) | In principle we are OK with both Alt 2(a) or 2(b). Considering some concerns on whether Alt 2(a) impose network to transmit DCI multiple times, we can accept Alt 2(b). Instead of saying “UE expects….”, the updated proposal 2 saying “UE does not expecting……” provides a better description and it is fine for us. Alt-3 is vague for our understanding since it does not address any assumption on whether UE need to continue perform decoding since we are not preferrable on this. Alt 1 (a)/(b) is a little bit restrictive since Alt 1 (a)/(b) is a consequence of the UE assumption described in Alt 2(b). Hence, we are more preferrable on Alt 2(b) |
| Huawei, HiSilicon | Alt-2(b) | Alt-1(a) and 1(b) specify the detailed UE behavior, therefore, they are not preferred. Alt-2(a) and Alt-2(b) are both OK. To resovle concern raise in the email discussion, Alt-2(b) is preferred to be agreed. Alt.3 still allows inconsistent indications actually. That means once a UE detects a DCI format 2_6 indicating not to wake-up, the UE still needs to monitoring the following monitoring occasions, because a wake-up indication would be transmitted later. |
| Samsung | Alt 3 | Current spec already supports Alt3. According to 213, UE will send wake-up indication to higher layer if receive a DCI format 2_6 with wake-up indication bit of 1. And RAN2 already captured detailed procedure about how to wake up. No new agreement is needed. |
| CMCC | Alt-2(b) | Alt-1(a) and Alt-1(b) is UE implemantation and don’t need to be sepcified. Alt-2(b) can restrict gNB’s indication than Alt-2(a). |
| MediaTek | Alt-2(b) | For Alt 1-(a)/(b), we don’t think the spec should specify the UE decoding behavior. For Alt-3, it means that UE may need to decode all monitoring occasions of DCI format 2_6 to determine whether to wake up, which somehow conflicts with the goal of power saving. |
| Intel | Neither/Alt-2(b) | Again, capturing none of these seem to be necessary. However, as a compromise, we could agree to Alt-2(b) to be captured as conclusion. Suggest to make Alt-2(b) simpler as follows: - UE does not expect to receive different information in the DCI format 2_6 detected on multiple monitoring occasions for the next DRX cycle. |
| Panasonic | Alt-1(a)/(b) | Atl-1(a)/(b) gives clear description of the UE behavior with minimum requirement, but in the meanwhile does not restrict the UE implementation. If UE wants to detect all the monitoring occasion, it is allowed to do so. No matter UE wakes up or not after detection a WUS, the consequent UE behavior is very well protected. This behavior description also indirectly “mandates” gNB to act perfectly. Alt-2(a)/(b) could have been okay only if repetition were always assumed and UE is supposed to detect all occasions and perform combining even. But it is not the case. Alt-3 is already supported by the current specification, which we agree with Samsung. |
| LG | Neither | If we reuse current spec (If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.), it could be concluded that the sentence means “If a UE detecs DCIs with inconsistent information, it is regarded as the DCIs are not detected.”. |
| ZTE | ALT-2(a) | From our perspective, ALT-2(a) and ALT-2(b) present the same intention. And ALT-2(a) is preferred. |

- Excluding DCI format 2_6 for DCI size budget for DCI size alignment

Proposal 3:

- TP for subclause 7.3.1.0 of 38.212 to exclude DCI format 2-6 from the maximum number of DCI sizes per cell.

****************************** Begin Text Proposal **********************************

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

-	DCI format 2-6 size is not counted as one of the 4 different DCI sizes configured per cell.

****************************** End of Text Proposal **********************************

| Company | Support/Not | Comments |
| --- | --- | --- |
| Ericsson | Support | It seems the common understanding is that DCI 2-6 size is not counted in the budget. Then, it seems also reasonable to capture it as a conclusion and update the spec. |
| Vivo | Support | A shorter DCI size, which can be configured by RRC, is preferred to achieve a low code rate to ensure high detection reliability for WUS. If DCI format 2-6 is counted as one of 3+1 DCI size budget, it will be padded to align with other DCI format in CSS. On the other hand, it will not be monitored at the same time instance as other PDCCH with UE specific RNTI during DRX active time, only one DCI size is monitored if the SI, paging PDCCH is not considered. In this case, it is not necessary to align the DCI size of WUS with other DCIs. Therefore, we support DCI format 2-6 size is not counted as one of the 4 different DCI sizes configured per cell |
| Huawei, HiSilicon | Support | We think this should be the common understanding in the WI discussion. |
| Samsung | Not | 38.212 discusses size matching for UE-specific DCI formats (USS). There is no treatment for DCI formats monitored on CSS as this is a gNB implementation issue since the fields are configurable. DCI size budget for PDCCH monitoring is captured in 38.213 and there is no ambiguity for the UE operation. No need for new specifications. |
| CMCC | Support | It need to be captured in the spec. |
| MediaTek | Supoort | In can avoid unncessary DCI size alignment for DCI format 2_6. |
| Intel | Support |  |
| Panasonic | Support | Agree with VIVO that zero padding is not necessary for DCI format 2_6 as it degraded performance and one more size will not increase UE decoding burden in DRX_OFF, even considering UE needs to decode paging and SI. |
| Spreadtrum | Support |  |
| LG | Support |  |
| ZTE | Support |  |

- Appendix: Summary from R1-2002698

DCI format 2_6 Monitoring and Related Procedures

Minimum time gap – values

RAN1#99 agreements Agreements: The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing. - The reporting is per SCS in units of slots of the respective SCS - The reported value for a SCS is taken from two possible values per SCS - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms - FFS impact of dormancy/non-dormancy transition RAN1#100-e agreements Agreements PS_offset range from {0.125ms to 15 ms} for all SCS. Agreements The PS_offset resolution is 0.125 ms. Agreements Candidate values for the minimum time gap are specified by RAN1 and shared with RAN4 · Minimum time gap is no more than 3 ms for all SCSs · Two values of minimum time gap for each SCS are proposed as - SCS 15kHz: {TBD, TBD} slots - SCS 30kHz {TBD, TBD} slots - SCS 60kHz {TBD, TBD} slots - SCS 120kHz {TBD, TBD} slots

During RAN1#100e email discussion, most companies prefer to determine the minimum time gap without considering the SCell dormancy/non-dormancy transition delay.   The exact values of minimum time gap will be determined at RAN1#100bis-e.

The definition of the minimum time gap was proposed to be clarified for SCell dormantcy indication by vivo

- Proposal 2: Further clarification of the minimum time gap for Scell dormancy indication, down-select from the following,

  - Alt 1: between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON

  - Alt 2: between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the time when the dormancy indication applies

Since MAC controls the start of DRX for all cells, which include PCell and SCells, in the same time, the two alternatives of SCell dormancy indication is the same.

The proposed values of minimum time gap in terms of number of slots for all SCS are as follows,

- SCS = 15 kHz

  - Low –

    - 0 – Huawei, HiSilicon, Sony, MediaTek,

    - 1 – ZTE, OPPO, Intel, CATT, Samsung, Nokia, NSB, Ericcson, DoCoMo, Qualcomm

  - High –

    - 2- Samsung,

    - 3 - Huawei, HiSilicon,ZTE, OPPO, Sony, MediaTek, Intel, CATT, Nokia, NSB, Ericcson, DoCoMo, Qualcomm

- SCS = 30 kHz

  - Low –

    - 0 – Huawei, HiSilicon, Sony, MediaTek,

    - 1 – ZTE, OPPO, Intel, CATT, Samsung, Ericcson, DoCoMo

    - 2 - Nokia, NSB, Qualcomm

  - High –

    - 4 - Samsung,

    - 5 – ZTE, Intel, CATT, Nokia, NSB, DoCoMo,

    - 6 - Huawei, HiSilicon, OPPO, Sony, MediaTek, Ericcson, Qualcomm

- SCS = 60 kHz

  - Low –

    - 0 - Sony

    - 1 – Huawei, HiSilicon, ZTE, OPPO, MediaTek, Ericcson,

    - 2 - Intel, CATT, Samsung, DoCoMo,

    - 3- Nokia, NSB, Qualcomm

  - High –

    - 8 - Samsung,

    - 9  - ZTE, Intel, CATT, Nokia, NSB, DoCoMo,

    - 12 - Huawei, HiSilicon, OPPO, Sony, MediaTek, Ericcson, Qualcomm

- SCS = 120 kHz

  - Low –

    - 0 - Sony

    - 1 – ZTE, OPPO, MediaTek,

    - 2 – Huawei, HiSilicon, Intel, Ericcson,

    - 4 - CATT, Samsung, DoCoMo,

    - 6 - Nokia, NSB, Qualcomm

  - High –

    - 16 - Samsung,

    - 18 - ZTE, Intel, CATT, Nokia, NSB, DoCoMo,

    - 24 - Huawei, HiSilicon, OPPO, Sony, MediaTek, Ericcson, Qualcomm

Proposal:  Regardless the processing time of BWP switching is needed or not for SCell dormancy indication,  two values of minimum time gaps in terms of slots per SCS are as follows,

| SCS (kHz) | Minimum Time Gap TminimumTimeGap (slots) |  |
| --- | --- | --- |
|  | Value 1 | Value 2 |
| 15 | 1 | 3 |
| 30 | 1 | 6 |
| 60 | 1 | 12 |
| 120 | 2 | 24 |

DCI format 2_6 Monitoring

Monitoring occasions and and conflict of information in the DCI format 2_6

- UE behaviour on conflict wakeup indication from more than one DCI decoding - – Companies had discussed the UE behaviour on inconsistent wakeup information from the contents of DCI format 2_6 by multiple DCP monitoring occasions (Huawei, HiSilicon, MediaTeck, )

Proposal:   These are implementation issues.

- UE behaviour on SCell dormancy with miss-detection or conflict information from the contents of DCI format 2_6 – Companies had discussed the UE behaviour on SCell dormancy when DCI format 2_6 was miss-detected (Huawei, HiSilicon, ZTE, ) or conflict information of power saving information in DCI format 2_6, such as UE not to wake up to SCell in non-dormancy and monitoring occasions (Huawei, HiSilicon, vivo, MediaTek).  One company (ZTE) would like to limit the detection of DCI format 2_6 for SCell dormancy indication  only for CORESET at first 3 symbols of slot.

Proposal:  UE behaviour on the SCell dormancy with miss-detection or conflict information from the contents of DCI format 2_6 should be discussed in AI-7.2.10.3 MR-DC/CA feature

Additional Invalid monitoring occasion –

A couple of companies (vivo, Nokia, NSB) discussed additional invalid monitoring occasions when the monitoring occasions of DCI format 2_6 PS-RNTI collide with monitoring of RA-RNTI or  TC-RNTI in case of CBRA or monitoring C-RNTI in search space given recoverySearchSpaceId for CFRA based BFR and intra-frequency RRM, RLM, BFD, CBD and L1-RSRP measurement defined in TS 38.133.   These issues had been discussed in RAN1#100e and understood by most companies that current specifications captured these invalid scenarios.

Proposal:  The proposed additional invalid monitoring occasions had been covered by current specification.

DCI format 2_6 not counting in the DCI size budget

Currently the DCI format size budget per UE is determined without separation for the capability between CONNECTED or IDLE/INACTIVE, nor between outside active time and inside active time.  The only separation is done via defining the RNTIs for UE monitoring. There are RNTIs (P-RNTI, SI-RNTI, RA-RNTI) that UE may need to monitor outside active time which the corresponding DCI formats are accounted in DCI-format size budget.  The intent of the DCI format 2_6 outside active time use would NOT be counted in the total budget of DCI format sizes.   Several companies (vivo, InterDigital, Ericsson) have proposals in excluding DCI format 2_6 account for the total budget of DCI format sizes.

- Proposal:  TP for subclause 7.3.1.0 of 38.212 to exclude DCI format 2-6 from the maximum number of DCI sizes per cell.

****************************** Begin Text Proposal **********************************

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

-	DCI format 2-6 size is not counted as one of the 4 different DCI sizes configured per cell.

****************************** End of Text Proposal **********************************

RAN1 and RAN2 Alignment -

Feature Interaction between WUS and Secondary DRX group

RAN2 had sent a LSt to RAN1[20] on the secondary DRX group for UE power saving with the following agreements as Rel-16 TEI.

Conditional on R1 acceptance:

- A separate drx-InactivityTimer and drx-onDurationTimer can be configured for the secondary DRX group. R2 understands that this has zero or almost zero impact in R1 and R4

- The combination of cross-carrier scheduling and secondary DRX group is not supported

- FFS if timers for FR2 DRX configuration are shorter than timers for FR1 DRX configuration.

- The intention is to apply secondary DRX configuration to FR2 and existing DRX configuration to FR1

The secondary DRX group is used for UE configured with CA, which contains carriers on both FR1 and FR2 cells. FR2 Cell has larger SCS than that of FR1 cell. The shorter drxInactivityTimer and OnDurationTimer can be configured for FR2 to enable the FR2 cells to go to sleep more quickly and thereby reduce power consumption.  During the email discussion in LS during RAN1#100e, there is no consensus on the RAN1 impacts and no LS reply had sent back to RAN2.   Several companies (Intel, CATT, Qualcomm) had discussed the feature interaction between UE adaptation to DRX and Secondary DRX group.   There are two proposals to minimize the specification impacts

- Seconddary DRX group is not configured with UE adaptation to DRX in Rel-16 – vivo, Intel, CATT

- Investigating and identify RAN1 specification impacts and change when seconddary DRX group is configured with UE adaptation to DRX - Qualcomm

Clarification the interaction between PHY and MAC layers

RAN2 sent a LS [19] to RAN1 on the MAC-PHY interactions for DCP (DCI with CRC scrambled by PS-RNTI) monitoring and the start of drx-onDurationTimer. The following understanding regarding how to capture DCP between MAC and PHY was agreed from RAN2 point of view:

MAC specification:

- MAC specifies the start of drx-onDurationTimer and Active Time, including:

- MAC should start drx-onDurationTimer according to indication provided by PHY

- MAC should start drx-onDurationTimer in case DCP is overlapped with Active time, measurement gap and BWP switching period

- MAC should start drx-onDurationTimer in case ps-Wakeup is set to true and no DCP indication is received from PHY

PHY specification:

- PHY specifies DCP monitoring, including:

- When to start the monitoring (ps_offset) and stop the monitoring (minimum gap based on UE capability)

- In case DCP is considered invalid from PHY perspective (scenarios FFS in RAN1), PHY should not monitor DCP and indicates to MAC to start the drx-onDurationTimer for the next DRX cycle

- PHY indicates to MAC whether a received DCP indicates to start the drx-onDurationTimer for the next DRX cycle or not.

- PHY should not specify the start of drx-onDurationTimer and Active Time.

Several companies (Huawei, HiSilicon, NEC, Nokia, NSB, Spreadtrum) had discussed the general procedures of UE adaptation to the DRX operation with DCP (DCI format 2_6 with CRC scrambled by PS-RNTI) captured in RAN1 and RAN2 specification.   Several companies had text proposals on the alignment of RAN1 and RAN2 specification regarding the DRX ON.

Proposal:  TP in replacing “start the drx-onDurationTimer” in Clause 10.3 of TS38.213

****************************** Begin Text Proposal **********************************

10.3	PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells

<text omitted>

-	a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6, where

-	the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '0', and

-	the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the Wake-up indication bit is '1'

<text omitted>

If a UE detects DCI format 2_6

-	if the value of the Wake-up indication bit is '0' and the UE is provided ps-WakeupOrNot, the UE is indicated by ps-WakeupOrNot whether the UE sends indication to higher layers for the next DRX cycle

-	if the UE is not provided ps-WakeupOrNot, and value of the Wake-up indication bit is '1', the UE shall send an indication to higher layers for the next DRX cycle

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE does not detect DCI format 2_6

-	if the UE is provided ps-WakeupOrNot, the UE is indicated by ps-WakeupOrNot whether the UE sends indication to higher layers may not start or whether the UE shall start the drx-onDurationTimer for the next DRX cycle

-	if the UE is not provided ps-WakeupOrNot, the UE may not start Active Time indicated by drx-onDurationTimer for the next DRX cycle

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

-	is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10, 11.1, 12, and in Clause 5.7 of [14, TS 38.321] for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or

-	does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle

the UE shall send the indication to higher layers start the drx-onDurationTimer for the next DRX cycle if the UE is not provided ps-WakeupOrNot or the ps-WakeupOrNot is set to false.

****************************** End Text Proposal **********************************

RAN2 LS on configuration of L1 Measurements  –

RAN1#99 Agreements: When drx_OnDurationTimer does not start, RAN1 agrees the following report(s) are impacted by the WUS indication - SP L1-RSRP reporting - SP-CSI - SRS Except: - by configuration, whether or not for periodic L1-RSRP reporting - by configuration, whether or not for periodic CSI - By default, both the above two are also impacted by the WUS indication Note: for the above two bullets (under Except), no additional RAN1 impact is expected in Rel-16 RAN1#100e Agreement: P-CSI and L1-RSRP reports are independently configured and to allow UE only to report periodic CSI apart from L1-RSRP.

RAN2 LS on L1-RSRP and P-CSI configuration

RAN2 sent a LS [19] to RAN1 on the configuration on the measurement configuration of L1-RSRP and P-CSI reporting  when drx-onDurationTimer is not running due to DCP. There are two options to interpret the two options in the configuration of L1-RSRP as follows,

- Option 1:

  - ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI, including L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

  - ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags cannot both be set to TRUE and it is not possible to control the UE only to report periodic CSI apart from L1-RSRP.

- Option 2:

  - ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

  - ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP)

In this option, the two flags are independent and it is possible to control the UE only to report periodic CSI apart from L1-RSRP.

RAN1 discussed two options in RAN1#100-e with the agreement that P-CSI and L1-RSRP reports are independently configured and to allow UE only to report periodic CSI apart from L1-RSRP.

.

Proposal:  Reply to RAN2 that RAN1 has concluded that the option 2 given in LS [19] is the assumed behaviour.

L1-SINR

RRC parameters PS-Periodic_CSI_TransmitOrNot and PS_Periodic_L1-RSRP_TransmitOrNot were introduced to allow P-CSI and L1-RSRP feedback at the configured UL resource when UE is indicated not to wakeup by DCI format 2_6.   In Rel-16, L1-SINR was also introduced as an additional measurement that UE can report. Three companies (CATT, Ericsson, and Qualcomm) has proposed to report L1-SINR even if drx-OndurationTimer is not started. This can be done by introducing a new RRC parameter (like above for L1-RSRP) or by reusing the existing parameter (PS_Periodic_L1-RSRP_TransmitOrNot).  Since it is instructed to have no new RRC parameter, reusing existing parameter (PS_Periodic_L1-RSRP_TransmitOrNot) with updated description is proposed.

Proposal:

- Rel-16 L1-SINR is supported as an alternative measurement report of L1-RSRP using the RRC parameter PS_Periodic_L1-RSRP_TransmitOrNot.

  - Rel-16 L1-SINR use existing RRC parameter PS_Periodic_L1-RSRP_TransmitOrNot

- TP on 5.2.2.5 of TS38.214

****************************** Begin Text Proposal **********************************

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS-Periodic_CSI_TransmitOrNot] to report CSI with the higher layer parameter reportConfigType set to ‘periodic’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS_Periodic_L1-RSRP_TransmitOrNot] to report L1-RSRP or L1-SINR with the higher layer parameter reportConfigType set to ‘periodic’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP or L1-SINR during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4.

****************************** End of Text Proposal **********************************

TP on reporting quantities

RAN1#100-e Agreements: Text Proposal to TS 38.213 5.1.6.1 CSI-RS reception procedure *** Unchanged text is omitted *** If the UE is configured with DRX, - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter[PS-Periodic_CSI_TransmitOrNot] to report CSI with the higher layer parameterreportConfigType set to ‘periodic’ when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter[PS_Periodic_L1-RSRP_TransmitOrNot] to report L1-RSRP with the higher layer parameterreportConfigType set to ‘periodic’ andreportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported; - otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.. 5.2.2.5 CSI reference resource definition *** Unchanged text is omitted *** When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS-Periodic_CSI_TransmitOrNot] to report CSI with the higher layer parameterreportConfigType set to ‘periodic’ when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated bydrx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS_Periodic_L1-RSRP_TransmitOrNot] to report L1-RSRP with the higher layer parameterreportConfigType set to ‘periodic’ and reportQuantity set to ‘cri-RSRP’ or ‘ssb-Index-RSRP’ when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to ‘cri-RSRP’ if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise..

RAN1#100e agrees the text proposals in TS38.214 to include the L1-RSRP/P-CSI measurements when UE is configured with DCI format 2_ 6 for UE power saving which the configured L1-RSRP and CSI measurements would not be performed outside Active Time if the UE is indicated not to wake up at the next DRX ON.  The L1-RSRP and P-CSI reports would be outdated and not useful when UE feedbacks to the gNB.   Several companies (ZTE, Samsung) had text proposals on the addition of reporting quantities on the text proposal.

Proposal: TP for Clauses of 5.1.6.1 and 5.2.5.2 of TS 38.214 V16.1.0

****************************** Begin Text Proposal **********************************

5.1.6.1           CSI-RS reception procedure

*** Unchanged text is omitted ***

If the UE is configured with DRX,

-	if  the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter [PS-Periodic_CSI_TransmitOrNot] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than cri-RSRP and ssb-Index-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter[PS_Periodic_L1-RSRP_TransmitOrNot] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.

5.2.2.5 CSI reference resource definition

*** Unchanged text is omitted ***

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS-Periodic_CSI_TransmitOrNot] to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than cri-RSRP and ssb-Index-RSRP when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter [PS_Periodic_L1-RSRP_TransmitOrNot] to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

****************************** End Text Proposal **********************************

Contributions summary and proposals

| Huawei, HiSilicon [1] | - Proposal 1: The capability values for minimum time gap between DCI format 2_6 monitoring occasions and ON Duration Timer are determined per SCS as the following table. NR Slot length (ms); Type 1; Type 2. 0; 1; 0; [3]. 1; 0.5; 0; [6]. 2; 0.25; 1; [12]. 3; 0.125; 2; [24]. - Proposal 2: Further discuss and narrow down between Alt.1 and Alt. 3 for UE behavior when dormancy indication is configured. - Proposal 3: The dormancy adaptations indicated by different DCI(s) on different monitoring occasions before the ON duration trigger the same BWP switching delay, which starts from the slot of the last monitoring occasions where dormancy indication(s) can be transmitted. - Proposal 4: UE may assume the Wake-up indication for one DRX cycle in multiple DCI format 2_6 monitoring occasions, if any, is consistent. Adopt the TP1 in TS 38.213. - Proposal 5: Adopt the TP2 in TS 38.213 to align with the RAN2 suggestions. |
| --- | --- |
| ZTE [2] | - Proposal 1: If the WUS PDCCH indicating dormancy behavior on the activated SCells outside Active Time is not detected and the UE is configured to wake up by higher layer signaling, UE should switch to non-dormancy behavior for the activated SCells. - Proposal 2: RAN1 clarifies whether DCI format 1_1/0_1 and DCI format 2_6 indicating SCell dormancy change are to be limited within the first 3 symbols of one slot. - TP: A UE expects to detect a DCI format 2_6, DCI format 1_1 or DCI format 0_1 indicating SCell dormancy change, as described in Clause 10.3, only if a corresponding PDCCH is received within the first 3 symbols of a slot. - Proposal 3: Adopt the following text- and reportQuantity set to quantities other than cri-RSRP and ssb-Index-RSRP - Proposal 4: Type 2 BWP switching delay can be taken as one of the two candidate values of minimum time gap. - Proposal 5: The value of 1 slot should be defined as another candidate value of minimum time gap. If the minimum time gap is not signaled by UE, the default value is 0. |
| vivo[3] | - Proposal 1: The UE capability on minimum time gap between last monitoring occasion of DCI format 2_6 and DRX On should be separated reported for with/without scell dormancy indication. - the BWP switching delay capability is reported to indicate the minimum time gap for WUS with Scell dormancy indication. - A new minimum time gap capability is reported to indicate the minimum time gap for WUS without Scell dormancy indication. As a starting point, the values can reuse the values for BWP switching delay. - Proposal 2: Further clarification of the minimum time gap for Scell dormancy indication, down-select from the following, - Alt 1: between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON - Alt 2: between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the time when the dormancy indication applies - Proposal 3: If UE is configured with different SCS for different serving cells and DL/UL BWPs, the switching delay should be determined by - the longer one between values corresponding to SCS before and after switching, and - the longest one among the values corresponding to SCS of the serving cells. - Proposal 4: The size budget of power saving DCI is not restricted by the existing DCI size budget (3+1) in Rel-15 which is used in Active Time. Capture TP in Appendix 1 in R1-2001682 for TS38.212. - Proposal 5: If monitoring occasion of DCI format 2-6 is not valid, due to scheduling availabilities for intra-frequency RRM, RLM, BFD, CBD and L1-RSRP measurement defined in TS 38.133, UE should start the drx-onDurationTimer for the next DRX cycle. - Capture TP in Appendix 2 in R1-2001682 for TS38.213. - Proposal 6: UE assumes the indication in multiple MOs in a DRX cycle for DCI format 2-6 is consistant. - Proposal 7: Among the N MO(s) before On Duration, - If all MOs are invalid, UE should wake up for the next DRX cycle; - If UE does not detect WUS on all WUS valid MO, UE should follow RRC configured UE behaviors (i.e., by RRC configured parameter ps-WakeupOrNot) - If any PDCCH WUS in a valid MO pass CRC, UE behavior should follow the indication by WUS. - Proposal 8: Clarify that if UE detects DCI format 2-6 with Wake-up indication bit '0', - UE does not report SP-CSI/L1-RSRP, and - UE does not report P-CSI/L1-RSRP if configured by RRC signaling not to. - And Capture TP in Appendix 3 in R1-2001682 for TS38.214. - Proposal 9: UE is not expected to be indicated by PDCCH WUS not to wake up while SCell group is indicated to non-dormancy state. Capture TP in Appendix 4 in R1-2001682 for TS38.213. - Proposal 10: In Rel-16 TEI, only consider the case where secondary DRX group is not configured simultaneously with DCP or SCell dormancy for a UE. - Proposal 11: The interaction with DCP or SCell dormancy indication for secondary DRX group, if needed, can be further considered in Rel-17, e.g. in the UE power saving enhancement WI. |
| OPPO [4] | - Proposal 1: Two values of minimum time gap for each SCS are proposed as -  15kHz: {1, 3} slots -  30kHz {1, 6} slots -  60kHz {1, 12} slots -  120kHz {1, 24} slots |
| Sony[5] | - Proposal 1: The minimum time gap capabilities for the different SCS are: - SCS 15kHz: {0,3} slots - SCS 30kHz {0,6} slots - SCS 60kHz {0,12} slots - SCS 120kHz {0,24} slots - Proposal 2: Minimum time gap capability does not account for dormancy / non-dormancy BWP switch in an Scell. - Proposal 3: UE can signal preferred minimum time gap as UE assistance information. |
| MediaTek[6] | - Proposal 1: Two candidate values of UE capability on minimum time gap are defined as follows: μ; NR slot length (ms); Minimum time gap (slots): the number of slots between the end of the slot of last monitoring occasion of wake-up signal and the first slot of drx-onDurationTimer. Candidate Value 1; Candidate Value 2. 0; 1; 0; 3. 1; 0.5; [0]; 6. 2; 0.25; 1; 12. 3; 0.125; 1; 24. - Proposal 2: If DCI format 2_6 including SCell dormancy indication is not detected by a UE and ps-Wakeup is set to true, the following alternatives can be considered. - Alt 1: SCell(s) is in non-dormancy behaviour in the corresponding DRX on-duration. - Alt 2: SCell(s) is in dormancy behaviour in the corresponding DRX on-duration. - Alt 3: Higher layer signaling on “dormancy or non-dormancy behaviour”. UE switches to (or stays in) dormant or non-dormant SCell(s) according to the signaling. Default is “non-dormancy behaviour”. - Proposal 3: UE does not expect to detect more than one DCI format 2_6 in multiple monitoring occasions with inconsistent DCI contents before a DRX cycle. |
| LG [7] | - Proposal 1: The monitoring occasion which has at least one actually monitored candidate is regarded as a valid |
| Intel [8] | - Observation 1: RAN1 impact of supporting secondary DRX can be minimal if separate WUS is not configured for secondary DRX group. - Proposal 1. Candidate two values of minimum time gap per SCS are - SCS 15kHz: {1, 3} slots - SCS 30kHz {1, 5} slots - SCS 60kHz {2, 9} slots - SCS 120kHz {2, 18} slots - Proposal 2: No change of invalid monitoring occasions in 10.3 of TS38.213 is needed. - Proposal 3: Support Option 2 in RAN2 LS R2-2002201 for CSI reporting - Option 2: - ps-TransmitPeriodicCSI = TRUE: Report all types of periodic CSI apart from L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP) - ps-TransmitPeriodicL1-RSRP = TRUE: Only report L1-RSRP (i.e. cri-RSRP and ssb-Index-RSRP) - Proposal 4: Keep configuration of PS_Periodic_L1-RSRP_TransmitOrNot and PS-Periodic_CSI_TransmitOrNot parameters per cell group. - Proposal 5: Existing SCell state remains valid if UE starts drx-ONduration-timer without receiving WUS indication. |
| CATT [9] | - Proposal 1: Two values of minimum time gap for each SCS are proposed as: -  15kHz: {1,3} slots -  30kHz: {1, 5} slots -  60kHz: {2, 9} slots -  120kHz: {4, 18} slots - Proposal 2: For each SearchSpace set, UE monitors DCI format 2_6 only in the 1st full “duration” of valid monitor occasion at or after the PS_offset, but before the DRX on-duration. - Proposal 3: Rel-16 L1-SINR could be considered to feedback indicated by the existing RRC parameter PS_Periodic_L1-RSRP_TransmitOrNot. - Proposal 4: The feature interaction between UE power saving with DRX adaptation and secondary DRX needs to be analyzed in detail with justification of the additional power saving gain before the support of both features in the same time. - Proposal 5: If secondary DRX group is configured, the UE adaptation to DRX should be disabled in Rel-16. |
| Samsung [10] | - Proposal #1: Support two UE capabilities of minimum time gap for each SCS with values determined by the Table below: Table 1: Minimum Time Gap between the end of the slot of last monitoring occasion of wake-up signal and the first slot of next DRX ON duration NR Slot length (ms); Minimum Time Gap TminimumTimeGap (slots). Type 1Note 1; Type 2Note 1. 0; 1; 1; 2. 1; 0.5; 1; 4. 2; 0.25; 2; 8. 3; 0.125; 4; 16. Note 1: Depends on UE capability.. - Proposal #2: When DCI format 2_6 is configured to indicate dormancy/non-dormancy transition, the sum of UE reported minimum gap value and UE reported BWP switching delay value shall be applied for determining the closest position for UE to detect format 2_6 before DRX ON. - Proposal #3: Wake-up indication in DCI format 2_6 indicates whether or not to start drx-onDurationTimer associated with all configured DRX groups. - Proposal #4: UE doesn’t expect to monitor DCI format 2_6 during extended Active Time corresponding to the drx-InActivitityTimer from primary cell. - Proposal #5: If the UE is configured with two DRX groups, the most recent CSI measurement occasion occurs in DRX active time for each DRX group overlapped with the DRX active time for CSI to be reported. - Proposal #6: Periodic CSI report associated with ps-TransmitPeriodicCSI includes all report quantities except ‘cri-RSRP’ and ‘ssb-index-RSRP’. |
| NEC [11] | - We are fine to confirm agreements in RAN2 LS in R1-2001507 LS on DCP RAN2, Huawei |
| CMCC [12] | - Proposal 1. Either alternative 2 or 3 could be considered for minimum time gap value. We slightly prefer Alt 2 as the minimum time gap design principle. - Alt 1. Both of the two values of minimum time gap take into account the SCell dormancy/non-dormancy transition delay - Alt 2. Neither of the two values of minimum time gap takes into account the SCell dormancy/non-dormancy transition delay - Alt 3. The smaller value of minimum time gap does not take into account the SCell dormancy/non-dormancy transition delay and the larger one takes into account the SCell dormancy/non-dormancy transition delay |
| Nokia, NSB [13] | - Proposal 1: If the monitoring occasions of DCI format 2_6 PS-RNTI collide with monitoring of RA-RNTI or TC-RNTI in case of CBRA or monitoring C-RNTI in search space given recoverySearchSpaceId for CFRA based BFR, UE should assume the legacy DRX operation (i.e. start the following drx-onDurationTimer). - Proposal 2: Capture the following text proposal to Section 10.3 of 38.213 Section 10.3 that monitoring occasions of DCI format 2_6 overlapping with RA-RNTI or TC-RNTI monitoring or monitoring C-RNTI in search space given recoverySearchSpaceId with are invalid and UE should follow the legacy DRX operation, based on following text proposal: - Proposal 3: The capability values for minimum gap between DCI format 2_6 monitoring and onDurationTimer could be determined per SCS as - 15kHz: {1 or 3} slots - 30kHz {2 or 5} slots - 60kHz {3 or 9} slots - 120kHz {6 or 18} slots - Proposal 4: Reply to RAN2 that RAN1 has concluded that the option 2 given in LS [1] is the assumed behaviour. - Proposal 5: Adopt following text proposal to Section 10.3 in 38.213 and provide feedback with alignment of RAN1 and RAN2 specs not regarding onDurationTimer controlled by MAC. |
| Spreadstrum[14] | - Proposal 1: For P-CSI/L1-RSRP measurement/report, consider to adopt TP in Appendix 5.1. - Proposal 2: The larger value of the minimum time gap can be 3ms, and the smaller value of the minimum time gap can be 1ms and is an optional capability. - Proposal 3: To clarify the real starting of monitoring is the beginning of the 1st full “duration”, consider to adopt TP in Appendix 5.2. - Proposal 4: To align parameters in RAN2, such as ps-Wakeup, ps-PositionDCI-2-6 and sizeDCI-2-6, consider to adopt TP in Appendix 5.2 and 5.3. |
| InterDigital [15] | - Proposal 1: Aggregation levels of the PDCCH-based power saving signal are limited to {4, 8, 16}. - Proposal 2: DCI format 2_6 is not counted in the DCI format size budget. |
| Ericsson [16] | - Proposal 1 Adopt theTP1 for 38.213 in subclause 10.3 to clarify the interaction between PHY and MAC layers. - Proposal 2 Value range for parameter SizeDCI_2 is 0 to maxSizeDCI_2-6. - Proposal 3 Two values of minimum time gap for each SCS are proposed as - SCS 15kHz: {1, 3} slots - SCS 30kHz {1, 6} slots - SCS 60kHz {1, [12]} slots - SCS 120kHz {2, [24]} slots The same value is used regardless of whether Scell dormancy indication is configured or not in DCI format 2-6. - Proposal 4 Adopt TP2 for 38.212 subclause 7.3.1.0 to exclude DCI format 2-6 from the maximum number of DCI sizes per cell. - Proposal 5 Rel-16 L1-SINR is supported in addition to L1-RSRP using the RRC parameter PS_Periodic_L1-RSRP_TransmitOrNot. - Proposal 6 Adopt TP3 for subclause 5.2.2.5, 38.214 to allow support for L1-SINR in addition to L1-RSRP using the RRC parameter PS_Periodic_L1-RSRP_TransmitOrNot. |
| NTT DoCoMo [17] | - Proposal 1: Update value range of PS_offset in RRC parameter list. -  Value range of PS_offset: 0.125, 0.25, 0.375, 0.5, …, 15ms - Proposal 2: - When DCI format 2_6 is configured to indicate dormancy/non-dormancy transition: - the UE reported minimum gap value shall be applied for determining the closest position for UE to detect format 2_6 before DRX ON. UE follows the behavior of BWP switching in Rel-15 on SCells if dormancy behavior change is indicated. - When DCI format 2_6 is configured only to indicate UE wakeup or not (i.e., indicate to higher layer whether to start the drx-onDurationTimer or not): - the UE reported minimum gap value shall be applied for determining the closest position for UE to detect format 2_6 before DRX ON. - Proposal 3: Two candidate values of UE reported minimum time gap for each SCS are: -  15kHz: {1, 3} slots -  30kHz {1, 5} slots -  60kHz {2, 9} slots -  120kHz {4, 18} slots |
| Qualcomm[18] | - Proposal 1: For the reported UE capability on the minimum time gap, the following sets of values can be considered: - SCS 15kHz: {1, 3} slots - SCS 30kHz: {2, 6} slots - SCS 60kHz: {3, 12} slots - SCS 120kHz: {6, 24} slots - Proposal 2: If a UE is configured to monitor DCI format 2_6, it can also be configured to report L1-SINR during the time duration indicated by drx-onDurationTimer outside DRX Active Time. - Proposal 3: UE reports periodic or semi-persistent CSI for any reported carrier(s) only when the reporting carrier for the CSI is in DRX active time, unless that CSI can be multiplexed in an overlapping PUSCH resource (as in legacy). - Proposal 4: PDCCH-WUS can be configured together with DRX groups and the existing RAN1 and RAN2 agreements on PDCCH-WUS are applied without any changes. More specifically, when DRX groups are configured, - PDCCH-WUS is configured only on SpCell and UE does not monitor PDCCH-WUS if SpCell is in DRX Active Time; - If a PDCCH-WUS occasion is not monitored because UE is already in Active Time on SpCell, UE starts DRX on duration timer of both DRX groups at their respective next occurrence; - If a PDCCH-WUS occasion is monitored, upon a wakeup indication, UE starts DRX on duration timers of both DRX groups at their respective next occurrence; - If no PDCCH-WUS is detected, UE follows configuration of ps-WakeupOrNot for both DRX groups at their respective next occurrence of DRX on duration. |

Reference

- R1-2001539 	Remaining issues on PDCCH based power saving	Huawei, HiSilicon

- R1-2001583		Remaining issues on WUS PDCCH		ZTE

- R1-2001682		Maintenance of PDCCH-based power saving signal	vivo

- R1-2001768		Remaining issues for Power saving signal	OPPO

- R1-2001819		Remaining issues on PDCCH-based WUS	Sony

- R1-2001843		Remaining issues on PDCCH-based power saving signal	MediaTek Inc.

- R1-2001943		Remaining issues on PDCCH-based power saving signal/channel	LG Electronics

- R1-2002008		Remaining details of PDCCH-based power saving signal/channel	Intel Corporation

- R1-2002093		Remaining issues on the Power Saving Signals/Channels	CATT

- R1-2002142		Remaining issues for PDCCH-based power saving signal	Samsung

- R1-2002189		TP to address RAN2 LS on DCP	NEC

- R1-2002215		Remaining issues on minimum time gap for PDCCH-based power saving signal/channel	CMCC

- R1-2002218		On open issues related to DCI format 2_6	Nokia, Nokia Shanghai Bell

- R1-2002261		Clarification on power saving signal	Spreadtrum Communications

- R1-2002366		Remaining Issues for PDCCH-based Power Saving Signal/Channel	InterDigital

- R1-2002414		Remaining issues for WUS	Ericsson

- R1-2002451		Maintenance for PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

- R1-2002555		Remaining issues for PDCCH-based power saving channel	Qualcomm Incorporated

- R1-2001507		LS on DCP	RAN2, 	Huawei

- R1-2000165, 	LS on secondary DRX group, RAN2, Ericsson.