3GPP TSG RAN WG1 Meeting #100bis-E															R1-200xxxx

e-Meeting, April 20 – 30, 2020

Source: 	Moderator (Intel Corporation)

Title:	Summary of email discussions for NR Mobility Enhancements

Agenda item:	7.2.9

Document for:	Discussion

- Introduction

In this contribution, we summarize the email discussion approved for discussion during RAN1 #100bis-E. Chairman has approved three email discussion threads for RAN1 #100bis-E. The following are the approved email discussions:

- [100b-e-NR-Mob-Enh-01] Email discussion/approval on UL cancellation in UL DAPS-HO by 4/24; if necessary, followed by endorsing the corresponding TP by 4/30 – Daewon (Intel)

- [100b-e-NR-Mob-Enh-02] Email discussion/approval on power sharing mode for UL DAPS-HO by 4/23; if necessary, followed by endorsing the corresponding TP by 4/29 – Daewon (Intel)

- [100b-e-NR-Mob-Enh-03] Email discussion/approval on PDCCH/PDSCH restrictions for DL DAPS-HO by 4/22; if necessary, followed by endorsing the corresponding TP by 4/28 – Daewon (Intel)

- Email Discussion [100b-e-NR-Mob-Enh-01]

[Copy discussion from the document for email thread-01]

- Email Discussion [100b-e-NR-Mob-Enh-02]

This discussion is regarding the power Sharing Mode for UL DAPS-HO (Issue #6 from [11]).

Issue and Proposal Summary:

Several companies provided discussion on how to correct the power sharing mode description for UL DAPS-HO. The following are list of proposals and corresponding TPs:

- Proposal by Intel [3] The indication from the gNB to UE to have the UE to no perform any power sharing operation and always drop the source cell transmission when it overlaps with target cell could be done by not providing the UplinkPowerSharingDAPS-HO-mode RRC configuration. Alternative method would be explicitly introducing a ‘no-powersharing mode’ indication for UplinkPowerSharingDAPS-HO-mode.

  - The following is proposed TP:

15 Dual active protocol stack based handover < Unchanged parts are omitted > If the UE does not provide indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE does not provide indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE is not provided with does not provides UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap are in overlapping time resources the UE transmits only on the target cell

- Two alternative TP are provided by Samsung [4]. The first alternative TP is proposed if the UE feature group 21-2 description is agreed with ALT 1 formulation (described in NTT Docomo’s contribution on UE feature list summary). The second alternative TP is proposed if the UE feature group 21-2 description is agreed with ALT 2 formulation.

  - ALT 1 formulation:

## 15 Dual active protocol stack based handover < Unchanged parts are omitted > If the UE does not provide indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE does not provide indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE is not provided with does not provides UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap are in overlapping time resources the UE transmits only on the target cell If - the UE is provided UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell ----omitted----

  - ALT 2 formulation:

## 15 Dual active protocol stack based handover ----omitted---- If - the UE does not provide UplinkPowerSharingDAPS-HO, or is not provided UplinkPowerSharingDAPS-HO-Mode and - UE transmissions on the target cell and the source cell are in overlapping time resources the UE transmits only on the target cell. If - the UE does not provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively. The UE determines intra-frequency as described in Clause 9.2.1 of [10, TS38.133]. ----omitted----

- Proposal by Nokia [5]: suggests to remove the UplinkPowerSharingDAPS-HO capability parameter description and replaces it with statement if which power control mode is used.

  - The following is the proposed TP:

If the UE indicates capability for UplinkPowerSharingDAPS-HO = Semistatic-mode1 power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates capability for UplinkPowerSharingDAPS-HO = Semistatic-mode2 power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates capability forUplinkPowerSharingDAPS-HO = Dynamic power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE isdoes not provided UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

- Proposal by Apple [6]: If gNB doesn’t configure the parameter UplinkPowerSharingDAPS-HO-mode, then no simultaneous UL transmission is allowed for UE with or without simultaneous transmission capability. UE drop the transmission to source cell if transmission collide in time domain resources. If gNB configures the parameter UplinkPowerSharingDAPS-HO-mode to UE with simultaneous transmission capability, if transmissions collide for intra-frequency intra-band and inter-frequency intra-band DAPS HO, then UE drops the transmission to source cell.

  - The following the proposed TP:

15 Dual active protocol stack based handover If - the UE is not provided with UplinkPowerSharingDAPS-HO-mode , and - UE transmissions on the target cell and the source cell are overlapping in time resources Or if - the UE is does not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

- Proposal by Ericsson [7]: If the NW does not signal to the UE how to distribute the transmit power between source and target, i.e., if the UE is not provided with the RRC parameter UplinkPowerSharingDAPS-HO-mode, the UE drops any UL transmission to the source if it overlaps with an UL transmission to target.

  - The following the proposed TP:

15 Dual active protocol stack based handover If - the UE does not provide UplinkPowerSharingDAPS-HO, or is not provided UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell.

- Proposal by Qualcomm [8]: Change UplinkPowerSharingDAPS-HO in “… as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO …” to NR-DC-PC-mode. Furthermore, we should align the terminology for Semi-static mode i.e., changing “Semistatic-mode” to “Semi-static-mode”.

  - Note: similar to proposal in [5]

  - The following the proposed TP:

15 Dual active protocol stack based handover <unchanged text omitted> If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. <unchanged text omitted>

Discussion Summary:

Companies are encouraged to provide comments on the proposal above. Comments should include views on whether proposal by Intel [3], Samsung [4], Nokia [5], Apple [6], Ericsson [7], and/or Qualcomm [8] is/are acceptable or not. Also, if companies have a merged proposal based on proposal from above companies, please do provide them below as well.

| Company Name | Comments/Views |
| --- | --- |
| Huawei/HiSilicon | The basic question, to our understanding, is whether there is correlation between UE indicates the capability of simultaneous transmission and does NOT indicate any power sharing mode. Our answer is yes. If UE is about to transmit UL simultaneously, similar to NR-DC, semi-static power sharing mode 1 should a basic feature. Otherwise, “no power sharing” would be meaningless. It is different from NW making decision whether enable/disable power sharing or simultaneous transmission. For example, if UE is not provided any power sharing mode, UE can just drop transmission to source cell. |
| Ericsson | In our understanding, two issues are discussed in parallel: the NWs ability to configure the UE to always transmit to target (and drop source if needed), and how to define the UE capability of power sharing. Again in our understanding, there seems to wide agreement that if the NW does not provide UplinkPowerSharingDAPS-HO-mode the UE would only transmit to target in case of collision. Regarding the issue on how to define the capability, the technical difference among the proposals seem small, and could be discussed on the UE capability threads. |
| Qualcomm | Below we provide suggested TP merging several TPs. In addition, we would like to add UE behavior for the case that the UE is provided UplinkPowerSharingDAPS-HO-mode different from UplinkPowerSharingDAPS-HO that the UE provides e.g., UE provides UplinkPowerSharingDAPS-HO = Semi-static-mode1 but it is provided with UplinkPowerSharingDAPS-HO-mode = Dynamic. Although this could be an error case, UE behavior should be well-specified in the spec. One question somewhat related to Samsung’s TP for Alt.2 “The UE determines intra-frequency as described in Clause 9.2.1 of [10, TS38.133].”: we have “if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band”. Does “intra-frequency and intra-band” mean intra-frequency case only, or mean both intra-frequency case and inter-frequency intra-band case? If it is former case, I wonder why we need to include intra-band here since intra-frequency by its own is already intra-band? Qualcomm’s suggested TP 15 Dual active protocol stack based handover <unchanged text omitted> If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE does not provide UplinkPowerSharingDAPS-HO, or is not provided UplinkPowerSharingDAPS-HO-mode, or is provided UplinkPowerSharingDAPS-HO-mode different from UplinkPowerSharingDAPS-HO that the UE provides, and - UE transmissions on the target cell and the source cell are in overlapping time resources the UE transmits only on the target cell. If - the UE provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell <unchanged text omitted>. |
| Intel | Quick question on the proposed TP from Qualcomm (above). The last text seems to be bit strange. “If - the UE provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell” This states that if the UE has indicated a capability and transmissions overlap, then it should only transmit on the target cell (regardsless of anything else). I think this may be updated similarly to what Apple suggested. |
| Samsung | We support the suggested TP from Qualcomm’s input. (above or modified version in the follow-up email). Regarding Qualcomm’s clarification about “intra-frequency and intra-band”, we think it is clear that “intra-frequency and intra-band” means intra-frequency case only. And “intra-band” can be removed in the text Qualcomm refers to. |
| ZTE | In our understanding, one of reason for UE to always drop source cell transmission in case of UL collision is that UE does not support any power sharing, i.e. nothing is reported by UE for power sharing. The other reason is that single Tx is reported as discussed in our contribution. There may be some other reasons which leads to drop source cell transmission. So we are agree that all the cases can be merged to one that the network configures UE to drop source transmission. This can be indicated by the absence of UL power sharing mode. So, we support Apples’s TP. For the description of power sharing reporting in TS38.213, we think these can be fixed after the relevant UE reporting is finished. |

- Email Discussion [100b-e-NR-Mob-Enh-03]

[Copy discussion from the document for email thread-03]

Reference

- R1-2001530, “Remaining issues on DAPS-HO,” Huawei, HiSilicon

- R1-2001624, “Remaining issues on NR mobility enhancements in physical layer,” ZTE

- R1-2002011, “Corrections to Physical layer aspects of NR mobility enhancement,” Intel Corporation

- R1-2002148, “Remaining issues for NR Mobility Enhancement,” Samsung

- R1-2002221, “Remaining physical layer aspects of dual active protocol stack based HO,” Nokia, Nokia Shanghai Bell

- R1-2002344, “On remaining issues on NR mobility enhancements,” Apple

- R1-2002490, “Correction to UL power sharing for DAPS HO,” Ericsson

- R1-2002558, “Maintenance for NR mobility enhancements,” Qualcomm Incorporated

- R1-2001531, “Remaining PHY aspects for CHO,” Huawei, HiSilicon

- R1-2001625, “Discussion on FR2 mobility interruption enhancements,” ZTE

- R1-2002010, “Issue Summary for NR Mobility Enhancements,” Moderator (Intel Corporation)