3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001527

E-meeting, April 20 – April 30, 2020

Agenda Item:	7.2.3.4

Source:	Huawei, HiSilicon

Title:	Remaining issues on the "case-1" OTA timing alignment

Document for:	Discussion and Decision

Introduction

In RAN2#109e, an LS was sent to RAN1 on T_delta in IAB [1]. According to RAN2 agreements, there is no SCS field in MAC-CE for T_delta and the length of the T_delta field is 11 bits (asking RAN1 whether it is inadequate). Besides, RAN4 provided a table on T_delta in the following [2]:

Table 1. Range of T_delta

| SCS [kHz] | Max T_delta [Tc] | Min T_delta [Tc] |
| --- | --- | --- |
| 15 | - /2 + 6256 | - /2 - 70528 |
| 30 | - /2 + 6128 | - /2 - 35328 |
| 60 | - /2 + 6032 | - /2 - 17664 |
| 120 | - /2 + 6032 | - /2 - 8816 |

In this contribution, the field length and index value mapping of T_delta are discussed.

Discussion

In NR, SCS with 15, 30 and 60 kHz can be applicable for FR1, while 60 and 120 kHz can be applicable for FR2. The length of T_delta field for different frequency ranges and different SCS was analyzed in [3].

Table 2. Analysis for T_delta index and signaling filed length

| FR | Granularity [Tc] | SCS [kHz] | Range [Tc] | Required index | Required length for signaling [bits] |
| --- | --- | --- | --- | --- | --- |
| FR1 | 64 | 15 | 6256+70528=76784 | ceil(76784/64)=1200 | 11 |
|  |  | 30 | 6128+35328=41456 | ceil(41456/64)=675 | 10 |
|  |  | 60 | 6032+17664=23696 | ceil(23696/64)=371 | 9 |
| FR2 | 32 | 60 | 6032+17664=23696 | ceil(23696/32)=740 | 10 |
|  |  | 120 | 6032+8816=14848 | ceil(14848/32)=464 | 9 |

In the table, the ceil function means rounding up. To capture the mapping between the index and value of T_delta, there are two possible options which will be discussed in following sub-sections.

Option 1: Different granularities for different FRs with given T_delta range

To map the index to value of T_delta, the relationship between them should be given separately for each FR. Considering different granularities in frequency ranges and min/max values for each SCS, one unified formula for T_delta can be summarized as:

Tdelta=-NTA,offset/2+min NT_delta,min+ nT_delta ∙GT_delta,   NT_delta,maxTc

Where nT_delta  is an index which can be provided to an IAB node. If we strictly follow RAN4 guidance [4] (64Tc for FR1 and 32Tc for FR2), NT_delta,max, NT_delta,min, and GT_delta should be captured as shown in Table 3

Table 3. Parameter table for determining T_delta

| FR | μ | NT_delta,max | NT_delta,min | GT_delta |
| --- | --- | --- | --- | --- |
| FR1 | 0 | 6256 | -70528 | 64 |
|  | 1 | 6128 | -35328 | 64 |
|  | 2 | 6032 | -17664 | 64 |
| FR2 | 2 | 6032 | -17664 | 32 |
|  | 3 | 6032 | -8816 | 32 |

Observation 1: A formula can be defined for T_delta with an additional table to capture various granularities and min/max values.

An example TP for option 1 is given as following:

14 Integrated access-backhaul operation =======================<Unchanged text is omitted>================================= If an IAB-node is provided an index ndelta from serving cell by the MAC CE as described in [11, TS38.321], the IAB node determines the value T_delta as follows: Tdelta=-NTA,offset/2+min NT_delta,min+ nT_delta ∙GT_delta, NT_delta,maxTc where NTA,offset is obtained as for a "UE" in Clause 4.2. NT_delta,max, NT_delta,min, and GT_delta are determined according to Table 14.2. Table 14.2. Parameter table for determining T_delta FR; μ; NT_delta,max; NT_delta,min; GT_delta. FR1; 0; 6256; -70528; 64. 1; 6128; -35328; 64. 2; 6032; -17664; 64. FR2; 2; 6032; -17664; 32. 3; 6032; -8816; 32. TdeltaThe IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time. ========================<Unchanged text is omitted>================================

Option 2: The same granularity for different FRs with modification on T_delta range

The values of T_delta range given by RAN4 are not easy for RAN1 to capture by a simple mapping formula. However, this issue can be solved by making some modifications on those values. For the maximum T_delta range, it can be set to (- /2 + 6256) for all SCS and FR which covered all given upper bound of ranges. Similar for the minimum number of T_delta range, they can be unified to a multiple of 8816 with a relation to its numerology.

Table 4. An example of modified range

| SCS [kHz] | Max T_delta [Tc] |  | Min T_delta [Tc] |  |
| --- | --- | --- | --- | --- |
|  | original | Modified | original | modified |
| 15 | - /2 + 6256 | - /2 + 6256 | - /2 - 70528 | - /2 - 70528 |
| 30 | - /2 + 6128 | - /2 + 6256 | - /2 - 35328 | - /2 - 35264 |
| 60 | - /2 + 6032 | - /2 + 6256 | - /2 - 17664 | - /2 - 17632 |
| 120 | - /2 + 6032 | - /2 + 6256 | - /2 - 8816 | - /2 - 8816 |

By doing those modifications, the formula of T_delta can be present as a unified formula:

Tdelta=(-NTA,offset/2+min(-70528/2μ+nT_delta∙32  , 6256))Tc

Where μ=0,1,2,3 and nT_delta = 0, 1, 2, ..., 2400,  nT_delta is the index provided by parent node. In this case, 12 bit field length is required for MAC CE signaling.

Observation 2: A simplified formula of T_delta can be defined by making some modifications on the ranges provided by RAN4. No additional table is needed in this case.

An example TP for option 2 is given as following:

14 Integrated access-backhaul operation =======================<Unchanged text is omitted>================================= If an IAB-node is provided an index ndelta from serving cell by the MAC CE as described in [11, TS38.321], the IAB node determines the value T_delta as: Tdelta=(-NTA,offset/2+min(-70528/2μ+nT_delta∙32 , 6256))Tc The IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time. =======================<Unchanged text is omitted>=================================

Summary

The comparison of two options above and their Pros and Cons are summarized in Table 5.

Table 5. Comparison of T_delta index mapping options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | Unified formula with T_delta values fully aligned to RAN4 agreements | Beside the formula, an extra table is required for capturing various ranges and granularities. |
| Option 2 | Unified formula for all SCS and FR | Potential need to check with RAN4 on those modified numbers |

Proposal: Considering either TP for option 1 or option 2 when capturing T_delta in TS 38.213.

Conclusions

In this contribution, several issues on T_delta indication signalling are discussed, we observe:

Observation 1: A formula can be defined for T_delta with an additional table to capture various granularities and min/max values.

Observation 2: A simplified formula of T_delta can be defined by making some modifications on the ranges provided by RAN4. No additional table is needed in this case.

And we propose:

Proposal: Considering either TP for option 1 or option 2 when capturing T_delta in TS 38.213.

- References

[1] R2-2002254, LS to RAN1 on T_delta in IAB, 3GPP TSG-RAN WG2 #109-e

[2] R4-1913101, RAN4#92bis meeting report

[3] R2-2000527, Remaining issue for the Timing Delta MAC CE, Huawei, HiSilicon

[4] R4-1912714, Reply LS on T_delta for IAB OTA timing alignment