3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001543

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.10.4

Source:	Huawei, HiSilicon

Title:	Remaining issues on the A-CSI RS triggering with different numerology

Document for:	Discussion and Decision

# Introduction

After RAN1#100-e meeting [1], most issues on A-CSI RS triggering with different numerology are completed and endorsed in the specifications. In this contribution, some remaining details on A-CSI RS triggering with different numerology are discussed and corresponding TP are proposed.

# Discussion

In current specification, the additional beam switching time delay d is defined in PDCCH symbols, the threshold of beamSwitchTiming + d∙2μCSIRS/2μPDCCH is converted to CSI-RS symbols when the reported value is one of the values of {14, 28, 48}. But when the reported value of beamSwitchTiming is one of the values of {224, 336}, the threshold of beamSwitchTiming + d is confusing on whether it is in PDCCH symbols or CSI-RS symbols. We propose to update the value as beamSwitchTiming + d∙2μCSIRS/2μPDCCH as in CSI-RS symbols.

Proposal 1: Adopt the following TP in TS 38.214.

q=-1q=-1

--------------------------------- Start TP for TS 38.214 sub-clause 5.2.1.5.1a --------------------------------------

<Unchanged parts omitted>

5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

<Unchanged parts omitted>

--------------------------------- End TP for TS 38.214 sub-clause 5.2.1.5.1a --------------------------------------

# Conclusion

According to the above discussions, we propose to adopt the above mentioned TP.

# References

- 3GPP RAN1 Chairman’s notes, RAN1#100-e, Feb.24– Mar.06, 2020.