3GPP TSG RAN WG1 #100bis                                                                                   R1-2001615

e-Meeting, April 20th – 24th, 2020

Source:			     ZTE

Title:	 Remaining issues on UL inter-UE multiplexing between eMBB and URLLC

Agenda item:	 7.2.5.5

Document for:  Discussion and Decision

Introduction

Most details of UL cancelation and UL power control schemes for UL inter-UE multiplexing were determined during the last RAN1 meetings. There are also some remaining issues needed to be further clarified and addressed, such as:

- A UL channel with which priority level can be cancelled by UL CI.

- Simultaneous UL prioritization/multiplexing for intra-UE and inter-UE cancellation.

- Determination of the time region for UL CI.

- Number of partitions within the time region excluding DL and SSB symbols.

- Clarification of the value of .

In this contribution, we mainly focus on the above issues and give the corresponding text proposals.

Priority level of UL channel for cancelation

The following options on priority level of UL channel for cancelation had been discussed during RAN1#100e meeting.

- Option 1-1: For a given UE, UL CI is only applicable to the UL transmissions indicated/configured as low priority level - Option 1-2: A UE is not expected to receive a UL CI indicating a cancellation of its high priority UL transmission - Option 2: For a given UE, UL CI is applicable to UL transmission irrespective of its priority level - Option 3: A new RRC parameter to configure between following behaviours - Behaviour #1: For the given UE, UL CI is only applicable to the UL transmissions indicated/configured as low priority level - Behaviour #2: For the given UE, UL CI is applicable to UL transmission irrespective of its priority level

The current two-level priority is defined only for intra-UE prioritization/multiplexing handling, while there is no priority level definition between inter-UEs. From the perspective of network, there may be more than two levels of priority coexisted in one cell considering various use cases with different requirements on reliability and latency. For example, there are two URLLC UEs, where UE#1 is a remote driving UE and UE#2 is a UE with intelligent transport traffic. As discussed in SI phase, UE#1 has a stringer latency requirement than UE#2. Together with eMBB UEs in the same cell, there could be more than 2 levels of priority from network perspective. Then, for both option 1-1 and option 1-2, it is impossible to cancel a high priority UL transmission from one UE, e.g. UE#2 by the network if it collides with a more vital UL transmission from another UE, e.g. UE#1.

Observation 1: There are cases that one UL transmission with high priority from a UE needs to be canceled due to collision with a more vital UL transmission from another UE.

Option 2 is the straightforward way without spec impact and gives some flexibility for gNB to optimize all transmissions in the cell as a whole.

Option 3 is a hybrid method by introducing an RRC signaling for switching between option 1 and option 2. At this late stage, introducing a new RRC parameter is not appropriate unless there is a clear motivation.

Proposal 1: NR should support the mechanism that UL CI is applicable to UL transmission irrespective of its priority level.

Simultaneous UL prioritization/multiplexing for intra-UE and inter-UE

In NR Rel-16, both intra-UE and inter-UE UL prioritization/multiplexing can be supported by a UE. The following options on simultaneous UL prioritization/multiplexing for intra-UE and inter-UE were discussed during RAN1#100 e-meeting.

- Option 1: Handling of intra-UE prioritization/multiplexing for overlapping UL transmissions is performed firstly and handling of inter-UE cancellation for UL transmission overlapping with resources by UL CI is performed secondly - Option 2: Handling of inter-UE cancellation for UL transmission overlapping with resources by UL CI is performed firstly and handling of intra-UE prioritization/multiplexing for overlapping UL transmissions is performed secondly - Option 3: UE performs intra-UE prioritization/multiplexing or inter-UE cancellation for the overlapped UL channels according to the time order which is determined by the receiving time order of PDCCH carrying DCI scheduling high priority transmission or DCI for UL CI.

In general, it is natural for a UE to perform intra-UE and inter-UE prioritization/multiplexing according to the DCI order in time, rather than for a UE to wait for a potential upcoming DCI before performing an operation. However, if a UE performs the operation only according to the time order of DCI, some UL transmissions may be dropped falsely. In Figure-1, a DL grant schedules a PDSCH with high priority and the corresponding PUCCH carrying the HARQ-ACK overlaps with a high priority PUSCH scheduled by an UL grant later. The UE drops the high priority PUCCH and multiplexes the UCI on the high priority PUSCH firstly. Then, the UE decides to cancel the high priority PUSCH after decoding the UL CI, which will result in cancelation of the high priority PUSCH and dropping of the UCI. However, it is reasonable to keep the PUCCH transmission because there will be no intra-UE multiplexing issue after canceling the PUSCH by UL CI. Similarly, if handling of intra-UE prioritization/multiplexing for overlapping UL transmissions is performed firstly, i.e. Option 1, some UL transmissions will be also dropped falsely because of intra-UE prioritization/multiplexing mechanism.

Figure-1 Example of simultaneous UL prioritization/multiplexing for intra-UE and inter-UE

Observation 2: If a UE performs intra-UE and inter-UE operations according to the time order of DCI or handling of intra-UE prioritization/multiplexing for overlapping UL transmissions firstly, some UL transmissions may be dropped falsely.

An effective way to solve the problem is not to handle intra-UE prioritization/multiplexing for overlapping UL transmissions firstly, but to cancel the PUSCH firstly according to the later UL CI. However, if the UE waits for the DCI carrying UL CI, it may not have enough time to perform the intra-UE prioritization/multiplexing after decoding UL CI. One way is to define a timeline, e.g. T = Tproc,2, between the end of UL CI and the beginning of a group of intra-UE overlapping channels. If the timeline is satisfied, it means that there will be enough time for handling intra-UE prioritization/multiplexing after decoding UL CI and the UE can perform inter-UE multiplexing firstly. Then, unnecessary dropping of UL transmission in cases as shown in Figure-1 can be avoided. If this timeline is not satisfied, the UE can perform intra-UE cancellation firstly.

The above method can be easily implemented according to the existing intra-UE prioritization/multiplexing mechanism, by assuming the time-frequency resource indicated by UL CI as a normal UL transmission with the highest priority.

Observation 3: If there is enough time for performing intra-UE prioritization/multiplexing after decoding UL CI, false dropping of UL transmissions can be avoided by performing inter-UE prioritization/multiplexing firstly.

Proposal 2: RAN1 should define a timeline between the end of last symbol of CORESET containing UL CI and the start of the earliest UL transmission among a group overlapping UL transmissions for determining whether handling of inter-UE prioritization firstly or not.

Proposal 3: A UE should perform simultaneous UL prioritization/multiplexing for intra-UE and inter-UE according to the timeline between the end of last symbol of CORESET containing UL CI and the start of the earliest UL transmission among a group overlapping UL transmissions.

●	If the timeline is satisfied, handling of inter-UE prioritization for UL transmission according to UL CI firstly and handling of intra-UE prioritization/multiplexing for overlapping UL transmissions secondly.

●	If the timeline is not satisfied, handling of intra-UE prioritization/multiplexing for overlapping UL transmissions firstly and handling of inter-UE prioritization for UL transmission according to UL CI secondly.

Determination of the time region for UL CI

As agreed in RAN1#98 and RAN1#98bis meetings, the starting symbol of time region for UL CI is related with UE processing time requirement for UL CI, which is based on N2 defined in Rel-15 UE cap#2. But it has not been discussed on which SCS should be taken as reference for N2 value determination.

Agreements: RAN1#98[1] - The UE processing time requirement for UL cancelation indication based on N2 defined in Rel-15 UE cap#2 is supported Agreements: RAN1#98bis[2] - The reference time region where a detected UL CI is applicable is determined by the following: - The reference time region starts from X symbols after the ending symbol of the PDCCH CORESET carrying the UL CI, where X is at least equal to the minimum processing time for UL cancelation

The corresponding description is captured as follows in TS 38.213[3], where the description highlight in yellow follows Rel-15 approach but was not explicitly discussed.

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4. <---------------------------Other parts are omitted ------------------------------->

As DCI format 2_4 is a group common DCI, a group of UEs who monitoring the same DCI may have PUSCH transmission or SRS transmission with different SCSs. According to current description, different UEs may use different starting points for the time region of the same UL CI according to different SCSs. For example, UE1 and UE2 are configured the same search space set for monitoring DCI format 2_4 with 30kHz, and scheduled for PUSCH transmissions with 15kHz and 30kHz respectively. Then, UE1 will determine  symbols in 15kHz, while UE2 will determine  symbols in 30kHz. Different starting points will be used by UE1 and UE2 respectively.

Observation 4: According to current description, different UEs may use different starting points for the time region of the same UL CI according to different SCSs.

In order to ensure different UEs have a uniform understanding on starting point of the time region. One simple way is to define the smallest SCS configuration between the SCS configurations of the PDCCH and SCSs provided in FrequencyInfoUL-SIB on the serving cell as reference SCS for starting point of the time region.

Proposal 4: Defining the smallest SCS configuration between the SCS configurations of the PDCCH and SCSs provided in FrequencyInfoUL-SIB on the serving cell as reference SCS for starting point of the time region for UL CI.

Based on the above analysis, we propose the following text proposal.

-----------------------------------------Text Proposal 1 for Section 11.2A in TS38.213[3]------------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or a SRS transmission on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by XXX. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and SCSs provided in FrequencyInfoUL-SIB on the serving cell. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 after a last symbol of a CORESET where the UE detects the DCI format 2_4. <---------------------------Other parts are omitted ------------------------------->

Number of partitions within the time region excluding DL and SSB symbols

According to the following agreements reached in RAN1#99 and #100e meetings[4][5]. The time region for UL CI will be divided to a configured number of partitions in the time domain after excluding DL symbols and SSB symbols.

Agreements: #99 - timeGranularityforCI is defined as number of partitions within the time region, and possible values are - {1,2,4,7,14,28} - The configured value of CI-PayloadSize shall be a multiple integer of the configured value of timeGranularityforCI Agreements: #99 - The DL symbols indicated by tdd-UL-DL-ConfigurationCommon are excluded from the reference time region for UL CI - The partition of reference time region is done after excluding the DL symbols - The symbols used for SSB are also excluded Agreements: #99 - Possible values for RRC parameter timedurationforCI can be: - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion - At least the same as the configured UL CI monitoring periodicity - FFS whether or not to additionally support multiple of UL CI monitoring periodicity Agreements: #99 Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity” - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity Agreements: #100e - Confirm that 14OS can be configured for timedurationforCI (when 1-slot is the configured UL CI monitoring periodicity with more than one monitoring occasions within 1 slot) - The possible values for CI-PayloadSize, are {1,2,4,5,7,8,10,14,16,20, 28,32,35,42,56,112}

The time domain partition method is described as follows in TS 38.213[3].

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a serving cell having an associated field in DCI format 2_4, for the field denote by - a number of bits provided by CI-PayloadSize - a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon,provided by timeDurationforCI in timeFrequencyRegion - a number of partitions for the symbols provided by timeGranularityforCI in timeFrequencyRegion sets of bits from the bits have a one-to-one mapping with groups of symbols where each of the first groups includes symbols and each of the remaining groups includes symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, bits from each set of bits have a one-to-one mapping with groups of PRBs where each of the first groups includes PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as and a number of contiguous RBs as from frequencyRegionforCI that indicates an offset and a length as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

As shown in Figure-2, the semi-static frame structure is configured as ‘DDDFU DDFUU’ for example. Within some time regions, e.g. time region #1, there are SSB symbols as shown in green blocks. In some other time regions, e.g. time region #2, there is no SSB symbol.

Figure-2: An example of time region partition

If the RRC parameters for UL CI indication are configured as follows.

- timeGranularityforCI =28

- CI-PayloadSize = 112

- timedurationforCI = 70 symbols

For the time region with SSB symbols, the number of remaining symbols within the time region excluding DL and SSB symbols is only 70-14*3-4*2=20.

According to current partition rules described in TS38.213[3], the remaining 20 symbols will still be divided into 28 time domain partitions. And each of the 28 partitions will be assigned 4 bits for cancelation resource indication. Then, each of the last 20 time domain partitions contains 1 symbol, while there is no symbol in the first 8 time domain partitions. The first 32 bits corresponds to the first 8 time domain partitions will be a big waste.

Observation 5: Some bits will be wasted if the number of partitions configured by timeGranularityforCI is larger than the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI.

The following method can be considered for making full use of all bits configured. Taking the minimum value between the number of partitions configured by timeGranularityforCI and the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI as the actual number of partitions.

Following the same configuration of Figure-2, the actual number of partitions will be min {28, 20} = 20. Then, each actual time domain partition contains 1 symbol. There will be bits for cancelation resource indication for each time domain partition. Comparing with 32bits invalid overhead, there are only 112-20*5=12bits being wasted.

Proposal 5: NR should support to take the minimum value between the number of partitions configured by timeGranularityforCI and the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI as the actual number of partitions.

Based on the above analysis, we propose the following text proposal.

--------------------------------------------Text Proposal 2 for Section 11.2A in TS38.213[3]---------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a serving cell having an associated field in DCI format 2_4, for the field denote by - a number of bits provided by CI-PayloadSize - a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon,provided by timeDurationforCI in timeFrequencyRegion - a number of partitions for the symbols provided by timeGranularityforCI in timeFrequencyRegion - min {, } sets of bits from the bits have a one-to-one mapping with groups of symbols where each of the first groups includes symbols and each of the remaining groups includes symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, bits from each set of bits have a one-to-one mapping with groups of PRBs where each of the first groups includes PRBs and each of the remaining groups includes PRBs. A UE determines a first PRB index as and a number of contiguous RBs as from frequencyRegionforCI that indicates an offset and a length as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

Clarification of the value of

The following agreements on time duration of UL CI are reached in RAN1#99 meeting[4].

Agreements: #99 - Possible values for RRC parameter timedurationforCI can be: - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion - At least the same as the configured UL CI monitoring periodicity - FFS whether or not to additionally support multiple of UL CI monitoring periodicity - Otherwise (i.e., >1 monitoring occasion within 1 slot when 1-slot is the configured UL CI monitoring periodicity) - {2, 4, 7, [14]} OS, which SCS is used when determine the time duration - SCS for the DL BWP carrying UL CI Agreements: #99 Regarding “FFS whether or not to additionally support multiple of UL CI monitoring periodicity” - If the configured UL CI monitoring periodicity is >1 slot or 1-slot with only one monitoring occasion, no additionally support that the time duration to be multiple of UL CI monitoring periodicity

The RRC parameter timeDurationForCI is captured in TS 38.331[6] as follows.

UplinkCancellation information element UplinkCancellation-r16 ::= SEQUENCE { ci-RNTI-r16 RNTI-Value, dci-PayloadSizeForCI-r16 INTEGER (0..maxCI-DCI-PayloadSize-r16), ci-ConfigurationPerServingCell-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF CI-ConfigurationPerServingCell-r16, ... } CI-ConfigurationPerServingCell-r16 ::= SEQUENCE { servingCellId ServCellIndex, positionInDCI-r16 INTEGER (0..maxCI-DCI-PayloadSize-r16-1), positionInDCI-ForSUL-r16 INTEGER (0..maxCI-DCI-PayloadSize-r16-1) OPTIONAL, -- Cond SUL-Only ci-PayloadSize-r16 ENUMERATED {n1, n2, n4, n7, n8, n14, n16, n28, n32, n56, n112}, timeFrequencyRegion-r16 SEQUENCE { timeDurationForCI-r16 ENUMERATED {n2, n4, n7} OPTIONAL, -- Cond SymbolPeriodicity timeGranularityForCI-r16 ENUMERATED {n1, n2, n4, n7, n14, n28}, frequencyRegionForCI-r16 INTEGER (0..37949), ... } } timeDurationForCI Configures the duration of the reference time region in symbols where a detected UL CI is applicable of this serving cell (servingCellId) (see TS 38.213 [13], clause 11.5). If the configured UL CI monitoring periodicity is larger than 1 slot or 1 slot with only one monitoring occasion, the UE applies the same as the configured UL CI monitoring periodicity, Editor 'note: FFS on n14 for timeDurationForCI.. Conditional Presence; Explanation. SymbolPeriodicity; This field is mandatory present if the configured UL CI monitoring periodicity is less than 1 slot with only one monitoring occasion, Need M, otherwise absent..

If the configured UL CI monitoring periodicity is larger than 1 slot or 1 slot with only one monitoring occasion, the UE applies the same as the configured UL CI monitoring periodicity. Then, there will be no value provided by RRC parameter timeDurationForCI for. While, according to current description in TS 38.213, the value of  is always provided by timeDurationForCI, which is not accurate.

Based on the above analysis, we propose the following text proposal.

--------------------------------------------Text Proposal 3 for Section 11.2A in TS38.213[3]---------------------------------

11.2A Cancellation indication <---------------------------Other parts are omitted -------------------------------> For a serving cell having an associated field in DCI format 2_4, for the field denote by - NCI a number of bits provided by CI-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, provided by timeDurationforCI in timeFrequencyRegion if the monitoring periodicity of DCI format 2_4 is one slot with more than one monitoring occasions within the slot, or equals to the number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, in monitoring periodicity of DCI format 2_4 if the monitoring periodicity is larger than 1 slot or 1 slot with only one monitoring occasion - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion GCI sets of bits from the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. <---------------------------Other parts are omitted ------------------------------->

Conclusions

In this contribution, the remaining issues on UL inter-UE multiplexing between eMBB and URLLC are discussed. The observations and proposals are made as follows, and text proposals are provided in corresponding sections respectively.

Observation 1: There are cases that one UL transmission with high priority from a UE needs to be canceled due to collision with a more vital UL transmission from another UE.

Observation 2: If a UE performs intra-UE and inter-UE operations according to the time order of DCI or handling of intra-UE prioritization/multiplexing for overlapping UL transmissions first, some UL transmissions may be dropped falsely.

Observation 3: If there is enough time for performing intra-UE prioritization/multiplexing after decoding UL CI, false dropping of UL transmissions can be avoided by performing inter-UE prioritization/multiplexing firstly.

Observation 4: According to current description, different UEs may use different starting points for the time region of the same UL CI according to different SCSs.

Observation 5: Some bits will be wasted if the number of partitions configured by timeGranularityforCI is larger than the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI.

Proposal 1: NR should support the mechanism that UL CI is applicable to UL transmission irrespective of its priority level.

Proposal 2: RAN1 should define a timeline between the end of last symbol of CORESET containing UL CI and the start of the earliest UL transmission among a group overlapping UL transmissions for determining whether handling of inter-UE prioritization firstly or not.

Proposal 3: UE should perform simultaneous UL prioritization/multiplexing for intra-UE and inter-UE according to the timeline between the end of last symbol of CORESET containing UL CI and the start of the earliest UL transmission among a group overlapping UL transmissions.

●	If the timeline is satisfied, handling of inter-UE prioritization for UL transmission according to UL CI firstly and handling of intra-UE prioritization/multiplexing for overlapping UL transmissions secondly.

●	If the timeline is not satisfied, handling of intra-UE prioritization/multiplexing for overlapping UL transmissions firstly and handling of inter-UE prioritization for UL transmission according to UL CI secondly.

Proposal 4: Defining the smallest SCS configuration between the SCS configurations of the PDCCH and SCSs provided in FrequencyInfoUL-SIB on the serving cell as reference SCS for starting point of the time region for UL CI.

Proposal 5: NR should support to take the minimum value between the number of partitions configured by timeGranularityforCI and the number of remaining symbols after excluding DL and SSB symbols from the time region for UL CI as the actual number of partitions.

References

- 3GPP RAN1 #98, RAN1 Chairman’s Notes.

- 3GPP RAN1 #98bis, RAN1 Chairman’s Notes.

- 3GPP TS 38.213-g10, Physical layer procedures for control, Release 16.

- 3GPP RAN1 #99, RAN1 Chairman’s Notes.

- 3GPP RAN1 #100e, List of email approvals.

- 3GPP TS 38.331-g00, Radio Resource Control (RRC) protocol specification, Release 16.