3GPP TSG-RAN WG1 Meeting #100bis		    				R1-2001684

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Other issues for R16 UE power saving

Agenda Item:	7.2.7.4

Document for:	Discussion and Decision

- Introduction

This contribution addresses other issues for Rel-16 UE power saving, especially interaction with SCell dormancy feature.

- Discussion

  - Conflict UE behaviour for WUS indication and Scell dormancy indication

In [1], we raised the issue that it is not clear about UE behavior of CSI measurement and UL transmission when SCell enters dormant BWP during DRX OFF.

For uplink, so far for cells which are in dormancy state, most uplink transmission is not transmitted, i.e., SRS transmission will not be supported and UE should stop all the UL behavior in case the DL BWP is switched to dormant BWP, i.e. stop any UL transmission, suspend any configured uplink grant Type 1, clear any configured uplink grant of configured grant Type 2 in the dormancy SCell. But for cells which are in DRX off state, most uplink transmission is not affected. UE can still transmit RACH, SR, configured grant, which will not be affected by DRX configuration.

For downlink, UE will not perform CSI measurement and report CSI during the time duration indicated by drx-onDurationTimer outside DRX active time if RRC parameter PS-Periodic_CSI_TransmitOrNot is configured. Howerver, UE need to perform periodic or semi-persistent CSI measurements on dormancy SCell.

Therefore, it is not clear the downlink/uplink transmission behavior when UE is indicated by WUS not to start drx-onDurationTimer (which means DRX OFF), but it is indicated by WUS to also enter in dormancy state. Proposal 1:  For a Scell in DRX OFF and dormancy state, following UE behavior is assumed

- UE does not perform CSI measurement and P-CSI report if RRC parameter PS-Periodic_CSI_TransmitOrNot is configured

- UE stop all the UL behavior

  - independent WUS indication and SCell dormancy indication

So far, TS38.212 restricts the ‘SCell dormancy indication’ can only be presence when ’Wake-up indication’ is presence. It is not allowed to indicate SCell dormancy without configuring ‘Wake-up indication’. Such restriction means that DCI 2_6 for dormancy indication cannot be operated with legacy DRX (i.e. without WUS function), and wake-up indication field shall always be preset in a DCI format 2_6.

Whether such operation (DCI format 2_6 with dormancy indication without wake-up indication) is valid or not should be discussed and decided. And from UE feature list perspective, whether FG18-4a “SCell dormancy outside active time” can be supported by the UE without supporting FG 19-1 “DRX Adaptation” component (1) ‘Configured PS_offset for the detection of  DCI format 2_6  with CRC scrambling by PS-RNTI and minimum time gap before DRX ON duration’ as prerequisite [3]. To achieve a DCI 2_6 without ‘wake-up indication’, there is no need for RRC parameter update, but only some RAN1 spec clarifications.

Proposal 2: For DCI format 2_6, to decide whether ‘SCell dormancy indication’ can be presence with absence of ‘wake-up indication’ i.e., ‘wake-up indication’ can be 0-bit.

Proposal 3: In UE feature session, to decide whether FG18-4a “SCell dormancy outside active time” can be supported by the UE without supporting FG 19-1 “DRX Adaptation –component 1)” as prerequisite

- Conclusion

This contribution addresses other issues for Rel-16 UE power saving, especially interaction with SCell dormancy feature.  And it is proposed as follows,

Proposal 1:  For a Scell in DRX OFF and dormancy state, following UE behavior is assumed

- UE does not perform CSI measurement and P-CSI report if RRC parameter PS-Periodic_CSI_TransmitOrNot is configured

- UE stop all the UL behavior

Proposal 2: For DCI format 2_6, to decide whether ‘SCell dormancy indication’ can be presence with absence of ‘wake-up indication’ i.e., ‘wake-up indication’ can be 0-bit.

Proposal 3: In UE feature session, to decide whether FG18-4a “SCell dormancy outside active time” can be supported by the UE without supporting FG 19-1 “DRX Adaptation –component 1) ” as prerequisite

# Reference

- R1-2001689	Remaining issues on SCell dormancy like behavior	vivo

- R1-2001682	Maintenance of PDCCH-based power saving signal	vivo

- R1-2001484	RAN1 UE features list for Rel-16 NR after RAN1#100-E	AT&T, NTT DOCOMO, INC.