3GPP TSG RAN WG1 #100b	                                              R1-2001688

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Remaining issues on uplink power control for NR-NR DC

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #100 e-meeting, following agreement was achieved on dynamic power sharing for NN-DC.

Update the previous agreement as follows (changes in red): Agreements: · For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0, - UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG - Otherwise, pwr_SCG <= Ptotal; - UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission. - (working assumption) No new RRC signaling is introduced for T_offset: - Alt.1: T_offset =<= T_proc,2 max⁡{Tproc,MCGmax,Tproc,SCGmax,}, where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc,SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and/or Tproc,CSImux as specified in TS38.213 and TS38.214 based on the configurations for the SCG. - This is the “DPS without look-ahead”. - Alt.2: T_offset =<= 2*T_proc,2max⁡{Tproc,MCGmax,Tproc,SCGmax,}, where: - Tproc,MCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux, and/or Tproc,2mux, as specified in TS38.213 and TS38.214 based on the configurations for the MCG. - Tproc,SCGmax is the maximum UE processing time among any of the possible values from Tproc,2, Tproc,CSI, Tproc,releasemux and/or Tproc,2mux, as specified in TS38.213 and TS38.214 based on the configurations for the SCG. - This is the “DPS with look-ahead”. - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms - To be addressed in the CR stage - A UE reports the UE capability of Alt.1 and/or Alt.2. - Details up to UE feature list discussion

In this contribution, we provide remaining issues and text proposals on NN-DC power control.

- Discussion

  - Semi-static power sharing

For semi-static power sharing, it was agreed Alt.1-2 only can be configured for synchronous DC. Regarding TDD UL-DL configurations for Alt.1-2, based on our understanding, at least tdd-UL-DL-ConfigurationCommon should be configured, but whether tdd-UL-DL-ConfigurationDedicated is configured or not is optional. Thus, the text of “by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated” is confusing and should be clarified.

Proposal 1:

- Agree the following text proposal to clarify the TDD UL/DL configuration for semi-static power sharing.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Subclauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Subclauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

  - Dynamic power sharing

The description of  PMCGactuali1 and PSCGactuali2 in dynamic power control part is not consistent with other parts. It is proposed to clarify the definitions of them (i.e., linear values).

Proposal 2:

- Agree the following text proposal for PMCGactuali1 and PSCGactuali2 definition in dynamic power sharing part to keep up with other parts.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided NR-DC-PC-mode = Dynamic

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the linear values of UE transmission power in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

  - RRC parameter alignment

The intention of following proposal and text proposal is to keep RRC parameters consistently between the endorsed MR-DC running CR of TS 38.331 and TS 38.213.

Proposal 3:

- Agree the following text proposal to keep RRC parameters consistently between the endorsed MR-DC running CR of TS 38.331 and TS 38.213.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

…

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 and/or bynrdc-PCmode-FR2-r16. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode1 or NR-DC-PC-mode = semi-static-mode2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3].

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = dynamic

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

If a UE

-	is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

- Conclusion

In this contribution, we present our views on remaining issues on NN-DC power control with the following proposals and text proposals.

Proposal 1:

- Agree the following text proposal to clarify the TDD UL/DL configuration for semi-static power sharing.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Subclauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Subclauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

Proposal 2:

- Agree the following text proposal for PMCGactuali1 and PSCGactuali2 definition in dynamic power sharing part to keep up with other parts.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

TS 38.213

7.6.2 NR-DC

…

If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided NR-DC-PC-mode = Dynamic

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the linear values of UE transmission power in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

Proposal 3:

- Agree the following text proposal to keep RRC parameters consistently between the endorsed MR-DC running CR of TS 38.331 and TS 38.213.

-------------------------------------------------------Start of text proposal-------------------------------------------------------

…

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2-r16 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2-r16 and with an inter-CG power sharing mode by nrdc-PCmode-FR1-r16 and/or bynrdc-PCmode-FR2-r16. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode1 or NR-DC-PC-mode = semi-static-mode2, the UE does not expect PMCG and PSCG to be configured such that PMCG+PSCG>PTotalNR-DC, where PMCG is the linear value of PMCG, PSCG is the linear value of PSCG, and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3].

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG overlapping with slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a power for the transmission on MCG or the SCG overlapping with slot i2, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

If the UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = dynamic

-	if UE transmission(s) in slot i1 of the MCG overlap in time with UE transmission(s) in slot i2 of the SCG and if PMCGactuali1+PSCGactuali2>PTotalNR-DC in any portion of slot i2 of the SCG, the UE reduces transmission power in any portion of slot i2 of the SCG so that PMCGactuali1+PSCGactuali2≤PTotalNR-DC in all portions of slot i2, where PMCGactuali1 and PSCGactuali2 are the UE transmission powers in slot i1 of the MCG and in slot i2 of the SCG, respectively, that the UE determines according to Clauses 7.1 through 7.5 using PMCG and PSCG, respectively, as the maximum transmission powers on the MCG and the SCG and PTotalNR-DC is the linear value of a configured maximum transmission power for NR-DC operation in FR1 as defined in [8-3, TS 38.101-3]

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a transmission power in slot i1 of the MCG or in slot i2 of the SCG as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

If a UE

-	is provided nrdc-PCmode-FR1-r16 or nrdc-PCmode-FR2-r16 = dynamic, and

-	indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that

-	are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, and

-	overlap with the transmission occasion on the SCG

…

--------------------------------------------------------End of text proposal-------------------------------------------------------

- References

- RAN1#100 e-meeting chairman’s notes