3GPP TSG RAN WG1 #100bis                                                     R1- 2001825

e-Meeting, April 20th – 30th, 2020

Source: MediaTek Inc.

Title: maintenance of low PAPR RS

Agenda Item: 7.2.6.5

Document for:  Discussion, Decision

# Introduction

In this contribution, we discuss some remaining issues of low PAPR RS based on V16.1.0 specifications [1][2].

# DCI 0_1 DMRS sequence initialization bit width

Sequence initialization for pi/2 BPSK DMRS according to DCI 0_1 is specified in 38.211 while it is missing in 38.212

In 38.211:

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by

cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231

where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in  [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSKscramblingID0 and pi2BPSKscramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by a PUSCH transmission with a configured grant;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSKscramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NIDnSCID=NIDcell otherwise;

-	otherwise, ru,vα,δ(n) is given by clause 5.2.2.

In 38.212

##### 7.3.1.1.2	Format 0_1

----------------< Unrelated parts are omitted > ----------------------

- DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

----------------< Unrelated parts are omitted > ----------------------

Text Proposal-1: for Section 7.3.1.1.2 of 38.212:

##### 7.3.1.1.2	Format 0_1

----------------< Unrelated parts are omitted > ----------------------

- DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled or transform precoder is enabled with DMRSuplinkTransformPrecoding-r16 configured; 0 bit otherwise.

----------------< Unrelated parts are omitted > ----------------------

# Cyclic shift value for PUCCH, format 3 and 4 with pi/2-BPSK DMRS

One remaining issue in previous e-Meeting is for low PAPR pi/2-BPSK PUCCH, format 3 and 4, whether, and if so how, the DMRS depends on the cyclic shift value α. From our view, R16 CGS sequences are designed to have better auto/cross-correlation only within a certain delay window. In particular, a window [-2,-1,1,2] for sequence’s circular shifts is examined for length-12 sequence. Therefore, if circular shift can be randomized in each cell, the cross-correlation between two shifted versions of DMRS base sequences is not guaranteed.  We propose the following text change in 38.211.

Text Proposal-2: for Section 6.4.1.3.3.1 of 38.211:

##### 6.4.1.3.3	Demodulation reference signal for PUCCH formats 3 and 4

###### 6.4.1.3.3.1	Sequence generation

The reference-signal sequence rlm shall be generated according to

where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration:

-	if the higher-layer parameter dmrsUplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α=0 depends on the cyclic shift hopping in clause 6.3.2.2.2.

-	otherwise, ru,vα,δ(m) is given by clause 6.3.2.2. The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with  for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4.

# References

[1] 3GPP TS 38.211 V16.1.0

[2] 3GPP TS 38.212 V16.1.0