3GPP TSG RAN WG1 Meeting #100bis 	R1-2001833

e-Meeting, April 20th – 30th, 2020

Agenda Item: 7.2.11.10

Source: MediaTek Inc.

Title: 	Views on Rel-16 UE features for MR-DC/CA

Document for: Discussion

- Introduction

This paper provides our views on Rel-16 UE features for MR-DC/CA. Detailed proposals are described in the following sections.

- Views on UE features for MR-DC/CA

For the current RAN1 UE feature table of MR-DC/CA, we think the alternative 18-1/1a/1b structure marked in [ ] for intra-FR DC UL power control is more clear. Hence, we support the alternative ones.

Proposal 1: Adopt the alternative 18-1/1a/1b structure marked in [ ] for intra-FR DC UL power control in current RAN1 UE feature table.

For FG 18-4, two Cases of SCell dormancy indication are supported when the indication is sent within DRX Active Time.

- Case 1 SCell dormancy indication:

  - DCI format 0_1 and 1_1 with additional bit field “SCell dormancy indication” are used.

  - Case 1 DCI can still schedule PDSCH/PUSCH, and the timeline for HARQ-ACK information feedback is the same as N1 in Rel-15.

  - 1 bit in “SCell dormancy indication” indicates SCell dormancy/non-dormancy for a group of SCells.

- Case 2 SCell dormancy indication:

  - DCI format 1_1 with some repurposed bit fields is used.

  - Case 2 DCI cannot schedule PDSCH but UE still needs to report HARQ-ACK information. Its HARQ-ACK report timeline is tighter than in Case 1. (working assumption: timeline is the same as in HARQ-ACK information report for SPS PDSCH release).

  - 1 bit of SCell dormancy indication indicates SCell dormancy/non-dormancy for a SCell.

Because of many differences between Case 1 and Case 2 SCell dormancy indication, it is more reasonable to have separated FGs for them. We suggest the following revisions:

Proposal 2:

- FG 18-4: SCell dormancy indication with data scheduling within active time

  - Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 scheduling PUSCH/PDSCH

- Add FG 18-4b: SCell dormancy indication without data scheduling within active time

  - Support for SCell dormancy indication sent within the active time on PCell via DCI format 1_1 without PDSCH scheduling

For FG 18-5: Cross-carrier scheduling with different SCS, it is sufficient to reuse the Rel. 16 FG 3-5b PDCCH monitoring to have multiple DCIs in one slot of the scheduling cell with lower SCS than the scheduled cell. This has the benefit of avoiding introducing additional impact to the spec (e.g., new design for HARQ-ACK codebook). We think the value of X is not needed. Besices, it is recommended to add ‘Per band’ to support the feature for CA within certain bands or not.

Proposal 3: RAN1 agree to reuse the Rel. 16 FG 3-5b PDCCH monitoring to have multiple DCIs in one slot of the scheduling cell with lower SCS than the scheduled cell. Delete the descriptions related to value X. Add ‘Per band’ to FG 18-5 to support the feature for CA within certain bands or not.

For FG [18-5a]: Default QCL assumption for cross-carrier scheduling with same SCS, we support to keep this capability for better UE implementation flexibility.

Proposal 4: Keep “18-5a Default QCL assumption for cross-carrier scheduling with same SCS” in current RAN1 UE feature table.

For FG [18-6a]: Default QCL assumption for cross-carrier A-CSI-RS triggering, we support to keep this capability for better UE implementation flexibility.

Proposal 5: Keep “18-6a Default QCL assumption for cross-carrier A-CSI-RS triggering” in current RAN1 UE feature table.

For FG 18-8: HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group, since this is a new Rel-16 feature, it should be optional with capability signaling.

Proposal 6: FG “18-8 HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group” should be optional with capability signaling.

# Proposed changes on current RAN1 UE features table

# MR-DC/CA enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18-1 | Synchronous NR-DC operation | NR-DC operation with synchronization between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - semi-static power-sharing mode 2 - dynamic power-sharing and the value of T_offset |  |  |  |  | Per band combination | N/A | N/A |  | Absence means synchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | 18-1a | Non-SFN synchronous NR-DC operation | Operation with non-zero slot offset between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - semi-static power-sharing mode 2 - dynamic power-sharing and the value of T_offset | 18-1 |  |  |  | Per band combination | N/A | N/A |  | Absence means non-SFN synchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | 18-1b | Asynchronous NR-DC operation | Operation with no slot alignment between MCG and SCG Power-sharing mode within the frequency range - semi-static power-sharing mode 1 - dynamic power-sharing and the value of T_offset | 18-1 |  |  |  | Per band combination | N/A | N/A |  | Absence means asynchronous NR-DC operation for the given band combination is not supported. 1) {Supported} 2) {not supported, supported} 3) {not supported, short, long} | Optional with capability signalling |
|  | 18-2 | Single UL TX operation for TDD PCell in intra-band EN-DC | TDM restriction to LTE TDD PCell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | EN-DC |  | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell 3) {not supported, supported} | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for singleUL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ sub-frame offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern (for type 1 UE) [5) if UE indicates that it does not support simultaneous UL transmissions as defined in TS 38.101-3 [4] using singleUL-Transmission, NR (SCG) UL transmission is dropped when an overlapping LTE (MCG) UL transmission is present (for type 1 UE).”] | 6-13 |  | N/A |  | Per band combination | Applicable to in FDD-LTE TDD-NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern | Optional with capability signalling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (for type 1 UE) 3) HARQ subframe offset 4) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe of the TDM pattern | EN-DC |  | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern | 18-2, 18-2a |  |  |  | Per UE |  |  |  |  | Optional with capability signaling |
|  | 18-4 | SCell dormancy indication with data scheduling within active time | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 scheduling PUSCH/PDSCH |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 18-4a | SCell dormancy outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 19-1 |  |  |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 18-4b | SCell dormancy indication without data scheduling within active time | Support for SCell dormancy indication sent within the active time on PCell via DCI format 1_1 without PDSCH scheduling |  |  | N/A |  | Per UE | No | No |  |  | Optional with capability signaling |
|  | 18-5 | Cross-carrier scheduling with different SCS | 1) The UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different [2 Default QCL assumption for cross-carrier scheduling with different SCS] |  |  | N/A |  | Per band combination | No | No |  | crossCarrierScheduling-OtherSCS 1) {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} | Optional with capability signalling |
|  | 18-5a | Default QCL assumption for cross-carrier scheduling with same SCS | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier scheduling with same SCS. | 6-10 |  |  |  | Per band combination |  |  |  |  | Optional with capability signalling |
|  | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS | 2-33 |  | N/A |  | Per band combination | No | No |  | 1) {PDCCH cell of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | 18-6a | Default QCL assumption for cross-carrier A-CSI-RS triggering | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier A-CSI-RS triggering. |  |  |  |  | Per band combination |  |  |  |  | Optional with capability signalling |
|  | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA |  |  | N/A |  | Per band combination | No | No |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 |  | N/A |  | Per UE | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | Optional with capability signalling |