3GPP TSG RAN WG1 Meeting #100bis-e                                                                R1-2001893

e-Meeting, April 20th – 30th, 2020

Agenda item:			7.2.3.4

Title:	FL summary on remaining issues in IAB case-1 timing

Source:					Moderator (ZTE)

Document for:     Discussion and Decision

# Introduction

This paper provides a summary of remaining issues identified for IAB case-1 timing, based on contributions submitted to RAN1 #100-bis-e, aiming to have an agreeable set of critical issues that are to be solved in RAN1 #100-bis-e discussion.

Observations and proposals in this paper are primarily related to the following WID objectives:

  - Specification of mechanism to support the “case-1” OTA timing alignment.

The current RAN1 specifications in TS 38.213 for IAB case-1 timing is copied below for quick reference.

If an IAB-node is provided a value Tdelta from a serving cell, the IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a “UE” in Subclause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time.

# Summary from the company contributions

There are 9 company contributions on IAB case-1 timing (7 submitted to AI 7.2.3.4 and 2 submitted to AI 5, discussing two issues:

- Issue #1: The mapping between T_delta index indicated in Timing Delta MAC-CE and T_delta value applied in DL-Tx timing formula;

- Issue #2: Whether NTA rounding allowed in Rel-15 UE TA adjustment is applicable to IAB-MT.

## Issue #1 (T_delta mapping between index in MAC-CE and value in DL-Tx timing formula)

Companies’ views are summarized in table below.

| Company (TDoc #) | Views, observations and proposals |
| --- | --- |
| Huawei, HiSi (R1-2001527) | Observation 1: A formula can be defined for T_delta with an additional table to capture various granularities and min/max values. Observation 2: A simplified formula of T_delta can be defined by making some modifications on the ranges provided by RAN4. No additional table is needed in this case. Proposal: Considering either TP for option 1 or option 2 when capturing T_delta in TS 38.213. Option 1 TP: ==============<Unchanged text is omitted>========================= If an IAB-node is provided an index ndelta from serving cell by the MAC CE as described in [11, TS38.321], the IAB node determines the value T_delta as follows: Tdelta=-NTA,offset/2+min NT_delta,min+ nT_delta ∙GT_delta, NT_delta,maxTc where NTA,offset is obtained as for a "UE" in Clause 4.2. NT_delta,max, NT_delta,min, and GT_delta are determined according to Table 14.2. Table 14.2. Parameter table for determining T_delta FR; μ; NT_delta,max; NT_delta,min; GT_delta. FR1; 0; 6256; -70528; 64. 1; 6128; -35328; 64. 2; 6032; -17664; 64. FR2; 2; 6032; -17664; 32. 3; 6032; -8816; 32. TdeltaThe IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time. ==================<Unchanged text is omitted>=========================== Option 2 TP: ================<Unchanged text is omitted>======================== If an IAB-node is provided an index ndelta from serving cell by the MAC CE as described in [11, TS38.321], the IAB node determines the value T_delta as: Tdelta=(-NTA,offset/2+min(-70528/2μ+nT_delta∙32 , 6256))Tc The IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time. =================<Unchanged text is omitted>============================ |
| Nokia, NSB (R1-2001792) | Proposal 1: RAN1 to agree with the RAN2 conclusion to specify mapping of T_delta values in RAN1 specification TS 38.213 aligned with the way how timing advance (TA) has been specified. Proposal 2a: A single value range for all SCSs is used for the mapping to MAC-CE information. Proposal 2b: Parent node shall be responsible for the signalled T_delta to be within the range valid for the SCS in concern. Proposal 3: Band indication (FR1/FR2) relevant for T_delta is implicitly known by the IAB node. Proposal 4: T_delta mapping tables are defined separately for FR1 and FR2. Proposal 5: RAN1 to confirm the usage of 11-bit information in MAC-CE for T_delta signalling. Proposal 6: RAN1 to agree on T_delta mapping to Tdelta according to Table 2 and 3 for FR1 and FR2, respectively. Corresponding mapping for FR1: NTdelta [Tc] = -NTA, offset/2 + Min_value_FR1 + 64*Tdelta, where - NTA, offset is the value used in the serving cell - Tdelta is the index value for MAC-CE information - Min_value_FR1 = -70528 Table 2. Mapping of T_delta values to indices, FR1 SCS [kHz]; Min value per SCS [Tc]; Max value per SCS [Tc]; Tdelta (index) range. 15; -70528; 6256; [0,1199]. 30; -35328; 6128; [550,1197]. 60; -17664; 6032; [826,1196]. Corresponding mapping for FR2: NTdelta [Tc] = -NTA, offset/2 + Min_value_FR2 + 32*Tdelta, where - NTA, offset is the value used in the serving cell - Tdelta is the index value for MAC-CE information - Min_value_FR2 = -17664 Table 3. Mapping of T_delta values to indices, FR2 SCS [kHz]; Min value per SCS [Tc]; Max value per SCS [Tc]; Tdelta (index) range. 60; -17664; 6032; [0,740]. 120; -8816; 6032; [277,740]. Proposal 7: The equation to calculate the DU timing adjustment is re-formulated to be (NTA + NTA,offset)*Tc/2+ NTdelta*Tc. |
| ZTE, Sanechips (R1-2001883) | Proposal 1: An index value ITdelta in Timing Delta MAC-CE, where 0≤ITdelta≤Imax, represents the T_delta value x satisfying Td,min+ITdelta∙∆≤x+NTA,offset2<Td,min+ITdelta+1∙∆ in unit of Tc, where Imax, Td,min and ∆ are given by the following. - Option 1 (same value range and same granularity for all SCS+FR combinations, requiring 12 bits) Imax=2399, Td,min=-70528, ∆=32 - Option 2 (both value range and granularity depend on FR but not on SCS, requiring 11 bits) FR for serving cell indicating T_delta; Imax; Td,min; ∆. FR1; 1199; -70528; 64. FR2; 740; -17664; 32. - Option 3 (granularity depends on value/index range per FR, requiring 11 bits) value range of x+NTA,offset2; ITdelta range; Imax; Td,min; ∆. FR1 range only: [-70528, -17664]; 0≤ITdelta≤825; 1573; -70528; 64. FR1/FR2 overlapping range: [-17664, 6256]; 826≤ITdelta≤1573; -44096; 32. Proposal 2: The parent node is not expected to indicated T_delta if the measured T_delta exceeds the range corresponding to 0≤ITdelta≤Imax. This may not be captured in specification. |
| LG Electronics (R1-2001953) (R1-2002187, submitted to AI 5) | Observation 1: 11 bits for the T_delta field (containing the index value of T_delta) are sufficient. Text proposal: # 14 Integrated access-backhaul operation -------------------------------------------------------- Omitted ----------------------------------------------------- If an IAB-node is provided a value Tdelta from a serving cell, the IAB-node may assume that NTA+NTA,offset⋅Tc/2+Tdelta NTA2+Tdelta_min+Tdelta_gran⋅Tdelta⋅Tc is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc2+Tdelta>0NTA2+Tdelta_min+Tdelta_gran⋅Tdelta⋅Tc>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell, Tdelta_min and Tdelta_gran are listed in Table 14-x, and Tdelta is given by Timing Delta command [11, 38.321]. The IAB-node may use the time difference to determine a DU transmission time. Table 14-x. Granularity and value range of Tdelta Frequency range (FR); Tdelta_gran [Tc]; SCS [kHz]; Tdelta_min [Tc]; Index range of Tdelta. FR1; 64; 15; -70528; 0,…,1200. 30; -35328; 0, … ,648. 60; -17664; 0, … ,371. FR2; 32; 60; -17664; 0, … ,741. 120; -8816; 0, … ,464. |
| NTT DOCOMO (R1-2002436) | Proposal 1: The mapping between T_delta index and actual value of T_delta should be independent for each FR and SCS. Proposal 2: Adapt text proposal for TS 38.213 which reflects proposal 1. == Start == ### 14 Integrated access-backhaul operation <omitted text> If an IAB-node is provided a Tdelta_index from a serving cell, the IAB-node derives a value Tdelta according to Tdelta_min+Tdelta_index⋅Tdelta_granularity, where Tdelta_min and Tdelta_granularity are selected based on FR and SCS of the serving cell as in Table 14.x, and may assume that NTA+NTA,offset⋅Tc/2+Tdelta is a time difference between a DU transmission of a signal from the serving cell and a reception of the signal by the IAB-node MT when NTA+NTA,offset⋅Tc/2+Tdelta>0, where NTA and NTA,offset are obtained as for a "UE" in Clause 4.2 for the TAG containing the serving cell. The IAB-node may use the time difference to determine a DU transmission time. Table 14.x: Tdelta_min and Tdelta_granularity for each FR and SCS FR of the serving cell; SCS of the serving cell (kHz); Tdelta_min (Tc); Tdelta_granularity (Tc). FR1; 15; - /2 - 70528; 64 Tc. 30; - /2 - 35328; 64 Tc. 60; - /2 - 17664; 64 Tc. FR2; 60; - /2 - 17664; 32 Tc. 120; - /2 - 8816; 32 Tc. <omitted text> == End == Proposal 3: Accordingly, we propose the following reply to RAN2. - RAN1 will capture the mapping between T_delta index and actual value of T_delta in the specification . - The 11-bit size of the T_delta field in Timing Delta MAC CE is adequate for the indication. |
| Qualcomm (R1-2002536) | Observation 1: The range for the index value of T_delta signalled via MAC-CE is adequate to meet the range requirements defined by RAN4 in [2]. Proposal 1: Adopt the following TP for section 14 of 38.213: ## <Unchanged text is omitted> An IAB-node can be provided with a value Tdelta obtained from the timing delta Tdelta [11, TS 38.321] by index values of Tdelta = 0, 1, 2, …, 1199, as Tdelta = min( Tdelta-min + Tdelta·Tdelta-step , Tdelta-max) where Tdelta-step is 64·Tc for FR1 and 32·Tc for FR2, Tdelta-min and Tdelta-max are provided in Table 14.2 as a function of the SCS of the PDSCH providing Tdelta, and NTA, offset is obtained as per Clause 4.2. Table 14.2: Tdelta-min and Tdelta-max values SCS; Tdelta-min; Tdelta-max. 15 KHz; -70528 – NTA, offset / 2; – NTA, offset / 2 + 6256. 30 KHz; -35328 – NTA, offset / 2; – NTA, offset / 2 + 6128. 60 KHz; -17664 – NTA, offset / 2; – NTA, offset / 2 + 6032. 120 KHz; -8816 – NTA, offset / 2; – NTA, offset / 2 + 6032. <Unchanged text is omitted> |
| Ericsson (R1-2002651) | - Observation 1: A granularity of 32Tc for T_delta signaling is not required in FR1 and therefore not for an SCS of 15kHz. - Observation 2: 11 bits are sufficient to represent all index values for T_delta, across FR1 and FR2 and any SCS, and is required for an SCS of 15kHz. - Proposal 1: RAN1 to inform RAN2 that it is adequate that the size of the T_delta field (containing the index value of T_delta in the MAC CE) is specified to 11 bits. |
| Samsung (R1-2002101, submitted to AI 5) | Proposal 1: A mapping between the T_delta index and the actual value of T_delta is captured in 38.213. Proposal 2: 12 bits is required for the T_delta signaling. |

All contributions listed above propose the T_delta mapping solutions that fits in RAN1 specification. There is no company raising concerns for solving the T_delta mapping in RAN1.

Summary observation #1: All interested companies show no concern for implementing the T_delta mapping in RAN1.

Regarding to whether 11-bits are sufficient to indicate T_delta, even though not all solutions see such sufficiency, almost every contribution (except 2101) provides at least one solution for which 11-bits are sufficient.

Summary observation #2: Though still depending on the detailed mapping solution, 11-bits can be sufficient from majority’s view.

As for the T_delta mapping solution, the key issue is the dependency of the T_delta mapping upon range and granularity, which is tightly related to whether and how to do the IAB-and-parent sync-up on SCS and FR. There are four categories for the proposed solutions.

  - T_delta mapping functions differ on different <range/SCS, granularity/FR> combinations (there are total 5 combinations).

Solutions in this category: 1527 (Option 1), 1792, 1953/2187, 2436, 2536.

Among these TDocs,

- {1792, 1953,2436} formulate T_delta = T_delta_min + index*granularity;

- {1527, 2536} formulate T_delta = min{T_delta_min + index*granularity, T_delta_max}

- {1792} applies the common T_delta_min in each FR (i.e., SCS-independent), while the mapping function domain (range of T_delta index) may or may not start from 0 depending on SCS (a behavior of parent node).

- {1527, 1953/2187, 2436, 2536} use SCS-dependent T_delta_min so that the index counts from 0.

Meanwhile, just two contributions clarify how to keep the IAB node and its parent applying the same FR and the same SCS, including:

- TDoc 2436: FR and SCS being applied are the FR and the SCS for the serving cell.

- TDoc 2536: SCS being applied is the SCS of PDSCH that carries the Timing Delta MAC-CE.

  - T_delta mapping functions differ on granularity/FR but not on range/SCS.

Solutions in this category: 1883 (Option 2), where FR being applied is the FR for the serving cell indicating the T_delta.

  - T_delta mapping functions differ on range/SCS but not on granularity/FR.

Solutions in this category: 1527 (Option 2).

  - No T_delta mapping selection based on either range/SCS or granularity/FR.

Solutions in this category: 1883 (Option 1 - needs 12 bits, Option 3 - needs 11 bits ), 2101 (largest range with finest granularity – needs 12 bits).

Summary observation #3: Among the four T_delta mapping solution categories from the contributions, the one differentiating all five <FR, SCS> combinations is proposed mostly often; however, there is no majority view shown on how to ensure the IAB node and its parent to apply the same FR and the same SCS to the mapping calculation.

## Issue #2 (applicability of NTA rounding)

Companies’ views are summarized in table below.

| Company (TDoc #) | Views, observations and proposals |
| --- | --- |
| ZTE, Sanechips (R1-2001883) | Proposal 3: IAB-node MT shall not perform NTA rounding on the uplink associated with the serving cell that indicates T_delta. - RAN1 to decide whether to capture this restriction in specification or just in RAN1 meeting minutes. |

# Preparation phase discussion

FL provides the following proposals for preparation phase discussion.

FL proposal 1: RAN1 spends one email thread budget on IAB case-1 timing.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE, Sanechips | Given the T_delta mapping in RAN1 spec is requested by RAN2, it has to be discussed in RAN1 #100bis-e. Meanwhile, the related technical issues are not as easy as being able to solve in preparation phase. So one official email discussion thread is necessary. |
| Qualcomm | Agree with FL proposal. |
| Intel | Agree with FL proposal. |
| DOCOMO | Agree. |
| Samsung | Agree with the FL proposal. |
| LG | Agree with the FL proposal. |
| Huawei | Agree with the FL proposal. |
| Ericsson | Agree |

FL proposal 2: IAB case-1 timing discussion thread includes following discussion points.

- #1: Whether/how the T_delta mapping depends on FR (granularity) and/or SCS (range), including how to make IAB node and its parent to use the same FR and the same SCS in the mapping.

- #2: How to deal with Rel-15 UE behaviour of NTA rounding in the context of IAB case-1 timing.

- #3: To finalize new TP for RAN1 spec and LS to RAN2/RAN4.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | We are ok to discuss Issue #1 and #2. Issue #3 is something that is anyways required based on the agreements and not required to mention as an issue. |
| Qualcomm | Agree with FL proposal. |
| Intel | Agree with FL proposal. |
| DOCOMO | Agree. |
| Samsung | Agree with the FL proposal. |
| LG | Agree with the FL proposal. |
| Huawei | Agree with the FL proposal. |
| Ericsson | Agree |

FL suggestion: In preparation phase, companies are encouraged to share their technical views for the following questions. No decision will be made in preparation phase discussion.

- Q3-1-1: Whether the T_delta granularity applied in the T_delta mapping should be FR-dependent or it can be 32 for both FR1 and FR2?

- Q3-1-2: For FR-dependent granularity, how to make IAB node and its parent to use the same FR?

- Q3-2-1: Whether T_delta range defined by RAN4 should still be needed in the spec once the number of signalling bits for T_delta is determined?

- Q3-2-2: In case of specifying RAN4-defined range which is SCS-dependent, how to make IAB node and its parent to use the same SCS?

- Q3-2-3: In case of specifying RAN4-defined range, should the range be reflected by the mapping function domain (the range of T_delta index) or the mapping function range (the range of mapped T_delta value)?

- Q3-2-4: In case the RAN4-defined range is reflected by the range of T_delta index, should this index range restriction be implemented in RAN1 spec or RAN2 spec (in either way the T_delta mapping is in RAN1 spec)?

- Q3-3-1: Views on NTA rounding for IAB-MT.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE, Sanechips | For Q3-1-1: From RAN1 perspective, the case-1 timing mechanism does not care what granularity has to be used in FR1 and what has to be used in FR2. For quantization purpose, even if the T_delta measurement is quantized with granularity of 64Tc, the indication of quantization result can still use a granularity of 32Tc. In our view, the granularity being used in T_delta mapping is a signaling design output. The key issue from RAN1 perspective is whether to deserve the cost paid to have FR-dependent granularity. For Q3-1-2: Given the carriers owned by one serving cell does not occupy more than one FR as of Rel-16, the FR used to interpret T_delta mapping granularity can be the FR of the serving cell indicating the T_delta. Using one explicit bit in MAC-CE to indicate FR1/FR2 does not provide any advantage, given doing so would result in the same signaling overhead as using granularity of 32Tc for both FR1 and FR2. For Q3-2-1: Similar to T_delta granularity, from RAN1 perspective, the case-1 timing mechanism does not care how large or small T_delta can be. What RAN1 sees is a fixed function relationship: func(TA, T_delta, one-way-propagation-delay)=0. If RAN1 uses one-to-one mapping from T_delta index to T_delta value, then from case-1 timing perspective, the IAB node just needs to calculate the one-way delay from above function under the direction of parent node who indicates one index of TA command and another index of T_delta. Both TA and T_delta can be seen to have symmetric contribution/interaction to the delay calculation; but under TA accumulation, the spec does not restrict the value range for TA. So a specific range limitation on T_delta (except the limitation by number of bits) seems unnecessary as well and may even introduce troublesome interpretation to the range of TA. Secondly, most of companies prefer to have a one-to-one mapping function from T_delta index to T_delta value. Here the mapped T_delta values just construct a sub-set of RAN4-defined value range. In other words, the RAN4-defined value range is the T_delta value range observed at the parent node side, but the one-to-one mapping focuses on IAB-node interpretation only. So we would rather interpret the RAN4-defined T_delta value range as a requirement to RAN2 signaling capability (to derive 11 or 12 bits). Once the number of bits is determined, whatever those bits can indicate should be acceptable by RAN1 spec. For Q3-2-2: RAN2 already confirms SCS is not explicitly indicated in MAC-CE. For the implicit indication, one cell can observe different SCS’s being used at the same time; what’s more, case-1 timing mechanism does not require the IAB node to know when the parent node measures/quantizes T_delta. All these facts would suggest more RAN1 discussions are needed on how to sync up IAB node and the parent node to use the same SCS. Unfortunately RAN4 LS did not clarify whether the T_delta range should be bound to certain channel/signal when it said the range is SCS-dependent. If SCS can be bound to any specific channel/signal as long as the IAB node and the parent node can have the same understanding, it probably means the range can be SCS-independent. For Q3-2-3: We think the range restriction (if needed to be specified) should be on T_delta index, not on mapped T_delta value. - Reason 1: For a one-to-one mapping from T_delta index to T_delta value, the T_delta values are discrete anyway, which means T_delta_max might not be reached. - Reason 2: Allowing the index being unlimited while forcing the mapped value to be limited would result in more than one index to be mapped to the same value, which would introduce incorrect case-1 timing calculation. - Reason 3: With the range restriction on T_delta index, there seems no need to have another restriction (with the same restriction effect) on mapped value. - Reason 4: RAN2 already endorsed certain range restriction to be applied to T_delta index (i.e., from 0 to 1199), rather than to the mapped value. For Q3-2-4: In our view, the restriction on index range (if specified) should stay where the index is defined, i.e., in RAN2 spec. In fact, as mentioned above, RAN2 already endorsed a TP to include certain index range. It would look strange to have a different range for the same index in RAN1 spec. Fr Q3-3-1, we think the simplest way to avoid UL-Tx timing ambiguity is to disallow NTA rounding for IAB-MT. Note that the current TS38.213 text of “The applicable value for an UL BWP with lower SCS may be rounded to align …” does not prevent UE from not performing NTA rounding. |
| Qualcomm | Our current assumption is that the T_delta ranges and granularity provided by RAN4 apply, noting that it was RAN1 that asked RAN4 to investigate and to provide guidance on this matter. |
| Intel | - Q3-1-1: Whether the T_delta granularity applied in the T_delta mapping should be FR-dependent or it can be 32 for both FR1 and FR2? We prefer the FR-dependent approach, i.e. 64Tc for FR1 and 32Tc for FR2 as defined in RAN4. - Q3-1-2: For FR-dependent granularity, how to make IAB node and its parent to use the same FR? One approach is to let parent IAB node transmit T_delta in the same FR as the FR that the T_delta will be applied in IAB node. Another approach is to introduce a FR indication using the reserved bits. - Q3-2-1: Whether T_delta range defined by RAN4 should still be needed in the spec once the number of signalling bits for T_delta is determined? Since T_delta range is different for different SCS, it can be remained depending on how the number of signalling bits for T_delta is defined. - Q3-2-2: In case of specifying RAN4-defined range which is SCS-dependent, how to make IAB node and its parent to use the same SCS? One approach is to let parent IAB node transmit T_delta using the same SCS as that of the T_delta will be applied in IAB node. Another approach is to introduce a SCS indication using the reserved bits. - Q3-2-3: In case of specifying RAN4-defined range, should the range be reflected by the mapping function domain (the range of T_delta index) or the mapping function range (the range of mapped T_delta value)? Either way since one can be calculated based on the other. Slightly prefer the range of T_delta index. - Q3-2-4: In case the RAN4-defined range is reflected by the range of T_delta index, should this index range restriction be implemented in RAN1 spec or RAN2 spec (in either way the T_delta mapping is in RAN1 spec)? RAN1 spec. - Q3-3-1: Views on NTA rounding for IAB-MT. Not sure. |
| DOCOMO | Q3-1-1 : We support T_delta mapping for FR/SCS dependent Q3-1-2 : IAB-node may select FR/SCS based on FR and SCS of the serving cell Q3-2-1 : The range is not necessary in the spec. Maximum or minimum value is captured in the spec to derive T_delta. Q3-2-2 : IAB-node may select FR/SCS based on FR and SCS of the serving cell |
| Samsung | Q3-1-1: Our preference is to have 32 for both FR1 and FR2. But, we are open to consider the FR-dependent as RAN4 LS suggested. Q3-2-1: T_delta range can be captured or max./min.value in the range per SCS can be captured. Q3-2-2: Implicit mapping (e.g. a SCS same as a SCS for active UL BWP) can be used. Q3-2-3: Not sure there is a difference. Either way is fine. Q3-2-4: RAN1 spec can capture it. Q3-3-1: Not sure for now. But, open to further discuss it. |
| LG | Q3-1-1: We prefer the FR-dependent approach. Q3-1-2: IAB-node may select based on FR of the serving cell. Explicit signaling of FR is not preferred. Q3-2-1: Maximum or minimum value of range can be captured in the spec to derive T_delta. Q3-2-2: IAB-node may select based on SCS of the serving cell. Q3-2-3: Either way is fine. Q3-2-4: In RAN1 spec. Q3-3-1: Not sure. |
| Huawei | Q3-1-1: The key deciding factor is whether there is any impact or specific considerations on the granularity in RAN4. Our understanding is there isn’t any. Hence we don’t need to stick with these any restrictions when design the signaling of T_delta. Q3-1-2: The FR of the PDSCH carrying T_delta is known to both IAB node and its parent hence can be used. Q3-2-1: Our understanding of the RAN4 discussion is that there is no strong reason to keep the exact range as long as it can be covered by the signaling. Q3-2-2: The SCS of the PDSCH carrying T_delta is known to both IAB node and its parent hence can be used. Q3-2-3: Both options are possible and there is no clear winner. Q3-2-4: Originally we thought the whole signaling design can be done in RAN2. Since RAN2 sent the LS to RAN1, we prefer to solve it in RAN1 if possible. Q3-3-1: Not sure. |
| Ericsson | Q3-1-1: Eventually, this should be part of the discussion phase since these are technical questions. Granularity should be FR dependent since this is what RAN4 provided and it allows for a cleaner specification. Q3-1-2: This relates to Q3-2-2. R1-2002436 and R1-2002536 provide useful ideas (item a. in Summary observation #2) to allow the IAB node and the IAB parent to assume the same FR and SCS. Q3-2-1: It may still be needed for implementation, e.g., supported min and max range. Q3-2-2: See Q3-1-2. Q3-2-3: RAN2 does not adequately specify range restrictions on T_delta index as a function of SCS. We prefer to limit the range in the mapping through the mapping function, like in Option 1 of R1-2001527. Q3-2-4: Additional RAN2 involvement for the sake of limiting the T_delta range should be avoided. Q3-3-1: Any change of NTA for the sake of rounding or any other reason would affect T_delta from the parent node and this would compensate for the changed NTA. Despite, rounding is an MT internal operation; it can compensate for that, if needed. |

Updated FL proposal for preparation phase conclusion (stable & final):

RAN1 spends one email thread budget to discuss following:

- #1: Whether/how the T_delta mapping depends on FR (granularity) and/or SCS (range), including how to make IAB node and its parent to use the same FR and the same SCS in the mapping.

  - Initial timeline: 4/20 - 4/24 technical discussion; 4/21 - 4/23 reply LS drafting; 4/24- 4/28 TP for RAN1 spec (if needed)

- #2: How to deal with Rel-15 UE behavior of NTA rounding in the context of IAB case-1 timing.

  - initial timeline: 4/23 - 4/28 technical discussion; 4/29 - 4/30 TP for RAN1 spec (if needed)

# Annex A. RAN1 agreements in earlier meetings (WI phase only)

RAN1 #100e

Conclusion: For the two issues raised in RAN1 #100e contributions on case-1 timing,

- For the missing of explicit binding/association between T_delta and NTA in current RAN1 specification,

  - A simple modification, as an example, of "where NTA and NTA,offset are obtained as for a “UE” in Subclause 4.2 for the TAG containing the serving cell" in 38.213 can explicitly fulfill the binding/association. There is no need to have TAG-ID in MAC-CE.

  - It is up to 38.213 editor whether to implement explicit binding/association between T_delta and NTA, with the following group common understanding reported to the editor

"The one-way propagation delay estimation defined in section 14 assumes that NTA and NTA,offset   associate with the same TAG as containing the serving cell that provides T_delta"

  - This issue is not further brought to official RAN1 #100e discussion.

    - If companies still believe the RAN1 spec needs correction after editor’s decision if applied, consider CR in RAN1 April meeting.

- For proposal of adding SCS to T_delta MAC-CE, the majority of participating companies do not think this SCS information is needed from RAN1 perspective. The issue is not brought to official RAN1 #100e discussion.

- Additional discussion relating to the T_delta MAC-CE is not required from RAN1 at this stage. Meanwhile, the group understand the RAN2 discussion may result in RAN2-RAN1 communication during RAN1 #100e.

RAN1 #99

Agreements:

To be captured in the specification:

- The proposal in the paragraph immediately after the “Conclusion from Wednesday offline session” in R1-1913316 is agreed

Agreements:

Adding in the specification the following:

The timing difference may be used by an IAB-node in the determination of its DU transmission timing.

Conclusion:

- In the CR stage, check further whether or not there is a need to further clarify in RAN1 spec about the usage of the timing difference based on RAN4 specifications

RAN1 #98bis

Agreements:

- From RAN1 perspective, Rel-16 NR IAB does not introduce signalling of accuracy/quality measure for IAB node DL-Tx timing.

Agreements:

An IAB node with multiple parents treats each parent as a separate synchronization source. The IAB node can also treat RAT-independent sources such as GNSS (if used) as a separate synchronization source.

- It is up to implementation how an IAB node determines its DL-Tx timing from multiple tentative DL-Tx timing, each of which is derived based on one synchronization source.

Agreements:

- For the TA and T_delta in (TA/2+T_delta), Opt-A is adopted with the following update:

  - Opt-A: T_delta is given by the latest T_delta signaling, and TA isrepresents the currentactual time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i, which is updated with the received TA command per Rel-15.

    - Note: it is understood that for T_delta, TA/2, and (TA/2+T_delta), they may be either current time interval or filtered over the latest two or more time intervals, up to implementation. If the filtering is applied, the resulting performance is intended to be improved (it doesn’t necessarily mean that there will be the corresponding RAN4 requirements, up to RAN4)  no RAN1 spec impact

Agreements:

- For the signalling to carry T_delta, MAC_CE is used

Send an LS to RAN2 informing the above two agreements – Wenfeng (ZTE), R1-1911497, updated to R1-1911546, which is endorsed by removing “Send an LS to RAN2 informing the above two agreements.” And by adding CCing to RAN4), with final LS in R1-1911548.  In the LS, also adding a note:

- There was one company raising concerns of the signalling reliability of using the MAC_CE to signal T_delta (causing misalignment between the parent and the child nodes), comparing with using the RRC approach, although some other companies commented that there are some ways to alleviate the concerns (e.g., by repeating the MAC_CE, by signaling T_delta along with TA command, etc.). There was another company raising concerns whether there is a need for the signaling as frequently as that can be offered by MAC_CE.

RAN1 #98

Agreements:

- According to RAN1 #96bis agreement, whether T_delta is a “target value” or an “actual value” is up to parent node implementation.

- For the TA and T_delta in (TA/2+T_delta), to down-select:

  - Opt-A: T_delta is given by the latest T_delta signaling, and TA is the current time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i, which is updated with the received TA command per Rel-15.

  - Opt-B: T_delta is given by the target T_delta signaling, and TA is an average of timing advance intervals (e.g., TA1, TA2, TA3…) updated by a series TA commands.

- Once down-selected, further discuss how to reflect it in RAN1 specs

RAN1 #97

Agreements:

In Rel-16, an IAB node is not expected to receive T_delta when the IAB node MT is not in RRC_Connected mode.

RAN1 #96bis

Agreements:

In order to align the DL TX timing of the IAB node with the DL TX timing of the parent node by setting DL TX timing of the IAB node (TA/2 + T_delta) ahead of its DL Rx timing, T_delta should be set to the (-1/2) of time interval at the parent node between the start of UL RX frame i for the IAB node and the start of DL TX frame i.

- The setting of T_delta is not necessarily specified.

- Note: The above setting of T_delta assumes that, for the same purpose, TA should be the time interval at the IAB node between the start of UL TX frame i and the start of DL RX frame i.

- Send LS to RAN4 for timing clarification. (Xinghua, Huawei)  R1-1905841, which is approved with the following updates:

  - IAB_cCore

  - Fix meeting location for the August meeting

  - Fix the top blue box in the appendex from UL to DL

Final LS in R1-1905842

Agreements:

- In case the calculated TA/2 + T_delta at IAB node is negative, the IAB node should not adjust its DL-Tx timing.

RAN1 #96

Agreements:

- T_delta is indicated by a parent to the child node independently from the existing Rel.15 TA indication from the parent node used to set the UL Tx timing of the child IAB node’s MT

  - T_delta is updated on an aperiodic basis determined by the parent node

  - The child IAB node should trigger its DL TX timing adjustment by TA/2 + T_delta after it receives the timing offset T_delta indication from its parent node, if it is using OTA Timing Case 1 to obtain its DL timing.

    - FFS: behavior if TA/2 + T_delta results in an effective negative timing offset

    - FFS: delay between receiving T_delta and application of T_delta at the child node

  - Separate value ranges/granularities may be considered for T_delta in FR1 and T_delta in FR2

- Send LS to RAN4 asking them to determine the exact values and granularity of T_delta and provide confirmation on RAN1’s assumption on the DL timing accuracy requirements for IAB nodes in case of OTA Case 1 timing is applied across multiple hops – R1-1903693 (Xinghua, Huawei), approved with final LS in R1-1903810

- RAN1 #AH1901

Agreements:

An IAB node should set its DL TX timing ahead of its DL Rx timing by TA/2 + T_delta

- T_delta is signalled from the parent node, where the value is intended to account for factors such the offset between parent DL Tx and UL Rx, if any due to factors such as Tx to Rx switching time, HW impairments, etc.

- TA is the timing gap between UL Tx timing and DL Rx timing, which is derived based on existing Rel-15 mechanism

- FFS (not necessarily an exhaustive list):

  - value range and granularity of Tdelta

  - need for aperiodic/periodic updates of Tdelta

  - other timing impairment factors for adjusting IAB node timing to be included in Tdelta

  - timing alignment when the IAB node has multiple parents

  - Note: once the design of the above FFS points is in a good shape, an LS to RAN4 may be necessary to solicit their input