3GPP TSG RAN WG1 Meeting #100bis_e                                                                	        R1-2002014

e-Meeting, April 20th – 30th, 2020

Agenda Item:		7.2.10.6

Source: 	Intel Corporation

Title:	Remaining issues on cross-carrier scheduling with different numerology

Document for: 	Discussion/Decision

# Introduction

The Rel-16 WI on MR-DC & eCA [1] was completed at RAN1#99 meeting. However, there are still several remaining issues need further clarification. Therefore, we provide our views on the remaining issues on cross-carrier scheduling with different numerology in this contribution.

# Discussion

Type-1 HARQ-ACK CB for cross-carrier SPS release

Type-1 HARQ-ACK CB for cross-carrier SPS release was discussed in RAN1#100_e, however there is no agreement yet. According to the email discussion, the following intermediate conclusion was reached, although not formally marked as “Agreed” by the RAN1 chairman:

- For cross-carrier SPS release, the HARQ-ACK generation of SPS release is based on the SPS PDSCH carrier

The open issue is how to define a SPS PDSCH slot corresponding to SPS PDSCH release. As discussed in the email discussion, two alternatives being considered are,

- Alt-1: the last PDSCH slot overlapping with the CORESET carrying SPS PDSCH release;

- Alt-2: the last PDSCH slot overlapping with the slot carrying SPS PDSCH release.

In general, both alternatives work. The only difference is, as shown in Figure 1, Alt-1 may result in an earlier PDSCH slot hence the earlier HARQ-ACK feedback. Therefore, therefore, Alt-1 is slightly preferred. Within the SPS PDSCH slot, a HARQ-ACK occasion for SPS PDSCH release is derived corresponding to the SLIV of SPS PDSCH.

Figure 1: Alternatives for SPS PDSCH slot corresponding to SPS release

Proposal 1: the last PDSCH slot overlapping with the SPS PDSCH release is used to derive a HARQ-ACK occasion in Type1 HARQ-ACK codebook. A HARQ-ACK occasion for SPS PDSCH release is derived corresponding to the SLIV of SPS PDSCH in the SPS PDSCH slot.

Text proposal for section 9.1.2.1 in 38.213-g10.

For the set of slot timing values , the UE determines a set of  occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for the SLIV of SPS PDSCH in the last slot that overlaps with the SPS PDSCH release. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for the SLIV of SPS PDSCH with the lowest SPS configuration index among the multiple SPS PDSCH releases in the last slot that overlaps with the SPS PDSCH release.

Type-1 HARQ-ACK CB considering multiple DL DCIs for a cell in a PDCCH MO

For the case of a lower SCS cell scheduling a higher SCS cell, the number of slots on the scheduled cell is N times of the scheduling cell. To keep the same level flexibility of PDSCH scheduling in a slot in the scheduled cell, it is expected that N times more PDCCHs need to be supported in a slot on scheduling cell. For example, as shown in Figure 1, multiple unicast DCIs could be transmitted in the same MO and schedule PDSCHs for which the HARQ-ACK are transmitted on the same PUCCH. Therefore, the HARQ-ACK bits ordering for the DCIs in same MO needs to be defined. A simplest way is relying on C-DAI as a third dimension of HARQ-ACK bits ordering. The maximum capability of 2-bit C-DAI is to differentiate up to 4 DL DCIs scheduling PDSCHs on a scheduled cell.

Proposal 2: Assuming X>1 DL DCIs scheduling unicast PDSCHs per scheduled cell can be transmitted in a PDCCH MO, C-DAI is used as a third dimension of HARQ-ACK bits ordering in Type2 HARQ-ACK CB.

# Conclusion

In this contribution, we discussed the remaining details of dynamic power sharing mechanisms for NR-NR DC operation. We make following proposals:

Proposal 1: the last PDSCH slot overlapping with the SPS PDSCH release is used to derive a HARQ-ACK occasion in Type1 HARQ-ACK codebook. A HARQ-ACK occasion for SPS PDSCH release is derived corresponding to the SLIV of SPS PDSCH in the SPS PDSCH slot.

Proposal 2: Assuming X>1 DL DCIs scheduling unicast PDSCHs per scheduled cell can be transmitted in a PDCCH MO, C-DAI is used as a third dimension of HARQ-ACK bits ordering in Type2 HARQ-ACK CB.

# References

[1] RP-182076, “WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements”, Ericsson, Nokia, Nokia Shanghai Bell, Huawei

[2] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting AH-1901, January, 2019.

[3] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #98, August, 2019.

[4] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #98bis, October, 2019.

[5] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #99, November, 2019.

[6] RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #100_e, February, 2020.

# ANNEX All agreements on Cross-carrier scheduling [2][3][4][5][6]

Conclusion: All the following 4 cases can be considered further, while there was yet no consensus if all the cases will be eventually specified. To be discussed further after work on solutions has progressed. - Support scheduling cell of lower SCS and scheduled cell of higher SCS downlink - Support scheduling cell of lower SCS and scheduled cell of higher SCS uplink - Support scheduling cell of higher SCS and scheduled cell of lower SCS downlink - Support scheduling cell of higher SCS and scheduled cell of lower SCS uplink Conclusion: Study further at least the following: - Determine the first possible PDSCH starting point based on the timing of the last or first symbol of the scheduling PDCCH. - Both Type A and Type B PDSCH allocation should be considered - Consider a possibility for introducing a single solution for Type A and Type B PDSCH allocation - PDCCH position cases 1-1, 1-2 and 2 should all be considered Agreements: - At least for the case of lower SCS PDCCH scheduling a higher SCS PDSCH the earliest possible starting point for the PDSCH is defined by the end of the PDCCH +  - >0. Detailed value(s) FFS - FFS other factor(s) impacting  Agreements: - The limit of BDs/CCEs (per slot in the scheduling CC) for the scheduled CC is determined based on the numerology of the scheduling CC. - Change the definition of NcellsDL, NcellsDL,μto “the number of configured DL-CCs whose scheduling cell is with active DL BWP having SCS configuration ” as in Section 10.1 of 38.213 Agreements: - For case 1-1 scheduling (PDCCH in the beginning of the slot), when a lower SCS PDCCH schedules a higher SCS PDSCH: - The  is determined a number of symbols based on PDCCH SCS counting from the end of the last symbol of the received PDCCH symbol to the beginning of the first symbol of the corresponding received PDSCH, quantized (using the granularity of PDSCH slot duration) to the next PDSCH slot boundary Agreements: - Delta-values for lower SCS PDCCH to higher SCS PDSCH case 1-1 scheduling - 15 kHz: 4 symbols - 30 kHz: 4 symbols - 60 kHz: 8 symbols - Case 1-2: use the same delta as the case 1-1 scheduling - With the quantization step - Case 2: - Use the same delta values as the case 1-1 scheduling INCLUDING the quantization step Agreements: - For the case of higher SCS PDCCH scheduling lower SCS PDSCH, for the scheduling timing, use the same definition as with lower SCS PDCCH scheduling higher SCS PDSCH scheduling, without quantization to the next PDSCH slot Agreements: - For high-SCS to low-SCS scheduling, the delta for 120 kHz SCS PDCCH: [12] symbols Agreements: - For the case of a lower SCS PDCCH scheduling a higher SCS PDSCH (or PUSCH), support using M DCIs in one slot for PDCCH to schedule M respective PDSCH (or PUSCH) transmissions in N slots for PDSCH (or PUSCH), where M<=N (for slot aggregation, M<N) following at least one of the following rules (potential down-selection can be done during Rel-16 UE feature discussion): - Alt 1a: Define max number of unicast DCIs that the UE is expected to decode in each span of PDCCH symbols - Alt 1b: Define an increased number of valid unicast DCIs per PDCCH monitoring occasions - Alt 1c: based on Rel-15 FG 3-5/3-5b - Further discussion whether some additional clarification is needed or not  potential clarification can be discussed under UE features Agreements: - timeDurationForQCL threshold is determined based on the numerology of the scheduled cells. Agreements: When PDSCH and its scheduling PDCCH are in the different CCs, if the PDCCH-to-PDSCH delay < Threshold-Sched-Offset timeDurationForQCL or if the TCI information is absent from the DCI, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell Conclusion: If the number of valid unicast DCIs at the same monitoring occasions is increased in the UE feature discussion, further discuss whether/how to update the HARQ-ACK codebook and PUCCH resource determination. Agreements: - In Rel-16, support enabling HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. - Note: vs. per cell group in Rel-15 Update after email discussion/approval [98-NR-23] Agreement: - Introduce two separate UE capabilities for the following: - Scheduling cell of lower SCS and scheduled cell of higher SCS - Scheduling cell of higher SCS and scheduled cell of lower SCS Agreements: At least the following two UE capabilities will be introduced - Scheduling cell of lower SCS and scheduled cell of higher SCS - Scheduling cell of higher SCS and scheduled cell of lower SCS Agreements: - The same additional beam switching timing as agreed for A-CSI under 7.2.13.4 is used for PDSCH being cross-carrier scheduled with different numerologies Agreements: For the time offset between PDCCH and PDSCH under cross-carrier scheduling with different numerologies: - The timing offsets 4,4,8,[12] for SCS 15, 30, 60, 120 kHz respectively are relaxed to 4,5,10,14 - The above-mentioned numbers assume that the minimum PDCCH-to-PDSCH timing in cross-carrier scheduling with different subcarrier spacings that from RAN1 spec perspective they are assumed to have zero time offset - Consider if there is a need to clarify the Rel-16 feature introduction CR on this aspect Agreements: For QCL other than TypeD: - Clarify that the following rule in Rel-15 is also applicable to cross-carrier scheduling with different numerologies: - If none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH. Conclusion: - Do not support cross-carrier PDCCH order in Rel-16 Agreements: - To extend the cross-carrier scheduling with different SCS to be operable with DCI format 1_2 defined for Rel-16 in eURLLC WI. - Note: to be discussed under UE feature discussion regarding whether or not to introduce separate UE capability to indicate the support for cross-carrier scheduling with different SCS to be operable with DCI format 1_1 and DCI format 1_2