3GPP TSG RAN WG1 #100bis                                                                                    R1-2002015

e-Meeting, April 20th – 30th, 2020

Source: 	Intel Corporation

Title:	Discussion on UE features for two-step RACH

Agenda item:	7.2.11.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#100-E meeting, an outcome of the email discussion/approval for Rel-16 NR UE features was captured in [1]. In particular, UE features regarding 2-step RACH is listed in the Appendix. In the contribution, we discuss remaining details of UE features for two-step RACH.

# Discussion on UE features for two-step RACH

As listed in the Appendix, two alternatives were discussed for basic feature group 9-1 for 2-step RACH.

- Alt 1: detailed description of basic feature group for 2-step RACH

- Alt 2: simplified decryption of basic feature group for 2-step RACH

In our view, given that the specification already describes detailed UE behaviour when supporting 2-step RACH, it is slightly preferable to consider a simplified description of basic feature group 9-1 for 2-step RACH. However, a minor modification needs to be considered as follows:

- MsgA PRACH and PUSCH configuration and transmission

- MsgB monitoring, reception, and feedback

- Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB

Proposal 1

- It is slightly preferable to consider a simplified description of basic feature group 9-1 for 2-step RACH.

- The following needs to be modified

  - 1)	MsgA PRACH and PUSCH configuration and transmission

If the simplified description of basic feature group for 2-step RACH is not agreeable, it would be more desirable to divide the basic feature group into multiple subsets to make it more organized. Table 1 illustrates the suggested update on feature groups for 2-step RACH.

Table 1. Basic UE feature groups for 2-step RACH

| Index | Feature group | Components |
| --- | --- | --- |
| 9-1a | MsgA PRACH and PUSCH configuration and transmission for 2-step RACH | - RACH type selection based on a SSB-based RSRP threshold - Separately configured ROs not applicable to from 4-step RO configuration - Same Shared ROs but with different preamble sequences partitioning with 4-step RO preamble sequences configuration - Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - MsgA PUSCH transmission including scrambling, DMRS sequences and ports, RV, etc - MsgA PRACH configuration and preamble formats - Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) as specified in Rel-15 - Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) as specified in Rel-15 - MsgA PRACH and PUSCH transmissions in different PRACH and PUSCH slots - 2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state - Intra-slot frequency hopping for MsgA PUSCH |
| 9-1b | MsgB monitoring, reception and feedback for 2-step RACH | - Monitoring of MsgB with PDCCH addressed to MsgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication - Support PUCCH transmission for HARQ-ACK feedback to a MsgB - Minimum TX gap between the last symbol of MsgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to MsgB PDSCH |
| 9-1c | Power control for MsgA and PUCCH for HARQ-ACK feedback to a MsgB for 2-step RACH | - MsgA open loop power control - PUCCH power control for HARQ-ACK feedback to a MsgB |

Proposal 2

- If the simplified description of basic feature group is not agreeable, consider Table 1 as starting point for discussion of feature groups for 2-step RACH.

# Conclusions

In this contribution, we discussed the remaining details of UE features for two-step RACH. Further, we summarize the proposals as follows:

Proposal 1

- It is slightly preferable to consider a simplified description of basic feature group 9-1 for 2-step RACH.

- The following needs to be modified

  - 1)	MsgA PRACH and PUSCH configuration and transmission

Proposal 2

- If the simplified description of basic feature group is not agreeable, consider Table 1 as starting point for discussion of feature groups for 2-step RACH.

# References

- R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, INC, e-Meeting, February 2020

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - MsgA PRACH and PUSCH transmission - MsgB monitoring, reception, and feedback - Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB FFS: if any of the components need to be more specific, i.e. the following components in Alt. 1 can be further discussed. [Alt 1 detailed feature group]: - [RACH type selection based on a SSB-based RSRP threshold] - [Separately configured ROs not applicable to 4-step RO configuration] - [Same ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration] - [Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective] - [Validation of MsgA PRACH and PUSCH] - [Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH] - [MsgA PUSCH transmission including scrambling, DMRS sequences and ports, RV, etc] - [MsgA open loop power control] - [Monitoring of MsgB with PDCCH addressed to msgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication] - [Support PUCCH transmission for HARQ-ACK feedback to a MsgB] - [MsgA PRACH configuration and preamble formats] - [PUCCH power control for HARQ-ACK feedback to a MsgB] - [Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [MsgA PRACH and PUSCH transmissions in different PRACH and PUSCH slots] - [Minimum TX gap between the last symbol of MsgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to MsgB PDSCH] - [2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state] - [Intra-slot frequency hopping for MsgA PUSCH] |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-2 | Supported 2 symbols DMRS for msgA PUSCH | Supported 2 symbols DMRS for msgA PUSCH (‘len2’) | 9-1, 2-18 | Yes | N/A | If UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources | per UE | No | Yes |  | Shall be aligned with 2-18 | Conditionallymandatory for UE supporting both 9-1 and 2-18 |
|  | 9-3 | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | 9-1 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination | N/A | N/A |  |  | Optional with capability signalling |