3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002046

e-Meeting, April 20th- 30th, 2020

Agenda Item:	7.2.8.1

Source:	Futurewei

Title:	Remaining Details on DL Reference Signals

Document for:	Discussion and decision

# Introduction

At RAN1#99, RAN1 completed the standardization work for NR Positioning Support. During the RAN1#100-e meeting, numerous CRs proposals were proposed and discussed. In this contribution, we revisit and provide our views on the remaining proposals [1] that were left out from previous discussion on the DL reference signals for Positioning.

# Discussions

## DL PRS Sequence ID

In [2], it was proposed to update the section 7.4.1.7.2 of the TS 38.211, to cover the case when dl-PRS-SequenceId-r16 is not configured. Text proposal is provided below:

“The pseudo-random sequence generator shall be initialised with

cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231

where ns,fμ is the slot number, the downlink PRS sequence ID nID,seqPRS∈0,1,…,4095 is given by the higher-layer parameter dl-PRS-SequenceId-r16, unless otherwise configured, nID,seqPRS is equal to physical cell ID, and l is the OFDM symbol within the slot to which the sequence is mapped.”

There was no agreement during RAN1#100-e on the above change and it was also suggested that the further discussion can continue in RAN2.

In Section 6.4.2.1 of TS 37.355 [3], the parameter ‘dl-PRS-SequenceId-r16’ is defined as follows:

dl-PRS-SequenceId-r16				INTEGER {0.. 4095}

The configurability allows for example more than one PRS resources transmitted from a TRP to have a common sequence ID if the network implementation can assure that the time-frequency resources are not overlapping. It should be noted that many PRS resources can be configured and not all such resources need to be assigned distinct sequence ID. Going back to the highlighted statement above in 38.211, it is not clear what it means to state ‘unless otherwise configured’, specifically what value or default value for nID,seqPRS would be in such a scenario.

Proposal: RAN1 should clarify the value of nID,seqPRS in the absence of configuration.

# Conclusions

# In this contribution, we have proposed the following:

Proposal: RAN1 should clarify the value of nID,seqPRS in the absence of configuration.

# References

[1] R1-2001233, “Output of email thread [100e-NR-Pos-DL-PRS-01],” Intel.

[2] R1-2000686, “Remaining issues of DL reference signals for NR Positioning,” LGE.

- [3] RP-200345, “RAN2 CRs to NR positioning support,” RAN2.