3GPP TSG RAN WG1 #100bis		R1-2002162

e-Meeting, April 20th – 30th, 2020

Source:	Panasonic

Title: 	Clarification on NR-U configured grant enhancement

Agenda Item:		7.2.2.2.4

Document for:	Discussion

Introduction

In this document, we propose the clarification on NR-U configured grant enhancement. It is related to issue #9 in the feature lead summary in RAN1#100-e [1]. In RAN1#100-e, it was not treated because issue #9 was categorized as low priority in preparation phase in the discussion.

Discussion

In RAN1#99, the following agreement was reached on NR-U configured grant enhancement.

Agreement: - DFI is transmitted using PDCCH scrambled with CS-RNTI - DFI size is similar to UL DCI size - Size is aligned with UL grant DCI format 0_1 size - To distinguish DCI for activation/deactivation CG transmission and DFI, a 1 bit flag (explicit indication) is used, when type 1 and/or type 2 CG PUSCH is configured - Content of DFI includes - UL/DL flag - CIF in the case of cross carrier scheduled is configured - 1 bit flag - HARQ Bitmap - TPC command – 2 bits - For type 1 configured grant, UE assumes DFI is only present when CG is configured. - For type 2 configured grant, UE assumes DFI is only present when CG is configured, and UE is in activated state for configured grant transmissions

The highlighted part in the above agreement was captured to 38.213[2] section 10.5 as follows (highlighted in green). On the other hand, there are no agreement related to yellow highlighted parts. Yellow highlighted parts seem to be intended that HARQ-ACK feedback for semi-persistent CSI reporting on PUSCH is indicated on DFI. However, since HARQ-ACK feedback for PUSCH by DFI needs HARQ process number, HARQ-ACK feedback for semi-persistent CSI reporting cannot be performed because HARQ process number is not assigned for semi-persistent CSI reporting on PUSCH. Therefore, we propose to remove yellow highlighted parts.

10.5 HARQ-ACK information for PUSCH transmissions A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on an XYZ field value, as described in [5, TS 38.212], if a PUSCH transmission is configured by ConfiguredGrantConfig or is an activated PUSCH transmission configured by semiPersistentOnPUSCH. The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for a number of HARQ processes provided by nrofHARQ-Processes for a serving cell of a PDCCH reception that provides DCI format 1_0 or, if DCI format 1_0 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field. For a PUSCH transmission configured by ConfiguredGrantConfig or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay-r16.

In addition, there are similar descriptions in 38.213 section 9 and 9.3 related to CG-UCI transmission. Since CG-UCI is not included in semi-persistent CSI reporting because of no data, these descriptions should also be removed.

9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, or in an activated PUSCH transmission configured by semiPersistentOnPUSCH, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-CG-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission. < Unchanged parts are omitted > 9.3 UCI reporting in physical uplink shared channel < Unchanged parts are omitted > For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK. < Unchanged parts are omitted >

Conclusion

We propose to revise the description on NR-U configured grant enhancement in 38.213 as following.

9	UE procedure for reporting control information

< Unchanged parts are omitted >

If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is configured by a ConfiguredGrantConfig, or in an activated PUSCH transmission configured by semiPersistentOnPUSCH, and includes CG-UCI [5, TS 38.212], the UE multiplexes the HARQ-ACK information in the PUSCH transmission if the UE is provided cg-CG-UCI-Multiplexing; otherwise, the UE does not transmit the PUSCH and multiplexes the HARQ-ACK information in a PUCCH transmission or in another PUSCH transmission.

< Unchanged parts are omitted >

9.3	UCI reporting in physical uplink shared channel

< Unchanged parts are omitted >

For a PUSCH transmission that is configured by a ConfiguredGrantConfig, or for an activated PUSCH transmission that is configured by semiPersistentOnPUSCH, and includes CG-UCI, the UE is provided by betaOffsetCG-UCI-r16 a IoffsetCG-UCI value, from a set of values, with the mapping defined in Table 9.3-1. If the UE multiplexes HARQ-ACK information in the PUSCH transmission, as described in Clause 9.2.5, the UE jointly encodes the HARQ-ACK information and the CG-UCI [5, TS 38.212] and determines a number of resources for multiplexing the combined information in a PUSCH using βoffsetHARQ-ACK.

< Unchanged parts are omitted >

10.5  HARQ-ACK information for PUSCH transmissions

A UE can be configured a number of search space sets to monitor PDCCH for detecting a DCI format 0_1 with CRC scrambled with a CS-RNTI provided by cs-RNTI. The UE determines that the DCI format provides HARQ-ACK information for PUSCH transmissions based on an XYZ field value, as described in [5, TS 38.212], if a PUSCH transmission is configured by ConfiguredGrantConfig or is an activated PUSCH transmission configured by semiPersistentOnPUSCH.

The HARQ-ACK information corresponds to transport blocks in PUSCH transmissions for a number of HARQ processes provided by nrofHARQ-Processes for a serving cell of a PDCCH reception that provides DCI format 1_0 or, if DCI format 1_0 includes a carrier indicator field, for a serving cell indicated by a value of the carrier indicator field.

For a PUSCH transmission configured by ConfiguredGrantConfig or for activated PUSCH transmissions configured by semiPersistentOnPUSCH, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission, or of any repetition of the PUSCH transmission, by a number of symbols provided by cg-minDFIDelay-r16.

< Unchanged parts are omitted >

- Reference

- R1-2000800, “Feature lead summary on NRU configured grant enhancement”, vivo

- 3GPP TS 38.213 V16.1.0 (2020-03)