3GPP TSG RAN WG1 #100bis			                           	                                                                 			R1-2002188

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.11.3

Source:	LG Electronics

Title:	Discussion on RAN1 UE feature for NR IAB

Document for:	Discussion and Decision

Introduction

In this contribution, we share our views on the RAN1 UE feature list based on the latest version in R1-2001484.

Discussion

Based on R1-2001484, we have following suggestions.

20-5

    - Component: Dynamic configuration/indication of UL-Flexible-DL slot formats is applied only for IAB MT. Also, according to FL’s summary in R1-2001484 as captured below, the majority view is RAN1 adds to the UE features framework only features that are IAB-MT specific. Thus, IAB DU should be removed.

Proposal by FL in R1-2001484

RAN1 adds to the UE features framework only features that are IAB-MT specific.

RAN1 documents non IAB-MT specific features for information to RAN3.

20-6

    - Component: For AI indication, DCI format 2_5 is used in the specification. Thus, it should be changed ‘4_0’ to ‘2_5’.

20-8

    - Component: Add UE features for supporting guard symbols for transition between IAB node MT and IAB node DU needs

Proposal: Adopt following changes on the RAN1 UE feature list for NR IAB.

- NR_IAB

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20. NR_IAB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-5 | UL-Flexible-DL slot formats | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-6 | Dynamic indication of soft resource availability | Support DCI Format 2_5 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-7 | Case 1 OTA timing alignment | Support T_delta reception. |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. For device supports IAB backhaul, it must report this FG is supported |
|  | 20-8 | Guard symbols | Support guard symbols for transition between IAB node MT to IAB node DU |  |  |  | Yes | N/A | Guard symbols for transition between IAB node MT to IAB node DU is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |