3GPP TSG-RAN WG1 Meeting #100b	R1-2002223

# e-Meeting, April 20th –30th , 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Specification alignment on SFI configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 specification TS38.331 lists that slotFormatCombToAddModList is optional in SlotFormatIndicator slotFormatCombToAddModList SEQUENCE (SIZE(1..maxNrofAggregatedCellsPerCellGroup)) OF SlotFormatCombinationsPerCell OPTIONAL, -- Need N. RAN1 specification TS38.212 mandates slotFormatCombToAddModList in SlotFormatIndicator The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI: - Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.. Furthermore, R16 NR-U has added several other fields to DCI format 2_0, such as availableRB-SetToAddModList-r16, searchSpaceSwitchTrigger-r16 and co-DurationPerCell-r16, which are indicated only if configured, as per current R16 TS38.212 |
| Summary of change: |  | Alignment of TS38.212 to TS38.331 to clarify that SFI is indicated by DCI format 2_0 only if slotFormatCombToAddModList is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R15, specification is ambiquous and therefore it is unclear whether NR-U R16 may opretate DCI format 2_0 without slotFormatCombToAddModList configured in SlotFormatIndicator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: It is common understanding that if R15 gNB configures DCI format 2_0 to a UE, it will as well configure UE with slotFormatCombToAddModList. Therefore, configuring SlotFormatIndicator without slotFormatCombToAddModList would be a clear misconfiguration in R15. There is no impact on existing R15 UEs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.3.1	Format 2_0

DCI format 2_0 is used for notifying the slot format.

The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI:

- 	If the higher layer parameter slotFormatCombToAddModList is configured

-	Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.

The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Subclause 11.1.1 of [5, TS 38.213].