3GPP TSG RAN WG1 #100bis-e	R1-2002253

e-Meeting, April 20th – 30th, 2020

Source:	Moderator (Nokia)

Title:	Summary#1 for AI 7.2.4.6 QoS management

Agenda item:	7.2.4.6

Document for:	Discussion and Decision

# Introduction

In this contribution we summarize the remaining issues in NR V2X QoS Management (agenda item 7.2.4.6).

# Issues

Email thread budget: Up to 2

Topic 1: Sidelink Congestion Control

Issue 1.1: How to meet CR limits – specify or leave it up to UE implementation?

Issue 1.2: CR – treatment of resources reserved, but not used due to HARQ feedback and/or pre-emption

Issue 1.3: Semi-persistent resource reservation disabled by congestion control

Issue 1.4: CBR reporting in cross-RAT scenarios

Issue 1.5: CBR-based power control in mode 1

Topic 2: TX Parameter restriction based on speed

Issue 2.1: TX Power restriction based on speed?

Topic 3: QoS

Issue 3.1: QoS-based Resource Pool Segregation/Prioritization

## Topic 1: Sidelink Congestion Control

### Issue 1.1: How to meet CR limits – specify or leave it up to UE implementation?

#### Background

One remaining open issue for the NR sidelink is whether UE behaviour on how to meet the upper bounds on CR needs to be specified. In LTE, this was up to UE implementation. TS 38.214 currently contains the following text:

[ It is up to UE implementation how to meet the above limits, including dropping the transmissions in slot n. ]

#### Views expressed in contributions

Up to UE implementation, like LTE: 4 [Huawei, HiSilicon], [vivo], [Nokia, NSB], [LGE].

Specify at least some aspects of behaviour: 1 [Intel]

#### Feature lead view

Needs to be discussed to finalize the specification.

### Issue 1.2: CR – treatment of resources reserved, but not used due to HARQ feedback and/or pre-emption

#### Background

One remaining issue is how resources which have been reserved but have not been used or will not be used due to HARQ feedback and/or pre-emption should be counted in channel occupancy ratio (CR).

This aspect is not directly covered by the existing agreements that LTE CR is the baseline, since the LTE sidelink does not support HARQ feedback or pre-emption.

The current CR definition in TS 38.215 counts resources “used for … transmissions in slots [n-a, n-1] and granted in slots [n, n+b]”. Hence, for the past segment of the evaluation window, slots [n-a, n-1], resources which have been reserved but released or pre-empted are not counted, since they have not been used for transmission. For the future segment of the evaluation window, slots [n, n+b], the current definition counts all the resources reserved for retransmissions, even if the UE knows already based on HARQ feedback that they will not be used.

#### Views expressed in contributions

Do not count unused resources in CR: [Nokia, NSB], [ZTE, Sanechips], [CATT], [Futurewei], [Apple]

Count unused resources in CR: [Huawei, HiSilicon]

For future segment, when sidelink HARQ feedback is enabled, UE assumes that the existing grant(s) can be released: [Samsung]

#### Feature lead view

Should be discussed to finalize the specification.

### Issue 1.3: Semi-persistent resource reservation disabled by congestion control

#### Views expressed in contributions

[InterDigital] propose that congestion control can disable semi-persistent resource reservation. This is opposed by [Futurewei], [CATT].

#### Feature lead view

Not critical.

### Issue 1.4: CBR reporting in cross-RAT scenarios

#### Views expressed in contributions

[OPPO] proposed support for reporting NR sidelink CBR to the eNB.

[ZTE, Sanechips] proposed support for reporting LTE sidelink CBR to the gNB.

#### Feature lead view

Can be discussed by RAN2.

### Issue 1.5: CBR-based power control in mode 1

#### Background

For the LTE V2X sidelink, CBR-based power control was only applied in resource allocation mode 3. The current specification in TS 38.213 on the other hand applies CBR-based power control regardless of resource allocation mode. So far there has not been any explicit discussion on this deviation from LTE behaviour. Supporting CBR-based power control in resource allocation mode 1 provides an additional tool for congestion control; moreover, if its use is not desired, it can be disabled by setting maximumtransmitPower-SL to its maximum value for all values of PPPP.

#### Views expressed in contributions

[Nokia, NSB] propose to make an explicit agreement that CBR-based power control is applied in both resource allocation modes.

#### Feature lead view

Not critical.

## Topic 2: TX Parameter restriction based on speed

### Issue 2.1: TX Power restriction based on speed?

#### Background

We have agreed the following:

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability

The agreement states “the above parameters can be restricted by reusing the same mechanism as in LTE”, and “Upper bound of TX power (including zero TX power)” is clearly one of the parameters listed above that phrase. On the other hand, the LTE mechanism did not restrict TX power based on speed, as can be seen from an examination of the LTE specs and the relevant agreement reached during the LTE_SL_V2V work item:

Agreement in RAN1 #85: - Alt 1 + Adapt MCS, the number of RBs, and number of transmission subframes depending on the UE absolute speed and UE synchronization source (e.g. GNSS or eNB) - Options for details of PSCCH - Working assumption which will be automatically confirmed if no problem is identified during this week - DMRS within a TTI for a transmission by a UE are not identical - No blind detection of DMRS is introduced - Details FFS - Working assumption: 2 consecutive PRB pairs in a subframe are used for each PSCCH if the number of SA bits is less than 64 including CRC. The exact size of SA is FFS including the CRC size and could be larger than 64 bits. - Options for details of PSSCH - Network configuration or preconfiguration can be used to associate the ranges of MCS, RB number for PSSCH, number of retransmission with the condition of the UE absolute speed. Different (pre)configuration and threshold is given for the different type (e.g., eNB, GNSS, UE) of the transmission synchronization reference. - RAN1 will study the proper range of these parameters

#### Views expressed in contributions

7 contributions propose to explicitly confirm that the upper bound of sidelink TX power not be restricted based on UE absolute speed: [vivo], [OPPO], [Nokia, NSB], [ZTE, Sanechips], [Intel], [CATT], [Ericsson]

[OPPO] go one step further and propose that MCS be the only TX parameter that can be restricted based on absolute speed.

#### Feature lead view

Not critical: Current specification is aligned with the unanimous view that upper bound of sidelink TX power not be restricted based on UE absolute speed. Benefit of additional agreement would be just to remove an ambiguity in existing agreements.

## Topic 3: QoS

### Issue 3.1: QoS-based Resource Pool Segregation/Prioritization

#### Background

R12 ProSe sidelink supported resource segregation based on QoS by associating a priority list with each transmit resource pool (priority-based resource pool selection). No such mechanism was defined for the LTE V2X sidelink, the LTE V2X sidelink was designed to support transmission of all QoS levels in the same resource pool and it defined mechanisms to take QoS into account for procedures such as resource selection and congestion control. For the LTE V2X sidelink there is no AS mechanism to map a QoS level to specific resource pools. However, there is a higher layer configuration mechanism which allows mapping a service to specific carrier frequencies (TS 24.385); so, if e.g. a service has stringent QoS requirements then that higher layer mechanism allows mapping that service to one or more specific carriers.

#### Views expressed in contributions

In the current meeting, this topic was addressed in one contribution:

- [InterDigital]: A resource pool can be configured with an allowed QoS for the data that can be transmitted using that resource pool

#### Feature lead view

Resource pool selection is a RAN2 topic, this can be discussed by RAN2.

# References

- RP-200129, “Revised WID: 5G V2X with NR sidelink”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- TR 38.885, Study on NR Vehicle-to-Everything (V2X)

# Background

## WI Objectives

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved ‎[1]. Two of the objectives are relevant for the present agenda item:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - … - Congestion control [RAN1, RAN2] 4. Specify support for QoS management [RAN2, RAN3, RAN1]

## Earlier Agreements

The following relevant agreements have been reached in previous meetings:

### QoS

Agreements: From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered: - Priority - latency - reliability

Agreements: RAN1 studies further how to use - priority, - latency, - reliability, - minimum required communication range (as defined by higher layers) if agreed to use in the physical layer aspects of at least - resource allocation and - congestion control and - resolution of in-device coexistence issues and - power control

In the Sidelink resource allocation mode 2 agenda item, the following working assumption was reached:

Working assumption: - An indication of a priority of a sidelink transmission is carried by SCI payload - This indication is used for sensing and resource (re)selection procedures - This priority is not necessarily the higher layer priority

Agreements: - For the priority indication in 1st stage SCI: - Up to RAN2 on how to define the mapping between the priority indication and the corresponding QoS - Size is 3 bits (as a working assumption)

### Sidelink Congestion Control

Agreements: - Introduce at least one congestion metric for NR sidelink - FFS details – to be done in WI phase (if included)

Agreements: - Congestion control is supported at least for sidelink mode 2 - Note: details of congestion control can be covered in the work item phase, not in this SI.

Conclusion: - It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB - Consequently, it is recommended to specify the corresponding details in the WI phase

Agreements: Support at least NR CBR as congestion metric for NR sidelink congestion control. - LTE CBR is the baseline for defining NR CBR.

Agreements: - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

Agreements: - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

Agreements: - For PSCCH/PSSCH multiplexing option 3, one CBR measurement over a resource pool is defined. - PSFCH resources, if (pre)configured, are excluded from this CBR measurement.

Agreements: Define NR sidelink Channel Occupancy Ratio (CR) measurement. - LTE CR is the baselines

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability

Agreements: Lookup table links CBR range with values of the transmission parameters and CRlimit for each value of the indication of a priority of a sidelink transmission carried by SCI payload (as per WA from RAN1#98), Lookup table is (pre)configured. Details up to RAN2. - Up to 16 (as a working assumption) CBR ranges are supported - The working assumption will be automatically confirmed in RAN1#99 if no further input

Agreements: - Sidelink RSSI (SL-RSSI) measurement is used for CBR estimation

Agreements: A sidelink resource is busy for the purpose of CBR measurement if Sidelink RSSI measured by the UE in that resource exceeds a (pre-)configured threshold.

Agreements: The CBR measurement time window size is 100 ms and 100 slots by (pre-)configuration. CR window size is { 1000 ms, 1000 slots } by (pre)-configuration

Agreement: - The future segment of the CR evaluation window reuses the same behaviour as in the LTE V2X sidelink. - FFS whether additional constraints on UE’s choice of values for a and b are needed

Agreement: For the constraints on past/future window in CR evaluation: - n+b shall not exceed the last transmission opportunity of the grant for the current transmission - b >= 0 - b < (a+b+1)/2 Notes: - in the first bullet point above, LTE’s “should” has been replaced by “shall”

Agreement: - UE evaluates CR and applies CR_limit for every (re)transmission.

Agreement: - The CBR processing time is given by UE capability according to the following table µ; Congestion process time 1 (slots); Congestion processing time 2 (slots). 0; 2; 2. 1; 2; 4. 2; 4; 8. 3; 8; 16. - A UE shall only apply a single CBR/CR processing time capability in SL. - CR processing time is the same as CBR processing time.

Agreement: - The slot index in the definition of CBR is the physical slot index. Agreement: - The slot index in the definition of CR is the physical slot index.

### TX Parameter Restrictions

Agreements: - Only TX parameter restriction based on absolute speed can be (pre)configured in Rel-16.

# Appendix: Contributions used as basis for the summary

| R1-2001556 | Remaining details of QoS management for NR sidelink | Huawei, HiSilicon |
| --- | --- | --- |
| R1-2001666 | Remaining issues on QoS management for sidelink | vivo |
| R1-2001752 | Remaining open issues on QoS | OPPO |
| R1-2001809 | Remaining details of QoS management for sidelink | Nokia, Nokia Shanghai Bell |
| R1-2001890 | Discussion on QoS management for NR sidelink | LG Electronics |
| R1-2001899 | Remaining issues on QoS | ZTE, Sanechips |
| R1-2001997 | Remaining details of QoS and congestion control for NR V2X design | Intel Corporation |
| R1-2002044 | Remaining details of QoS for sidelink | Futurewei |
| R1-2002081 | Remaining issues on QoS management in NR V2X | CATT |
| R1-2002130 | On QoS Management for NR Sidelink | Samsung |
| R1-2002239 | QoS management for NR sidelink | Ericsson |
| R1-2002303 | Remaining Issues on Congestion control and QoS Management for NR-V2X | InterDigital, Inc. |