3GPP TSG RAN WG1 #100bis					  	R1-2002365

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.2.1.3

Source:	NEC

Title:              	TPs on uplink signals in NRU

Document for:	Discussion and Decision

- Introduction

In this contribution, we have some text proposals on uplink signal in NRU.

- Discussion

In NR-U uplink SRS transmission, the SRS starting symbol is enhanced from symbol #0. This agreement is captured in TS38.211, while it’s not reflected in TS38.214 which may confuse the reader.

So we propose that:

- Text proposal 1

>>> Text proposal for 38.214 Section 6.2.1 >>>

< Unchanged parts are omitted >

The UE may be configured by the higher layer parameter resourceMapping or resourceMapping-r16 in SRS-Resource with an SRS resource occupying  adjacent symbols within the last 6 symbols of the slot by resourceMapping or anywhere within the slot by resourceMapping-r16, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter [SRS-for-positioning] the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

< Unchanged parts are omitted >

>>> End Text proposal >>>

- Conclusion

In this contribution, we provided some text proposals on uplink signal in NRU.

- References

- TS38.213 v16.1.0