3GPP TSG RAN WG1 Meeting #100bis                                          	                    R1-2002397

e-Meeting, April 20th –  30th, 2020

Source:	CATT

Title:	              Feature lead summary on AI 7.2.4.3 #2 Sidelink synchronization mechanism – Email discussion outcomes

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

- Introduction

In RAN1#100bis-e meeting, 4 email discussion threads were taken place.

- [100b-e-NR-5G_V2X_NRSL-SYNC-01]

Email discussion/approval related to PSBCH contents - Indication of TDD configuration

(a,k.a. issue 1-1) by 4/24, with potential TPs by 4/29 (CATT, Teng)

- [100b-e-NR-5G_V2X_NRSL-SYNC-02]

Email discussion/approval related to DM-RS sequence initialization for PSBCH and QCL for S-SSB

(a,k.a. issues 2 and 3) by 4/24, with potential TPs by 4/29 (CATT, Teng)

- [100b-e-NR-5G_V2X_NRSL-SYNC-03]

Email discussion/approval related to sync procedure - SL SSIDs/sync resources for each priority and Lower SLSS ID with higher priority for P6/P6’ UE

(a,k.a. issues 4-1 & 4-2) by 4/24, with potential TPs by 4/29 (CATT, Teng)

- [100b-e-NR-5G_V2X_NRSL-SYNC-04]

Email discussion/approval related to

- Slot number/sidelink timing derived from GNSS

- Resource sets for S-SSB transmission

- Timing determination of S-SSB

(a,k.a. issues 5,6,7) by 4/24, with potential TPs by 4/29 (CATT, Teng)

After long time discussion, we achieved some agreements and endorsed TP, which are captured in Section 2 in this document.

- Agreements and endorsed LS/TP

[100b-e-NR-5G_V2X_NRSL-SYNC-01]

Agreements:

- For indication of TDD configuration:

- X=1 bit indicates the number of patterns

  - Value 0 indicates one pattern is used.

  - Value 1 indicates two patterns are used.

- Y=4 bits indicate the periodicity information

  - When one pattern is used, Y indicates the periodicity of the pattern.

  - When two patterns are used, Y jointly indicates the periodicities of the two patterns.

- Z=7 bits indicate the UL slots

  - UL slots are jointly indicated by 7 bits when two patterns are configured.

  - FFS other details.

[100b-e-NR-5G_V2X_NRSL-SYNC-02]

Agreements:

- The DM-RS sequence initialization for PSBCH can be c_init=N_ID^SL

- QCL mechanism is not supported for S-SSB receptions in Rel-16 NR V2X.

-------------------------------------------------Start of Draft TP for 38.211---------------------------------------------

8.4.1.4 Demodulation reference signals for PSBCH

8.4.1.4.1 Sequence generation

The reference-signal sequence rm for an S-SS/PSBCH block is defined by

rm=121-2cm+j121-2cm+1

where cn is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each S-SS/PSBCH block occasion with

cinit=NIDSL.

--------------------------------------------------------End of Draft TP for 38.211 ----------------------------------------

(FL: This TP with full information and cover page can found in subsection 2.6)

[100b-e-NR-5G_V2X_NRSL-SYNC-03]

Conclusion:

- The ambiguity of SSID(SSID=337) between P2 and P6 exists when the GNSS is the highest priority, and it can be resolved by proper configuration (e.g. only configure 2 synchronization resources).

To check 4/30 – update on 4/30, for issue#2, no consensus, not to be further discussed in Rel-16.

Email thread is closd.

[100b-e-NR-5G_V2X_NRSL-SYNC-04]

Agreements:

- The slot number within a frame used for NR sidelink communication is derived from the following formula:

SlotNumber= Floor ((Tcurrent –Tref–offsetDFN)*2μ) mod (10*2μ),

Where Tcurrent, Tref, offsetDFN are defined in 38.331, and μ=0/1/2/3 corresponding to the 15/30/60/120 kHz of SCS for SL, respectively.

- Send and LS to RAN2 regarding the above.

Conclusion:

- (for issue #7 in the summary) The previous agreements includes that the synchronization resource set is following LTE-V2X mechanism.

- (for issue #8 in the summary) There is no more clarification is needed, since the current S-SSB timing determination mechanism has no issue left.

LS in [100b-e-NR-5G_V2X_NRSL-SYNC-04]

The approved LS can be found in R1-2002990 which is uploaded to the inbox.

TP in [100b-e-NR-5G_V2X_NRSL-SYNC-02]

The endorsed TP can be found in next page with full information and cover page.

3GPP TSG-RAN WG1 #100bis-e	R1- 200XXXX

e-Meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | - | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to V2X |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specification of DM-RS signals for PSBCH, the scrambling sequence generator is imcompleted, and the initialization part should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For DM-RS signals in PSBCH, the scrambling sequence generator shall be initialized with cinit=NIDSL, which is agreed in meeting RAN1#100bis-e. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for V2X. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.4.1.4 Demodulation reference signals for PSBCH

8.4.1.4.1 Sequence generation

The reference-signal sequence rm for an S-SS/PSBCH block is defined by

rm=121-2cm+j121-2cm+1

where cn is given by clause 5.2. The scrambling sequence generator shall be initialized at the start of each S-SS/PSBCH block occasion with

cinit=NIDSL.