3GPP TSG RAN WG1 #100bis                                                                    R1-2002521

e-Meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | xxxx | rev | - | Current version: | 15.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on type-A and Type-B Carrier Switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In DCI Format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA, when the SRS request field is configured, one of the 1st or 2nd or 3rd set of CCs can be chosen according to the Table 7.3.1.1.2-24. However, if the SRS request is not present, it is not clear to which set of serving cells the TPC command should correspond. Furthermore, in RAN1#90bis meeting, we have following agreements for SRS carrier switching: Agreement: - Specify NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching design including - Periodic/aperiodic/semi-persistent SRS on a CC without PUCCH/PUSCH configured - TA (through PRACH) on TAG without PUSCH/PUCCH configured - Power control separated from that of PUSCH - Group common DCI for aperiodic SRS triggering and TPC - DL/UL interruptions and collision handling due to SRS switching In LTE the typeA and typeB mechanisms were used only when the number of carriers is more than 5 or less than 5 respectively. Thus, there was not case that both types would be configured simultaneously. In NR. a clarification is missing whether TypeA and TypeB can be configured simultaneously to the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Only one set of serving cells can be configured if the SRS request field is not present in DCI Format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA. Clarify that both type-A and type-B cannot be configured simultaneously to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be unclear to which set of serving cells the TPC commands should be applied when SRS request is absent. It will be unclear whether typeA and typeB can be configured to the UE simultaneously in different serving cells. If this is indeed possible, additional clarifications would be needed to ensure correct operation of such a feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: It is expected that a gNB would not configure more than one sets of serving cells to the UE if the SRS request is absent, or that it has implemented this feature with the SRS request always present. It is expected that a gNB already does not configure both typeA and typeB to a UE. Therefore, no interoperability issues are expected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24, otherwise, only one set of serving cells can be configured by higher layers.

-	TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeB for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block or more blocks is configured for the UE by higher layers where each block applies to an UL carrier, with the following fields defined for each block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Subclause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command –2 bits

A UE does not expect to be configured simultaneously with a higher layer parameter srs-TPC-PDCCH-Group = typeA and a higher layer parameter srs-TPC-PDCCH-Group = typeB in two different serving cells.

The number of information bits in format 2_3 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_3 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_3 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

< Unchanged parts are omitted >