3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002599

# E-Meeting, April 20 - April 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.211 | CR | xxxx | rev | - | Current version: | V14.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NRS presence in non-anchor carriers for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2020-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In Rel-14 NB-IoT, it was found in R1-1804898 that the frequency/time tracking loops will become unstable and the UE may need to spend more power to regain a sync state if the eNB only transmits NRS when there is a detected NPRACH. Thus the following conclusion was made in RAN1#92Bis. But the conclusion only applies for non in-band case in current spec. Conclusion: - RAN1 to clarify that the UE can assume that the eNB always transmits NRS in the Type-2 CSS configured by higher layers, as well as 10 NB-IoT DL subframes before and 4 NB-IoT DL subframes after each Type-2 CSS, regardless of whether the eNB detected an NPRACH transmission or not. - In Rel-14 NB-IoT, the following agreements were made about NRS presence on non-anchor carriers in RAN1#88. RAN1#88 agreements: - For RAR monitoring, the UE can assume that NRS are present in subframes {0,4,9} for in-band, {0,1,3,4,9} for standalone/guard-band, and valid DL subframes at least during the following: - 10 valid subframes before the start of each Type-2 CSS in the RAR window - 4 valid subframes after the end of each Type-2 CSS in the RAR window In current spec, NRSs are present in 4 valid subframes after the start of each Type-2 CSS in the RAR window for in-band case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Apply the RAN1#92Bis conclusion for NRS presence to in-band case. - Delete “at the start of” in the sentence “in 4 NB-IoT DL subframes after the start of each Type-2 CSS”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Without the first change, UEs in network with in-band operation mode will suffer unstable frequency/time tracking loops and more power consumption, when the UEs attempt to decode the NPDCCH with DCI scrambled by RA-RNTI in a non-anchor carrier. - Without the second change, the performance for UEs in network with in-band operation mode is degraded, when the UEs decode the NPDCCH with DCI scrambled by RA-RNTI in a non-anchor carrier. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

### 10.2.6	Narrowband reference signal (NRS)

#### < Unchanged parts are omitted >

On an NB-IoT carrier for which DL-CarrierConfigCommon-NB is present and inbandCarrierInfo is present:

-	When an NB-IoT UE is configured by higher layers to decode NPDCCH with CRC scrambled by the P-RNTI, the UE may assume NRSs are transmitted in the NPDCCH candidate where the UE finds a DCI with CRC scrambled by the P-RNTI. The UE may also assume NRSs are transmitted in10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after the NPDCCH candidate, where NB-IoT DL subframes without NRS are not counted. If the DCI with CRC scrambled by the P-RNTI schedules a NPDSCH, the UE may assume NRSs are transmitted in the NB-IoT DL subframes carrying the NPDSCH as well as 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted.

-	During the window controlled by higher layers where the UE shall attempt to decode the NPDCCH with DCI scrambled by RA-RNTI (see [8], subclause 5.1.4), the UE may assume NRSs are transmitted in the Type-2 CSS configured by higher layers, as well as in 10 NB-IoT DL subframes before and in 4 NB-IoT DL subframes after each Type-2 CSS, where NB-IoT DL subframes without NRS are not counted. If a DCI scrambled by the RA-RNTI is detected, the UE may assume NRSs are transmitted in the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, as well as in 4 NB-IoT DL subframes before and after the scheduled NPDSCH, where NB-IoT DL subframes without NRS are not counted. In addition, when the UE attempts to decode a DCI with CRC scrambled by the RA-RNTI as well as receiving the NPDSCH scheduled by the DCI scrambled by the RA-RNTI, the UE may assume NRSs are transmitted in subframes #0, #4 and #9.

#### < Unchanged parts are omitted >