3GPP TSG RAN WG1 #100b e-Meeting	R1- 2002687

April 20th – 30th, 2020

Agenda item:	7.2.11.1	3

Source: 	Qualcomm

Title: 	Discussion on UE features

Document for:	Discussion and Decision

# 1	Introduction

In this document, we propose new FGs for Rel. 15 NR. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for scheduling 3 TBs in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot. In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2.

# 2         Proposed Feature Groups

| 5-11c | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-12c | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 5-13d | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a or 5-5b | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
| 5-13g | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |

| 11-3a | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-5a or 5-5b | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13 | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13a | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13c | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3e | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13d | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |