3GPP TSG RAN WG1 #100bis  		R1-2002971

e-Meeting, April 20th – 30th, 2020

Agenda item: 6.2.1.2 Support for transmission in preconfigured UL resources

Source:  Moderator (Sierra Wireless)

Title:  Text Proposals for email [100b-e-LTE-eMTC5-PUR-02] on PUR Power Control

Document for: Discussion and decision

# Introduction

This document includes the endorsed text proposals from [1] R1-2002800  for email topic:

[100b-e-LTE-eMTC5-PUR-02] Power control correction – includes power control issues in section 4.2 of R1-2002740 by 4/24 and corresponding TP (if any) by 4/30 – Gus (Sierra Wireless)

This document also includes:  “reasons for change”, “summary of changes”, “Specs/Sections impacted”, and “consequences if not approved” for each text proposal.

# Missing  definition

Reasons for change:  The definition of PO_UE_PUSCH,c(3) is unclear for transmissions using pre-configured UL resources.

Summary of changes:  Added definition where PO_UE_PUSCH,c(3) = p0-UE-PUSCH in PUR-Config

Specs/Sections impacted: TS 36.213 section 5.1.1

Consequences if not approved:  Unclear definition of PO_UE_PUSCH,c(3) for transmissions using pre-configured UL resources.

Text proposal for TS 36.213:

---------------------------------------------- Text start -------------------------------------------------------

5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	is a parameter composed of the sum of a component  provided from higher layers for j=0, 1 and 3 and a component  provided by higher layers for j=0, 1 and 3 for serving cell . For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1, for PUSCH (re)transmissions corresponding to the random access response grant then j=2 and for BL/CE UE PUSCH (re)transmission using preconfigured uplink resource then j=3 and PO_UE_PUSCH,c3 = p0-UE-PUSCH in PUR-Config.  and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell .

-------------------------------------------------- Text end ----------------------------------------------------------

# Missing Zeroing of power control loop

Reasons for change:  The specification is unclear that the closed-loop component fc(i) is zero for transmissions using pre-configured UL resources.

Summary of changes:  Added text to indicate that closed-loop component fc(i) is zero for transmissions using pre-configured UL resources.

Specs/Sections impacted: TS 36.213 section 5.1.1

Consequences if not approved:  Unclear specification that the closed-loop component fc(i) is zero for transmissions using pre-configured UL resources.

Text proposal for TS 36.213:

---------------------------------------------- Text start -------------------------------------------------------

5.1.1.1	UE behaviour

<Unchanged parts are omitted>

-	For both types of  (accumulation or current absolute) the first value is set as follows:

-	If  value is changed by higher layers and serving cell  is the primary cell or, if  value is received by higher layers and serving cell  is a Secondary cell or, if a BL/CE UE transmits using preconfigured uplink resources,

-

Note: fci = 0 for i = 0,1,… up to the last subframe of the PUSCH transmission using preconfigured uplink resource

-	For serving cell , when the UE is configured with higher layer parameter shortTTI or when there is a change in configuration corresponding to the higher layer parameter shortTTI,  for the first following PUSCH transmission in a slot or subslot in a given subframe is set to the value of  associated with PUSCH of the previous uplink subframe.

-	Else

-	If the UE receives the random access response message for a serving cell

-------------------------------------------------- Text end ----------------------------------------------------------

# References

R1-2002800 Outcome summary of [100b-e-LTE-eMTC5-PUR-02] on PUR Power Control Moderator(Sierra Wireless)

R1-2002740 Preconfigured UL resources feature lead summary #2 RAN1 #100bis-e Moderator(Sierra Wireless)

R1-2001517 LS on open PUR issues for NB-IoT/eMTC RAN2, Ericsson

R1-2001849 Discussion on RAN2 LS on open PUR issues ZTE

R1-2002501 On the LS on open PUR issues for NB-IoT/eMTC Ericsson

R1-2002603 Draft reply LS on open PUR issues for NB-IoT/eMTC Huawei, HiSilicon

R1-2002356 LTE-M Pre-configured UL Resources Design Considerations Sierra Wireless, S.A.

R1-2002503 Corrections for Preconfigured UL resources for LTE-MTC Ericsson

R1-2001567 Corrections on transmission in preconfigured UL resources Huawei, HiSilicon

R1-2001850 Remaining issues for transmission in preconfigured UL resources for MTC ZTE

R1-2002173 Support for transmission in preconfigured UL resources Qualcomm Incorporated

R1-2002641 Remaining issues for preconfigured UL resources Nokia, Nokia Shanghai Bell

R1-1913673 “L1 Parameter list to RAN2” RAN1, Reno, USA, November 18th – 22nd, 2019

R2-1916424 “RAN2 agreements, RAN2, Reno, USA, November 18th – 22nd, 2019