3GPP TSG RAN WG1 Meeting #100bis                    	R1-2003030

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.3

Source:	Moderator (Huawei)

Title:	Draft TPs for NRU HARQ Issue A9

Document for:	Discussion and Decision

# Introduction

This document provides TP proposals on issue A9 based on proposals (pending approval) in [2].

[100b-e-NR-unlic-NRU-HARQ-02] Email discussion/approval on following issues related to Type-2 enhanced HARQ-ACK codebook by 4/24; if necessary, followed by endorsing the corresponding TPs by 4/30 – David (Huawei)

- A9: How to determine NFI, number of requested groups and PUCCH occasions i(g) and i((g+1) mod 2) when multiple DCIs provide these values

The agreements below were declared in the email discussion [3].

Agreement:

- The 1 MSB bit is the NFI for the scheduled PDSCH group, and the 1 LSB bit is the NFI for the non-scheduled PDSCH group.

Develop a TP by 4/29

Agreement:

- It is not expected to receive DCIs with q=0 pointing to the same PUCCH transmission occasion for different PDSCH groups

Develop a TP by 4/29

Conclusion:

It is not expected to receive a DCI with q=0 after receiving a DCI with q=1 pointing to the same PUCCH transmission occasion.

- No TP needed

Conclusion:

It is not expected to receive DCIs on different cells in the same monitoring occasion if the DCI formats indicate the same PDSCH group and different values of h(g). It is not expected to receive DCIs on different cells in the same monitoring occasion if the DCI formats indicate different values of q.

- No TP needed

Agreement:

If a first DCI format scheduling PDSCH reception and providing the first indication for a PUCCH transmission occasion in a slot does not include a New_Feedback indicator field, the value of h(g) for this PDSCH reception is set only if h(g) is provided by another DCI format providing a value of h(g), and the same value of g and a value of k indicating the same PUCCH transmission occasion in the slot. This first DCI determines m=0.

Develop a TP by 4/29

Agreement:

Clarify that g (scheduled group) and q  (number of requested groups) are obtained from the last non-fallback DCI format 1_1 providing these values for a PUCCH transmission occasion

Develop a TP by 4/29

# Text proposals

TP#1 for 38.212 v16.1.10

| Reason for change | The mapping of the 2 bits of the New feedback indicator DCI field to the PDSCH group g or (g+1)mod2 (TS38.213 clause 9.1.3.3) is undefined. |
| --- | --- |
| Summary of changes | Specify how the 2 bits of the New feedback indicator DCI field are mapped to PDSCH group g or (g+1)mod2 defined in TS38.213 clause 9.1.3.3. |
| Specs/Sections impacted | 38.212 v16.1.10 section 7.3.1.2.2 |
| Consequences if not approved | The UE cannot determine the NFI for PDSCH group g and (g+1)mod2 |

================== Beginning of text proposal 1 ===================

7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

*** Unchanged text omitted ***

-	New feedback indicator – 0, 1 or 2 bits.

-	1 bit if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 and the higher layer parameter NFI-TotalDAI-Included-r16 is not configured;

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 and the higher layer parameter NFI-TotalDAI-Included-r16 = enable; the MSB corresponds to the scheduled PDSCH group, and the LSB corresponds to the non-scheduled PDSCH group, as defined in [TS38.213] clause 9.1.3.3;

-	0 bit otherwise.

================== End of text proposal 1 ===================

TP#2 for TS 38.213 v16.1.10

| Reason for change | When the UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the following behaviors are not clearly specified: how to determine NFI for a DCI format 1_0 and how the first PDCCH monitoring occasion for type-2 HARQ-ACK codebook is determined when the first DCI providing a slot for a PUCCH transmission occasion is a DCI format 1_0? From which DCI the UE obtains g (scheduled group) and q (number of requested groups) for a PUCCH transmission occasion? What is the UE behavior if the UE receive DCIs with q=0 pointing to the same PUCCH transmission occasion for different PDSCH groups? |
| --- | --- |
| Summary of changes | When the UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16: correct how to determine NFI for a DCI format 1_0 and correct how the first PDCCH monitoring occasion for type-2 HARQ-ACK codebook is determined when the first DCI providing a slot for a PUCCH transmission occasion is a DCI format 1_0. Clarify that g (scheduled group) and q (number of requested groups) are obtained from the last non-fallback DCI format 1_1 providing these values for a PUCCH transmission occasion. Clarify that a UE is not expected to receive DCIs with q=0 pointing to the same PUCCH transmission occasion for different PDSCH groups. |
| Specs/Sections impacted | TS38.213 v16.1.10 section 9.1.3.3 |
| Consequences if not approved | UE may not report HARQ-ACK information correctly when it is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16 |

================== Beginning of text proposal 2 ===================

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

If a UE is provided pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16, the UE determines HARQ-ACK information for multiplexing in a PUCCH transmission occasion according to the following procedure.

Set g to the value of a PDSCH group index field in the last DCI format that provides a value of g and that indicates a PUCCH transmission occasion. g=0

Set i(g) to denote a PUCCH transmission occasion for multiplexing HARQ-ACK information

Set k to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g. If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK

Set h(g) to the value of a first New_Feedback indicator field in the last DCI format providing the value of g.

Set hg+1mod2(g) to a value of a second New_Feedback indicator field in the last DCI format providing the value of g.

Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2 in the last DCI format providing the value of g. If g=1 and a DCI format not including a PDSCH group index field is detected after the last DCI format providing the value of g and indicating the same slot for the PUCCH transmission occasion, set VDAIg+1mod2=∅.

Set q to the value of a number of requested PDSCH group(s) field, in the last DCI format providing the value of g.

A UE does not expect to receive DCI formats each indicating Number of requested PDSCH group(s) = 0 if the PDSCH-to-HARQ_feedback timing field values in those DCI formats indicate a same PUCCH transmission occasion and the DCI formats indicate different values for the PDSCH group index field.

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Clause 9.1.3.1, where

-	the first HARQ-ACK information corresponds only to detections of DCI formats each providing a same value of g, of h(g), and to detections of DCI formats not providing a value of g and h(g) but each associated with a same value of g, of h(g), and at least one of the DCI formats providing a value of k indicating the slot

-	at least one of the DCI formats provides a h(g) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g or that is associated with a value of g, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g)

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), and to detections of DCI formats not providing a value of g+1mod2, of h(g+1mod2),  but each associated with a same value of g+1mod2, of h(g+1mod2),

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2 or that is associated with a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2)

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Clause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

------------------ Unchanged part omitted ------------------------

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

If a first DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, this PDSCH reception is associated with PDSCH group 0 and the value of h(g) associated with the first DCI format is set only if h(g) is provided by another DCI format providing a value of h(g) for PDSCH group 0 and indicating the same slot for the PUCCH transmission occasion.

------------------ Unchanged part omitted ------------------------

================== End of text proposal 2 ===================

# Companies comments

| Company | Comments |
| --- | --- |
| QC | For TP#2: Comment 1: The last DCI should be defined, i.e., among which DCIs? We suggest the following (including some editorial suggestions): Set g to the value of a PDSCH group index field in a last DCI format that provides a value of g among the DCI formats that indicate a same slot for a PUCCH transmission occasion Comment 2: Once this last DCI format is defined, the other values should be based on that. Also, “if any” is not needed for the case of h(g) as that DCI format always indicates NFI for the group. Hence, we suggest: Set h(g) to the value of a first New_Feedback indicator field, in the last DCI format providing the value of g Set hg+1mod2(g) to a value of a second New_Feedback indicator field, if any, in the last DCI format providing the value of g Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2, if any, in the last DCI format providing the value of g Set q to the value of a number of requested PDSCH group(s) field, in the last DCI format providing the value of g . Comment 3: For VDAIg+1mod2, we should be careful when the last DCI format providing the value of g (as determined above) is not the absolute last DCI (which means the absolute last DCI is a fallback DCI). Then there are 2 cases: - Case 1: g=0. In this case, there is no issue - Case 2: g=1. In this case, VDAIg+1mod2 should be set to empty and should not be used (since DAI is updated in absolute last DCI that is a fallback DCI associated with group 0). If we do not do that, then codebook construction will be wrong for the second HARQ-Ack information, i.e., in the following part “if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.” Based on above, we suggest the following: Set VDAIg+1mod2 to the value of a total DAI field for group g+1mod2, if any, in the last DCI format providing the value of g. If g=1 and a DCI formats not including a PDSCH group index field is detected after the last DCI format providing the value of g and indicating the same slot for the PUCCH transmission occasion, set VDAIg+1mod2=∅. Comment 4: For proposal 6, we have the following editorial suggestions to simplify (also g, h(g) , etc. is for a DCI format in the rest of the section and not for a PDSCH repletion) If a first DCI format indicating a slot for a PUCCH transmission occasion does not include a New_Feedback indicator field, the value of h(g) for the first DCI format is set only if h(g) is provided by another DCI format providing a value of h(g) for g = 0 and indicating the same slot for the PUCCH transmission occasion. Also, perhaps this text above can be added to the second part of the TP where other assumptions about fallback DCI is explained. |
| MediaTek | - On TP for Proposal 2: We support the TP from FL On TP for Proposal 3: Agree with FL’s and Nokia’s comment that spec should avoid ambiguous signaling. Thus, we suggest the following TP for proposal 3: If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and if the DCI formats indicate different values of the PDSCH group index field, the UE does not expect that none of the DCI formats indicates Number of requested PDSCH group(s) = 1.. - On TP for Proposal 6: Do we preclude DCI 1_0 to indicate DLSPS release when eType2 codebook is configured? If not, we suggest the following changes: If a first DCI format providing the first indication for a PUCCH transmission occasion in a slot does not include a New_Feedback indicator field, the value of h(g) associating with the DCI format is set only if h(g) is provided by another DCI format providing a value of h(g) for g = 0 and a value of k indicating the same PUCCH transmission occasion in the slot.. However, if common understanding is DLSPS release is not included, I am fine with this TP. - On TP for Proposal 7: Prefer to more clearly specify how to determine the last DCI that provides a value of g or q for the PUCCH transmission occasion - The last DCI with a respective PDSCH-to-HARQ_feedback timing field value indicating the PUCCH transmission occasion - The last DCI provides a value of g or q in a corresponding field of the DCI Thus, we suggest the following TP for proposal 7: Set g to the value of a PDSCH group index field in the last DCI format that provides a value of g and a value of PDSCH-to-HARQ_feedback timing field indicating the PUCCH transmission occasion. Set q to the value of a number of requested PDSCH group(s) field, in the last DCI format providing the value of q and a value of PDSCH-to-HARQ_feedback timing field indicating the PUCCH transmission occasion.. On the end of TP: I am a bit puzzled about the TP. According to the condition, if there is no DCI format is DCI format 1_0, then the UE assumes PDSCH group index 0 for a PDSCH reception scheduled by detections of DCI format 1_0. However, none of the detected DCIs is format 1_0. |
| vivo | - On TP for Proposal 2: We support the TP from FL. - On TP for Proposal 3: We share the same view as MediaTek. - On TP for Proposal 6: Agree with QC and MediaTek. DCI 1_0 indicating DL SPS release when eType2 codebook is configured should also be involved. - On TP for Proposal 7: Either QC's and MediaTek's update is OK. The set of DCI formats should be clarified. - On the end of TP: We have the same concern as MediaTek. Maybe the condition for 'if' should be changed to 'none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0'. |
| Intel | On following TP related to proposal 2, better to directly capture that UE doesn’t expect to receive such information in DCI formats, instead of not generating HARQ-ACK codebook, to better align with proposal 2. We suggest the following revision. A UE is not expected to receive a DCI format indicating Number of requested PDSCH group(s) = 0 if the received DCI format corresponds to a same PUCCH transmission occasion as another DCI format which indicates a different value for the PDSCH group index field. We support some comments in QC’s reply regarding clarification of ‘last DCI’ Other parts of the proposed TP by FL are fine to us. |
| Samsung | On the TP for proposal 2, we support the TP proposed by FL. On the TP for proposal 3, we agree with Nokia and Intel that it is better to specify UE does not expect to receive such information in DCI formats rather than not generate HARQ-ACK information. On the TP for proposal 6, agree with MTK and vivo that DL SPS release should also be included. On the TP for proposal 7, we support the TP proposed by FL. Regarding where to capture g=0 for DCI 1_0, we think it would be more clear to capture it before the description for first HARQ-ACK information generation rather than at the end of the section. Maybe it can be captured in the paragraph for proposal 6 as below, If a first DCI format scheduling PDSCH reception and providing the first indication for a PUCCH transmission occasion in a slot does not include a New_Feedback indicator field, the value of g for this PDSCH reception is set to g=0 and h(g) for this PDSCH reception is set, only if h(g) is provided by another DCI format providing a value of h(g) for g = 0 and a value of k indicating the same PUCCH transmission occasion in the slot. |

# References

R1-2002696 Feature lead summary#1 on NR-U HARQ, RAN1#100b-e

R1-2002923 Feature lead summary#1 on email discussion 100b-e-NR-unlic-NRU-HARQ-02

Chairman's Notes RAN1#100b-e 7.2.2 v009