3GPP TSG RAN WG1 #101-e			R1-2003261

e-Meeting, May 25th – June 5th, 2020

3GPP TSG-RAN WG2 Meeting #109bis-e 	R2-2004188

# e-Meeting, 20-30 April, 2020

Title:	LS to RAN1 on agreements related to 2-step RACH

Response to:

Release:	Rel-16

Work Item:	NR_2step_RACH-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name: 	Eswar Vutukuri

Tel. Number:	+44 7722129788

E-mail Address:	eswar.vutukuri@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

As part of the 2-step RACH work item, RAN2 has made the following agreements.

Agreements 1 Once the UE transmits PRACH preamble then it shall transmit the associated PUSCH regardless of the occurrence of measurement gap 2 No changes are made to the MAC spec for the issue related to the MSGA PUSCH resources overlapping with measurement gap 3 To handle the scenario that some of the parameters within the PUSCH configuration may include optional UE features; Nothing is specified (assumption being the network only uses the optional PUSCH configuration features on dedicated BWPs and only for UEs that support these features) – no change to specification needed. Inform RAN1 about this conclusion 4 If there is no valid PUSCH resource associated to the PRACH resource selected: o then MAC doesn’t determine the UL grant or the associated HARQ information o MAC will only indicate the preamble resource to L1 o The UE will still generate the MAC PDU based on the TB size 5 msgA-TransMax is configured for 2 step CFRA in rachConfigDedicated and that the UE is not allowed to switch to 4-step RACH if this is not configured in rachConfigDedicated 6 From RAN2 perspective, if the UE supports 2-step RA, it shall support 2-step CFRA for the SSB case (i.e. same as 4-step RACH) 7 FFS If RAN1 indicates support for CSI-RS for 2-step CFRA, there is no need for a separate capability bit for this in 38.306 from RAN2 perspective. 8 Change the LCID to one byte eLCID for the Absolute Timing Advance Command 9 Move the initialisation of RSRP_THRESHOLD_RA_TYPE_SELECTION after the BWP operation since the threshold can be different in different BWPs

2. Actions:

To RAN1

ACTION: 	RAN2 respectfully asks RAN1 to take the above into account in their specification work:

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #110-e	01-12 June 2020	emeeting

TSG-RAN2 Meeting #111	24-28 Aug 2020	Toulouse, FR